3GPP TSG-RAN WG4 Meeting #116bis		                         R4-2513094

Prague, Czech Republic, October 13th – 17th, 2025

Title: 	Discussion on RRM performance requirements for LP-WUSWUR

Source: 	OPPO

Agenda item:	6.16.3

Document for:	Approval

Introduction

Based on the timeline of LP-WUS/WUR work item and the agreed work plan [1], the performance requirements discussion needs to be continued in this meeting. In this contribution, we will provide our considerations on the open issues captured in [2].

Discussion

Based on the conclusions of core part discussion, for UE with LP-WUR, the following states has been defined and the corresponding evaluation requirements are also defined:

- Relaxed measurement for both serving cell and neighbor cell (case 3)

- Serving cell measurement offloading (case 1)

- LP-WUS monitoring

From testing perspective, we think for UE with LP-WUR, the tests need to be defined to verify the evaluation period requirements of the entry and exit conditions for each case. And the evaluation requirements of entry and exit condition can be tested in one test. To cover the different switching cases, the following scenarios can be considered:

- Scenario 1: UE enter Case 1 from legacy and exit to legacy

- Scenario 2: UE enter Case 3 from legacy and exit to legacy

- Scenario 3: UE enter Case 1 from Case 3

In addition, according to the CR split, the following measurement requirements are defined for UE with LP-WUR in both IDLE and INACTIVE state.

4.X.2.2 [LP-WUR serving cell measurement and evaluation requirements]

4.X.2.3 [MR serving cell measurement and evaluation requirements]

4.X.2.4 [Measurements of intra-frequency NR cells for UE with LP-WUR]

4.X.2.5 [Measurements of inter-frequency NR cells for UE with LP-WUR]

4.X.2.6 [Measurements of inter-RAT E-UTRAN cells for UE with LP-WUR]

When UE enters case 3, the relaxed serving cell and neighbor cell measurement requirements are defined, and during case 1, UE is allowed to perform relaxed measurement for high priority layers. For these two cases, the corresponding cell reselection test case are needed and the legacy cell reselection tests can be used as baseline except that the test requirements need to be updated based on the relaxed measurement.

In summary, the following test cases in proposal 1 can be considered for UE with LP-WUR and only IDLE mode tests are defined which is same as legacy.

Proposal 1: Define the following test cases for UE with LP-WUR in IDLE mode:

- Test case for the evaluation requirements of entry/exit conditions for:

  - Scenario 1: UE enter Case 1 from legacy and exit to legacy

  - Scenario 2: UE enter Case 3 from legacy and exit to legacy

  - Scenario 3: UE enter Case 1 from Case 3

- Test case for cell reselection when UE in case 3 for:

  - Intra-frequency NR cells

  - Inter-frequency NR cells

  - Inter-RAT E-UTRAN cells

- Test case for cell reselection when UE in case 1 and high priority layers are configured

- Test case for OOK based LP-WUR serving cell measurement based on LP-SS

- Test case for OFDM based LP-WUR serving cell measurement based on PSS/SSS

Proposal 2: For cell reselection related test cases, the legacy test configurations can be used as baseline.

For LP-WUR serving cell measurement, UE is not required to report the measurement results in real deployment. But to verify the requirements, in the test case for LP-WUR serving cell measurement for PSS/SSS and LP-SS, test mode can be used to let UE report the measurement results.

Proposal 3: Define test mode for LP-WUR serving cell measurement tests.

Conclusions

In this paper, we provided our views on the performance requirements for LP-WUSWUR. And the following proposals are provided.

Proposal 1: Define the following test cases for UE with LP-WUR in IDLE mode:

- Test case for the evaluation requirements of entry/exit conditions for:

  - Scenario 1: UE enter Case 1 from legacy and exit to legacy

  - Scenario 2: UE enter Case 3 from legacy and exit to legacy

  - Scenario 3: UE enter Case 1 from Case 3

- Test case for cell reselection when UE in case 3 for:

  - Intra-frequency NR cells

  - Inter-frequency NR cells

  - Inter-RAT E-UTRAN cells

- Test case for cell reselection when UE in case 1 and high priority layers are configured

- Test case for OOK based LP-WUR serving cell measurement based on LP-SS

- Test case for OFDM based LP-WUR serving cell measurement based on PSS/SSS

Proposal 2: For cell reselection related test cases, the legacy test configurations can be used as baseline.

Proposal 3: Define test mode for LP-WUR serving cell measurement tests.

Reference

- R4-2505669 Work plan for performance part for LP-WUSWUR, vivo, RAN4#115

- R4-2509065 Topic summary for [116][222] NR_LPWUS, vivo, RAN4#116

- R4-2512139 WF on RRM requirements for NR_LPWUS, vivo, RAN4#116