3GPP TSG-RAN WG4 Meeting #116bis		 						    R4-2513104

Prague, Czech Republic, October 13th – 17th, 2025

Title: 	On accuracy of CSI-RS based L1 measurement for SBFD operation

Source: 	OPPO

Agenda item:	6.14.3.2

Document for:	Discussion

Introduction

In last meeting, a WF on RRM requirements for NR_duplex_evo was approved [1].

In this contribution, we will provide our views on RRM performance impact for SBFD operation.

Discussion

#### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements (Monday online session) - For UE supporting simultaneous DL reception in 2 DL subbands, - For measurement period: - For UE indicating CSI processing time scaled by 2, - Add a lower bound of 8ms on top of existing measurement period, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2. - For UE not indicating CSI processing time scaled by 2, - Lower bound is not needed and existing measurement period applies - Further discuss the measurement accuracy requirements in Perf part of the WI - For UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB for Case 3 apply. - Option 2: accuracy requirements based on 24 RB for Case 3 apply. - For UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB for Case 3 apply. - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

Background: o Different cases for DUD - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). Agreements (RAN4#115) o For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs • In the test case, not model the interfering UE.

For CSI-RS L1 measurement with puncturing in frequency domain, we prefer RAN4 to define relaxed accuracy requirements based on 24 RBs for case 3, regardless of UE indicating CSI processing time scaled by 2 or not.

Based on the agreed simulation assumption, the L1 measurements for both 24RBs and 48 RBs are simulated and the accuracy results are summarized in Table 1 below. About 0.6~0.8 dB performance loss is observed for 24 RBs compared with 48 RBs.

Table 1. Simulation results for CSI-RS L1-RSRP accuracy in Case 3

|  | SCS | CSI-RS (RB) | 5% | 95% | max (5%, 95%) |
| --- | --- | --- | --- | --- | --- |
| FR1 | 15kHz | 24 | -1.73 | 1.30 | 1.73 |
|  |  | 48 | -1.15 | 0.91 | 1.15 |
|  | 30kHz | 24 | -1.77 | 1.32 | 1.77 |
|  |  | 48 | -1.07 | 0.91 | 1.07 |
| FR2 | 120kHz | 24 | -1.81 | 1.32 | 1.81 |
|  |  | 48 | -1.09 | 0.98 | 1.09 |

Observation 1: For CSI-RS L1 measurement, 0.6~0.8 dB performance loss is observed for 24 RBs compared to 48 RBs.

To be conservative, we propose to introduce 1dB margin for the relaxed accuracy requirement, based on legacy CSI-RS based L1-RSRP accuracy requirements defined in clause 10.1.19.2 in TS38.133.

Proposal 1: For accuracy requirement for CSI-RS L1 measurement with 24 RBs in case 3, consider 1dB relaxation compared to the legacy accuracy with 48 RBs.

Conclusions

In this paper we provided our views on RRM impact for SBFD operation.

Observation 1: For CSI-RS L1 measurement, 0.6~0.8 dB performance loss is observed for 24 RBs compared to 48 RBs.

Proposal 1: For accuracy requirement for CSI-RS L1 measurement with 24 RBs in case 3, consider 1dB relaxation compared to the legacy accuracy with 48 RBs.

Reference

- R4-2512133, WF on RRM requirements for NR_duplex_evo