3GPP TSG-RAN WG4 Meeting #116-bis               	                R4-2513118

Prague, CZ, 13th - 17th Oct 2025

Title:	RRM Perf requirements on event triggered L1 measurement reporting

Source:	China Telecom

Agenda item:			6.17.2.1

Document for:	Discussion

- Introduction

In the RAN4 #116 meeting, the measurements related enhancements were discussed and WF [1] has been approved. In this paper, we will continue to discuss the RRM Perf requirements on event triggered L1 measurement reporting for LTM.

- Discussion

Issue 4-1-1: test case list for event-triggered L1 reporting

Recommended WF: - Further discuss based on the following directions - For SSB based L1 RSRP measurement reporting: - Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases. (E///, Apple) - Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. (MTK, HW) - For CSI-RS based L1 RSRP measurement reporting: - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting. - For UE supporting event triggered reporting, UE only needs to pass 2) - For UE not supporting event triggered reporting, UE only needs to pass 1)

In Previous meetings, we have discussed the core requirements on event triggered L1 measurement reporting for LTM. And the difference between event-triggered reporting and legacy periodic reporting is delay and evaluation conditions. The main measurement behaviour is same.

So, we think it is not necessary to duplicate all R18 test cases, maybe we could only add some necessary new test cases to verify event-triggered reporting behavior. Such as,

-	Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1.

-	Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2.

-	Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1.

Proposal1: keep R18 test cases, develop necessary new R19 test to verify reporting behavior.

-	Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1.

-	Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2.

-	Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1.

Issue 4-1-2: test configuration for event-triggered L1 reporting

Agreement: - On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met.

In previous RAN2 meetings, they have agreed to define the following L1 events for LTM reporting:

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

We think it is not necessary to test all events. And we should choose an event involved serving cell and candidate cell, in this case, Event LTM3 is best to verify event-triggered L1 reporting, For Event LTM5, we think we could discuss if it is necessary.

Proposal2: Event LTM3 is best to verify event-triggered L1 reporting, For Event LTM5, discuss that if it is necessary.

- Summary

In this paper, we provide our views on measurements related enhancements for LTM. From this discussion we have derived the following proposals:

Proposal 1: keep R18 test cases, develop necessary new R19 test to verify reporting behavior.

-	Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1.

-	Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2.

-	Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1.

Proposal 2: Event LTM3 is best to verify event-triggered L1 reporting, For Event LTM5, discuss that if it is necessary.

- References

[1] R4-2512141, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple Inc.