3GPP TSG-RAN WG4 Meeting #116bis									R4-2513132

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	8.3

Source:	CMCC

Title:	Discussion on 6GR system parameters

Document for:	Discussion

Introduction

In RAN#108, the technology study item for 6G radio (6GR) has been approved [1]. The anticipated services of 6GR range from immersive communication and massive communication to integrated sensing and communication. Furthermore, 6G services can be provided by both terrestrial network (TN) and non-terrestrial network (NTN).

- In this contribution, we show our initial analysis on waveform, modulation, CBW, FFT, numberology, sync raster, spectrum utilization, irregular channel bandwidth, PA modelling.

Discussion

  - Waveform

Agreement (first agreement for 6G!!) CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

RAN1 has approved to use NR waveform design as the baseline for the 6GR study. Besides, the PAPR reduction design is proposed in RAN1 with benefit of RF front-end implantation of both BS and UE in many services. Better PAPR performance will contribute to better ACLR, leading to less MPR and better UL coverage performance. On the other side, the PAPR reduction design will have potential impact on signal quality (e.g. degraded EVM performance) and implementation complexity of both transmitter and receiver. RAN4 is suggested to be involved for above analysis.

- RAN4 is suggested to be involved for the justification of PAPR reduction design study with following aspects:

- PAPR reduction gain (e.g., ACLR, MPR).

- Impact on signal quality (e.g., degradation of EVM).

- Impacts on implementation (e.g., RF implementation complexity for Tx and Rx unit).

Modulation

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

Higher-order modulation is essential to meet the 6G peak data rate targets, e.g., 50Gbps outlined in the ITU recommendation document [2]. However, a performance gap of approximately 1.53dB persists between current high-order QAM modulation and the theoretical bound. Constellation shaping is a promising technique to bridge this gap, primarily through two approaches: geometric shaping and probabilistic shaping.

For the new constellation shaping, existing EVM metric is not applicable and an EVM-like metric is indispensable to validate the feasibility and performance of constellation shaping, which involves RAN4 workloads.

- RAN4 is suggested to be involved early to study the EVM like metric for feasibility validation of new constellation shaping modulation schemes.

CBW

- Last meeting RAN1 agreement is listed as below

Agreement For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth. - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different.

- Different from RAN1, following aspects will impact the minimum CBW definition in RAN4.

- Operators’ spectrum allocation

- Sync raster design

- SSB minimum bandwidth

- Device type

- In 5G, the minimum CBW is 5MHz for below 3GHz and 10MHz for other FR1 and 50MHz for FR2. For the legacy bands, current minimum CBW lead to much dense sync raster with longer search time. Furthermore, for certain NR key bands e.g. n41, n77, n78, n79, operator’s spectrum allocation is much larger than current minimum CBW definition. It is reasonable to enlarge minimum CBW definition for most legacy bands. For the existing bands that operators’ spectrum is fragment with smaller bandwidth, existing minimum CBW could be reused. For the new bands, larger minimum CBW is suggested since operators spectrum is not fragment in theory.

- RAN4 further discuss the possibility of band basis minimum CBW rather than frequency range basis. The basic design principle is to define larger minimum CBW for most legacy operation bands and only consider smaller minimum CBW for the bands that operators spectrum is fragment with smaller spectrum bandwidth.

- In addition to, for the same bands, the minimum CBW could be different for different devices considering different device energy consumption, RF front end and use case.

- RAN4 to further discuss the possibility of different minimum CBW for different device on the same operating bands.

FFT

The FFT is based on max CBW. For the around 7GHz new bands, 400MHz CBW is suggested considering available spectrum with 30kHz SCS

If 30KHz SCS is used, a 400MHz CBW can be achieved by following approaches:

- BS requirement: Support with a 16K FFT (FFT size = 16384).

- UE requirement: the following options can be considered for further studies

  - Option 1 (1x16K_FFTx1CC): UE operates one 16K FFT (FFT size = 16384) in baseband to support 400MHz in one carrier

  - Option 2 (2x8K_FFTx1CC): UE operates two 8K FFT (FFT size = 8192) in baseband to support 400MHz in one carrier

  - Option 3 (1x8K_FFTx2CC): UE operates 8K FFT (FFT size = 8192) in baseband for each carrier to support 400MHz in two carriers, i.e., 2CC carrier aggregation.

- If 30KHz SCS is used, a 400MHz CBW can be achieved for around 7GHz by following approaches:

  - -BS requirement: Support with a 16K FFT (FFT size = 16384).

  - -UE requirement: the following options can be considered for further studies

    - Option 1 (1x16K_FFTx1CC): UE operates one 16K FFT (FFT size = 16384) in baseband to support 400MHz in one carrier

    - Option 2 (2x8K_FFTx1CC): UE operates two 8K FFT (FFT size = 8192) in baseband to support 400MHz in one carrier

    - Option 3 (1x8K_FFTx2CC): UE operates 8K FFT (FFT size = 8192) in baseband for each carrier to support 400MHz in two carriers, i.e., 2CC carrier aggregation.

Numerology

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion. Conclusion Numerologies for sensing is up to sensing agenda discussion. Agreement 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR. - FFS potential need for other CP.

In order to facilitate MRSS between 5G NR and 6GR within legacy bands, 6GR should support the SCS configurations currently predominant in 5G NR deployments for those bands. For new spectrum around 7GHz in 6GR, 30 kHz SCS utilizing the existing NCP configuration from 5G-NR can be considered as the baseline to ensure coverage robustness, particularly for wide-area deployments where resilience against multi-path delay spread is critical. Meanwhile, other SCS options are not precluded for discussion.

- For new spectrum around 7GHz, consider 30kHz SCS utilizing the existing NCP configuration from 5G-NR as baseline and study the support of 60 kHz SCS.

The SCS of SSB will impact sync raster design. 5G commercial network have approved the robust of the same SCS between SSB and data/signal channel. RAN4 can use the assumption of unified SCS for all channel/signals for sync raster discussion and further update if RAN1 support additional SCS for SSB.

- RAN4 can use the assumption of unified SCS for all channel/signals for sync raster discussion and further update if RAN1 support additional SCS for SSB.

Spectrum utilization, guard band, channel raster, sync raster requirements all will be based on the numerology conclusion. RAN4 can discuss such requirements based on current RAN1 numerology conclusion and further update if additional numerology is proposed. Furthermore, the additional SCS can be introduced as band basis and from operators request.

RAN4 can wait for the conclusion of RAN1 on the numerology discussion before discuss spectrum utilization, guard band, sync raster, etc.

- RAN4 can discuss spectrum utilization, guard band, sync raster requirements based on current RAN1 numerology discussion and further update as band basis and from operators request if additional numerology is proposed.

Synchronization raster

In 5G NR, the sync raster is defined based on minimum transmission bandwidth configuration, SSB bandwidth and channel raster. The minimum CBW for <3GHz, >3GHz FR1 and FR2 frequency range is 5MHz, 10MHz and 50MHz respectively. Such minimum CBW based sync raster will introduce much denser raster and enlarge search time with higher power consumption. Usually operators have wider spectrum than above minimum channel bandwidth. For example, CMCC’s spectrum for n41 and n79 is 160MHz and 100MHz respectively. Following show the candidate sync raster numbers for different CBW for <3GHz. UE has to search more sync raster for larger CBW based on current spec, which is not necessary for actual deployment.

RAN4 is suggested to study the possibility of defining sparser sync raster based on larger CBW which align with operators wider spectrum profile with the benefits of shorter search time and lower power consumption.

- RAN4 further study the possibility of defining sparser sync raster based on larger CBW which align with operators wider spectrum profile with the benefits of shorter search time and lower power consumption.

Spectrum utilization

Agreement Study and identify the lessons learned from NR spectrum utilization and aggregation framework. - DC is subject to RANP decision in June 2026. - Note: MRSS aspects are separate discussion.

The minimum guardband for each 5G NR UE channel bandwidth and SCS is referred as below for information. In 5G, maximum transmission bandwidth is specified one by one rather than based on unified spectrum efficiency. Specially, the minimum guard band for less CBW is even larger than that of larger CBW, e.g. the 30MHz and 35MHz minimum guard band as below is even larger than that of 40MHz. Then access failure occurs for such CBW with larger minimum guard-band when BS CBW is larger than UE CBW, e.g. for 700MHz as proposed in 5G. some resolutions are proposed by shifting the transmission bandwidth by 1PRB.

Table 5.3.3-1: Minimum guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 142.5 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1065 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

- In 6G, maximum transmission bandwidth configuration should be designed to avoid above issues and minimum guard band is defined with the logic that less CBW, less guard band.

- In 6G, maximum transmission bandwidth configuration should be specified with following principle that less CBW, less minimum guard band.

Irregular channel bandwidth

In NR R15, only the CBW multiples of 5MHz have been introduced as mandatory. In the following WI, operators proposed CBW that does not conform to existing NR channel bandwidths, commonly referred to as "irregular channel bandwidth. In NR Release 18, a Study Item (SI) titled "FS_NR_eff_BW_util" was established to facilitate more efficient utilization of such CBW with the general conclusions (TR 38.844 ) that two promising methods was proposed, 1)overlapping channels from network perspective and 2) the next larger channel. Both methods have pros and cons. Finally, RAN4 defines native irregular channel BWs, e.g. 7MHz into spec.

For 6GR, it’s better to consider such irregular channel bandwidth at first version for forward compatible. Following lists candidate 3 suggestions for further discussion.

- Option 1: specify the minimum RF requirements for min and max CBW as baseline and add scaling factor on top of the baseline for other CBWs.

  - exception is allowed for certain RF requirements when it’s hard to simplify RF requirements by scaling factor for different CBW.

- Option 2: consider the possibility of defining RF requirements based on actual configured/activated bandwidth i.e. BWP-like basis rather than CBW basis

- Option 3: same method as NR, in the first version define RF requirements for all CBW that has defined for NR or proposed by operators and consider other CBW in future release

- RAN4 consider irregular CBW in the first version with following options:

  - Option 1: in the first version define RF requirements for all CBW that has defined for NR or proposed by operators and consider other CBW in future release

  - Option 2: based on the two promising method as concluded from NR study phase

  - Option 3: specify the minimum RF requirements for min and max CBW as baseline and add scaling factor on top of the baseline for other CBWs.

    - exception is allowed for certain RF requirements when it’s hard to simplify requirements by scaling factor for different CBW.

  - Option 4: consider the possibility of defining all/part of RF requirements based on actual configured/activated bandwidth i.e. BWP-like basis rather than CBW basis.

    - One example, RAN4 define RF requirements per RB basis rather than CBW basis. There is translation from baseline RB configuration to other configurations.

PA modeling

PA modeling will be used for RF front end non-linearity simulation, e.g. MPR requirement analysis and as the baseline for RF AI analysis. In 5G phase, polynomial PA modeling is widely used among different work items but lack of detailed analysis of the difference from commercial PA device. Some companies also use actual testing data for MPR requirements analysis but without unified PA modeling. In 6GR, PA modeling study and assumption is indispensable at study phase which is better to be based on commercial testing data. Following PA models are used in some RF simulation software, e.g. AM-PM modeling which can be further discussed in RAN4.

- RAN4 is suggested to start the PA modelings analysis in study phase based on actual commercial testing data, following models can be further optimized based on commercial testing data

  - AM-PM

  - Polynomial

  - Others are not precluded

Device types

According to the RAN #109 device type discussion, we have following services

Proposal 1: 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT Proposal 2: For 6G Massive Communication (IoT), the minimum number of UE receive/transmit antenna is 1 for lowest-tier device - Reflect this agreement in the device type section of the TR Moderator proposal 1: It is agreed to capture following into TR 38.914 section 5.4: - Mobile broadband - Immersive Communication - Massive Communication (IoT) - Sensing - AI - Voice - Regulatory services Moderator proposal 2: It is agreed that ‘Observability’ will be discussed in architecture requirements. Moderator proposal 3: Whether to capture following into TR 38.914 section 5.4 will be further discussed: - [Non-Terrestrial Network] - [Aerial] - [FWA] - [Vehicles] - [Mission Critical Communication] - [Positioning and/or Navigation] - [TSN] - [HRLLC] Moderator proposal 4: - 6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. 6GR should study cooperative sensing/multi-static sensing mechanism involving multiple UEs and TRPs

For NR FR2, the UE type is coupled with power class and RAN4 define RF requirements based on the category of UE power class. For NR FR1, RF spec use suffix to differentiate different RF requirements for different types, especially some special UE type use specific operating band that is only applicable for such UE types, e.g. NTN service and vehicle UE types. In 6G, RAN4 should further discuss the methodology to different UE types capability, following list some options for further discussion.

- Option 1: Different UE types is coupled with different UE capability sets

  - Option 1a: Define minimum capability sets. Different UE types is coupled with different UE capability sets on top of the minimum capability sets

- Option 3: LTE category like UE types, different types are classified based on different throughput

- Option 4: combine LTE category like UE types and FR2 power class related method. I.e. different UE types are classified based on the combination of throughput and RF front end categories.

- Proposal 13: RAN4 is suggested to be evolved for UE types discussion based on following options.

- Option 1: Different UE types is coupled with different UE capability sets

  - Option 1a: Define minimum capability sets. Different UE types is coupled with different UE capability sets on top of the minimum capability sets

- Option 3: LTE category like UE types, different types are classified based on different throughput

- Option 4: combine LTE category like UE types and FR2 power class related method. I.e. different UE types are classified based on the combination of throughput and RF front end categories.

Conclusion

This contribution makes the following observations and proposals.

- RAN4 is suggested to be involved for the justification of PAPR reduction design study with following aspects:

- PAPR reduction gain (e.g., ACLR, MPR).

- Impact on signal quality (e.g., degradation of EVM).

- Impacts on implementation (e.g., RF implementation complexity for Tx and Rx unit).

- RAN4 is suggested to be involved early to study the EVM like metric for feasibility validation of new constellation shaping modulation schemes.

- RAN4 further discuss the possibility of band basis minimum CBW rather than frequency range basis. The basic design principle is to define larger minimum CBW for most legacy operation bands and only consider smaller minimum CBW for the bands that operators spectrum is fragment with smaller spectrum bandwidth.

- RAN4 to further discuss the possibility of different minimum CBW for different device on the same operating bands.

- If 30KHz SCS is used, a 400MHz CBW can be achieved for around 7GHz by following approaches:

  - -BS requirement: Support with a 16K FFT (FFT size = 16384).

  - -UE requirement: the following options can be considered for further studies

    - Option 1 (1x16K_FFTx1CC): UE operates one 16K FFT (FFT size = 16384) in baseband to support 400MHz in one carrier

    - Option 2 (2x8K_FFTx1CC): UE operates two 8K FFT (FFT size = 8192) in baseband to support 400MHz in one carrier

    - Option 3 (1x8K_FFTx2CC): UE operates 8K FFT (FFT size = 8192) in baseband for each carrier to support 400MHz in two carriers, i.e., 2CC carrier aggregation.

- For new spectrum around 7GHz, consider 30kHz SCS utilizing the existing NCP configuration from 5G-NR as baseline and study the support of 60 kHz SCS.

- RAN4 can use the assumption of unified SCS for all channel/signals for sync raster discussion and further update if RAN1 support additional SCS for SSB.

- RAN4 can discuss spectrum utilization, guard band, sync raster requirements based on current RAN1 numerology discussion and further update as band basis and from operators request if additional numerology is proposed.

- RAN4 further study the possibility of defining sparser sync raster based on larger CBW which align with operators wider spectrum profile with the benefits of shorter search time and lower power consumption.

- In 6G, maximum transmission bandwidth configuration should be specified with following principle that less CBW, less minimum guard band.

- RAN4 consider irregular CBW in the first version with following options:

  - Option 1: in the first version define RF requirements for all CBW that has defined for NR or proposed by operators and consider other CBW in future release

  - Option 2: based on the two promising method as concluded from NR study phase

  - Option 3: specify the minimum RF requirements for min and max CBW as baseline and add scaling factor on top of the baseline for other CBWs.

    - exception is allowed for certain RF requirements when it’s hard to simplify requirements by scaling factor for different CBW.

  - Option 4: consider the possibility of defining all/part of RF requirements based on actual configured/activated bandwidth i.e. BWP-like basis rather than CBW basis.

    - One example, RAN4 define RF requirements per RB basis rather than CBW basis. There is translation from baseline RB configuration to other configurations.

- RAN4 is suggested to start the PA modelings analysis in study phase based on actual commercial testing data, following models can be further optimized based on commercial testing data

  - AM-PM

  - Polynomial

  - Others are not precluded

- Proposal 13: RAN4 is suggested to be evolved for UE types discussion based on following options.

- Option 1: Different UE types is coupled with different UE capability sets

  - Option 1a: Define minimum capability sets. Different UE types is coupled with different UE capability sets on top of the minimum capability sets

- Option 3: LTE category like UE types, different types are classified based on different throughput

- Option 4: combine LTE category like UE types and FR2 power class related method. I.e. different UE types are classified based on the combination of throughput and RF front end categories.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO