3GPP TSG-RAN WG4 Meeting #116bis                        R4-2513139                                                                                                       Prague, CZ, 13th – 17th October 2025

Source:	China Telecom

Title:	Discussion on UE demodulation performance requirements for 6Rx

Agenda Item:	6.1.2

Document for:	Discussion

Introduction

The RAN4#114bis is the first meeting we discuss the UE demodulation performance requirements for 6Rx. According to the WID, RAN4 will define 6-layer performance requirements for FWA and 4-layer performance requirements for FWA and handheld UE devices. During RAN4#116, the Way Forward is approved in [1]

In this contribution, our views on the remaining open issues are given.

Discussion

Test applicability rule for 6Rx test cases

- Option 1: Introduce the test applicability rules by following core part agreement. - 6 MIMO layers performance requirements will be only applicable for FWA devices - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices. - Option 1a: Introduce UE declaration for the proper test applicability design if no explicit capability signalling finally confirmed by RAN2 - Option 2: Postpone the discussion until RAN2 concludes the UE capability signalling discussion

It is now decided in the September RAN meeting that RAN2 will introduce unique new maximum 6 layer capability for FWA UE [2]:

maxNumberMIMO-LayersPDSCH, maxNumberMIMO-LayersPDSCH-v1900 Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2 (except for FR2-NTN). If supportOf2RxXR is indicated, for single CC standalone NR, it is mandatory with capability signalling to support 2 MIMO layers in the bands specified in Table 7.3.2-2b in TS 38.101-1 [2]. If absent, the UE does not support MIMO on this carrier. The maxNumberMIMO-LayersPDSCH-v1900 is only included for FWA UE as defined in TS 38.101-1 [2]. The UE supporting maxNumberMIMO-LayersPDSCH-v1900 shall also indicate support of maxNumberMIMO-LayersPDSCH for the corresponding component carrier. When maxNumberMIMO-LayersPDSCH and maxNumberMIMO-LayersPDSCH-v1900 are reported together for a component carrier, the network which is able to decode maxNumberMIMO-LayersPDSCH-v1900 ignores maxNumberMIMO-LayersPDSCH. For the bands where pdsch-1024QAM-2MIMO-FR1-r17 is indicated, MIMO layers for 1024 QAM is the smaller value between 2 and maxNumberMIMO-LayersPDSCH.; FSPC; CY; N/A; N/A.

The above RAN2 capability makes it easier for RAN4 to define the applicability rule for 6Rx UEs:

- 6 MIMO layers performance requirements are applicable for all 6Rx UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6.

- 4 MIMO layers performance requirements will be applicable for all 6Rx UEs that supporting maxNumberMIMO-LayersPDSCH = 4 or maxNumberMIMO-LayersPDSCH-v1900 >= 4.

It is now unclear whether RAN4 will introduce one set or two sets of requirements for FWA and handheld UEs for rank 4 tests. If there will be 2 sets of requirements, RAN4 may still require introducing new UE declaration for different UE types to link the UE type with the test cases.

Proposal 1: RAN4 to define the applicability rule for 6Rx UEs requirements as below

- 6 MIMO layers performance requirements are applicable for all 6Rx UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6.

- 4 MIMO layers performance requirements will be applicable for all 6Rx UEs that supporting maxNumberMIMO-LayersPDSCH = 4 or maxNumberMIMO-LayersPDSCH-v1900 >= 4.

Proposal 2: If there will be 2 sets of requirements for FWA and handheld UE, RAN4 may still require introducing new UE declaration for different UE types to link the UE type with the test cases.

Specification structure

- Option 1: Add a new clause for 6Rx after the current 8Rx performance requirements - Section 5.2.5 6Rx requirement - Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification - Section 5.2.3 4Rx requirements - Section 5.2.4 6Rx requirements - Section 5.2.5 8Rx requirements

There is only one thing we would like to point it out is that the current requirements for 1/2/4/8Rx has been introduced in 38.101-4 without a Void section for 6Rx, which means 6Rx related sections may need to be specified under 8Rx.

We would like to check if it is possible to modify the section numbering and put 6Rx requirements between 4Rx and 8Rx in the Rel-19 specification.

Proposal 3: RAN4 to discuss whether it is possible to modify the section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification.

Test cases for 4 MIMO layers for FWA and Handheld UE

- Option 1: One single test case that is applicable for both FWA and Handheld UE - Option 1a: α = 0 and β = 0.07179 - Option 1b: α = 0 and β = 0 - Option 2: α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively

It can be optimal to define 2 sets of requirements for different UE types since the performance difference is larger than 1dB between the 2 correlations. However, this may bring new issue on how to differentiate the test cases and UE types.

So we are also ok to define requirements for single correlation, and option 1a is supported since it bring additional interference at the UE side and makes the test for difficult to pass.

Proposal 4: Ok to define requirements for single correlation, and option 1a is supported.

Test applicability rule for FWA

- Option 1: FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests - Option 2: Not introduce additional applicability rule to skip corresponding 4 layers tests if it passes the FWA 6 layers tests.

For 8Rx FWA, both rank 2, 4 and 8 PDSCH demodulation requirements are defined without additional applicability rule to skip some of the cases. Therefore, we propose to follow the same concept for 8Rx FWA and not to introduce additional applicability rule to skip corresponding 4 layers test if it passes the FWA 6 layers tests.

Proposal 5: Follow the same concept for 8Rx FWA and not to introduce additional applicability rule to skip corresponding 4 layers test if it passes the FWA 6 layers tests.

DMRS configuration for 6 MIMO layers case

- Option 1: Rel-15 Type 1 DMRS 2+2 - Option 2: Rel-18 eType1 DMRS 1+1

We propose not to combine 6 MIMO layer test cases with other new features defined in 5G-A.

Proposal 6: Rel-15 Type 1 DMRS 2+2.

CQI test for 6Rx

- Option 1: Not define CQI test - Option 2: Only define one CQI requirements with rank 4 under static channel

RI test

- Option 1: Not define RI test for 6 layers - Option 2: Define RI test for 6 layers

For CQI and RI testing for 6Rx, we support to follow the same test coverage for 2/4Rx, i.e., Periodic CQI reporting requirements under AWGN and RI test is defined.

Proposal 7: Define periodic CQI reporting requirements under AWGN, and RI test for 6Rx.

Conclusion

Proposal 1: RAN4 to define the applicability rule for 6Rx UEs requirements as below

- 6 MIMO layers performance requirements are applicable for all 6Rx UEs supporting maxNumberMIMO-LayersPDSCH-v1900 = 6.

- 4 MIMO layers performance requirements will be applicable for all 6Rx UEs that supporting maxNumberMIMO-LayersPDSCH = 4 or maxNumberMIMO-LayersPDSCH-v1900 >= 4.

Proposal 2: If there will be 2 sets of requirements for FWA and handheld UE, RAN4 may still require introducing new UE declaration for different UE types to link the UE type with the test cases.

Proposal 3: RAN4 to discuss whether it is possible to modify the section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification.

Proposal 4: Ok to define requirements for single correlation, and option 1a is supported.

Proposal 5: Follow the same concept for 8Rx FWA and not to introduce additional applicability rule to skip corresponding 4 layers test if it passes the FWA 6 layers tests.

Proposal 6: Rel-15 Type 1 DMRS 2+2.

Proposal 7: Define periodic CQI reporting requirements under AWGN, and RI test for 6Rx.

Reference

R4-2512617, Way Forward for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx, 3GPP TSG RAN4 Meeting #116, India, Aug, 2025

RP-252808, RAN2 endorsed CRs for Introduction of UE capability on 6 DL MIMO	RAN2 layers, 3GPP TSG RAN Meeting #109, Beijing, Sep, 2025