3GPP TSG-RAN WG4 Meeting#116bis		R4-2513149

Prague, Czech, Oct 13th -17th, 2025

Agenda item:	8.6

Source: 	Spreadtrum, UNISOC

Title: 	Views on 6G spectrum

Document for:	Discussion

Introduction

In RAN4#116 meeting, a Plan [1] about RAN4 6G guidance for information was provided by RAN4 chair. In this contribution, we want to provide some initial views on 6G spectrum.

6G topics; Scope; Scope in Q4’2025 and Q1’2026. System parameters; waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types; waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types. General RF and UE RF; non-spectrum UE RF, MIMO operation, duplexing, spectrum aggregation, initial access, multi-TRP, UE RF related coverage and energy efficiency and other joint UE-BS RF issues; non-spectrum and non-AI UE RF, spectrum aggregation，UE RF related coverage and energy efficiency. BS RF; BS RF, coexistence study, BS RF energy efficiency, duplexing, MSR; non-spectrum and non-AI BS RF, MSR, coexistence study. Spectrum; band/band combination definition and simplification, defintion of frequency ranges, spectrum related regulatory survey; band/band combination definition and simplification, defintion of frequency ranges, spectrum related regulatory survey. RRM; RAN4 driven non-AI RRM topics, MIMO operation, mobility, initial access, spectrum aggregation, multi-TRP, RRM related coverage and energy efficiency, other PHY signal/channel/procedure related RRM; RAN4 driven non-AI RRM topics,. Demod; RAN4 driven non-AI demod topics, MIMO operation, channel coding, multi TRP, demod related coverage and energy efficiency, other PHY signal/ channel/procedure related demod; RAN4 driven non-AI demod topics. AI; RAN4 driven use cases, RAN4 impacts of other WG driven use cases, RAN for AI; RAN4 driven use cases. 5G and 6G spectrum sharing; multi-RAT spectrum sharing for migration, mobility between 5G NR and 6G; Topics, if identified, which are not RAN1/2 dependent. Sensing; PHY functions and procedure, RF, coexistence and testability; Topics, if identified, which are not RAN1/2 dependent. OTA and testability; Testability methodology framework and key assumptions, including both AI and non-AI features; Testability methodology framework and key assumptions, including both AI and non-AI features. RAN4 operation efficiency; CR, spec improvement and modernization in general, new tools for band/ band combinations, other aspects; CR, spec improvement and modernization in general, new tools for band/ band combinations, other aspects.

Discussion

Frequency ranges

In 5G, spectrum was divided into FR1 and FR2 for TN: FR1 (from 410MHz to 7125MHz) and FR2-1(from 24.25GHz to 52.6GHz). For NTN, spectrum was divided into FR1-NTN and FR2-NTN: FR1-NTN (from 410MHz to 14.5GHz) and FR2-NTN (from 10.7GHz to 30GHz).

According to the RAN1 and RAN plenary discussion, there are two controversial issues for TN:

- Issue 1: How to categorize the frequency range? Should the spectrum resource from 7.125GHz to 8.4GHz be categorized into the lower group or the higher group? - Option 1: extend FR1 to 8.4 GHz and define a separate mid-high band (8.4–24.25 GHz) with its own single numerology. - Option 2: extend FR1 to 8.4 GHz and merge 8.4–24.25 GHz into FR2, creating a unified FR2 family. - Option 3: define separate FR for 7.125 – 24.25 GHz. - Issue 2: whether we need to highlight the specific 15GHz in the study or simply use a frequency range, e.g., Sub-24.25 GHz to cover 15GHz (Lower bound is dependent on the detailed definition of around 7)? - Potential way forward: for frequency range 24.25 – 52.6 GHz, we directly use frequency range rather a specific frequency point in the agreement, perhaps we can go with same logic, simply use frequency range, e.g., below 24.25 GHz rather than 15GHz. As to which frequency in this range is to be used for evaluation, it can be up to the output of RAN requirements (if confirmed in this meeting)

The most important principle of frequency range is numerology, now the numerology is under discussion in RAN1 and RAN plenary. In addition, a single SCS for sub frequency range should be follow in 6GR.

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

From RAN4 perspective, we can wait for the numerology discussion for around 7GHz and around 15GHz. Harmonized design for TN and NTN in 6GR is expected, and we need to avoid the situation that the existing two SCSs among the same frequency range like 5G NTN.

Proposal 1: Study how to define the frequency ranges of FR3 (e.g., based on the numerology), and keep a single SCS in sub-frequency ranges/band in 6GR.

Band/band combination definition and simplification

In 5G NR, for FR1 range, there are a total of 51 bands, for FR2, there are 7 bands. 5G NR FR1 and FR2-1 bands will be re-farming and reused in 6G, and FR3 will be introduced in 6GR. With regard to operators’ requests, there are so many bands and band combinations defined. However, most of the band combinations are not commercial. In order to support these band combinations, the complexity of UE implementation will obviously be increased. Thus, it is necessary to simplify the band combinations.

How to simplify the band combinations, a concept of band group is popular in 6GR. Based on the concept, the number of band combinations will obviously decrease. The concept of band group is that bands with different frequency bands are combined, switched within groups and aggregated between groups. Such as sub-1GHz can be group1, n1, n3, n39, etc. can be group2, n7, n41, etc. can be group3. Within one band group, we can choose one band by switching. In this one band group, requirements for these bands can be same. Inter band groups can be band combinations. In our understanding, the concept of band group can be studied in 6G.

Proposal 2: To simplify band combinations definition, band group can be studied in 6G.

Spectrum related regulatory survey

From the spectrum perspective, FR3 (7.125-15.35GHz) is identified as 6G new band. Especially the frequency range from 7.125GHz to 8.4GHz is the focus. It has the advantages of coverage and throughput. Regarding the frequency range, we have done relevant regulatory survey, and the results are as follows.

The regulatory about the frequency range (7.125GHz to 8.4GHz) in US [2].

This band is just above the unlicensed 6 GHz band (5.925 – 7.125 GHz) which is currently used primarily for extensive fixed services but was recently designated for unlicensed use in the U.S. on a non-interference basis to existing incumbents. Figure 1 shows the current allocations in the 7.125 - 8.5 GHz band, with the following principal characteristics:

FS, FSS and MSS are the largest current allocations and will be the biggest challenge for sharing with terrestrial mobile systems.

● approximately 20% of FS use is by the Department of Defense (DoD), and the satellite allocations also include DoD operations.

● other uses may not be ubiquitous and hence perhaps more amenable to sharing.

Figure 1: 7.125-8.5GHz spectrum allocations

In addition to federal allocations for fixed and satellite services, the 8400 – 8500 MHz band is allocated for SRS (DL).

The federal agencies use of this band is mostly for fixed point-to-point microwave communication systems. This includes the Federal Aviation Administration’s (FAA) use of this band for fixed point-to-point microwave communications networks to connect remote long-range aeronautical radio-navigation radars to air traffic control centers. However, the use of the band for fixed assignments in the 7.125 – 8.5 GHz has been declining.

This band is also used for FSS and MSS. Federal agencies operate the Defense Satellite Communications Systems (DSCS) series of geostationary satellites (DL) in this frequency band. Federal agencies also operate the Wideband Gap filler Satellite (WGS) in this band. FSS uses 7.25 – 7.75 GHz as downlink and 7.9 – 8.4 GHz as uplink; this includes support for both DSCS and WGS.

Observation1: Based on the regulatory of FCC in US, there is only 7.375-7.45GHz frequency range used for Mobile service.

The regulatory about the frequency range (7.125GHz to 8.4GHz) in China [3].

Table1: Radio frequency allocation (MHz) in China (7.125GHz -8.4GHz)

| China |  |  | ITU region3 frequency allocation |
| --- | --- | --- | --- |
| The Chinese mainland | Hong kong | Macao |  |
| 7075—7145 Fixed Mobile CHN45 5.458 CHN23 | 7075—7250 Fixed Mobile | 7075-7145 Fixed Mobile | 7075—7145 Fixed Mobile 5.458 5.459 |
| 7145—7190 Fixed Mobile SPACE RESEARCH (Earth-to-space) 5.460 5.458 CHN23 |  | 7145—7190 Fixed Mobile SPACE RESEARCH (deep space) (Earth-to-space) | 7145—7190 Fixed Mobile SPACE RESEARCH (deep space) (Earth-to-space) 5.458 5.459 |
| 7190—7235 Earth exploration-satellite (Earth-to-space) 5.460A5.460B Fixed Mobile SPACE RESEARCH (Earth-to-space) 5.460 5.458 CHN23 |  | 7190—7235 Earth exploration-satellite (Earth-to-space) Fixed Mobile SPACE RESEARCH (Earth-to-space) | 7190—7235 Earth exploration-satellite (Earth-to-space) 5.460A5.460B Fixed Mobile SPACE RESEARCH (Earth-to-space) 5.460 5.458 5.459 |
| 7235—7250 Earth exploration-satellite(Earth-to-space) 5.460A Fixed Mobile 5.458 CHN23 |  | 7235—7250 Earth exploration-satellite (Earth-to-space) Fixed Mobile | 7235—7250 Earth exploration-satellite(Earth-to-space) 5.460A Fixed Mobile 5.458 |
| 7250—7300 Fixed FIXED-SATELLITE (Space-to-earth) Mobile 5.461 | 7250—7375 Fixed Mobile | 7250—7300 Fixed FIXED-SATELLITE (Space-to-earth) Mobile | 7250—7300 Fixed FIXED-SATELLITE (Space-to-earth) Mobile 5.461 |
| 7300—7375 Fixed FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile 5.461 |  | 7300—7375 Fixed FIXED-SATELLITE (Space-to-earth) MOBILE except aeronautical mobile | 7300—7375 Fixed FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile 5.461 |
| 7375—7450 Fixed FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile Satellite water movement(Space-to-earth) 5.461AA 5.461AB | 7375—7750 Fixed Mobile Satellite water movement(Space-to-earth) | 7375—7450 Fixed FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile Satellite water movement(Space-to-earth) | 7375—7450 Fixed FIXED-SATELLITE (Earth-to-space) MOBILE except aeronautical mobile Satellite water movement(Space-to-earth) 5.461AA 5.461AB |
| 7450—7550 Fixed FIXED-SATELLITE (Space-to-earth) Satellite meteorology(Space-to-earth) MOBILE except aeronautical mobile Satellite water movement(Space-to-earth) 5.461AA 5.461AB 5.461A CHN18 |  | 7450—7550 Fixed FIXED-SATELLITE (Space-to-earth) Satellite meteorology(Space-to-earth) MOBILE except aeronautical mobile Satellite water movement(Space-to-earth) | 7450—7550 Fixed FIXED-SATELLITE (Space-to-earth) Satellite meteorology(Space-to-earth) MOBILE except aeronautical mobile Satellite water movement(Space-to-earth) 5.461AA 5.461AB 5.461A |
| 7550—7750 Fixed FIXED-SATELLITE (Space-to-earth) MOBILE except aeronautical mobile Satellite water movement(Space-to-earth) 5.461AA 5.461AB CHN18 |  | 7550—7750 Fixed FIXED-SATELLITE (Space-to-earth) MOBILE except aeronautical mobile Satellite water movement(Space-to-earth) | 7550—7750 Fixed FIXED-SATELLITE (Space-to-earth) MOBILE except aeronautical mobile Satellite water movement(Space-to-earth) 5.461AA 5.461AB |
| 7750—7 900 Fixed Satellite water movement(Space-to-earth) 5.461B MOBILE except aeronautical mobile CHN18 | 7750—7900 Fixed | 7750—7 900 Fixed Satellite water movement(Space-to-earth) 5.461B MOBILE except aeronautical mobile | 7750—7 900 Fixed Satellite water movement(Space-to-earth) 5.461B MOBILE except aeronautical mobile |
| 7900—8025 Fixed FIXED-SATELLITE (Earth-to-space) Mobile 5.461 CHN18 | 7900—8025 Fixed | 7900—8025 Fixed FIXED-SATELLITE (Earth-to-space) Mobile | 7900—8025 Fixed FIXED-SATELLITE (Earth-to-space) Mobile 5.461 |
| 8025—8175 Earth exploration-satellite (space -to- Earth) Fixed FIXED-SATELLITE (Earth-to-space) Mobile 5.463 5.462A CHN18 | 8025—8400 Earth exploration-satellite (space -to- Earth) Fixed | 8025—8175 Earth exploration-satellite (space -to- Earth) Fixed FIXED-SATELLITE (Earth-to-space) Mobile | 8025—8175 Earth exploration-satellite (space -to- Earth) Fixed FIXED-SATELLITE (Earth-to-space) Mobile 5.463 5.462A |
| 8175—8215 Earth exploration-satellite (space -to- Earth) Fixed FIXED-SATELLITE (Earth-to-space) Satellite meteorology(Earth-to-space) Mobile 5.463 5.462A |  | 8175—8215 Earth exploration-satellite (space -to- Earth) Fixed FIXED-SATELLITE (Earth-to-space) Satellite meteorology(Earth-to-space) Mobile | 8175—8215 Earth exploration-satellite (space -to- Earth) Fixed FIXED-SATELLITE (Earth-to-space) Satellite meteorology(Earth-to-space) Mobile 5.463 5.462A |
| 8215—8400 Earth exploration-satellite (space -to- Earth) Fixed FIXED-SATELLITE (Earth-to-space) Mobile 5.463 5.462A |  | 8215—8286 Earth exploration-satellite (space -to- Earth) Fixed FIXED-SATELLITE (Earth-to-space) Mobile | 8215—8400 Earth exploration-satellite (space -to- Earth) Fixed FIXED-SATELLITE (Earth-to-space) Mobile 5.463 5.462A |
|  |  | 8286—8363 Fixed |  |
|  |  | 8363—8400 Earth exploration-satellite (space -to- Earth) Fixed FIXED-SATELLITE (Earth-to-space) Mobile |  |

The regulatory about the frequency range (7.125GHz to 8.4GHz) in CEPT [4].

Table 2: Radio frequency allocation (MHz) in Europe (7.125GHz -8.4GHz)

| Frequency band | Europe |
| --- | --- |
| 7075 MHz - 7145 MHz | Allocations: FIXED, Earth Exploration-Satellite (passive), MOBILE Applications: Passive sensors (satellite), UWB applications, LPR, Fixed, Video PMSE |
| 7145 MHz - 7190 MHz | Allocations: Space Operation (Earth-to-space), FIXED, MOBILE, SPACE RESEARCH (DEEP SPACE) (EARTH-TO-SPACE) Applications: Fixed, Video PMSE, LPR, UWB applications |
| 7190 MHz - 7235 MHz | Allocations: EARTH EXPLORATION-SATELLITE (EARTH-TO-SPACE), FIXED, SPACE RESEARCH (EARTH-TO-SPACE), MOBILE Applications: UWB applications, Passive sensors (satellite), Fixed, LPR, Video PMSE |
| 7235 MHz - 7250 MHz | Allocations: FIXED, EARTH EXPLORATION-SATELLITE (EARTH-TO-SPACE), Space Research (Earth-to-space) Applications: Video PMSE, Fixed, LPR, UWB applications, Passive sensors (satellite) |
| 7250 MHz - 7300 MHz | Allocations: FIXED, FIXED-SATELLITE (SPACE-TO-EARTH), MOBILE Applications: MSS Earth stations, Satellite systems (military), Land military systems, UWB applications, Fixed, LPR, Video PMSE |
| 7300 MHz - 7375 MHz | Allocations: FIXED, FIXED-SATELLITE (SPACE-TO-EARTH), MOBILE EXCEPT AERONAUTICAL MOBILE Applications: Video PMSE, UWB applications, Fixed, LPR, Land military systems, Maritime military systems, Satellite systems (military), MSS Earth stations |
| 7375 MHz - 7450 MHz | Allocations: FIXED, FIXED-SATELLITE (SPACE-TO-EARTH), MOBILE EXCEPT AERONAUTICAL MOBILE, MARITIME MOBILE-SATELLITE (SPACE-TO-EARTH) Applications: MSS Earth stations, Satellite systems (military), Maritime military systems, Land military systems, Fixed, LPR, UWB applications, Video PMSE |
| 7450 MHz - 7550 MHz | Allocations: MARITIME MOBILE-SATELLITE (SPACE-TO-EARTH), FIXED, FIXED-SATELLITE (SPACE-TO-EARTH), METEOROLOGICAL-SATELLITE (SPACE-TO-EARTH), MOBILE EXCEPT AERONAUTICAL MOBILE Applications: Video PMSE, Fixed, UWB applications, LPR, Land military systems, Maritime military systems, Satellite systems (military), Weather satellites |
| 7550 MHz - 7750 MHz | Allocations: FIXED, FIXED-SATELLITE (SPACE-TO-EARTH), MOBILE EXCEPT AERONAUTICAL MOBILE, MARITIME MOBILE-SATELLITE (SPACE-TO-EARTH) Applications: Satellite systems (military), Maritime military systems, Land military systems, LPR, UWB applications, Fixed, Video PMSE |
| 7750 MHz - 7900 MHz | Allocations: FIXED, METEOROLOGICAL-SATELLITE (SPACE-TO-EARTH), MOBILE EXCEPT AERONAUTICAL MOBILE Applications: Video PMSE, Fixed, UWB applications, LPR, Weather satellites |
| 7900 MHz - 8025 MHz | Allocations: FIXED, MOBILE, FIXED-SATELLITE (EARTH-TO-SPACE) Applications: Satellite systems (military), Land military systems, LPR, UWB applications, MSS Earth stations, Fixed, Video PMSE |
| 8025 MHz - 8175 MHz | Allocations: FIXED-SATELLITE (EARTH-TO-SPACE), EARTH EXPLORATION-SATELLITE (SPACE-TO-EARTH), FIXED, MOBILE Applications: Video PMSE, Fixed, UWB applications, LPR, Land military systems, Satellite systems (military), Land mobile, Earth exploration-satellite |
| 8175 MHz - 8215 MHz | Allocations: EARTH EXPLORATION-SATELLITE (SPACE-TO-EARTH), FIXED, METEOROLOGICAL-SATELLITE (EARTH-TO-SPACE), MOBILE, FIXED-SATELLITE (EARTH-TO-SPACE) Applications: Land mobile, Satellite systems (military), Land military systems, LPR, UWB applications, Earth exploration-satellite, Fixed, Video PMSE |
| 8215 MHz - 8400 MHz | Allocations: FIXED-SATELLITE (EARTH-TO-SPACE), EARTH EXPLORATION-SATELLITE (SPACE-TO-EARTH), FIXED Applications: Video PMSE, Fixed, UWB applications, LPR, Land military systems, Satellite systems (military), Radio astronomy, Earth exploration-satellite |
| 8400 MHz - 8500 MHz | Allocations: FIXED, SPACE RESEARCH (SPACE-TO-EARTH), Radiolocation Applications: Space research, LPR, Fixed, UWB applications, Video PMSE |

Based on the CEPT regulatory, until now, even though there are no explicit applications for mobile service, but they have mobile applications.

Observation 2: Table 1/2 and figure 1 can be the information for the regulatory for the frequency range from 7.125GHz to 8.4GHz.

Conclusions

In this paper, we provide our initial views on 6G spectrum, we have made the following proposals and observations.

Proposal 1: Study how to define the frequency ranges of FR3 (e.g., based on the numerology), and keep a single SCS in sub-frequency ranges/band in 6GR.

Proposal 2: To simplify band combinations definition, band group can be studied in 6G.

Observation1: Based on the regulatory of FCC in US, there is only 7.375-7.45GHz frequency range used for Mobile service.

Observation 2: Table 1/2 and figure 1 can be the information for the regulatory for the frequency range from 7.125GHz to 8.4GHz.

- Reference

- R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 chair. TSG-RAN4#116 meeting

- A Preliminary View of Spectrum Bands in the 7.125 - 24 GHz Range; and a Summary of Spectrum Sharing Frameworks

- https://www.srrc.org.cn/law.aspx

- https://efis.cept.org/view/search-general.do