3GPP TSG-RAN WG4 Meeting #116bis                                                                                                    R4-2513163

Prague, CZ, 13rd –17th October, 2025

Source: 	CATT

Title: 	Further discussion on LP-WUS demodulation requirements

Agenda item:	6.16.4

Document for:	Approval

- Introduction

In RAN4#116 meeting, a WF for [116][330] NR_LPWUS_Demod was agreed[1], and the remaining issues on LP-WUS demodulation requirements in WF require further discussion.

In this contribution, we provide our views on LP-WUS demodulation requirements.

- Discussion

Whether to introduce FAR test

As per the WF [1], the remaining open issue concerning whether to introduce FAR test is shown as below:

Issue 2-1-2: Whether to introduce FAR test - Proposals - Option 1: Yes - Option 2: No

As WF, RAN4 agrees to introduce MDR test for LP-WUS, since the MDR test is dependent on FAR test, like PRACH FAR, It is necessary to introduce FAR test.

Proposal 1: Adopt Option 1 for whether to introduce FAR test as below:

  - Option 1: Yes.

Test metric for FAR test (if introduced)

As per the WF [1], the remaining open issue concerning test metric for FAR test is shown as below:

Issue 2-1-3: Test metric for FAR test (if introduced) - Proposals - Option 1: 1% FAR - Option 2: 0.1% FAR

For PRACH, its FAR was defined 0.1%, since it is for establishing Uplink Synchronization, so we think that the 0.1% FAR for PRACH is reasonable. Referring to PRACH, LP-WUS is mainly used for terminal devices to receive network signals in a low-power mode when they are not connected, thereby achieving energy saving and fast wake-up functions. LP WUS does not require smaller FAR, we think that 1% FAR is sufficient.

Proposal 2: Define Option 1 for test metric for FAR test as below:

  - Option 1: 1% FAR

Carrier frequency

As per the WF [1], the remaining open issue concerning carrier frequency is shown as below:

Issue 2-1-12: Carrier frequency - Proposals - Option 1: Use fc=900MHz for FR1 FDD and fc=2.6GHz for FR1 TDD as starting point - FFS for FR2 TDD

It is sufficient to use fc=900MHz for FR1 FDD and fc=2.6GHz for FR1 TDD for LP-WUS demodulation, for FR2-TDD, it is sufficient to use fc=30GHz for FR2 TDD for LP-WUS demodulation.

Proposal 3: Use fc=900MHz for FR1 FDD and fc=2.6GHz for FR1 TDD, and use fc=30GHz for FR2 TDD.

Antenna configuration

As per the WF [1], the remaining open issue concerning antenna configuration for FR2 is shown as below:

Issue 2-1-13: Antenna configuration - Agreement: - 1T1R for FR1 - FFS: 1T1R for FR2

We think that 1T1R is sufficient for FR1 and FR2 for LP-WUS demodulation. We propose,

Proposal 4: Adopt 1T1R for FR2 for antenna configuration.

Whether to define single requirement for Type 1 and Type 2 receivers

As per the WF [1], the remaining open issue concerning whether to define single requirement for Type 1 and Type 2 receivers is shown as below:

Issue 2-1-21: Whether to define single requirement for Type 1 and Type 2 receivers - Proposals - Option 1: RAN4 to study the performance for both receiver types (Type 1: OOK and Type 2: OFDM) and further discuss based on simulation results if one or two sets of requirements are to be defined. - Option 2: Do not define separate demodulation requirements for Type-1 and Type-2 LP-WUR capable UEs if those are not tied to Envelope vs Sequence Detection.

For Option 1, it is reasonable to further discuss based on simulation results if one or two sets of requirements are to be defined. Form our understanding, because one shot OFDM receiver and OOK receiver have similar performance, so it is sufficient to define a set of requirements for Type 1 and Type 2 receivers. So we think that Option2 is also reasonable.

Proposal 5: Adopt Option 2: Do not define separate demodulation requirements for Type-1 and Type-2 LP-WUR capable UEs if those are not tied to Envelope vs Sequence Detection.

Inclusion of requirements in the specification

As per the WF [1], the remaining open issue concerning inclusion of requirements in the specification is shown as below:

Issue 2-1-27: Inclusion of requirements in the specification - Proposals - Option 1: - RAN4 to add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR1. - RAN4 to add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR2.

Since LP-WUS demodulation requirement for FR1 is conducted requirement, and LP-WUS demodulation requirement for FR2 is radiated requirement. So, it is reasonable to add conducted requirement and radiated requirement in corresponding clauses.

Proposal 6: Adopt Option 1 for inclusion of requirements in the specification as below:

  - Option 1:

    - RAN4 to add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR1.

    - RAN4 to add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR2.

- Conclusion

This contribution provides analysis on LP-WUS demodulation requirements. The following proposal is concluded as follows:

Proposal 1: Adopt Option 1 for whether to introduce FAR test as below:

  - Option 1: Yes.

Proposal 2: Define Option 1 for test metric for FAR test as below:

  - Option 1: 1% FAR

Proposal 3: Use fc=900MHz for FR1 FDD and fc=2.6GHz for FR1 TDD, and use fc=30GHz for FR2 TDD.

Proposal 4: Adopt 1T1R for FR2 for antenna configuration.

Proposal 5: Adopt Option 2: Do not define separate demodulation requirements for Type-1 and Type-2 LP-WUR capable UEs if those are not tied to Envelope vs Sequence Detection.

Proposal 6: Adopt Option 1 for inclusion of requirements in the specification as below:

  - Option 1:

    - RAN4 to add a new subclause ‘5.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR1.

    - RAN4 to add a new subclause ‘7.7 Wake-up signal (WUS) demodulation requirements’ in TS 38.101-4 for FR2.

- References

- R4-2512526, WF for [116][330] NR_LPWUS_Demod, Moderator (Qualcomm), RAN4#116

- R4-2508626, WF for [115][332] NR_LPWUS_Demod, Moderator (Qualcomm), RAN4#115

- R4-2507936, WF for [115][136] NR_LPWUS_UERF, vivo, RAN4#115