3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513192

# Prague , CZ, 13th – 17th Oct, 2025

Title: 	Discussion on RRM performance requirements for on-demand SSB SCell operation

Source: 	CATT

Agenda item:	6.15.2.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, the core part of WI for Netw_Energy_NR_enh was completed on time [1]. In this contribution, we discuss and provide our initial views on the performance requirements for on-demand SSB SCell operation based on the agreed core requirements.

- Discussion

  - Scenario- General

For the test scenario, since case 1/case 2 and scenario #2/scenario #2A/scenario #3B has been introduced in core part and different requirements have been defined for these scenarios, so RAN4 need to define the test cases for both case 1/case 2 and scenario #2/scenario #2A/scenario #3B for OD-SSB SCell operation, and the test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell. Therefore, the following test cases can be considered.

•	In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

•	In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

Proposal 1: RAN4 to define the following test cases including deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

•	In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

•	In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

For case 2, there is AO-SSB and OD-SSB and RAN4 has defined the core requirements for Alt. Time C1 and Alt. Time C2, but RAN4 didn’t define the requirements when OD-SSB and AO-SSB are within different frequencies (Alt scenario 3). Therefore, we believe that RAN4 only need to define the test cases to verify the UE behaviour in Alt. Time C1 and Alt. Time C2.

Proposal 2: RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1 and Alt. Time C2.

  - Scenario – known/unknown Scell activation

For OD-SSB based SCell activation, since known condition has been defined and the core requirements include known and unknown SCell cases, so RAN4 to define the related test case for known SCell and unknown SCell.

Proposal 3: RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

  - Test case list

In core part, RAN4 defined the T2 and discussed UE’s behaviour for re-activating the OD-SSB transmission. However, we think that the test case can mainly focus on single OD-SSB transmission, and not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

Proposal 4: RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

Based on the above, and according to the core part requirements, we propose that at least the following test cases need to be defined:

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

- OD-SSB based SCell Activation in Case 1

- OD-SSB based SCell Activation in Case 2 Alt Time C-1

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2 Alt Time C-1

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement for deactivated SCell in Case 1

- Intra-frequency measurement for deactivated SCell in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

Proposal 5: RAN4 at least to define the following test cases for OD-SSB SCell operation:

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

- OD-SSB based SCell Activation in Case 1

- OD-SSB based SCell Activation in Case 2 Alt Time C-1

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2 Alt Time C-1

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement for deactivated SCell in Case 1

- Intra-frequency measurement for deactivated SCell in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

  - Test configuration

For the general principle of test configuration, the candidate proposals are copied as follows [1]:

RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed. - Both FR1 and FR2 - Test OD-SSB periodicities with 5ms, 20ms. 10ms - Only test non-DRX mode - The test without SSB time index detection - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting - SMTC and measCycleSCell as 40ms/160ms - Only NCD-SSB for OD-SSB test case.

In our view, all proposals for test configuration are acceptable for us, so we propose that:

Proposal 6: The following principles to design the OD-SSB test case can be agreed:

- Both FR1 and FR2

- Test OD-SSB periodicities with 5ms, 20ms. 10ms

- Only test non-DRX mode

- The test without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

- SMTC and measCycleSCell as 40ms/160ms

- Only NCD-SSB for OD-SSB test case.

- Conclusion

This document discussed the performance requirements for on-demand SSB SCell operation and presented following proposals:

Proposal 1: RAN4 to define the following test cases including deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

•	In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

•	In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

Proposal 2: RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1 and Alt. Time C2.

Proposal 3: RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

Proposal 4: RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

Proposal 5: RAN4 at least to define the following test cases for OD-SSB SCell operation:

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

- OD-SSB based SCell Activation in Case 1

- OD-SSB based SCell Activation in Case 2 Alt Time C-1

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2 Alt Time C-1

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement for deactivated SCell in Case 1

- Intra-frequency measurement for deactivated SCell in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

Proposal 6: The following principles to design the OD-SSB test case can be agreed:

- Both FR1 and FR2

- Test OD-SSB periodicities with 5ms, 20ms. 10ms

- Only test non-DRX mode

- The test without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

- SMTC and measCycleSCell as 40ms/160ms

- Only NCD-SSB for OD-SSB test case.

- Reference

[1] 	R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#116