3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513193

# Prague , CZ, 13th – 17th Oct, 2025

Title: 	Discussion on RRM performance requirements for adaptation of common signal channel transmission

Source: 	CATT

Agenda item:	6.15.2.2

Document for:	Discussion

- Introduction

In last RAN4 meeting, the core part of WI for Netw_Energy_NR_enh was completed on time [1]. In this contribution, we discuss and provide our initial views on the performance requirements for adaptation of common signal channel transmission.

- Discussion

  - Test scope

For the test case list, the agreement in the last meeting is copied as follows [1]:

Issue 3-3-1: Test case list Agreement RAN4 to at least define the following SSB adaptation test case. - L3 intra-frequency measurement RAN4 to discuss whether the following SSB adaptation test case list can be defined. - L1-RSRP - L1-SINR - CBD

RAN4 has agreed to at least define SSB adaptation test case for L3 intra-frequency measurement and RAN4 need to further discuss whether to define test cases for L1-RSRP, L1-SINR and CBD. In general, according to the core part requirements, we propose that L1 measurement test cases including the following also need to be defined for SSB adaptation:

- L1-RSRP measurement

- L1-SINR measurement

- CBD

Proposal 1: RAN4 to define L1 measurement test cases including the following also need to be defined for SSB adaptation:

- L1-RSRP measurement

- L1-SINR measurement

- CBD

  - General principle

For the general principles to design the SSB adaptation test cases, the way forward is copied as below [1]:

Issue 3-3-2: General principle RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed. - RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only. - RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9. - RAN4 to reuse legacy trigger events for measurement report after SSB adaptation. - Both FR1 and FR2. - RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement. - Only test non-DRX mode. - without SSB time index detection.

Regarding the above proposals, we agree with Proposals 4, 5, 6 and 7. Firstly, both FR1 and FR2 test cases should be covered. Furthermore, to save the testing effort, only to test non-DRX mode and without SSB time index detection is enough.

Proposal 2: Regarding general principles to design the SSB adaptation test cases, the following can be supported:

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- Without SSB time index detection.

- Conclusion

This document discussed performance requirements for adaptation of common signal channel transmission and presented following proposals:

Proposal 1: RAN4 to define L1 measurement test cases including the following also need to be defined for SSB adaptation:

- L1-RSRP measurement

- L1-SINR measurement

- CBD

Proposal 2: Regarding general principles to design the SSB adaptation test cases, the following can be supported:

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- Without SSB time index detection.

- Reference

[1] 	R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#116