3GPP TSG-RAN WG4 Meeting #116-bis	R4-2513201

# Prague , CZ, 13th – 17th Oct, 2025

Title: 	Discussion on RRM requirements for (e)RedCap UE enhancement

Source: 	CATT

Agenda item:	7.4.3.2

Document for:	Discussion

- Introduction

A new RAN4 leading work item for NR RRM Phase 6 was approved in RAN#109. At the first stage, (e)RedCap UE enhancement including relaxed RLM/BFD RRM requirements and support of less than 5 MHz channel bandwidth will be discussed. The objectives of core part for NR RRM Phase 6 are as following [1]:

- Specify the following requirements for (e)RedCap UEs [RAN4] - Relaxed RLM/BFD RRM requirements - Use R17 RLM/BFD relaxation for legacy UE as a baseline - Support of less than 5 MHz channel bandwidth (15 PRB transmission bandwidth configuration and 12 PRB SSB) - RF requirements, considering example bands: n100, n106, n26, n28 and n85 - RRM requirements including relaxed RLM/BFD RRM requirements - Note 1: Use R18 3 MHz requirements for non-RedCap UE as a baseline - Note 2: Consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD - For intra-band co-located contiguous CA, specify necessary RRM requirements to support data scheduling on SCell prior to UE transmitting valid CSI report (to be started in Q4 2026) [RAN4] - Note 3: No new PDSCH demodulation requirement shall be defined.

According to the schedule of this WI, this document will firstly discuss the general RRM issues about (e)RedCap UE enhancement and present our proposals.

- Discussion

Topic 1: Relaxed RLM/BFD RRM requirements

- Sub-topic 1-1: General principle

Power saving is important for (e)RedCap UEs due to their lower cost and complexity and smaller battery size in devices such as wearables. Therefore, the further enhancement on power saving for (e)RedCap UE is needed. Given that the existing Rel-17 RLM/BFD relaxation for power saving WI has been specified only for non-RedCap UEs in FR1 and FR2, it is natural to extend it to (e)RedCap UEs [1].

According to Rel-17 RLM/BFD relaxation, the following requirements were defined for non-RedCap UEs for power saving:

Table 1 RRM Requirements defined for RLM/BFD relaxation in Rel-17

| RLM | - Minimum requirement of SSB based radio link monitoring for UE fulfilling relaxed measurement criteria - Minimum requirement of CSI-RS based radio link monitoring for UE fulfilling relaxed measurement criteria |
| --- | --- |
| LR | - Minimum requirement of SSB based beam failure detection for UE fulfilling relaxed measurement criteria - Minimum requirement of CSI-RS based beam failure detection for UE fulfilling relaxed measurement criteria |

Therefore, minimum requirement of SSB/CSI-RS based RLM and BFD for RedCap UE fulfilling relaxed measurement criteria need to be defined for (e)RedCap UEs. Also, RLM/BFD relaxation requirements in R20 should consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD impact.

Proposal 1: RAN4 to define the following requirements for (e)RedCap UEs in R20:

- Minimum requirement of SSB based radio link monitoring for RedCap UE fulfilling relaxed measurement criteria

- Minimum requirement of CSI-RS based radio link monitoring for RedCap UE fulfilling relaxed measurement criteria

- Minimum requirement of SSB based beam failure detection for RedCap UE fulfilling relaxed measurement criteria

- Minimum requirement of CSI-RS based beam failure detection for RedCap UE fulfilling relaxed measurement criteria

Proposal 2: The existing RLM/BFD relaxation requirements for non-RedCap UE defined in Rel-17 can be used as a baseline for (e)RedCap UEs.

- Requirements should consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD impact.

- Sub-topic 1-2: Specific RRM requirements

In detail, at least for 2Rx (e)RedCap UEs, we believe that the existing RLM/BFD relaxation requirements for non-RedCap UE can be reused as a baseline. For 1Rx (e)RedCap UEs, RAN4 need to discuss the further relaxation on the top of relaxation due to reduction in the number of UE Rx branches for specific requirements.

Proposal 3: For 2Rx (e)RedCap UEs, the existing RLM/BFD relaxation requirements for non-RedCap UE can be reused as a baseline.

Proposal 4: For 1Rx (e)RedCap UEs, RAN4 need to discuss the further relaxation on the top of relaxation due to reduction in the number of UE Rx branches for specific requirements.

The specific RRM impact will be discussed case by case as following:

- Issue 1-2-1: RLM/BFD relaxation

In R17 requirements of SSB/CSI-RS based RLM for RedCap UE fulfilling relaxed measurement criteria, only the relaxations for TEvaluate_out_SSB/CSI-RS were considered for FR1 and FR2, and relaxation factor K1 and the relaxation factor for the lower bound K3 were introduced for non-RedCap UE. In R20, the relaxations for TEvaluate_out_SSB/CSI-RS also need to be considered for (e)RedCap UEs.

In R17 requirements of SSB/CSI-RS based BFD for RedCap UE fulfilling relaxed measurement criteria, only relaxations for TEvaluate_BFD_SSB/CSI-RS were considered for FR1 and FR2, and relaxation factor K1 and the relaxation factor for the lower bound K3 were introduced for non-RedCap UE. In R20, the relaxations for TEvaluate_BFD_SSB/CSI-RS also need to be considered for (e)RedCap UEs.

It is worth emphasizing that when discussing the specific further relaxations, the legacy relaxation due to reduction in the number of UE Rx branches need to be taken into consideration together to avoid relaxing too much.

Proposal 5: For RLM, RAN4 to consider the relaxations for TEvaluate_out_SSB/CSI-RS on the top of RedCap requirements.

Proposal 6: For BFD, RAN4 to consider the relaxations for TEvaluate_BFD_SSB/CSI-RS on the top of RedCap requirements.

Proposal 7: When discussing the specific further relaxations, the legacy relaxation due to reduction in the number of UE Rx branches need to be taken into consideration together to avoid relaxing too much.

Topic 2: RRM requirements for less than 5 MHz channel bandwidth

- Sub-topic 2-1: General principle

Considering that it is important to extend 3 MHz support from normal UE to (e)RedCap UEs to reduce complexity and cost and the demand of spectrum resource utilization from some operators, support of less than 5 MHz channel bandwidth is very significant, and 15 PRB transmission bandwidth configuration and 12 PRB SSB can be reused.

In general, from RRM requirements perspective, since less than 5MHz requirements for non-RedCap UE have been defined in Rel-18, so the existing requirements can be used as a baseline for (e)RedCap UEs. Also, less than 5MHz requirements in R20 should consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD impact.

Proposal 8: The existing less than 5MHz requirements for non-RedCap UE defined in Rel-18 can be used as a baseline for (e)RedCap UEs.

- 15 PRB transmission bandwidth configuration and 12 PRB SSB will be reused.

- Requirements should consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD impact.

Firstly, RAN4 need to identify what the impact of less than 5MHz on RRM requirements, the following requirements were defined for less than 5MHz for non-RedCap UE in Rel-18 [2]:

Table 2 RRM Requirements defined for less than 5MHz for non-RedCap UE in Rel-18

| RRC_CONNECTED state mobility | - Handover - NR Handover |
| --- | --- |
| Signalling characteristics | - Radio Link Monitoring - Link Recovery Procedures |
| Measurement Procedure | - NR intra-frequency measurements - NR inter-frequency measurements |

In Rel-20, in our view, it is naturally to define less than 5MHz requirements for (e)RedCap UEs based on the requirements for (e)RedCap UEs in Rel-17/Rel-18, so all the special features of (e)RedCap UEs are still retained, and then consider the impact on the forementioned  requirements of less than 5MHz defined for non-RedCap UE in Rel-18.

According to the above, RAN4 need to define less than 5MHz related requirements on the top of those RedCap requirements in Table 2. For the specification structure of less than 5MHz requirements for (e)RedCap UEs in TS 38.133[2],  it will be captured under clauses defined for (e)RedCap UEs.

In addition, less than 5MHz requirements for RedCap will include relaxed RLM/BFD RRM requirements according to WID, so RLM/BFD relaxation related RedCap requirements will also need to consider the impact of less than 5MHz.

Observation 1: It is naturally to define less than 5MHz requirements for (e)RedCap UEs based on the requirements for (e)RedCap UEs in Rel-17/Rel-18, and all the special features of (e)RedCap UEs are still retained.

Proposal 9: RAN4 to define the following requirements for less than 5MHz for (e)RedCap UEs in R20:

- Handover

- Radio Link Monitoring (include RedCap requirements for UE fulfilling relaxed measurement criteria)

- Link Recovery Procedures (include RedCap requirements for UE fulfilling relaxed measurement criteria)

- NR intra-frequency measurements

- NR inter-frequency measurements

Proposal 10: RAN4 to define less than 5MHz related requirements on the top of those RedCap requirements in Table 2, which will be captured under clauses defined for (e)RedCap UEs.

- Sub-topic 2-2: Specific RRM requirements

In detail, at least for 2Rx (e)RedCap UEs, we believe that the existing less than 5MHz requirements for non-RedCap UE can be reused as a baseline. For 1Rx (e)RedCap UEs, RAN4 need to discuss the further relaxation due to less than 5MHz on the top of relaxation due to reduction in the number of UE Rx branches for specific requirements.

Proposal 11: For 2Rx (e)RedCap UEs, the existing less than 5MHz requirements for non-RedCap UE can be reused as a baseline.

Proposal 12: For 1Rx (e)RedCap UEs, RAN4 need to discuss the further relaxation due to less than 5MHz on the top of relaxation due to reduction in the number of UE Rx branches for specific requirements.

The specific RRM impact will be discussed case by case as following:

- Issue 2-2-1: Handover

RAN4 only need to define the requirements for the scenario of NR FR1 - NR FR1 Handover for (e)RedCap UEs. For the interruption time, T∆ should consider the impact of less than 5MHz, and RAN4 need to consider the relaxation for T∆ for both known and unknown target cells operating with 12 PRB SSB BW on the top of RedCap requirements.

Proposal 13: For FR1 - NR FR1 Handover, RAN4 to consider the relaxation for T∆ in interruption time for both known and unknown target cells operating with 12 PRB SSB BW on the top of RedCap requirements.

- Issue 2-2-2: Signalling characteristics

Regarding the requirements for SSB based radio link monitoring, RAN4 only need to define PDCCH transmission parameters for out-of-sync/ in-sync evaluation for a RedCap UE operating on a cell with less than 5MHz BW.

Furthermore, since there was no impact for minimum requirement due to less than 5MHz, so the requirements for SSB based radio link monitoring for RedCap UE fulfilling relaxed measurement criteria can be directly reused for (e)RedCap UEs with less than 5MHz.

Proposal 14: RAN4 to define PDCCH transmission parameters for out-of-sync/ in-sync evaluation for a RedCap UE operating on a cell with less than 5MHz BW.

Proposal 15: The requirements for SSB based radio link monitoring for RedCap UE fulfilling relaxed measurement criteria can be directly reused for (e)RedCap UEs with less than 5MHz.

Regarding the requirements for SSB based beam failure detection, RAN4 also need to define PDCCH transmission parameters for beam failure instance for a RedCap UE operating on a cell with less than 5MHz BW.

Furthermore, since there was no impact for minimum requirement due to less than 5MHz, so the requirements for minimum requirement of SSB based beam failure detection for RedCap UE fulfilling relaxed measurement criteria can be directly reused for (e)RedCap UEs with less than 5MHz.

Proposal 16: RAN4 to define PDCCH transmission parameters for beam failure instance for a RedCap UE operating on a cell with less than 5MHz BW.

Proposal 17: The requirements for SSB based beam failure detection for RedCap UE fulfilling relaxed measurement criteria can be directly reused for (e)RedCap UEs with less than 5MHz.

- Issue 2-2-3: Measurement Procedure

For NR intra/inter-frequency measurements without and with measurement gaps, the time period for time index detection (Frequency range FR1) for a target cell with 12 PRB SSB need to be defined for (e)RedCap UEs, and RAN4 need to consider the relaxation for TSSB_time_index_intra/TSSB_time_index_inter for target cells operating with 12 PRB SSB BW on the top of RedCap requirements.

Proposal 18: The time period for time index detection (Frequency range FR1) for a target cell with 12 PRB SSB need to be defined for (e)RedCap UEs including the following, and RAN4 to consider the relaxation for TSSB_time_index_intra/TSSB_time_index_inter for target cells operating with 12 PRB SSB BW on the top of RedCap requirements:

- NR intra-frequency measurements without measurement gaps for RedCap.

- NR intra-frequency measurements with measurement gaps for RedCap.

- NR inter-frequency measurements with measurement gaps for RedCap.

- NR inter-frequency measurements without measurement gaps for RedCap.

- Conclusion

This document discussed preliminary RRM requirements for (e)RedCap UE enhancement RRM requirements and presented following observation and proposals:

Proposal 1: RAN4 to define the following requirements for (e)RedCap UEs in R20:

- Minimum requirement of SSB based radio link monitoring for RedCap UE fulfilling relaxed measurement criteria

- Minimum requirement of CSI-RS based radio link monitoring for RedCap UE fulfilling relaxed measurement criteria

- Minimum requirement of SSB based beam failure detection for RedCap UE fulfilling relaxed measurement criteria

- Minimum requirement of CSI-RS based beam failure detection for RedCap UE fulfilling relaxed measurement criteria

Proposal 2: The existing RLM/BFD relaxation requirements for non-RedCap UE defined in Rel-17 can be used as a baseline for (e)RedCap UEs.

- Requirements should consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD impact.

Proposal 3: For 2Rx (e)RedCap UEs, the existing RLM/BFD relaxation requirements for non-RedCap UE can be reused as a baseline.

Proposal 4: For 1Rx (e)RedCap UEs, RAN4 need to discuss the further relaxation on the top of relaxation due to reduction in the number of UE Rx branches for specific requirements.

Proposal 5: For RLM, RAN4 to consider the relaxations for TEvaluate_out_SSB/CSI-RS on the top of RedCap requirements.

Proposal 6: For BFD, RAN4 to consider the relaxations for TEvaluate_BFD_SSB/CSI-RS on the top of RedCap requirements.

Proposal 7: When discussing the specific further relaxations, the legacy relaxation due to reduction in the number of UE Rx branches need to be taken into consideration together to avoid relaxing too much.

Proposal 8: The existing less than 5MHz requirements for non-RedCap UE defined in Rel-18 can be used as a baseline for (e)RedCap UEs.

- 15 PRB transmission bandwidth configuration and 12 PRB SSB will be reused.

- Requirements should consider both 1Rx and 2Rx (e)RedCap UEs and HD-FDD impact.

Observation 1: It is naturally to define less than 5MHz requirements for (e)RedCap UEs based on the requirements for (e)RedCap UEs in Rel-17/Rel-18, and all the special features of (e)RedCap UEs are still retained.

Proposal 9: RAN4 to define the following requirements for less than 5MHz for (e)RedCap UEs in R20:

- Handover

- Radio Link Monitoring (include RedCap requirements for UE fulfilling relaxed measurement criteria)

- Link Recovery Procedures (include RedCap requirements for UE fulfilling relaxed measurement criteria)

- NR intra-frequency measurements

- NR inter-frequency measurements

Proposal 10: RAN4 to define less than 5MHz related requirements on the top of those RedCap requirements in Table 2, which will be captured under clauses defined for (e)RedCap UEs.

Proposal 11: For 2Rx (e)RedCap UEs, the existing less than 5MHz requirements for non-RedCap UE can be reused as a baseline.

Proposal 12: For 1Rx (e)RedCap UEs, RAN4 need to discuss the further relaxation due to less than 5MHz on the top of relaxation due to reduction in the number of UE Rx branches for specific requirements.

Proposal 13: For FR1 - NR FR1 Handover, RAN4 to consider the relaxation for T∆ in interruption time for both known and unknown target cells operating with 12 PRB SSB BW on the top of RedCap requirements.

Proposal 14: RAN4 to define PDCCH transmission parameters for out-of-sync/ in-sync evaluation for a RedCap UE operating on a cell with less than 5MHz BW.

Proposal 15: The requirements for SSB based radio link monitoring for RedCap UE fulfilling relaxed measurement criteria can be directly reused for (e)RedCap UEs with less than 5MHz.

Proposal 16: RAN4 to define PDCCH transmission parameters for beam failure instance for a RedCap UE operating on a cell with less than 5MHz BW.

Proposal 17: The requirements for SSB based beam failure detection for RedCap UE fulfilling relaxed measurement criteria can be directly reused for (e)RedCap UEs with less than 5MHz.

Proposal 18: The time period for time index detection (Frequency range FR1) for a target cell with 12 PRB SSB need to be defined for (e)RedCap UEs including the following, and RAN4 to consider the relaxation for TSSB_time_index_intra/TSSB_time_index_inter for target cells operating with 12 PRB SSB BW on the top of RedCap requirements:

- NR intra-frequency measurements without measurement gaps for RedCap.

- NR intra-frequency measurements with measurement gaps for RedCap.

- NR inter-frequency measurements with measurement gaps for RedCap.

- NR inter-frequency measurements without measurement gaps for RedCap.

- Reference

[1]	  RP-252955, WID: NR Radio Resource Management (RRM) Phase 6, Apple

[2]      3GPP TS 38.133, NR Requirements for support of radio resource management