3GPP TSG-RAN WG4 Meeting # 116bis                                                        R4-2513207

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Discussion on SBFD BS to SBFD BS adjacent channel coexistence

Source: 	CATT

Agenda item:	7.2.4

Document for:	Discussion

- Background

According to WID[1], further study is required on the adjacent-channel coexistence scenario between SBFD BS and SBFD BS, and the requirements of BS Tx and Rx RF in this scenario also need to be considered. This contribution provides our analysis of such a coexistence scenario.

- Spectrum configuration and requirements analysis

In WID[1], the analysis content required for the adjacent channel coexistence scenario between SBFD BS and SBFD BS is shown below.

SBFD BS to SBFD BS adjacent channel co-existence

- SBFD BS to SBFD BS adjacent channel coexistence

  - Focus on FR1 WA.

  - Consider both scenarios including SBFD capable BS being co-located and non-collocated.

  - Focus on bands: n104, n79, n41.

  - Specify necessary (and if feasible) BS TX and RX RF requirement for SBFD-capable BS for the scenarios of SBFD BS to SBFD BS adjacent channel coexistence. The BS RF requirements include

    - TX unwanted emission

    - RX in-band blocking

  - Note 1: whether to comply with requirements is declared by BS.

  - Note 2: The existing TDD requirements apply for non-SBFD BSs

  - Spectrum configuration

(a)

(b)

Figure 1 Interference rejection diagram

According to [2] of RAN4#113, for two SBFD-capable BSs with adjacent operating bands, both BSs configured as {DU}, and two DL sub-bands are adjacent is the best configuration. In this spectrum configuration, the frequency offset between aggressor channel to victim channel is max, and this configuration can optimize the rejection at victim BS UL sub-band by RF BPF used after PA, as shown in figure 1 (a). Additionally, when operating in SBFD mode, the UL hardware link utilizes an RF BPF whose passband exclusively covers the UL operating frequency band. The maximum separation between this passband and the aggressor BS DL interference frequency band results in relatively high suppression, as illustrated in Figure 1(b).

Proposal 1: For two SBFD-capable BSs with adjacent operating bands, both BSs configured as {DU}, and two DL sub-bands are adjacent is the best configuration.

  - Tx unwanted emission requirements

Table 1: Feasibility analysis of SBFD BS co-location

| Parameters | Value |
| --- | --- |
| Output power of aggressor SBFD-capable BS(dBm) | 49 |
| BW(MHz) | 100 |
| Degradation of victim SBFD BS sensitivity(dB) | [1] |
| Required total isolation(dB) | 144 |

For FR1 aggressor WA SBFD BS, the output power is 49dBm, bandwidth of channel is 100MHz, if assume the degradation of victim SBFD BS sensitivity is 1dB, the required total isolation between the two BSs is 144dB, that is, the sum of ACLR, couple loss of co-location and the rejection  of RF BPF　used after PA must larger than 144dB.

Proposal 2: We propose that the degradation of the victim SBFD BS sensitivity be limited to 1dB.

Observation 1: The sum of ACLR, couple loss of co-location and the rejection of RF BPF used after PA must larger than 144dB.

Table 2: For the BPF used after PA, the required suppression levels

| Parameters | 2.6GHz | 4.9GHz | 7GHz |
| --- | --- | --- | --- |
| 1st ACLR(dB) | 45 | 45 | 45 |
| Coupling loss of co-location (dB) | 31 | [46] | [55] |
| For the BPF used after PA, the required suppression levels (dB) | 68 | 49 | 44 |

According to WID [1], the applicable bands focus on band n41, n79 and n104, it is assumed that the co-location coupling losses for these operating bands are the values shown in [3]. ACLR is 45dB. For the BPF used after PA, the required suppression levels on the UL subband of the victimized BS are shown in Table 2. If the value of the co-located coupling loss is updated later, the BPF suppression requirements should be adjusted accordingly.

Observation 2: For the BPF used after PA, the required suppression levels on the UL subband of the victimized BS are shown in table 2.

If channel bandwidth is 100MHz, the DL sub-band widths of two BSs are both 80MHz and UL sub-band width of two BSs are both 20MHz, when the two DL sub-bands are adjacent, victim BS UL sub-band is 80MHz offset to the edge of aggressor BS passband, the BPF after PA can provide suppression. The higher the rejection requirement, the more cavities the filter needs, which increases its size and insertion loss. Of course, this also depends on the specific frequency. The required suppression for the 2.6GHz, 4.9GHz, and 7GHz RF BPFs shown in Table 2 are achievable. However, in practical applications, factors such as the volume and weight of BS also need to be taken into consideration.

Observation 3: The required suppression for the 2.6GHz, 4.9GHz, and 7GHz RF BPFs shown in table 2 are achievable. However, in practical applications, factors such as the volume and weight of BS also need to be taken into consideration.

Proposal 3: The best configuration does not require the introduction of additional ACLR and/or OBUE requirements for FR1 WA SBFD BS.

  - Rx in-band blocking requirement

Table 3: For the RF BPF used in victim BS UL link,

| Parameters | 2.6GHz | 4.9GHz | 7GHz |
| --- | --- | --- | --- |
| Output power of aggressor SBFD-capable BS(dBm) | 49 | 49 | 49 |
| Coupling loss of co-location (dB) | 31 | [46] | [55] |
| Interference signal power level at TAB of victim SBFD-capable BS (dBm) | 18 | 5 | -6 |
| Existing in-band blocking of NR BS (dBm) | -43 | -43 | -43 |
| For the RF BPF used in victim BS UL link, required rejection on interference signal (dB) | 61 | 46 | 37 |

In table 3, it is assumed that the co-location coupling losses at 2.6GHz, 4.9GHz, and 7GHz are the values shown in [3]. If the co-location coupling loss values are updated later, the BPF suppression requirements shall be adjusted accordingly.

For FR1 aggressor WA SBFD-capable BS, the output power is 49dBm. If the co-location blocking is 16dBm, the coupling loss of co-location must lager than 33dB. The co-location coupling losses at 4.9GHz and 7GHz are both greater than 33dB，but the co-site coupling loss at 2.6GHz is 2dB less than the required value, this 2dB difference can be effectively compensated by the RF BPF used in the UL link.

If the in-band blocking of victim SBFD-capable BS is -43dBm, the required total isolation between transmission of aggressor SBFD-capable BS and the reception of victim SBFD-capable BS is 92dB. The total isolation includes co-location coupling losses, and the rejection on interference signal of RF BPF used in victim BS UL link.

Observation 4: If the in-band blocking of victim SBFD-capable BS is -43dBm, the required total isolation between transmission of aggressor SBFD-capable BS and the reception of victim SBFD-capable BS is 92dB.

Observation 5: The sum of the co-location coupling loss and the suppression of the interfering signal by the RF BPF used in the victim BS UL link must be greater than 92dB.

If channel bandwidth is 100MHz, the DL sub-band widths of two BSs are both 80MHz and UL sub-band width of two BSs are both 20MHz, when the two DL sub-bands are adjacent, victim BS UL sub-band is 80MHz offset to the edge of aggressor BS passband, the RF BPF used in  victim BS UL link can provide suppression.

The higher the rejection requirement, the more cavities the filter needs, which increases its size and insertion loss. Of course, this also depends on the specific frequency. The required suppression for the 2.6GHz, 4.9GHz, and 7GHz RF BPFs shown in Table 3 are achievable. However, practical implementation also requires consideration of the associated costs.

Observation 6: For the RF BPF used in UL link of victim BS, the required suppression levels on the DL subband of the aggressor BS are shown in table 3.

Observation 7: The required suppression for the 2.6GHz, 4.9GHz, and 7GHz RF BPFs shown in table 3 are achievable. However, in practical applications, factors such as the volume and weight of BS also need to be taken into consideration.

Proposal 4: The best configuration does not require the introduction of additional in-band blocking for FR1 WA SBFD BS.

- Summary

In this contribution, we have the following proposals and observations.

Proposal 1: For two SBFD-capable BSs with adjacent operating bands, both BSs configured as {DU}, and two DL sub-bands are adjacent is the best configuration.

Observation 2: For the BPF used after PA, the required suppression levels on the UL subband of the victimized BS are shown in table 2.

Observation 3: The required suppression for the 2.6GHz, 4.9GHz, and 7GHz RF BPFs shown in table 2 are achievable. However, in practical applications, factors such as the volume and weight of BS also need to be taken into consideration.

Proposal 3: The best configuration does not require the introduction of additional ACLR and/or OBUE requirements for FR1 WA SBFD BS.

Observation 4: If the in-band blocking of victim SBFD-capable BS is -43dBm, the required total isolation between transmission of aggressor SBFD-capable BS and the reception of victim SBFD-capable BS is 92dB.

Observation 5: The sum of the co-location coupling loss and the suppression of the interfering signal by the RF BPF used in the victim BS UL link must be greater than 92dB.

Observation 6: For the RF BPF used in UL link of victim BS, the required suppression levels on the DL subband of the aggressor BS are shown in table 3.

Observation 7: The required suppression for the 2.6GHz, 4.9GHz, and 7GHz RF BPFs shown in table 3 are achievable. However, in practical applications, factors such as the volume and weight of BS also need to be taken into consideration.

Proposal 4: The best configuration does not require the introduction of additional in-band blocking for FR1 WA SBFD BS.

- Reference

[1] RP-252905, “New WID on NR base station (BS) RF requirement evolution for FR1 and testing phase 2”, RAN#109

[2] R4-2417852,“Discussion on modification of existing Tx requirements for SBFD operation”，CATT, RAN4#113

[3] R4-2508657, “Way Forward for [115][303] NR_BS_RF_Part2_CLTA_SE: CLTA”, Huawei, HiSilicon, RAN4#115