3GPP RAN WG4 Meeting #116bis	R4-2513211

Prague, Czech, October 13th – October 17th, 2025

Title: 	Discussion on the maintenance of RRM core requirements of low band CA via switching

Source: 	CATT

Agenda item:	4.8.2

Document for:	Discussion

Introduction

RAN4 completed the discussion on the impacts of low-low band carrier aggregation via switching on RRM core requirements in RAN4#116 meeting and the core part was successfully closed. The agreements are captured in WF [1]. In this contribution, we will discuss some maintenance issues in [2] and present our views.

Discussion

CQI/L1-RSRP reporting during SDL SCell activation in LB-CA

According to the SCell activation requirement defined in 38.133, UE is requested to report CQI/L1	-RSRP for the SCell to be activated during activation procedure. Before last meeting, RAN4 agreed that the RRC configured switching pattern does not apply the SDL SCell in activation procedure and therefore the CQI/L1-RSRP reporting for SDL SCell on PCell may collide with reference signal receptions on SDL SCell. However RAN4 revised the agreement in last meeting:

RAN4#116: Issue 1-3-5: Activation of Switching Pattern [Agreement]: - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline.

Based on the latest RAN4 agreement, the RRC configured switching pattern for SDL SCell is applied and therefore network clearly knows the time periods during which UE stays on SDL SCell and then it can avoid the collisions between configurations of UL resources on PCell and the SDL SCell ON periods. With this assumption, both the newly defined requirement of SDL SCell activation in LB-CA and requirement of CQI/L1	-RSRP reporting for SCell can apply.

Observation 1: With RRC configured switching pattern applied for SDL SCell, network can avoid the collision between configurations of UL resources on PCell and the SDL SCell ON periods.

Proposal 1: RAN4 do not consider the case where CQI/L1	-RSRP reporting collides with RS resources in SDL SCell.

Activation/deactivation of switching pattern

Another issue is when UE performs switching from PCC to SDL SCell. In our understanding, after UE decodes SCell activation command, it knows on which cell it should stay on with the knowledge of switching pattern. If it should stay on SDL SCell according to the switching pattern, it needs to switch to SDL SCell immediately. Similarly, if should stay on PCell, it can switch to SDL SCell at the next switching period. In addition, on the value of TLBCA, we do not have a strong view as long as the value proposed by proponents is reasonable.

Proposal 2: UE performs switching to SDL SCell following the switching pattern, i.e.,

- UE switches to SDL SCell immediately after Y if it should stay on SDL SCell according to the switching pattern, or

- UE switches to SDL SCell after Y if it should stay on PCell according to the switching pattern.

Where Y = n+(THARQ+3ms+ TLBCA)/slot-length for MAC CE based SCell activation and Y = n+(TRRC_Process + TLBCA)/slot-length for RRC based direct SCell activation.

For deactivation of switching pattern, the following issue also needs more discussion:

Issue 1-3-5a: Deactivation of Switching Pattern [Agreement]: - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length). - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

Firstly, after completion of SCell deactivation, UE needs to stay on PCell. Hence we are supportive for the FFS part which is quite straightforward. For TLBCA, the value, in our opinion, in the activation procedure of switching pattern can be reused.

Proposal 3: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

Proposal 4: RAN4 to reuse the value of TLBCA in the activation procedure of switching pattern.

RLM requirement for PCell

There is one open part in the RLM requirement for PCell as follows:

[Agreement]: - For RLM in LB-CA, follow principle in issue 1-1-1 to define RLM requirements for PCell in LB CA. - P factor in the existing RLM measurement requirements can be updated, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of RLM-RS in W window. - Navailable is number of available RLM-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure.

In the case where radio link failure or beam failure is triggered on the PCell, we also agree it is important to rapidly recover the link/beam in PCell. Therefore we support that the switching pattern does not apply until the link recovery is completed.

Proposal 5: RAN4 to agree that the switching pattern does not apply when radio link failure or beam failure is triggered on the PCell until link recovery is completed.

Conclusions

This contribution discussed the impacts of LBCA on RRM core requirements and some proposals are provided:

Observation 1: With RRC configured switching pattern applied for SDL SCell, network can avoid the collision between configurations of UL resources on PCell and the SDL SCell ON periods.

Proposal 1: RAN4 do not consider the case where CQI/L1	-RSRP reporting collides with RS resources in SDL SCell.

Proposal 2: UE performs switching to SDL SCell following the switching pattern, i.e.,

- UE switches to SDL SCell immediately after Y if it should stay on SDL SCell according to the switching pattern, or

- UE switches to SDL SCell after Y if it should stay on PCell according to the switching pattern.

Where Y = n+(THARQ+3ms+ TLBCA)/slot-length for MAC CE based SCell activation and Y = n+(TRRC_Process + TLBCA)/slot-length for RRC based direct SCell activation.

Proposal 3: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

Proposal 4: RAN4 to reuse the value of TLBCA in the activation procedure of switching pattern.

Proposal 5: RAN4 to agree that the switching pattern does not apply when radio link failure or beam failure is triggered on the PCell until link recovery is completed.

References

[1] R4-2512119, WF on RRM requirements for NR_LBCA_Sw, Moderator (Apple), RAN4#116.

[2] R4-2509049, Topic summary for [116][206] NR_LBCA_Sw, Moderator (Apple), RAN4#116.