3GPP RAN WG4 Meeting #116bis	R4-2513213

Prague, Czech, October 13th – October 17th, 2025

Title: 	Discussion on the maintenance of RRM core requirements of SBFD

Source: 	CATT

Agenda item:	4.19.3

Document for:	Discussion

Introduction

RAN4 completed the discussion on the impacts of SBFD on RRM core requirements in RAN4#116 meeting and the core part was successfully closed. The agreements are captured in WF [1]. In this contribution, we will discuss some maintenance issues in [2] and present our views.

Discussion

Requirement applicability of L1 CLI measurements

In RAN4#115 [3] and RAN4#116, RAN4 discussed the requirement applicability for L1 CLI measurement:

- Issue 1-1-1: Requirement applicability - RAN4#115: - Agreement (Monday online session) - For method #2, 3 and non-basic UE operation of method #1 - The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is configured within active DL BWP - when CLI-RSSI measurement resource is configured within active DL BWP - FFS for basic UE operation for method #1 - Note: The assumption for basic UE operation is that UE cannot receive simultaneous PDSCH reception on two DL subbands, and whether the basic UE operation exists or not is pending on RAN1 discussion. - RAN4#116: - Agreement - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

In RAN4#122 meeting, RAN1 further discussed the UE feature list for SBFD and the following agreement was made [4]:

- RAN1#122: Agreement: - FG 60-1 to include support of PDSCH/CSI-RS reception in two DL subbands  Add a new component to report the number of partial PRGs supported Candidate values: {2, 4}  Add a new component to report CSI timeline, active CSI-RS ports counting and active CSI-RS resource counting for a single CSI-RS resource across two DL subbands  Candidate values: {“with factor-of-two relaxation”, “without factor-of-two relaxation”}  Add a note: “FFS: details of counting”  Add a new component of “maximum of one DL-UL switching point per SBFD slot for actual DL/UL transmission(s)”  Add a new component of “Determination of TBS of PDSCH/PUSCH in SBFD symbols”

According to the RAN1 agreements, UEs have different capabilities of supporting simultaneous DL receptions in both two subbands from RAN1 perspective. Therefore the basic UE operation exists, for which the requirements need to be defined by RAN4. Regarding the requirement applicability of L1 CLI measurements, the current description, in our understanding, is quite general and it also applies for the basic UE operation.

Proposal 1: 	The requirement applicability of L1 CLI measurements applies for the basic UE operation. No updates are needed.

In addition, RAN1 differentiate that whether UE can support reception of CLI-RSSI measurement resources in one DL subband only or across two DL subbands only, which means UE can measure CLI-RSSI resource in one DL subband at a time or in both subbands simultaneously depending on its capability, but current spec cannot reflect this. Therefore we submitted a companion draft CR to update the introduction clause of L1-CLI-RSSI requirements to reflect the impact of UE capability [5].

Proposal 2: 	RAN4 to update the L1-CLI-RSSI requirements to reflect that whether it is capable of receiving two DL subbands simultaneously or not.

CSI-RS L1 measurement – puncturing in frequency domain

In RAN4#116 meeting, RAN4 discussed the puncturing in frequency domain for CSI-RS based L1 measurement and the following agreement was achieved:

RAN4#116: - Agreements (Monday online session) - For UE supporting simultaneous DL reception in 2 DL subbands, - For measurement period: - For UE indicating CSI processing time scaled by 2, - Add a lower bound of 8ms on top of existing measurement period, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2. - For UE not indicating CSI processing time scaled by 2, - Lower bound is not needed and existing measurement period applies - Further discuss the measurement accuracy requirements in Perf part of the WI - For UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB for Case 3 apply. - Option 2: accuracy requirements based on 24 RB for Case 3 apply. - For UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB for Case 3 apply. - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

Based on the discussion in clause 2.1, it is clear that the basic UE operation exists and therefore RAN4 need to confirm that the lower bound is not needed in the CSI-RS based L1 measurement requirements when UE can receive CSI-RS in only one DL subband at one time. But considering that the current description can already serve the purpose, no extra updates are needed.

In addition, the measurement accuracy requirements for CSI-RS based L1 measurements are discussed in our paper [6].

Proposal 3: RAN4 to confirm that the lower bound is not needed in the CSI-RS based L1 measurement requirements when UE can receive CSI-RS in only one DL subband at one time.

Condition for SSB based measurement with basic UE operation

Another open issue is to discuss whether there is any condition for SSB based measurement when UEs do not support simultaneous reception in two DL subbands. The main concern is that UE may be scheduled to receive DL transmission in one DL subband while SSB is transmitted in the other DL subband, in which case UE cannot receive SSBs. In our understanding, if UE reports it does not support simultaneous reception in two DL subbands, then network clearly knows the DL useable PRBs for this UE and it is reasonable to schedule this UE to perform DL reception in the DL useable PRBs. If SSB transmission is outside of DL usable PRBs, then UE can measure it using measurement gaps, which is the same as the legacy UE behaviour and RAN4 have defined corresponding requirements. Therefore RAN4 can assume that DL scheduling always happens in the DL useable PRBs for the UEs with basic operation.

Proposal 4: RAN4 can assume that DL scheduling always happens in the DL useable PRBs for the UEs with basic operation.

Conclusions

This contribution discussed the maintenance issues of SBFD for RRM core requirements and the following proposals are provided:

Proposal 1: 	The requirement applicability of L1 CLI measurements applies for the basic UE operation. No updates are needed.

Proposal 2: 	RAN4 to update the L1-CLI-RSSI requirements to reflect that whether it is capable of receiving two DL subbands simultaneously or not.

Proposal 3: RAN4 to confirm that the lower bound is not needed in the CSI-RS based L1 measurement requirements when UE can receive CSI-RS in only one DL subband at one time.

Proposal 4: RAN4 can assume that DL scheduling always happens in the DL useable PRBs for the UEs with basic operation.

References

[1] R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN4#116.

[2] R4-2509061, Topic summary for [116][218] NR_duplex_evo, Moderator (Huawei, HiSilicon), RAN4#116.

[3] R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon, RAN4#115.

[4] R1-2506409, Session Notes of AI 9.3, RAN1#122.

[5] R4-2513214, DraftCR to update RRM core requirements of SBFD, CATT, RAN4#116bis.

[6] R4-2513221, Discussion on RRM performance requirements of SBFD operation, CATT, RAN4#116bis.