3GPP TSG-RAN WG4 Meeting #116bis				                                           R4-2513226

Prague, Czech, October 13th – October 17th, 2025

Title:	Discussion on the impacts of NR MIMO ph6 on RAN4 requirements

Source:	CATT

Agenda Item:	7.10.3

Document for:	Discussion

1 Introduction

In RAN#109 meeting, RAN approved a new WI on NR MIMO Phase 6 [1] with the following objectives:

The detailed objectives are as follows: RAN1: - On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible. - Cross-slot SRS between one U slot and one adjacent S slot within a single SRS resource set - When used for one SRS with repetition, cross-slot SRS symbol mapping is limited to within one SRS resource, with a common timing advance (TA), a common UL spatial filter, and common transmit power for the SRS resource across the two consecutive slots - On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively - For 48, 64, and 128 CSI-RS ports aggregated over multiple CSI-RS resources per legacy specification: CSI-RS density of 1/3, 1/4, 1/6, and 1/8 RE/RB/port while fully reusing legacy CSI-RS RE mapping per RB for CDM group generation (including OCC length, CDM group type, number of CDM groups), without impacting the legacy specification for rate matching on CSI-RS REs - Density of 1/3 and 1/6 RE/RB/port are intended only for 48 ports RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In this contribution, we will discuss the potential impacts of this WI on RAN4 requirements and present our views.

2 Discussion

Impacts of early SRS/CSI/CSI-RS triggering

According to the 2nd objective in the WID, the early SRS/CSI/CSI-RS triggering mechanism will be applied to the three procedures in FR1:

1. UE transitioning from IDLE/INACTIVE to CONNECTED mode;

2. SCell activation;

3. Switching out of SCell dormancy in CONNECTED mode.

2. On enhancing DL CSI acquisition, targeting FR1, specify the following enhancements: - Early SRS/CSI/CSI-RS triggering for UE transitioning from IDLE/INACTIVE to CONNECTED mode via MSG4 of 4-step RACH, as well as for SCell activation and switching out of SCell dormancy in CONNECTED mode via the legacy signaling mechanisms (based on legacy SCell activation MAC CE, and legacy switching DCI out of SCell dormancy, respectively) - Reusing the legacy design, limit the scope only to SRS for antenna switching, aperiodic wideband CSI with Rel-15 Type-I Single-Panel codebook (targeting FDD) and PMI-free report (targeting TDD) via the legacy UCI, as well as CSI-RS for CSI and (only for IDLE/INACTIVE to CONNECTED mode) CSI-RS for tracking - For IDLE mode, UE capability for early triggering and resource/reporting configuration are signaled via MSG3 and System Information (SI), respectively …

In RAN1#122 meeting RAN1 discussed the application of early SRS/CSI/CSI-RS triggering in UE RRC state transition and SCell activation procedures, and some agreements were made [2]:

RAN1#122: Agreement: For a UE transition from IDLE to CONNECTED mode, support the following procedure at least for early aperiodic SRS-AS/CSI-RS/CSI triggering (i.e., early triggering of aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting): - Step-1: The UE receives the resource/reporting configuration(s) for early SRS-AS/CSI-RS/CSI triggering provided in the system information before MSG3. - FFS: Which SIB is used to carry the resource/reporting configuration(s) for early SRS-AS/CSI/CSI-RS triggering. - Step-2: The UE reports its capability on early SRS/CSI-RS/CSI triggering through MSG3. - Step-3: The UE receives MSG4 that triggers early SRS-AS/CSI-RS/CSI based on the capability reported by the UE. - Step-4: The UE performs aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting. - FFS: Timeline of the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, aperiodic CSI reporting. Note: The term “capability” above does not mean legacy RRC based UE capability. FFS: For a UE transition from INACTIVE to CONNECTED mode, whether above procedure can be reused at least for early aperiodic SRS-AS/CSI-RS/CSI triggering. Note: Whether the aperiodic SRS-AS transmission, aperiodic CSI-RS reception, and/or aperiodic CSI reporting can be configured/triggered simultaneously will be discussed separately. Agreement: For early triggering of SRS-AS when SCell transition from deactivation to activation, support at least aperiodic SRS-AS transmission on a SCell, triggered based on legacy SCell activation command activating the SCell. - FFS: Timeline of the aperiodic SRS-AS transmission (requirement is up to RAN4) - FFS: How to trigger the aperiodic SRS-AS transmission based on legacy SCell activation command - FFS: Whether/what new information is provided in SCell activation command? Agreement: For early triggering of CSI/CSI-RS when SCell transition from deactivation to activation, support aperiodic CSI reporting for a SCell triggered based on legacy SCell activation command activating the SCell. - The aperiodic CSI reporting is assosicated with at least aperiodic CSI-RS for CSI on the SCell - FFS: Timeline of the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI (requirement is up to RAN4) - FFS: How to trigger the aperiodic CSI reporting and corresponding aperiodic CSI-RS for CSI based on legacy SCell activation command? - FFS: Whether/what new information is provided in SCell activation command?

Based on RAN1 discussions, RAN4 need to discuss the impacts on SRS antenna switching and SCell activation. For both procedures, RAN4 defined interruption requirements, e.g., clause 8.2.2.2.16 and 8.2.2.2.17 in 3GPP TS 38.133 [3]. Therefore RAN4 can discuss whether any updates on these requirements are required.

Proposal 1: RAN4 to discuss whether any updates are needed for SRS antenna switching and SCell activation interruption requirements due to support of early triggering of SRS/CSI/CSI-RS.

In addition, if CSI-RS reception and CSI reporting on SCell is triggered by legacy SCell activation command, the current SCell activation requirement will be affect and the delay may be shortened considering that the end point of SCell activation procedure may be moved ahead in time domain. RAN4 need to discuss the impacts of early triggering of CSI/CSI-RS on SCell activation requirements.

Proposal 2: RAN4 to discuss the impacts of early triggering of CSI/CSI-RS on SCell activation requirements.

For the application of early triggering of SRS/CSI/CSI-RS in the switching out of SCell dormancy in CONNECTED mode, since RAN1 has not made any agreements on this scenario yet, RAN4 can discuss the impacts later.

Proposal 3: RAN4 to discuss the impacts of early triggering of CSI/CSI-RS on switching out of SCell dormancy in CONNECTED mode after RAN1 make progress.

SRS phase continuity issue

As per the note in WID document [1], in the case of multiple frequency-domain starting positions for SRS repetition symbols in each SRS frequency hop for RPFS, SRS should reuse the legacy conditions and requirements of phase continuity for DMRS bundling as much as possible.

## 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI The detailed objectives are as follows: RAN1: - On enhancing UL capacity and coverage, specify the following enhancements: - For SRS, reusing the legacy port numbering for SRS resource, comb design, SRS sequence, SRS power control, symbol-level dropping rules (when SRS collides with another UL signal), TCI/UL-spatial-relation frameworks, and SRS bandwidth configuration for SRS frequency hopping, with the maximum number of SRS resources per set following the legacy specification: - Multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop for RB-level partial frequency sounding - Note: On phase continuity, the applicable conditions and requirements from the legacy RAN4 spec for DMRS bundling should be retained as much as possible.

Apparently, the requirements and conditions in legacy RF spec TS 38.101-1 cannot be directly applied. Therefore, this section is discussing the issues that should be considered when reusing legacy requirements.

The phase continuity requirements and measurement conditions for DMRS bundling specified in clause 6.4.2.5 in [4] are extracted as below:

#### 6.4.2.5 Phase continuity requirements for DMRS bundling For bands that UE indicates the support of DMRS bundling, when the UE is configured with DMRS bundling, the maximum allowable difference between the measured phase value in any slot p-1 and slot p, or slot 0 and any slot p for each antenna connector shall satisfy the requirements as listed in Table 6.4.2.5-1 for the measurement conditions defined in Table 6.4.2.5-2, within a measurement time window limited by the UE capability of maximum duration for DMRS bundling maxDurationDMRS-Bundling-r17, and defined for each frequency band separately. The phase value for each slot is measured as shown in Annex F.9. These requirements apply to PUCCH and PUSCH transmissions with DFT-s-OFDM and CP-OFDM waveforms. Unless otherwise specified, pi/2 BPSK in this clause refers to both variants of pi/2 BPSK referenced in Table 6.2.2-1. Table 6.4.2.5-1: Maximum allowable phase difference for DMRS bundling UL channel; Modulation order; Phase difference between any slot p-1 and slot p (NOTE 2); Phase difference between slot 0 and any slot p (NOTE 3). PUSCH; Pi/2 BPSK, QPSK; [25] degrees; [30] degrees. PUCCH; Pi/2 BPSK, BPSK, QPSK. NOTE 1: The UE capability of the length of maximum duration refers to the maximum time duration during which UE is able to meet the phase continuity requirements, assuming no phase consistency violating events defined in TS 38.214 in between. NOTE 2: This requirement applies for FDD and TDD bands, for supported DMRS bundling configurations ≤ 8 slots. NOTE 3: This requirement applies only for FDD bands, for supported DMRS bundling configurations of 16 slots.. The above requirements are applicable when all the following conditions are met within the measurement time window: - RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated.、 - No network commanded TA takes effect. - The TPMI precoder does not change. - There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE. - UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions. - For TDD, no downlink slot(s) or downlink symbol(s) or flexible symbol(s) with/without DL monitoring occasion configured in-between the PUSCH or PUCCH transmissions. Table 6.4.2.5-2: Measurement conditions for the maximum allowable phase difference Parameter; Unit; Level. UE Output Power; dBm; PCMAX,f,c in clause 6.2.4, P-MPR = 0. UE downlink received power; Not change. Operating conditions; Normal conditions. Transmission bandwidth; Confined within FUL_low + [4] MHz and FUL_high – [4] MHz. DL signal frequency; Not change before and during the measurement window. DL signal timing; Maintained constant before and during the measurement window. UL slots for testing; Tested on consecutive UL slots. PUSCH waveform for testing; DFT-s-OFDM.

We can easily notice that following aspects are unsuitable for SRS signal in the requirements:

- The DMRS phase difference is specified at slot granularity, whereas the phase difference for SRS should be specified at symbol granularity.

- The format of Table 6.4.2.5-1 is not fully applicable for SRS. No need to consider UL channel and modulation order. And the notes are not suitable for SRS as well.

- The phase difference measurement conditions for DMRS bundling requires “frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated”, which is not applicable for SRS RPFS with multi-frequency domain starting positions scenario.

- Measurement conditions in table 6.4.2.5-2 are not applicable for SRS, such as PUSCH waveform

Observation 1: The phase continuity requirements for DMRS bundling in TS 38.101-1 6.4.2.5 are not suitable for SRS RPFS with multi-frequency domain starting positions scenario. A new sub-clause is needed.

Observation 2: Appropriate table formats and measurement conditions should be considered for SRS to reuse the legacy phase difference requirements.

Proposal 4: The phase continuity requirements for SRS should be specified in a new separate sub-clause in TS 38.101-1 with appropriate modifications to the legacy DMRS bundling phase difference requirements and measurement conditions.

Moreover, to measure the SRS phase difference, applicable TDD pattern (A.2) and phase measurement procedure should be introduced into TS 38.101-1 Annex A.2 and Annex F.

Proposal 5: New TDD pattern and phase measurement procedure should be defined in TS 38.101-1 Annex A.2 and Annex F for RB-level partial frequency sounding with multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop.

3 Conclusion

In this paper, we discussed potential impacts of NR MIMO Ph6 WI on RAN4 requirements and the following proposals are provided:

Proposal 1: RAN4 to discuss whether any updates are needed for SRS antenna switching and SCell activation interruption requirements due to support of early triggering of SRS/CSI/CSI-RS.

Proposal 2: RAN4 to discuss the impacts of early triggering of CSI/CSI-RS on SCell activation requirements.

Proposal 3: RAN4 to discuss the impacts of early triggering of CSI/CSI-RS on switching out of SCell dormancy in CONNECTED mode after RAN1 make progress.

Observation 1: The phase continuity requirements for DMRS bundling in TS 38.101-1 6.4.2.5 are not suitable for SRS RPFS with multi-frequency domain starting positions scenario. A new sub-clause is needed.

Observation 2: Appropriate table formats and measurement conditions should be considered for SRS to reuse the legacy phase difference requirements.

Proposal 4: The phase continuity requirements for SRS should be specified in a new separate sub-clause in TS 38.101-1 with appropriate modifications to the legacy DMRS bundling phase difference requirements and measurement conditions.

Proposal 5: New TDD pattern and phase measurement procedure should be defined in TS 38.101-1 Annex A.2 and Annex F for RB-level partial frequency sounding with multiple frequency-domain starting positions for SRS repetition symbols within each SRS frequency hop.

4 References

- RP-252936, Revised WID: NR MIMO Phase 6, MediaTek, RAN#109.

- RAN1 chair note, RAN1#122.

- 3GPP TS 38.133, NR; Requirements for support of radio resource management, V19.1.0.

- TS 38.101-1 Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V19.2.0.