3GPP TSG RAN WG4 Meeting #116bis		R4-2513237

Prague, Czech Republic, Oct 13th – 17th , 2025

Title: 	Discussion on BS RF requirements for 6MHz channel bandwidth

Agenda Item:	7.1.5.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN#109, a work item for further UE RF enhancement was approved, and one of the objectives is to add 6MHz channel bandwidth support [1]:

6MHz channel bandwidth - Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4] - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements (OTA and conducted) for UL and DL - Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed - Additional Maximum Power Reduction (A-MPR) - Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2] - Note This should be similar to the signalling for 7 MHz channel bandwidth - Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3] - Note This should be similar to the signalling for 7 MHz channel bandwidth NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent. General for all the areas - Specify release independence requirements for the core part in TS 38.307, if needed and feasible.

In this contribution, we discuss the corresponding BS RF requirements for this new channel bandwidth.

# 2 Discussion

For maximum transmission bandwidth configuration, BS should have the same as UE side, and our companion paper proposes to set 31 PRBs for 6MHz channel bandwidth with SCS 15kHz [2]. And the corresponding guard-band size is 202.5kHz.

Proposal 1: RAN4 to specify maximum transmission bandwidth configuration as 31 and the corresponding guard-band size as 202.5kHz for 6MHz channel bandwidth with SCS 15kHz.

For REFSENS, the situation is different from that at US side, which means that REFSENS is not scaled by channel bandwidth. And in the current TS 38.104, 5/10/15MHz channel bandwidths have the same REFSENS for different BS types and classes. Hence it is reasonable to set the same level as that for 5/10/15MHz.

Proposal 2: RAN4 to specify REFSENS for 6MHz channel bandwidth as the same as that for channel bandwidths of 5/10/15MHz.

The below tables illustrate the changes accordingly.

BS type 1-C and 1-H

For wide area BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-7 (Note 1) | -103.6 |
|  |  | G-FR1-A1-21 (Note 6) | -103.6 |
| 5, 6, 7, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -101.7 |
|  |  | G-FR1-A1-10 (Note 3) | -101.7 (Note 2) |

For middle range BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-7 (Note 1) | -98.6 |
|  |  | G-FR1-A1-21 (Note 6) | -98.6 |
| 5, 6, 7, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -96.7 |
|  |  | G-FR1-A1-10 (Note 3) | -96.7 (Note 2) |

For local area BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-7 (Note 1) | -95.6 |
|  |  | G-FR1-A1-21 (Note 6) | -95.6 |
| 5, 6, 7, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -93.7 |
|  |  | G-FR1-A1-10 (Note 3) | -93.7 (Note 2) |

BS type 1-O

For WA BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 6, 7, 10, 15 | 15 | G-FR1-A1-1 | -101.7 - ΔOTAREFSENS |

For MR BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 6, 7, 10, 15 | 15 | G-FR1-A1-1 | -96.7 - ΔOTAREFSENS |

For LA BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 6, 7, 10, 15 | 15 | G-FR1-A1-1 | -93.7 - ΔOTAREFSENS |

For Rx dynamic range, the current specs indicates that the interfering signal mean power is scaled with channel bandwidth for conducted requirement, which means a scaling factor of 0.9dB applied for 6MHz channel bandwidth on top of that for 5MHz channel bandwidth, i.e., -82.5+0.9=-81.6dBm for WA, -77.5+0.9 = -76.6dBm for MR and -74.5+0.9 = -73.6dBm for LA.

Proposal 3: For conducted Rx dynamic range, RAN4 to specify the interfering signal mean power as -81.6dBm for WA, -76.6dBm for MR and -73.6dBm for LA.

The following tables show the required changes.

BS Type 1-C and 1-H:

For WA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A2-15 | -73.6 | -84.7 | AWGN |
| 5 | 15 | G-FR1-A2-1 | -70.7 | -82.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
| 6 | 15 | G-FR1-A2-1 | -70.7 | -81.6 | AWGN |
| 7 | 15 | G-FR1-A2-1 | -70.7 | -81 | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.7 | -79.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.7 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |

For MR BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (Note 2) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A2-15 | -68.6 | -79.7 | AWGN |
| 5 | 15 | G-FR1-A2-1 | -65.7 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4 |  |  |
| 6 | 15 | G-FR1-A2-1 | -65.7 | -76.6 | AWGN |
| 7 | 15 | G-FR1-A2-1 | -65.7 | -76 | AWGN |
| 10 | 15 | G-FR1-A2-1 | -65.7 | -74.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4 |  |  |
|  | 60 | G-FR1-A2-3 | -63.4 |  |  |

For LA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A2-15 | -65.6 | -76.7 | AWGN |
| 5 | 15 | G-FR1-A2-1 | -62.7 | -74.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.4 |  |  |
| 6 | 15 | G-FR1-A2-1 | -62.7 | -73.6 | AWGN |
| 7 | 15 | G-FR1-A2-1 | -62.7 | -73 | AWGN |
| 10 | 15 | G-FR1-A2-1 | -62.7 | -71.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.4 |  |  |
|  | 60 | G-FR1-A2-3 | -60.4 |  |  |

And similarly, for radiated requirements, the interfering signal mean power could also be scaled as -81.6- ΔOTAREFSENS for WA, -76.6- ΔOTAREFSENS for MR, and -73.6- ΔOTAREFSENS for LA.

Proposal 4: For radiated Rx dynamic range, RAN4 to specify the interfering signal mean power as -81.6- ΔOTAREFSENS for WA, -76.6- ΔOTAREFSENS for MR, and -73.6- ΔOTAREFSENS for LA.

The below tables illustrate the required changes.

WA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -82.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |
| 6 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -81.6- ΔOTAREFSENS | AWGN |
| 7 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -81- ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -79.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |

MR BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -77.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4- ΔOTAREFSENS |  |  |
| 6 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -76.6- ΔOTAREFSENS | AWGN |
| 7 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -76- ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -74.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.4- ΔOTAREFSENS |  |  |

LA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -74.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.4- ΔOTAREFSENS |  |  |
| 6 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -73.6- ΔOTAREFSENS | AWGN |
| 7 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -73- ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -71.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.4- ΔOTAREFSENS |  |  |

# 3 Conclusion

In this contribution, we have the following proposals for BS RF requirements for 6MHz channel bandwidth:

Proposal 1: RAN4 to specify maximum transmission bandwidth configuration as 31 and the corresponding guard-band size as 202.5kHz for 6MHz channel bandwidth with SCS 15kHz.

Proposal 2: RAN4 to specify REFSENS for 6MHz channel bandwidth as the same as that for channel bandwidths of 5/10/15MHz.

Proposal 3: For conducted Rx dynamic range, RAN4 to specify the interfering signal mean power as -81.6dBm for WA, -76.6dBm for MR and -73.6dBm for LA.

Proposal 4: For radiated Rx dynamic range, RAN4 to specify the interfering signal mean power as -81.6- ΔOTAREFSENS for WA, -76.6- ΔOTAREFSENS for MR, and -73.6- ΔOTAREFSENS for LA.

# Reference

[1] RP-252952, “New WID: UE RF enhancements for NR FR1, Phase 5”, Moderator(vivo)

[2] R4-2513236, “Discussion on system parameters and UE RF requirements for 6MHz channel bandwidth”, CATT