3GPP TSG RAN WG4 Meeting #116bis		R4-2513239

Prague, Czech Republic, Oct 13th – 17th , 2025

Title: 	Discussion on general aspects from RAN4 perspective for 6G study

Agenda Item:	8.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

Following the completion of 5G Release-15 and subsequent enhancements in Releases-16 to -19, 3GPP RAN has initiated a RAN level 6G study item and a WG level 6G study item respectively [1][2]. The overall objectives for the WG level 6G study item can be found below [2]:

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion. - Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1 - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - User Plane architecture and protocol design [RAN2] - Control Plane architecture (including RRC states) and protocol design [RAN2, RAN3] - Access stratum security aspects, in alignment with requirements from SA3 [RAN2] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Data transfer design to support various types of data (e.g. AI/ML, Sensing, etc) [RAN2, RAN3] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4] - Radio Access Network architecture, interface protocols and procedures considering support of various services and functionalities (e.g. AI/ML, sensing, etc). [RAN3, RAN2] - Overall RAN architecture aspects [RAN3, RAN2] - RAN-CN functional split, interface, protocol stack and procedures [RAN3] - RAN internal functional split, interfaces, protocol stacks and procedures, [RAN3, RAN2] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4] RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML - Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

As usual in previous releases, RAN4 is expected to play a critical role in defining the radio performance requirements, spectrum aspects, and coexistence evaluations for 6G systems. 3GPP RAN4 leadership has developed a preliminary plan and arrangement for undertaking these works [3].

This contribution provides our general perspective and highlighted topics on 6G study in RAN4.

# 2 Discussion

## 2.1 Role of RAN4 in 6G study

Compared with 5G WG Study Item [4], the objectives related to RAN4 are much clearer in 6G WG Study Item [2]. RAN4 traditionally specifies the RF (including system parameters), RRM and demodulation requirements for UEs and BSs, defines band-specific parameters, and evaluates coexistence for new spectrum bands. For 6G study, this role is expected to continue and expand into new potential areas, including:

- Support user-centric network:

UE centric network is a comprehensive design, which involves many aspects across different layers, including, e.g., UE-specific service subscription, optimal mobility, smart link selection, customized energy saving, efficient control and scheduling, etc.

- Support UL/DL decoupling.

- Definition of RF requirements for new frequency ranges (up to 52.6GHz according to 6G WG SID [2]).

- Performance evaluation for advanced antenna configurations (massive MIMO, extremely large antenna arrays).

- Coexistence and sharing with incumbent services in new bands (e.g. 7-8 GHz, 15 GHz).

- Support for novel device types (e.g. low-complexity 6G IoT devices, sensing-enabled UEs).

- Alignment with ITU-R IMT-2030 requirements.

Proposal 1: RAN4 to be prepared for expanding into new potential areas in addition to traditional tasks.

## 2.2 Potential Technical Topics for Study

### 2.2.1 RF system aspects

From RF system perspective, at least the following aspects needs to be considered in 6G studies:

- FR1 evolution: Extension towards 7-8 GHz bands, or below 410MHz.

- FR3 definition: study of channel bandwidths, RF requirements, and co-existence constraints.

- irregular bandwidths: study potential ways to handle and introduce the support of irregular bandwidths efficiently

- Wide channel bandwidth (≥200 MHz),

For 6G, FR1 is expected to evolve beyond the current 5G allocations. Two specific directions are being considered:

- Extension upwards towards 7–8 GHz:

  - Provides additional mid-band spectrum with wider contiguous bandwidths (e.g., ≥200 MHz).

  - Potential reuse of the existing 3.5 GHz site grid for deployments around 7 GHz, targeting comparable coverage to 5G mid-band.

  - Requires new RF requirements and coexistence studies, since this range is also heavily utilized by incumbent services (Wi-Fi, satellite, fixed links, etc.).

  - Brings challenges for device implementation due to increased front-end complexity and coexistence with harmonics of lower bands.

- Extension downwards below 410 MHz:

  - Offers improved coverage and penetration, especially in rural and deep-indoor environments.

  - Supports emerging 6G use cases such as massive IoT and wide-area sensing, where ultra-long range is prioritized over capacity.

  - Demands careful consideration of spectrum sharing with legacy narrowband systems.

Together, these extensions at both ends of FR1 highlight the need for scalable and flexible RF/RRM frameworks in 6G, spanning ultra-wide frequency ranges while balancing coverage, capacity, and coexistence requirements.

Additionally, FR3 (7/8–24 GHz) in 6G is expected to become a key frequency range, complementing FR1 and FR2 by combining wider channel bandwidths with improved coverage compared to mmWave, therefore positioned as a coverage–capacity bridge for 6G, demanding careful balancing of performance, complexity, and coexistence considerations in RAN4.

Moreover, in 6G, spectrum allocations may increasingly come in non-standardized or fragmented sizes that do not align neatly with today’s standardized channel bandwidths (e.g., 5G’s 5/10/20/40/50/100/200/400 MHz). From a RAN4 perspective, it is required to establish a methodology for introducing irregular channel bandwidths into specifications without exponentially increasing the number of band-specific annexes, and consider parameterization (e.g., scalable FFT sizes, numerology) rather than enumerating each bandwidth explicitly, hence RAN4 can enable efficient and scalable support of irregular bandwidths, avoiding specification proliferation while maximizing spectrum utilization in diverse regional allocations.

For 6G, a key design target is to support significantly wider channel bandwidths compared to 5G NR. While Release 15/16 introduced up to 400 MHz per carrier in FR2, and Release 17/18 extended aggregation across multiple GHz, 6G aims to establish ≥200 MHz per carrier as a baseline for mid-band and upper-mid-band (FR3) deployments. Supporting wide channel bandwidths as a standard feature will allow 6G to achieve multi-Gbps throughput, ultra-low latency, and spectral efficiency improvements, while ensuring harmonization across FR1, FR3, and beyond.

Proposal 2: From RF system aspects RAN4 to consider at least extension of FR1, new FR3, handling irregular channel bandwidth efficiently and wider channel bandwidths etc. in its 6G study.

### 2.2.2 Considerations on RF Requirements and Test Methodologies

For 6G studies in RAN4 on RF requirements and test methodologies, the following aspects could be considered:

(1) Requirements vs. Complexity/Efficiency

- Balancing strictness with efficiency:
Requirements need to be ambitious enough to enable 6G performance targets, but not so strict that they lead to impractical device complexity or excessive power consumption. Trade-offs between spectral efficiency, device implementation cost, and energy efficiency will be central to RAN4’s evaluations.

- BS and UE RF scalability:
Define scalable RF requirements that support very wide carrier bandwidths (up to several GHz), while ensuring consistent performance across both wideband and narrowband allocations.

(2) Band and Band Combination Framework

- New band definition enabling decoupled UL and DL operations.

- Harmonization of band definitions:
Work toward globally aligned FR1/FR3 band definitions to simplify device design and promote worldwide roaming.

- Simplified band combination framework:
Develop a new approach to band/band combination specification, avoiding the complexity and redundancy that emerged in LTE/NR. A structured and database-driven framework may be needed for 6G.

(3) Device types and power classes

- New power classes:
Define new UE power classes tailored to handhelds, wearables, XR devices, and industrial terminals, addressing diverse form factors and thermal constraints.

- Various device types:
Ensure that requirements are scalable across a wide range of device types, from high-performance UEs (e.g. laptops, XR headsets) to low-complexity IoT nodes.

Extensive discussions on the definition of various device types were held at RAN#109, triggered by the related RAN1 debate [5]. However, from a RAN4 perspective, the focus is limited to those attributes that have a direct impact on RF requirements. Consequently, the definitions adopted in RAN4 may differ from those used in other working groups.

(4) Testability and Conformance Methods

- Ensuring testability and practicality:
Requirements must remain verifiable with practical conformance testing methods, both in conducted and OTA environments.

- OTA and advanced test methods:
Extend OTA testing methodologies to cover the new frequency ranges and extremely large antenna arrays.

(5) Coexistence Studies

- TN and NTN coexistence:

  - Conduct coexistence studies between terrestrial (TN) and non-terrestrial (NTN) networks in overlapping or adjacent frequency bands. This includes evaluation of adjacent-channel interference and joint operation in shared bands under updated assumptions.

- Coexistence study for new FR3:

  - Evaluate feasible minimum and maximum bandwidths in FR3, targeting ≥200 MHz per carrier as a baseline, with options for multi-carrier aggregation up to several GHz.

  - Harmonize raster definitions and numerology choices to enable efficient deployment

Proposal 3: RAN4 to consider at least trade-off between requirements and complexity/efficiency, harmonized band definition and simplified band combination framework, various devices types and new power classes, testability and conformance methods and coexistence studies for TN/NTN and new FR3 for RF requirements and test methodologies in its 6G studies.

### 2.2.3 RRM Requirements

One of the expected evolutions in 6G is the introduction of “user-centric” operation, where the network takes a more active role in service delivery, including mobility management. This may substantially impact radio resource management (RRM), as procedures such as handover could be initiated by the network rather than being UE-triggered.

Another important aspect for 6G RRM is the integration of terrestrial (TN) and non-terrestrial (NTN) operation, with particular focus on ensuring mobility support in challenging environments.

In addition, RRM will need to address the requirements of a multi-functional RAN, where communication and sensing functionalities are jointly supported.

These representative challenges, and their differences from 5G operation, should be carefully considered and studied in the context of 6G, which may lead to a new RRM framework or requirements.

Proposal 4: RAN4 to consider and study potential new RRM framework or requirements to address the user-centric operation, integrated TN and NTN and multi-functional RAN etc.

# 3 Conclusion

RAN4 is expected to play a critical role in 6G studies by shaping spectrum usage, defining RF/RRM requirements, and ensuring coexistence with other services. Early alignment on technical priorities will facilitate timely contributions to ITU-R IMT-2030 deliverables and prepare for normative work in later releases. In this contribution, we have the following proposals on general aspects from RAN4 perspective for 6G study:

Proposal 1: RAN4 to be prepared for expanding into new potential areas in addition to traditional tasks.

Proposal 2: From RF system aspects RAN4 to consider at least extension of FR1, new FR3, handling irregular channel bandwidth efficiently and wider channel bandwidths etc. in its 6G study.

Proposal 3: RAN4 to consider at least trade-off between requirements and complexity/efficiency, harmonized band definition and simplified band combination framework, various devices types and new power classes, testability and conformance methods and coexistence studies for RF requirements and test methodologies in its 6G studies.

Proposal 4: RAN4 to consider and study potential new RRM framework or requirements to address the user-centric operation, integrated TN and NTN and multi-functional RAN etc.

# Reference

[1] RP-251395, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom

[2] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[3] R4-2511652, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple)

[4] RP-160671, “New SID Proposal: Study on New Radio Access Technology”, NTT DOCOMO

[5] RP-252873, “Moderator's summary on 6G diverse device types (incl. minimum UE bandwidth)”, Apple