3GPP TSG-RAN WG4 Meeting #116bis	R4-2513242

Prague, Czech Republic, Oct 13th – 17th , 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.774 | CR | draftCR# | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TR 38.774 on transmitted signal quality (clause 7.2.4) for LP-WUS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, [Ericsson] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The endorsed TP R4-2512667 was not captured into the TR because it was based on an oder version. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The same change adapted to the latest approved TR RP-252465 is made |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EVM discussion is not completely recorded. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 7.2.4	Transmitted signal quality

It is agreed that overlaid OFDM sequence is designed for LP-WUS and the sequence is ZC sequence. This applies in the time domain before the DFT transform for both OOK-1 and OOK-4. RAN1 also agrees on the processing for the OOK signal, and this is illustrated in Figure 7.2.4-1. After the DFT transform, the OOK-4 frequency domain pattern is illustrated in Figure 7.2.4-2.

Figure 7.2.4-1: OOK-4 signal processing and subcarrier mapping before IFFT

Figure 7.2.4-2: ZC sequence in frequency domain after DFT processing before IFFT

Fig. 7.2.4-3, Transmitted signal generation for LP-WUS

Figure 7.2.4-3 illustrates the LP-WUS signal generation process based on the completed RAN1 signal design. Up to 5 information bits are channel coded, followed by rate matching of the coded bits as the first bit block. The output is then Manchester coded, doubling the bit length as 2*E0, where

- E0=NOS∙M2

- NOS is the number of OFDM symbols allocated for the information bits configured by higher layer

- M is the chip rate which means the number of ON-OFF chips per OFDM symbol.

In parallel, there is also another path directly from information bits to generate a corresponding overlaid OFDM sequence according to the outputs of both Manchester coding and the rate matching for the second bit block if present, and the generated overlaid OFDM sequence is to be overlaid on ON-chips after ON-OFF waveform generation. Both paths are performed on a per-block basis. With the overlaid OFDM sequency, an OFDM-based receiver can decode the information bits earlier than the final OFDM symbol corresponding to the information bits. Whereas a symbol-rate processing comes afterward, where for OOK-4 with M chips, all bits are grouped into M-bit tuples with each tuple associated with one OFDM symbol, and for each tuple an ON-OFF waveform is generated and then M ON-chips are replaced with M complex symbols sequentially extracted from the overlaid OFDM sequence. After the replacement, the waveform is transmitted in one symbol via a DFT module, and then the output is multiplexed across N sub-carriers with other NR modulated symbols followed by an IFFT operation to generate a time-domain signal for transmission.

NR RF requirements for LP-WUS operation in the present release are specified assuming that LP-WUS signal transmitting signal quality is assured in NR BS with NR signal transmission.

<End of change>