# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513250

# Prague, CZ, 13th – 17th Octobor, 2025

Source: 	vivo

Title: 	Initial views on 6G system parameters

Agenda Item:	8.3

Document for:	Approval

Introduction

A study item on 6G radio [1][2] was approved. In the last RAN4 meeting, RAN4 6G plan and scopes for 6G topics were established [3]. System parameters topic is one of the most important aspects for 6G, including waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types. In this contribution, initial views on 6G system parameters are provided.

Discussion

Waveform, modulation

Waveform

The agreement of RAN1 #122 meeting regarding waveform for 6G is as follows:

Agreement of RAN1#122: CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink

As can be seen, CP-OFDM and DFT-s-OFDM waveforms in 5G NR would be considered as benchmark in 6G stage. And based on the assumption, some remaining issues about the waveforms should be studied further in 6G, such as coverage enhancement. One of the effective approaches is to design the low-PAPR waveform.

The waveform with high PAPR necessitates significant power back-off to avoid operating in the nonlinear region of the PA, which would otherwise lead to saturation. This reduction in power decreases PA efficiency and limits output power. To comply with requirements such IBE, EVM, ACLR, and SEM, a higher MPR is inevitably required.

Observation 1: Waveform with high PAPR would decrease PA efficiency and further reduce maximum output power, which is a key requirement in RAN4.

Proposal 1: The basic waveform is decided by RAN1, but the PAPR and PA efficiency related aspect can be discussed and decided in RAN4.

Low-PAPR scheme specified in NR R15 and R18: FDSS

From the discussion details in RAN1, in addition to the baseline waveform discussion, the 6G waveform enhancements: low-PAPR waveform for coverage extension in UL has also been discussed extensively in RAN1. In 5G Rel-18 coverage enhancement WI, due to limited TU and consideration for avoiding complex impact on RAN1, similar discussion is converged to a transparent spectrum shaping scheme: FDSS. PAPR is reduced by using a certain shaping filter to attenuate specific high frequency components, which are the source of peaks in the signal. FDSS was specified earlier in R15 for pi/2 BPSK and expanded the utilization to QPSK in R18. FDSS doesn’t have any impact on RAN1 and is considered as a transparent spectrum shaping scheme, which is much simpler but has limited performance improvement for PAPR reduction. In RAN4 spec, finally there is 1/0.5dB power boost for inner region.

Observation 2: Transparent spectrum shaping schemes to reduce PAPR such as FDSS have been discussed extensively in 5G R15 and R18 stage.

Remaining scheme in NR R18: FDSS-SE

In R18-stage, some non-transparent approaches have also been discussed, the most typical one is FDSS-SE: FDSS with spectrum extension. Compared to FDSS, FDSS-SE first performs symmetrical expansion on both sides of the spectrum to place the copied in-band data at the edge of the allocated RBs, followed by frequency-domain shaping of the expanded spectrum as a whole. Although the expanded spectrum portion would be attenuated by filtering, the receiver can still improve reception performance through combining.

Comparing transparent and non-transparent approaches, it could be seen that in the transparent scheme, the specific waveform shaping used by the transmitter is not explicitly signalled to the network side, preventing the receiver from performing adaptive equalization. It would in turn restricts the choice of shaping parameters at the transmitter, making it hard to achieve optimal shaping. In contrast, the non-transparent scheme, with its superior coordination, enables the receiver to configure appropriate equalization parameters. As a result, constraints on filtering parameters at the transmitter can be relaxed, allowing better waveform designs to further reduce PAPR.

It should be pointed out that the performance comparation of no filtering, FDSS, and FDSS-SE in R18 was based on the assumption of keeping the total allocated bandwidth, the spectral efficiency and resource in time domain the same for all compared cases. Also, for FDSS-SE these expanded frequency-domain resources could not be allocated to other users.

Figure 1. Illustration of FDSS and FDSS-SE

In RAN1 #122 meeting, there are more advanced schemes to reduce PAPR proposed by different companies, including some transparent schemes and non-transparent schemes, such as FDSS (approximating GMSK) [4], FDSS-CE[5], pre-coded DFT-s-OFDM[6], CFR-SE[7] and so on.

Recommended scheme for 6G: CFR-SE

Clipping, i.e., Crest Factor Reduction (CFR), is typically implemented in UEs to reduce the PAPR of uplink signals, thereby enabling lower PA power back-off. A common CFR approach involves iterative clipping and filtering operations: Clipping removes signal peaks to achieve lower PAPR, while subsequent filtering mitigates the out-of-band leakage caused by the clipping process. Given filtering will introduce new peaks, usually multiple iterations of this clipping-filtering operations are needed. One way to reduce the number of iterations and further improve PAPR is CFR with spectrum extension (CFR-SE), where filtering is carried out based on extend spectrum to retain more sidelobes (in frequency) for clipping signals. These sidelobes can help to further reduce PAPR. As shown in Figure 2, actual PRBs are used to carry UL transmission block while indicated PRBs are used to determine filter. In addition to better PAPR performance, the extended PRBs can be shared by other UEs because no actual data signal is transmitted using this part. Further, as clipping algorithm is sufficiently mature and implemented in commercial UEs, and CFR-SE only extends the frequency band of the filtering, the additional complexity and effort to implement CFR-SE is marginal. Moreover, it can be applicable to both CP-OFDM and DFT-s-OFDM.

Figure 2. Schematic of CFR-SE[7]

Some initial simulation results could be seen as follows, the CCDF of PAPR is illustrated to show the performance of different schemes. The relative simulation assumptions are as shown in Annex. Comparing the baseline and five different spectrum shaping schemes, CFR-SE demonstrates better PAPR and DCM performance than FDSS, FDSS-SE, CFR and TR.

Observation 3: CFR-SE demonstrates superior performance in terms of PAPR compared to FDSS, FDSS-SE, CFR and TR.

From RAN4 side, some evaluation regarding diverse low-PAPR waveform schemes could also be started, including the impact of spectrum shaping on RF requirements such as ACLR and EVM. And the requirements in the extended RBs may also need further analysis as in some schemes these extended RBs don’t transmit the actual data and are not part of the allocated RBs. Finally, the value of MPR reduction or power boosting should be simulated as the key point of PAPR reduction. And after the final scheme is specified, EVM spectrum flatness also need re-evaluation to accommodate the shaping.

Proposal 2: Study transparent and non-transparent techniques to further reduce PAPR, including CFR-SE. RAN 4 could start the evaluation of affected RF requirements, such as EVM, ACLR, MPR and applicable requirements in the extended RBs if needed. Other related spec impact needs further identification.

In R18 power boosting topic, the net gain metric is considered as high-comprehensive requirement to evaluate the performance of a low-PAPR waveform. The formula is excerpted as follows, PoutFilter i-PoutRef. filter represents the coverage enhancement gain, i.e., power back-off gain caused by PAPR reduction. While SNR10%Ref. filter-SNR10%Filter i is the SNR gain to achieve 10% BLER compared against the reference. A high-performance waveform should achieve good power efficiency without much degradation of the spectral efficiency, thus it is recommended to reuse the net gain metric to evaluate the low-PAPR schemes in 6G as a starting point.

Net Gain dB=PoutFilter i-PoutRef. filter+SNR10%Ref. filter-SNR10%Filter i

Observation 4: Net gain is adopted as an efficient metric of evaluating the comprehensive performance of a low-PAPR waveform in R18.

Proposal 3: Net gain (defined as MPR gain minus SNR loss at a BLER of 0.1) could be considered as the starting point of the metric to evaluate the comprehensive performance of low-PAPR waveform scheme in 6G.

Another important issue is the selection of PA model. The evaluation of MPR needs a certain PA model to simulate the power back-off value, and different PA model would bring about diverse output performance and different saturation point. It is recommended to align a consensus on the PA model assumption for more accurate simulation.

Proposal 4: Recommend to align a widely accepted PA model for MPR simulation.

Modulation

Agreement of RAN1 #122: - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes Some outcomes of online discussion: ##### Proposal 2.1-3 (replaced) For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM without constellation shaping are supported as basis for CP-OFDM - Further study 4096QAM, including applicable scenarios, challenges, requirements, and solutions ##### Proposal 2.1-4 (replaced) For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM without constellation shaping are supported as basis for both DFT-s-OFDM and CP-OFDM - Further study 1024QAM, including applicable scenarios, challenges, requirements, and solutions - Recommendation for the Chair to decide: Any enhancements to uniform QAM targeting lower PAPR are to be studied in waveform AI or in this AI ##### Proposal 2.1-8 (for online) For the study of 4096QAM for DL and 1024QAM for UL, need to consider performance, complexity, requirements, applicable scenarios, and challenges and solutions. ##### Proposal 2.2-7 (for online) For 6GR constellation shaping evaluation for CP-OFDM, (Geometric shaping and probabilistic shaping with optimized MCS), the proposed scheme will be compared with non-shaping with NR MCS table. The evaluation and comparison should consider at least the following: - BLER performance under AWGN channel and fading channel (SISO, and MIMO with rank=1 and rank>1) - Throughput performance under fading channel (SISO, and MIMO with rank=1 and rank>1) - Transmitter and receiver complexity, latency, parallelism implementation, and storage requirements, - PAPR - Expected spec impact - FFS: Simulation assumptions, such as - For MIMO channel evaluation, needs to provide assumptions on MIMO precoder (e.g., open loop MIMO or closed loop MIMO (as baseline)) and receiver assumed (e.g., MMSE or rML, genie channel or realistic channel estimation) - FFS MU-MIMO - For throughput evaluation, needs to provide assumptions on rate adaptation (e.g., target BLER for 1st transmission, maximum # of retransmissions) - Other KPI not excluded, such as EVM, MPR/A-MPR - FFS: System level simulations

In RAN1, some discussions regarding higher modulation order are currently underway, including 1024 QAM for UL and 4096QAM for DL. For the new modulation schemes beyond 5G, it needs feasibility validation from RAN4, especially for some hardware metrics like EVM. Also, the MPR should also be evaluated based on the higher feasible modulation. However, from RAN4 aspect we need to wait for RAN1’s conclusion to trigger the feasibility study.

Observation 5: In RAN1, some discussions regarding higher modulation order are currently under discussion, including 1024 QAM for UL and 4096QAM for DL.

Proposal 5: If the study of higher modulation orders beyond 5G is confirmed by RAN1, the feasibility validation from RAN4 perspective is necessary, including UE/BS Tx EVM.

CBW, FFT, numerology

For maximum channel bandwidth,

For supported UE channel bandwidths, they are closely dependent with frequency ranges. In 5G NR, various channel bandwidths are supported from 5MHz to 100MHz in FR1, 50MHz to 400MHz in FR2-1. Different channel bandwidths can support enough flexibility for deployment and high date peak rate for 5G. In 6GR, we need to reconsider the maximum channel bandwidth and minimum channel bandwidth. For 6GR refarming bands, reusing the channel bandwidth is our preference, that is maximum 100MHz for Sub-6GHz and 3/5MHz for minimum channel bandwidth. For 6G new spectrum, U6G and FR3 have abundant spectrum, which can support more than 100MHz. We can extend 100MHz to 200MHz for these new 6G spectrum, achieving higher peak date rate for 6G and competing with 5G. For 6GR FR2-1, we think reusing channel bandwidth from 5G is enough.

For minimum channel bandwidth,

From our perspective, 6G minimum channel bandwidth needs to cover SSB bandwidth, which needs RAN1’s confirmation. Also, it depends on the request from operators. For spectrum sub-6GHz, 6G still need to utilize the 5G refarming bands which are fragmented. Minimum channel bandwidth 5MHz is still the baseline choice for normal UE, however, 3MHz may still be needed considering some low-tier device and spectrum holdings from some operators. For U6G and newly introduced FR3, we propose 20MHz as minimum channel bandwidth for the efficient use of these abundant spectrums.

Proposal 6: For 6GR FR1, it is proposed that minimum channel bandwidths 5MHz and maximum channel bandwidth 200MHz can be supported.

Proposal 7: For operators those has only 3MHz spectrum resources, RAN1 may need to specify a mechanism to support the operation of 3MHz spectrum usage.

Numerology and FFT size:

Numerology:

As usual, 6GR numerology is within RAN1’s scope. The agreement on 6GR numerology from RAN1#112 was captured as follows. Many RAN4 discussion is tightly relying on the decisions on 6GR numerology. We can take below agreements as working assumptions in RAN4.

Agreements from RAN1#112:

In current NR bands, multiple numerologies were defined, 15/30/60 for FR1 and 60/120/240 for FR2-1. However, in real deployment, only one numerology is adopted for a specific band. Avoiding this redundancy and complexity in 6GR, it is proposed to support only one numerology for a specific frequency range/ band.

Proposal 8: For a specific frequency range/band, it is proposed to support only one numerology.

FFT size is tightly dependent on the number of RE for the specific channel bandwidth. For example, if we consider 100MHz with 30kHz, the total RE number would be 3276 (273*12), then the best suitable FFT size is 4096. For sub-6GHz, we think FFT size 4096 is OK if we consider maximum 100MHz channel bandwidth. However, it may not be enough if we consider 200MHz for around 7GHz and FR3. The RE number would be doubled for 200MHz, which is approximately 2*3276. In this case, FFT size 8192 can be used.

Proposal 9:

For Sub-6GHz, maximum FFT size 4096 is suggested for maximum 100MHz with 30kHz SCS;

For FR1 U6G and FR3, maximum FFT size 8192 is suggested for the expected maximum 200MHz with 30kHz.

Channel raster, synchronization signal and raster, spectrum utilization

Channel raster

Channel raster defines a set of RF frequency points for the possible RF positions for channel bandwidth. In 5G NR, 100kHz channel raster was used for the LTE refarming bands, and SCS based channel raster for 5G new bands.

In 6GR, we suggest to adopt SCS based channel raster, since it covers all the possible channel deployment from various operators from all over the world. However, for 5G refarming bands to 6GR, it may need further alignment between 5G channel raster and 6G channel raster.

Figure 1 shows an example of 5G and 6G raster alignment. For some 5G bands, 100kHz channel raster was used. If 6G adopts general SCS based channel raster, e.g. 15kHz channel raster. There is channel raster misalignment between 5G and 6G. In this case, frequency raster shift is needed for 6G in co-existence with 5G.

Figure 3. An example of 5G and 6G raster alignment

Proposal 10: It is suggested to only adopt SCS based channel raster for 6GR, additional frequency raster shift can be considered to align with 5G channel raster.

Synchronization signal and raster

Synchronization signal discussion is within RAN1’s scope. RAN1 kicks off the discussion for 6GR sync signal and the agreement from RAN1#122 was captured:

For sync raster design, it is highly related to RAN1’s sync signal design. While, waiting for RAN1’s decision on 6GR SSB design, we can discuss some high-level aspects regarding sync raster.

Sync raster concept was firstly introduced in 5G NR with the basic principle that any Min channel bandwidth can cover at least one SSB, as shown in Figure 2. Min CBW with X RB was placed in the channel raster points for a specific band. SSB BW with Y RB was covered in the Min CBW. In order to ensure the sync raster interval is sparse enough and SSB can be covered by at least one Min CBW, the sync raster should satisfy the equation: X-Y+ΔFCH,Raster.

Figure 4. Concept of sync raster

We think this basic concept of sync raster established in 5G is hard to shake. However, it needs further confirmation whether to reuse sync raster design principle for 6GR in RAN4.

Proposal 11: To discuss whether the fundamental sync raster design principle can still be applied to 6GR:

Sync Raster interval =Min BWCHANNEL-BWSSB+ ΔFCH,Raster

Under above sync raster design principle, sync raster interval is dependent the min channel bandwidth, the SSB bandwidth and channel raster, which require more discussion in both RAN1 and RAN4.

Additionally, RAN1 may also have some new considerations for SSB design. For example, RAN1 may adopt 160 ms SSB periodicity. It may have more demand on sparser sync raster design in RAN4. RAN1 may consider different SSB bandwidth, which may also have a big impact on sync raster. We can wait for SSB design in RAN1.

Spectrum Utilization

In RAN4 discussion, spectrum utilization means the maximum transmission bandwidths NRB or the percentage of the maximum transmission for a given channel bandwidth. For 5G refarming bands to 6G, reusing current SU for existing channel bandwidths can be a baseline. For newly introduced channel bandwidths for 6G, it needs further discussion.

In 5G NR, FR1 can achieve 95%~98% spectrum utilization for CBW larger than 50MHz and 95% spectrum utilization for FR2-1, as shown in below figures. In order to keep the same high level with 5G NR, current spectrum utilization can be used as a starting point for existing channel bandwidths in 6GR refarming bands. For newly introduced channel bandwidth e.g. 200MHz for U6G and FR3, we think spectrum utilization evaluation is needed based on RF requirements (SEM/EVM/ACLR) defined for 6GR, which will be discussed in 6GR RAN4 RF part.

Figure 5. Spectrum Utilization for FR1

Figure 6. Spectrum Utilization for FR2-1

Proposal 12: For existing channel bandwidths 0-100 MHz, reusing current 5G spectrum utilization for 6GR can be a starting point. For newly introduced channel bandwidth, e.g. 200MHz, spectrum utilization evaluation is needed, considering the RF requirements (SEM/EVM/ACLR) for U6G and FR3.

Irregular channel bandwidth

The irregular CBW in NR refer to the CBW other than the values that specified in the spec, e.g., 7MHz, 13MHz. Based on the 6G SID, at least 200MHz will be introduced, so RAN4 first need to discuss what is CBW granularity in 6G, especially when CBW is larger than 100MHz.

For CBW≤100MHz, NR already define a lot of CBW which is based on the commercialized request from operators and in 6G, these CBW can be the starting point and whether it is possible to reduce the number of candidate CBW in 6G can be discussed. For CBW>100MHz, we prefer not to use fine granularity as NR to limit the number of CBW, and 50 or 100 MHz can be considered. For other CBW, single cell multi-carrier or intra-band contiguous CA can be possible ways to utilize the spectrum since from UE perspective, the architecture would be same for intra-band contiguous CA and single carrier.

Proposal 13: RAN4 need discuss the granularity for the CBW in 6G

- RAN4 should limit the number of mandatory CBW in the spec

- For CBW≤100MHz, CBW specified in NR can be the starting point

- For CBW > 100MHz, 50 or 100MHz as granularity can be considered

  - Other CBW can be supported by single cell multi carrier or intra-band contiguous CA

The techniques to efficiently utilize operator spectrum allocations for irregular channel bandwidth was also studied in NR. From UE side, honestly the benefit of supporting such irregular channel bandwidth doesn’t attractive. On the one hand, for the UE work under the operator who holds irregular CBW, it could be a rare case that NW scheduling the whole spectrum to a single UE, which means the UE performance gain could be low. On the other hand, UE needs to implement with additional design, e.g., filter, which increase the cost.

Observation 6: The performance gain of irregular CBW at UE side is lack of justification.

In R19, a WI for introducing dedicated 7MHz was finished. After checking the spec impact, we find that almost all RF requirements are similar to 5MHz, and main changes happen on regulation related requirement, e.g., A-MPR evaluation. This means that if UE only support irregular CBW at DL, the requirement will be easier to be discussed and defined.

Observation 7: If the irregular CBW is only supported in DL, the RF requirements can be easier defined based on closed nominal CBW.

In addition to create a dedicated channel bandwidth, there are four possible approaches are identified:

Figure 7. 4 Possible approach to deal with irregular channel bandwidth

In 6G from UE side, we prefer support the irregular CBW in transparent way as much as possible. The UE impact of Overlapping UE CBW and Larger CBW are small, which can be the starting point for 6G. In addition, the single cell multi-carrier operation is discussed in RAN1 and such operation could provide another angle to help on the irregular channel bandwidth utilization.

Proposal 14: Study the solution to utilize the irregular channel bandwidth which is transparent to UE design in 6G.

- Overlapping UE CBW and Larger CBW in TR38.844 can be the starting point.

- Check whether single cell multi-carrier operation can help on utilizing irregular channel bandwidth

- FFS whether only enable irregular CBW in DL

Device types

In LTE, many UE categories are defined in TS36.306 which are mainly designed based on baseband capability, e.g., up to 12 UE categories for combined DL and UL capabilities.

The UE category can classify the UE performance and expect more efficient communication between UE and NW. However, most of UE categories are not commercialized in the end and this also proves that too fragmented device type is over-designed. In NR, the device type isn’t defined and in RAN4, power class can implicitly reflect the device type, e.g., PC1 is FWA and PC2/3 is handheld UE, but in R19 6Rx WI, it was a long discussion how to distinguish FWA and handheld for 6layer, and with more and more different device types appear in 6G, such discussion could be unavoidable.

Observation 8: The lesson from LTE and NR is that to classify UE type to a certain extent (or high-level category) is helpful for classifying the UE performance and feature but too fragmented device type is over-designed.

In 6G, we suggest to study on defining limited device types (or Cats) in RAN4 based on BB and RF capability. The Figure x show a possible example:

Figure 8. Example for 6G device types (or Cats) design in RAN4

Based on the experience in NR, devices can be categorized into 3 types. From high-tier device type A including Smartphones, immersive eMBB, to low-tier device type C for IoT, and device type B in-between for reduced capability eMBB, can be considered. It is noted that, ambient IoT is a different type of IoT evolved in a separate track, thus is not considered here. To avoid similar problem in R19 6Rx, we add a new tier for FWA/CPE device.

Proposal 15: Study on introducing limited number of device types (or Cats) and corresponding baseline BB/RF capability in RAN4. The following 4 types can be considered as starting point.

- Type A+:  FWA, CPE

- Type A:  Smart phone, immersive eMBB device

- Type B:  Reduced capability eMBB device, wearable/XR

- Type C:  IoT

In RAN#109, the following agreement was made:

Once the device types are introduced in RAN4, the maximum channel bandwidth can be different for each device type, and RAN4 should discuss for each device type, what the value of maximum channel should be.

Proposal 16: The maximum channel bandwidth for each device type can be different, and the following value can be considered as starting point for further discussion:

- Type A+:  FWA, CPE 200MHz，400MHz optional

- Type A:  Smart phone, immersive eMBB device 200MHz，400MHz optional

- Type B:  Reduced capability eMBB device, wearable/XR 100MHz

- Type C:  IoT 20 MHz

For a specific device type, e.g., smartphone, the RF capability could be different in different frequency range.The number of Tx and Rx is the most fundamental RF capability, which has a significant impact on the RF requirements. In 5G more Tx and Rx numbers are always the pursuit of the industry. However, for handheld device, we always need to seek a balance between design complexity and power consumption, form factor restriction and cost. In 6GR, we still prefer 1Tx as baseline for sub-6GHz. Higher 2Tx can be used for U6G and FR3. For the number of Rx, 2Rx, 4Rx, 6Rx can be supported for handheld in NR. 6Rx was recently supported with maximum 4 DL MIMO layer for handheld UE. In 6GR, we prefer 2Rx for Sub-1GHz, 4Rx for 1GHz~6425MHz (2Rx Optional), 4Rx for U6G and FR3 (6Rx Optional).

Based on above discussion, 6G smartphone baseline capability is listed in the following table

Proposal 17: For device type A (smartphone), the following table can be considered as starting point of RF capability:

| Frequency Range | < 1GHz | 1 GHz ~ 6425 MHz | 6425~7125MHz | [FR3] |
| --- | --- | --- | --- | --- |
| Default Power class | PC2 | PC2 | PC2 | PC2 |
| Minimum Channel bandwidth | 5MHz | 5MHz | 20MHz | [20MHz] |
| Maximum Channel bandwidth | 100MHz | 100MHz | 200MHz | 200MHz |
| Numerology | 15kHz for FDD; 30kHz for TDD; | 15kHz for FDD; 30kHz for TDD; | 30/60 kHz can be studied; | 30/60/120 kHz can be studied |
| Maximum FFT size | 4K FFT | 4K FFT | 8K FFT | 8K FFT |
| #Rx, #Tx | 1Tx/2Rx | 1Tx/4Rx， optional 2Tx | 2Tx/4Rx, optional 6Rx | 2Tx/4Rx，optional 6Rx |

Conclusion

This contribution discusses system parameters for 6GR. The following observations and proposals are made:

Waveform

Observation 1: Waveform with high PAPR would decrease PA efficiency and further reduce maximum output power, which is a key requirement in RAN4.

Proposal 1: The basic waveform is decided by RAN1, but the PAPR and PA efficiency related aspect can be discussed and decided in RAN4.

Observation 2: Transparent spectrum shaping schemes to reduce PAPR such as FDSS have been discussed extensively in 5G R15 and R18 stage.

Observation 3: CFR-SE demonstrates superior performance in terms of PAPR compared to FDSS, FDSS-SE, CFR and TR.

Proposal 2: Study transparent and non-transparent techniques to further reduce PAPR, including CFR-SE. RAN 4 could start the evaluation of affected RF requirements, such as EVM, ACLR, MPR and applicable requirements in the extended RBs if needed. Other related spec impact needs further identification.

Observation 4: Net gain is adopted as an efficient metric of evaluating the comprehensive performance of a low-PAPR waveform in R18.

Proposal 3: Net gain (defined as MPR gain minus SNR loss at a BLER of 0.1) could be considered as the starting point of the metric to evaluate the comprehensive performance of low-PAPR waveform scheme in 6G.

Proposal 4: Recommend to align a widely accepted PA model for MPR simulation.

Modulation

Observation 5: In RAN1, some discussions regarding higher modulation order are currently under discussion, including 1024 QAM for UL and 4096QAM for DL.

Proposal 5: If the study of higher modulation orders beyond 5G is confirmed by RAN1, the feasibility validation from RAN4 perspective is necessary, including UE/BS Tx EVM.

CBW, FFT, numerology

Proposal 6: For 6GR FR1, it is proposed that minimum channel bandwidths 5MHz and maximum channel bandwidth 200MHz can be supported.

Proposal 7: For operators those has only 3MHz spectrum resources, RAN1 may need to specify a mechanism to support the operation of 3MHz spectrum usage.

Proposal 8: For a specific frequency range/band, it is proposed to support only one numerology.

Proposal 9:

For Sub-6GHz, maximum FFT size 4096 is suggested for maximum 100MHz with 30kHz SCS;

For FR1 U6G and FR3, maximum FFT size 8192 is suggested for the expected maximum 200MHz with 30kHz.

Channel raster, synchronization signal and raster, spectrum utilization

Proposal 10: It is suggested to only adopt SCS based channel raster for 6GR, additional frequency raster shift can be considered to align with 5G channel raster.

Proposal 11: To discuss whether the fundamental sync raster design principle can still be applied to 6GR:

Sync Raster interval =Min BWCHANNEL-BWSSB+ ΔFCH,Raster

Proposal 12: For existing channel bandwidths 0-100 MHz, reusing current 5G spectrum utilization for 6GR can be a starting point. For newly introduced channel bandwidth, e.g. 200MHz, spectrum utilization evaluation is needed, considering the RF requirements (SEM/EVM/ACLR) for U6G and FR3.

Irregular channel bandwidth

Proposal 13: RAN4 need discuss the granularity for the CBW in 6G

- RAN4 should limit the number of mandatory CBW in the spec

- For CBW≤100MHz, CBW specified in NR can be the starting point

- For CBW > 100MHz, 50 or 100MHz as granularity can be considered

  - Other CBW can be supported by single cell multi carrier or intra-band contiguous CA

Observation 6: The performance gain of irregular CBW at UE side is lack of justification.

Observation 7: If the irregular CBW is only supported in DL, the RF requirements can be easier defined based on closed nominal CBW.

Proposal 14: Study the solution to utilize the irregular channel bandwidth which is transparent to UE design in 6G.

- Overlapping UE CBW and Larger CBW in TR38.844 can be the starting point.

- Check whether single cell multi-carrier operation can help on utilizing irregular channel bandwidth

- FFS whether only enable irregular CBW in DL

Device types

Observation 8: The lesson from LTE and NR is that to classify UE type to a certain extent (or high-level category) is helpful for classifying the UE performance and feature but too fragmented device type is over-designed.

Proposal 15: Study on introducing limited number of device types (or Cats) and corresponding baseline BB/RF capability in RAN4. The following 4 types can be considered as starting point.

- Type A+:  FWA, CPE

- Type A:  Smart phone, immersive eMBB device

- Type B:  Reduced capability eMBB device, wearable/XR

- Type C:  IoT

Proposal 16: The maximum channel bandwidth for each device type can be different, and the following value can be considered as starting point for further discussion:

- Type A+:  FWA, CPE 200MHz，400MHz optional

- Type A:  Smart phone, immersive eMBB device 200MHz，400MHz optional

- Type B:  Reduced capability eMBB device, wearable/XR 100MHz

- Type C:  IoT 20 MHz

Proposal 17: For device type A (smartphone), the following table can be considered as starting point of RF capability:

| Frequency Range | < 1GHz | 1 GHz ~ 6425 MHz | 6425~7125MHz | [FR3] |
| --- | --- | --- | --- | --- |
| Default Power class | PC2 | PC2 | PC2 | PC2 |
| Minimum Channel bandwidth | 5MHz | 5MHz | 20MHz | 20MHz |
| Maximum Channel bandwidth | 100MHz | 100MHz | 200MHz | 200MHz |
| Numerology | 15kHz for FDD; 30kHz for TDD; | 15kHz for FDD; 30kHz for TDD; | 30/60 kHz can be studied; | 30/60/120 kHz can be studied |
| Maximum FFT size | 4K FFT | 4K FFT | 8K FFT | 8K FFT |
| #Rx, #Tx | 1Tx/2Rx | 1Tx/4Rx， optional 2Tx | 2Tx/4Rx, optional 6Rx | 2Tx/4Rx，optional 6Rx |

- References

[1] RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, RAN#108.

[2] RP-252912, Revised SID: Study on 6G Radio,	NTT DOCOMO, CMCC, AT&T, Vodafone.

[3] R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair, RAN4#116.

[4] R1-2505913, Waveforms for 6GR air interface, Apple, RAN1#122.

[5] R1-2506306, Discussion on Waveform, NTT DOCOMO, INC., RAN1#122.

[6] R1-2506020, Waveform for 6GR air interface, MediaTek Inc., RAN1#122.

[7] R1-2505416, Discussion on waveform for 6GR air interface, vivo, RAN1#122.

- Annex

Simulation Assumption

| Parameter | Value |
| --- | --- |
| Channel | PUSCH, 14 symbols |
| Carrier frequency and scenario | 7GHz (Urban) |
| Channel BW | 100MHz (for 7GHz band) |
| UE power class | 23dBm |
| SCS | 30 kHz |
| Channel model | TDL-C 300ns for FR1 Urban (7GHz) |
| UE speed | 3km/h |
| Waveform | DFT-s-OFDM |
| Modulation | π/2-BPSK、QPSK |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 8 |
| Number of DMRS symbols | 2 |
| Number of PUSCH data symbols | 12 |
| HARQ configuration | No retransmissions |
| Frequency hopping | Disabled |
| Number of PRBs | BLER evaluation: 40 PRBs |
| Extension factor [FDSS-SE] / sideband size [TR][CFR-SE] (α) | [1/4] as starting point |
| BLER | 10% |