3GPP TSG-RAN WG4 Meeting #116bis	R4-2513253

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussion on 6G Spectrum sharing

Agenda Item:	8.10

Document for:	Discussion

- Introduction

The RAN WG study on 6G radio has been approved by RAN#108 as SID in [1].The relevant objectives of RAN4 are copied below for reference.

- Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Radio interface protocol architecture and procedures for 6GR [RAN2, RAN1, RAN4, RAN3] - Radio signalling framework for UE capabilities, aiming at improvements and simplification compared to 5G NR [RAN2, RAN1, RAN4] - Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3] - 6GR core and performance requirements - General RF aspects [RAN4] - Intra-3GPP co-existence studies - Study RF related system parameters and requirements - BS RF requirement aspects including band [RAN4] - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles - UE RF requirement aspects including band and band combination [RAN4] - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination - Study UE RF capabilities considering different device types and implementations - RRM aspects for 6GR [RAN4, RAN1, RAN2] - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR - Study how to improve 6G requirement specification, including structure and drafting principles - Demodulation and performance aspects [RAN4] - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS - Aspects related to the testability [RAN4] - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity - Other aspects - Handling irregular channel bandwidths including the definition [RAN4, RAN1] - Definition of ‘frequency range(s)’[RAN4] - Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR: - 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3] - Study if any additional migration mechanism is necessary. [RAN] [RAN2, RAN1, RAN3, RAN4] NOTE: the start of this study objective (b) should be triggered by RAN plenary in time to guarantee proper completion of the WG study. - Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4] - AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

In the SID, one of the objectives is to study the migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR.

- Discussion

General:

At the time of introducing a new mobile communication system, a smooth and cost-effective transition from the legacy one to the new one is crucial.

MRSS, which has similar intention with DSS (Dynamic Spectrum Sharing), the technique used to facilitate the transition from 4G to 5G, is essential to ensure operators can introduce 6GR services gradually, cost effectively however rapidly without fully relying on spectrum re-farming, through allowing 5G NR and 6GR coexist and dynamically sharing the same frequency band.

DSS for 4G to 5G:

Some basic situation for DSS From 4G to 5G is as following:

The basic design principle of DSS is backward compatibility, that is introduction of 5G be transparent to LTE devices. That would require 5G NR be adaptive to LTE, particularly those “always-on” signal. This would means that NR transmission would have to fit into the time and spectral gaps in LTE signal structure.

More specifically, the MBSFN subframes is used in LTE carrier to create space for NR signals. The legacy LTE devices would omit those specific subframes for unicast data reception, and leave a clean “gap” for 5G NR to transmit essential singlas such as SSB and CSI-RS. In addition, in non-MBSFN LTE subframes, rate mating is used for 5G NR scheduler to “puncture” the always on LTE CRS signal, and also the 5G device would also ignore them during decoding.

There are also some other collision avoidances in the control and data planes and other schemes.

Considerations for MRSS:

The basic backward compatibility need for MRSS should be similar to DSS, so the need to avoid collision of the “always on” signals is similar. However, since the 5G is already using much less “always on” signal compared to LTE, the scheme may be simpler to achieve similar effects.

In addition, if basic thinking of 5G SSB and sync raster is reused for 6G, there may be a need to discuss the collision scenarios and discuss what kind of assumptions are reasonable. For instance, there may be corner condition, that MRSS would be used for a very narrow carrier which can not fully avoid SSB collisions between 5G/6G, and how to cope these situation may need further discussion. This kind of discussion may involve both RAN1 and RAN4 system parameters discussion.

Observation 1: The scenarios of MRSS is closely related to system parameters, and would need more discussion. E.g. whether (partially) overlapping SSBs are possible for 5G/6G.

The detail spectrum sharing techniques will be discussed in RAN1. Currently a few issues have been discussed at RAN1 122 meeting. For example on how to multiplexing 6G signal and 5G signals and channels, TDM, FDM and rate matching are proposed. Other aspects such as spectrum efficiency of the MRSS, overhead reduction etc.

It is also proposed that 5G NR initial access signals and reference signal can be utilized for 6G UE’s cell identification and measurement however this design will bring complexity on the UE side. More detailed RAN1 discussion on MRSS can be found at [2].

Based on the current situations, the impact to RF and the system parameters would need more RAN1 agreements.

Proposal 1: For MRSS, the impact on system parameters and RF will be clearer when more RAN1 agreements are available.

Based on all existing discussions from RAN4 RRM perspective, the impact on the RRM will only be efficiently analysed when more detail of RAN1 design is available. From RRM measurement point of view, reusing of legacy NR signal/channels for 6G UE’s measurement purpose is not preferred.

Proposal 2: For MRSS, the impact on the RRM will only be efficiently analysed when more detail of RAN1 design is available.

Proposal 3: For MRSS, from RRM measurement point of view, reusing of legacy NR signal/channels for 6G UE’s measurement purpose is not preferred.

- Conclusion

In this contribution, the initial study regarding 6G spectrum sharing is provided and the following proposals are available:

Observation 1: The scenarios of MRSS is closely related to system parameters, and would need more discussion. E.g. whether (partially) overlapping SSBs are possible for 5G/6G.

Proposal 1: For MRSS, the impact on system parameters and RF will be clearer when more RAN1 agreements are available.

Proposal 2: For MRSS, the impact on the RRM will only be efficiently analysed when more detail of RAN1 design is available.

Proposal 3: For MRSS, from RRM measurement point of view, reusing of legacy NR signal/channels for 6G UE’s measurement purpose is not preferred.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

- RAN1 122 meeting report