3GPP TSG RAN WG4 Meeting #116bis	R4-2513271

Prague, CZ, 13th – 17th, Oct. 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Views on 6G spectrum aspect

# Agenda item:	8.6

# Document for:	Approval

# 1 Introduction

The new SID[1] was approved in RAN #108 meeting, and the work for RAN4 will be started in RAN4 #116bis meeting.

In this contribution, we give some initial views on the 6GR spectrum aspect.

# 2	Discussion

## 2.1	General

For 6GR, it should not only consider the spectrum in existing 5G FRs(low/medium/high) for refarming, but also consider the possible new spectrum, as stated in the approved 6G SID [1], where the SID study will address frequency ranges up to 52.6GHz, including at least the full range of FR1 (up to 7.125GHz), the range between FR1 and FR2-1 (including around 7GHz), and FR2-1 (24.25 GHz-52.6GHz)., as shown in Figure 2.1-1.

Figure 2.1-1.Frequency range for 6G according to the SID.

For the spectrum in the frequency range between 7.125GHz to 24.25GHz, some proposals/recommendations for IMT-2030/6G spectrum can be found in the World Radio communication Conference 2023 (WRC-23) study, including:

- 7.125-8.4 GHz

- 10.0-10.5 GHz

- 14.8-15.35 GHz

For the spectrum in 10.0~10.5GHz range, it was already confirmed in WRC-23 that the spectrum in 10~10.5 GHz has been identified for IMT usage in region 2, but for the other two ranges, there is no decision yet. In addition to the above three spectrum ranges, another  spectrum range of 4.4-4.8 GHz (in some regions) was also proposed, although this spectrum falls into the existing FR1 range.

- Observation 1: There are some proposals for the spectrum in WRC-23 conference, i.e. 4.4-4.8GHz, 7.125-8.4 GHz and 14.8-15.35 GHz. However the final decisions for the IMT-2030/6G spectrum will be made in the WRC-27 conference.

## 2.2	Band Definition

Band definition is one critical parameter to regulate the official paired or unpaired spectrum which could be used by industry to provide the wireless communication system. From the RF implementation perspective, band definition would have the direct impacts on the frond-end design as shown in Figure 2.2-1 and Figure 2.2-2 (e.g. Duplex for FDD bands and filter design for TDD bands).

For 6G system, it could be expected that lots of 6G spectrum will be refarmed from the legacy RAT. In other words, the 6GR band definition should also consider the backward compatibility design to enable the support the legacy RAT with the same RF front-end or reusing the existing commercially available front-end duplex/filter on the mature market to enable 6G service, therefore we propose to use the legacy NR FDD/TDD band definition method as basis for 6G design to guarantee the backward compatibility at least from RF front-end perspective.

Figure 2.2-1. Illustration of example RF front-end of FR1 low bands (lower than 1GHz)

Figure 2.2-2. Illustration of example RF front-end of FR1 middle bands (around 2GHz)

Proposal 1: For 6GR band definition, it is proposed to use the legacy NR FDD/TDD band definition as basis for 6GR RF requirement definition to guarantee the backward compatibility from RF front-end perspective.

In addition, there were some initial discussions or concerns on the implementation complexity of RF front-end due to the massive band definition originated from the different regulators/regions/countries, this has already made lots of implementation burdens from UE side since UE needs to roam globally without any barrier to get the service from the regional operators by the mutual contract from the operators, and the UE would be also sold to different regions/countries with the unified design to simplify the implementation cost from regions to regions. However it could be easily found that band definition from different regulators are not well aligned or coordinated unfortunately in past 20 years due to the independent regulator’s decisions. If in the 6G phase, 3GPP want to redefine the band definition e.g. by grouping any DL/UL spectrum come from the unified band definition to serve the different regions, as shown in Figure 2.2-3 and Figure 2.2-4, it might be an impossible mission from our understanding especially considering the UL-DL spectrum from different regions is flipped.

In the current implementation, we also want to highlight that if bands are adjacent to each other or partially overlapped and with DL/UL direction are aligned, then the simplified duplex or filter design is possible and achievable on the commercial UE to save the implementation cost/complexity. In other words, we could see that it is better to not redefine the band definition (e.g. group the proximity spectrum or overlapping spectrum into single band) in 6G phase and keep it up to the implementation optimization if possible from RF front-end design perspective.

Figure 2.2-3. iIllustration of 3GPP band definition below 1GHz

Figure 2.2-4. Illustration of 3GPP band definition from 1GHz to 2GHz

Proposal 2: For 6GR band definition, it is proposed not to define the new band group definition to replace the legacy band definition to cause the discrepancy between 3GPP definition and regional regulator’s definition.

Therefore, for 6GR band definition, we think some experiences of 5G NR can be referred, which are summarized in the following:

1. LTE refarming bands

2. New NR bands

For LTE refarming bands, LTE band numbers are reused for 5G NR, which is also reused the same mechanism as UTRA and LTE where the same number was used for the same frequency range, while written in a different way.

For NR new bands, new spectrum ranges were introduced and defined as 5G NR bands for FR1 and FR2, where the band numbering 1 to 256 is reserved for NR FR1 bands and 257 to 512 are reserved for NR FR2 bands with increasing band number in general. For satellite bands, decreasing band number approach is used.

As of today, except for the satellite bands, there are only 110 band numbers are used in FR1, and only 7 bands (n257~n263) are defined in FR2, which means there are lots of unused band number in each frequency range even counting all of the satellite bands.

Note that all the NR new bands (including satellite bands) obey the rules called “first come first served”, and in order to distinguish different RATs, all of the NR band numbers are written with a prefix “n”.

Observation 2: 5G NR band includes LTE refarming bands and New NR bands, prefix “n” is used for the band numbers to distinguish different RATs.

Observation 3: In 5G NR, band numbering 1 to 256 is reserved for FR1 bands and 257 to 512 are reserved for FR2 bands. There are lots of unused band number in each frequency range.

Similarly, at least both 5G NR refarming bands (including satellite bands) and new 6GR bands should be considered for 6GR bands:

- For the refarming band, the band numbers of 6GR are same with the band numbers of 5G NR;

- For new 6GR bands, it should be introduced on “first come first served” basis.

Furthermore, all of the 6GR band numbers should be written with a new prefix, such as ‘r’, ‘s’, ‘z’, in order to distinguish different RATs.

Proposal 3: 6GR bands, both 5G NR refarming bands (including satellite bands) and new 6GR bands should be considered. The band numbers of refarming bands in 6GR should be same with the band numbers of 5G NR, and new 6GR bands should be introduced on “first come first served” basis.

Proposal 4: All of the 6GR band numbers should be written with a new prefix.

## 2.3	Band combination definition and simplification

In 5G NR, several types of band combinations including NR CA, NR DC, SUL, ENDC and NEDC are introduced. In the past decade, thousands of band combinations have been introduced in the specification but unfortunately, very few of (less than 10%) have been implemented. Although there were lots of guidance/rules generated to reduce and simply the work, RAN4 still spent a lot of time to discuss/analysis the band combination specific requirements and also the other associated works like requesting new combination, maintenance CR, status report updates and etc.

For 6GR, how to simplify the band combination work has become a popular topic and companies are keen on this simplification work, and it seems become a urgent task.

One of a concept is to use ‘band group’, which is to divide all of bands into some (e.g. 5) groups by using frequency metric. For example, group 1 is to group all sub-1 GHz bands, and group 2 is to group all bands belong to 1400-2200MHz range, and etc. By using the ‘band group’ conception, the bands between two different band groups can support inter-band CA band combinations by default. In this way, there is no need to request inter-band CA band combinations as did in 5GR. In addition to that, it might also reflect actual implementation due to some RF proponents like PA/LNA are common to several close-frequency bands, and also it can save Rx chain which can make UE support more bands. However, for the bands in the same band group, no CA is supported, instead of using switching approach among bands, which means only one band is allowed to operate at a time.

Observation 4. In ‘band group’ conception, the bands between two different band groups can support inter-band CA band combinations by default, while for the bands in the same groups, CA is not supported and only one band is allowed to operate at a time.

We have to admit that introduction of ‘band group’ concept for band combination can largely reduce the band combination number in the specification. However, there are some cons where some problems are needed to considered.

First, band combination is not supported for the bands in the same band groups which would not reflect operator’s demand who have such bands but want to support band combination. For example, n1 and n3 will not be allowed to be used as inter-band CA due to they are from the same band group, while CA_n1-n3 is supported in TS38.101-1. if operator would like to implement CA for the bands in the same ‘band group’, then traditional approach would be needed, but it would cause mix approaches are used in 6GR.

Second, for HPUE inter-band combinations, due to neither all UL constitute single band nor all inter-band UL CA supports HPUE. It seems the ‘band group’ concept will not well reflect the power class for UL configuration. Furthermore, no all band support UL MIMO/TxD, it seems the ‘band group’ concept will not reflect 3Tx HPUE inter-band combinations either.

Third, for high order band combination, for example 3DL inter-band CA nX-nY-nZ, it would be difficult to define the IMD MSD requirements for nZ caused by UL nX-nY since we have no idea what are the exact bands nX, nY and nZ as they are the band in the ‘band group’.

Observation 5. From band combination aspect, although ‘band group’ concept can largely reduce the band combination number, there are cons e.g. it may not reflect operator’s demand in the same band group, HPUE and high order band combination and etc..

With the experiences in 5G, there are several factors in the following which cause heavy basket WID workload:

1. Band combination specific requirements (i.e. ΔTIB/ΔRIB, MSD)

2. Huge band configuration number with several BCS.

3. Requesting and Status report updates.

4. Maintenance (Errors checking)

Band combination specific requirements

For band combination specific requirements (i.e. ΔTIB/ΔRIB, MSD), they are usually the extensive discussed not only in flagging period, but also in online/offline meeting period, which occupy a significant portion of the time before, during and post meeting.

In the PRD[2], there are some guidance captured in clause 5.4.2 and 5.4.3 to derive the ΔTIB/ΔRIB requirements, we think it can be used as baseline for the band combination ΔTIB/ΔRIB requirements derivation in 6GR, and also we think it is feasible to remove the big tables for ΔTIB and ΔRIB requirements in the specification, instead of using equation-based/LUT-like approach in the specification. In this way, there is no need to discuss and study the ΔTIB/ΔRIB requirements for each band combination. Noted that ΔTIB/ΔRIB requirements are defined as power class agnostic, the same ΔTIB/ΔRIB requirements as PC3 are applied for HPUE band combination.

For MSD requirements, LUT MSD approach were agreed to be used for HPUE band combination in Rel-19. Thanks to LUT MSD approach, the calculation/discussion on MSD values are not needed anymore and TP to TR is not needed, which can largely alleviate HPUE basket work. Although for PC3 band combination, there are no such LUT approach, we see it is also feasible to come up with some approaches (LUT-like) for PC3 MSD derivation, instead of discussing and studying the ΔTIB/ΔRIB requirements for each band combination.

In addition to that, the PC3 MSD derivation were largely based on calculation with common RF parameters assumption. However, some of these RF parameters such PCB isolation (=65dB) and antenna isolation(=10dB) do not reflect practical implementation, which cause high MSD values (even more than 35dB) in the specification. We don’t think high MSD make sense, and that’s why RAN4 discussed lower-MSD topic in Rel-18 however unfortunately it is not implemented in the filed. The justification for lower-MSD topic showed that the practical RF parameters will far improve the MSD value which make the actual MSD value is far better than the value define in the specification.

Therefore, we propose to improve the MSD values for the band combination in 6GR by using practical RF parameters and simplify the MSD framework with LUT-like approaches.

Proposal 5. To improve the PC3 band specific requirements (i.e. MSD, ΔTIB/ΔRIB).

- For ΔTIB/ΔRIB, to use equation-based/LUT-like approach based on PRD guidance.

- For MSD, to use LUT-like approach and use practical RF parameters to derive the MSD value.

By simplification of band combination specific requirements, we think the band combination requirements workload can be largely reduced as well.

Huge band configuration number with several BCS

As mentioned as above, very few of band configurations in thousands of configuration in the specification are implemented in the filed, which make a very painful and tough basket work. For 6GR, we must learn the lessons from 5G NR.

Regarding band configuration requesting, RAN4 should focus on the band configurations for real deployment. In this way, we think the band configuration number can be largely reduced. Thus RAN4 should discuss the criteria on how to judge the BCs are for real deployment.

Proposal 6: In 6GR, RAN4 should focus on the band configurations for real deployment and RAN4 should discuss the criteria on how to judge it.

In 5G NR, BCS 0, 1, 2, 3, 4 and 5 are supported, in which different supported channel bandwidth in each BCS and all possible channel bandwidths in each band are supported in BCS 4 and 5.

The intention to introduce BCS 4 and 5 is to alleviate the new band configuration requesting in the case of operator have urgent demands for some channel bandwidths which are not supported in the existing configuration, but usually it have at least 3 months delay due to the basket WID procedure. However, although BCS4 and 5 is used, the traditional BCSs are still allowed which relies on proponent. In the end, there are numbers of band configurations have more than one BCS, for example CA_n1A-n78A in the following.

Numbers of BCS cause high workload and occupy a lot of specification pages to include all the BCS for configurations. In 6GR, we think it needs to discuss how to improve this situation. To keep only one BCS may be a promising solution, but it seems there is no different with no BCS.

Actually, for inter-band ENDC configurations, there is no BCS applied and all the channel bandwidths in either LTE band or NR band are supported by default. From this perspective, one follow question would be why not the inter-band ENDC approach cannot be applied for inter-band NR CA.

Proposal 7. Discuss the necessity of BCS for inter-band NR CA in 6GR.

Requesting and Status report updates

In 5G, proponents sent email attached with Excel file to the reflector for their new configuration request before the meeting, in which at least min. 3 supported companies is mandatory, and proponent/rapporteur do their status report updates for the completed/ongoing band configurations in the post meeting. Both are time-consuming works.

Due to RAN4 will use database approach for the Rel-20 band configuration work, and we are wondering if it is feasible to use database approach for the configuration requesting and status reports updates work, to replace the traditional way.

Proposal 8. Discuss if it is feasible to use database approach for the configuration requesting and status reports updates work.

Maintenance (Errors checking)

This is also a time-consuming but usually non-technical work, such as formatting issue, missing fallback, typos and so on. In our understanding, due to RAN4 will use database approach to include band combination from Rel-20 onwards, we think the band configuration tables can be removed from the specifications, instead of storing the band configuration tables in the database via JSON file inputs. In this case, we are also wondering if it is feasible for the band configuration errors self-checking and RAN4 should come up with some methods to reduce this time-consuming check work.

Proposal 9. Discuss if it is feasible for the band configuration errors self-checking in the database/JSON file.

## 2.4 Definition of frequency ranges

In last RAN #109 meeting, there are three options for how to categorize the frequency range:

- Option 1: extend FR1 to 8.4 GHz and define a separate mid-high band (8.4–24.25 GHz) with its own single numerology. - Option 2: extend FR1 to 8.4 GHz and merge 8.4–24.25 GHz into FR2, creating a unified FR2 family. - Option 3: define separate FR for 7.125 – 24.25 GHz.

As we mentioned in section 2.1, the final decision for the IMT-2030/6G spectrum will be made in WRC-27 conference although there are some proposals on the spectrum between 7.125 – 24.25 GHz in WRC-23, which means the spectrum between between 7.125 – 24.25 GHz have not yet clearly been specified so far.

Here are our initial understandings on the implications on the frequency range definition in 3GPP:

- Frequency range definition in 3GPP will provide the general guidance or indication on the RF component design in the industry. For example, lots of commercially available RF components are designed with targeted upper frequency as 6GHz without the optimization/application for higher frequency range which is not officially confirmed by 3GPP/ITU-R to save the implementation cost. If 3GPP starts to officially extend the FR definition without the IMT confirmation, this might cause some confusion from both the RF component’s design and also relevant discussion in ITU-R in the industry.

- Frequency range definition is also somehow relevant with how the RF requirement is defined. For example, from UE RF perspective, RF requirement for FR1 UE is based on the assumption of the conductive RF connectors accessible for the conformance testing; however for RF requirement for FR2 UE is based on the integrated antenna and RF RFIC within AIP module. For the potential new candidate spectrum between 7-24GHz, especially for 15GHz, the UE antenna implementation (e.g. omini antenna or phase antenna array) is not well discussed and confirmed during the Rel-19 WP5D LS, we also think that it is premature to have the discussions on the frequency range extension.

- Lastly, frequency range definition will also impact on other WG’s specification drafting. Back to the Rel-17 for NR-U standardization from 52.6-71GHz, the FR2-1 and FR2-2 definition is also confirmed together with other WGs in RAN-P meeting aim to minimize the spec impacts across the WGs.

In last RAN plenary meeting, it was agreed that keep using “around 7GHz” instead of using detailed frequency range definition in RAN1’s and RAN4’s study. From this perspective, it would be premature to discuss the frequency range with detailed frequencies.

In short, we propose to postpone the discussion on FR extension and focus on the relevant technical discussions in 6G SI phase.

Proposal 10: Postpone the discussion on FR extension and focus on the relevant technical discussions in 6G SI phase.

# 3 Conclusion

In this contribution, we give some initial discussion on 6GR spectrum aspect, the conclusions are summarized below:

- General

- Observation 1: There are some proposals for the spectrum  in WRC-23 conference, i.e. 4.4-4.8GHz, 7.125-8.4 GHz and 14.8-15.35 GHz. However the final decisions for the IMT-2030/6G spectrum will be made in the WRC-27 conference.

- Band Definition

Proposal 1: For 6GR band definition, it is proposed to use the legacy NR FDD/TDD band definition as basis for 6GR RF requirement definition to guarantee the backward compatibility from RF front-end perspective.

Proposal 2: For 6GR band definition, it is proposed not to define the new band group definition to replace the legacy band definition to cause the discrepancy between 3GPP definition and regional regulator’s definition.

Observation 2: 5G NR band includes LTE refarming bands and New NR bands, prefix “n” is used for the band numbers to distinguish different RATs.

Observation 3: In 5G NR, band numbering 1 to 256 is reserved for FR1 bands and 257 to 512 are reserved for FR2 bands. There are lots of unused band number in each frequency range.

Proposal 3: 6GR bands, both 5G NR refarming bands (including satellite bands) and new 6GR bands should be considered. The band numbers of refarming bands in 6GR should be same with the band numbers of 5G NR, and new 6GR bands should be introduced on “first come first served” basis.

Proposal 4: All of the 6GR band numbers should be written with a new prefix.

Band combination definition and simplification

Observation 4. In ‘band group’ conception, the bands between two different band groups can support inter-band CA band combinations by default, while for the bands in the same groups, CA is not supported and only one band is allowed to operate at a time.

Observation 5. From band combination aspect, although ‘band group’ concept can largely reduce the band combination number, there are cons e.g. it may not reflect operator’s demand in the same band group, HPUE and high order band combination and etc..

Proposal 5. To improve the PC3 band specific requirements (i.e. MSD, ΔTIB/ΔRIB).

- For ΔTIB/ΔRIB, to use equation-based/LUT-like approach based on PRD guidance.

- For MSD, to use LUT-like approach and use practical RF parameters to derive the MSD value.

Proposal 6: In 6GR, RAN4 should focus on the band configurations for real deployment and RAN4 should discuss the criteria on how to judge it.

Proposal 7. Discuss the necessity of BCS for inter-band NR CA in 6GR.

Proposal 8. Discuss if it is feasible to use database approach for the configuration requesting and status reports updates work.

Proposal 9. Discuss if it is feasible for the band configuration errors self-checking in the database/JSON file.

Definition of frequency ranges

Proposal 10: Postpone the discussion on FR extension and focus on the relevant technical discussions in 6G SI phase.

# 4	Reference

[1]RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

[2] Permanent Reference Document (PRD); Rules, guidelines and ways of working for introduction of band combinations in NR and LTE