# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513272

# Prague, CZ, 13-17 Oct, 2025

Agenda Item:	8.3

Source: 	Xiaomi

Title: 	Overview for 6G system parameter design

Document for:	Discussion

- Introduction

According to the endorsed 6G work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. System parameter is one of major agenda with following candidate scope:

In this contribution, our preliminary view on 6G system parameter design is provided on following areas:

- Waveform, modulation

- CHBW/Numerology/FFT/Spectrum utlization

- Channel raster/Sync raster

- Irregular channel bandwidth

- Device type and RF/BB capabilities

- Discussion

  - Overall scope

Following design principle was considered in NR on system parameter design:

- 4K FFT size with maximum 3300 sub-carriers as baseline

- Configurable numerologies and multiple CHBW sets as per numerology basis introduced in NR

- SSB design and sync raster is pending on Minimum CHBW assumption

- Maximum CHBW is pending on SCS, FFT size, UE processing capacity, and available contiguous spectrum

- NR majorly focus on eMBB UE in day 1 design

Pain points from 5G:

- Flexible numerologies and multiple CHBW sets as per band basis increase gNB, UE design and conformance test complexity

- 3MHz CHBW was introduced in later stage with PBCH design refinement (NBC issue)

- Diverse device types were introduced in late stage with 1/2 Rx exception (XR, Redcap) and RAN4 requirements device dependency (FWA, CPE, Handheld, non-handheld)

Proposal 1: RAN4 initial study on system parameters focus on following direction:

- Unify candidate numerologies (data and SSB) as sub-frequency range basis

- Decide minimum CHBW and maximum CHBW compatible with diverse device types

- Simplify sync raster and channel raster design compatible with variable CHBW and device types

- Evaluate solutions to support irregular bandwidths with minimized complexity from both gNB and UE side

- Harmonize design for diverse device types with basic RF/BB capabilities

  - Waveform /Modulation order

6G SID: - Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

Basic waveform and modulation design majorly belong to RAN1 centric items. According 6G SID, 5G NR waveform and modulation is considered as baseline for 6GR. Following agreements reached in August RAN1 meeting:

Observation 2-1: RAN1 agreed to reuse NR waveforms and modulation orders as baseline, with necessary enhancement which pending on further discuss in RAN1.

The major work expected from RAN4 on modulation order/waveform is feasibility study with associated RF requirements impact e.g., MPR/Power boosting, Tx EVM, and applicable scenarios (DL/UL, and frequency ranges). Without sufficient input from RAN1 on basic waveform and modulation order structure, it’s hard for RAN4 to have efficient discussion.

Proposal 2-1: Postpone waveform/modulation order discussion until sufficient progress reached in RAN1 e.g., start from Q2’ 26.

  - Numerology/FFT/CHBW/Spectrum utilization

    - Numerology

According to 6G SID, minimum channel bandwidth and maximum channel bandwidth and numerologies required joint effort from both RAN1 and RAN4. Also, from 6G SID, 6G aims to harmonize numerology per sub-frequency range.

6G SID - Physical Layer structure for 6GR, - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

Based on initial discussion in RAN1 and further confirmed in RAN#109 with following agreements: at least 15kHz is supported for FDD and 30kHz SCS is supported for TDD for below 6GHz bands and 30kHz/60KHz as candidate for around 7GHz.

One of major pain point in NR, NR introduced flexible numerologies configuration and almost each bands support all the numerologies i.e., 15kHz, 30kHz and 60kHz with corresponding channel bandwidth sets in FR1.  However, from real deployment, for each band only single numerology was deployed. These flexible numerologies with multiple options in each bands bring unnecessary complexity to both gNB and UE without any benefits/usage on commercialization.

Another issue, in NR mixed numerologies between SSB and data channel were supported in the same carrier and this bring UE implementation complexity and pending on UE capability, UE may not support mixed numerologies which will bring interruption in data channel reception when performing intra-cell SSB measurement/detection.

In existing NR deployed spectrum/bands, it’s better to align the numerology between 5G and 6GR to simplify the scheduling complexity considering MRSS.

The choice of numerologies selection needs to consider several factors including phase noise robust, maximum CHBW, Cell size/CP length.  On around 7GHz, based on previous release evaluation, we didn’t see any issue on phase noise impact with 30kHz SCS.  Another point, we aim to achieve same coverage as C bands (3.5 GHz) for around 7GHz as 6GR design target.

6G SID The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

With above consideration, it’s better to align numerology between around 7GHz range and C-bands. With 8K FFT operation, single carrier can achieve 200MHz CHBW for 30kHz which also meet the demand on wider channel bandwidth.

Proposal 3-1: Limit to single numerology as per sub-frequency range/per band basis at least for DL/UL control channel and data channel except PRACH

Proposal 3-2: Same numerology is applied for SSB (initial cell access) and DL channel in same band

Proposal 3-3: On 5G migration spectrum/bands, harmonize the numerology between 5G and 6G by taking existing NR commercialization deployment choice into account.

Proposal 3-4: Following numerology choice is proposed on different sub-frequency ranges

| Frequency range | SCS for data/control channel except PRACH | SCS for PBCH |
| --- | --- | --- |
| Below 3GHz (FDD bands) | 15kHz | 15kHz |
| Below 3GHz (TDD bands) | 30kHz | 30kHz |
| 3GHz ~ 7.125GHz | 30kHz | 30kHz |
| 7.125GHz ~8.4GHz | 30kHz | 30kHz |
| 24.25 GHz -52GHz | 120kHz | 120kHz |

    - CHBW/Spectrum utilization

The detailed CHBW sets per numerology, corresponding spectrum utilization per combination of {CHBW, SCS} and supported CHBW sets per band belong to RAN4 scope.

Minimum CHBW is pending on following factors:

- “Minimum available spectrum from 6GR deployment” (RAN4 scope)

- Overall system performance impact to ensure comparable performance as NR (RAN1 scope on common channel design)

- Lowest device capability that 6GR design can be applied with smallest maximum CHBW (RAN1/RAN4 joint effort)

From available spectrum perspective, 5MHz CHBW was specified in NR as minimum CHBW across all the bands. And in later stage, RAN4 also identified 3MHz for special RMR system operation, and in some specific bands operators may hold 3MHz spectrum under below 1GHz bands for example band n28, n85, n26. On some NTN deployment scenario, operators may also hold relatively small block contiguous spectrum.

In terms, maximum CHBW from system perspective, we can assume 8K FFT as baseline for 6G. Also, the potential   spectrum that operators on 6GR deployment also needs to be considered.

Proposal 3-5: Considering both 3MHz and 5MHz as minimum CHBW pending on operating bands

Proposal 3-6: Take 8K FFT as baseline assumption

Proposal 3-7: Specify the minimum CHBW and maximum CHBW based on numerologies

| SCS | Min CHBW | Max CHBW |
| --- | --- | --- |
| 15kHz | 5MHz 3MHz (below 1GHz bands) | 100MHz |
| 30kHz | 10MHz | 200MHz |
| 120kHz | 50MHz | 800MHz |

Regarding spectrum utilization, NR specified transmission bandwidth configuration and guard-band for each BW to decide SU as per {CHBW, SCS} basis.

Figure 5.3.1-1: Definition of the channel bandwidth and the maximum transmission bandwidth configuration for one NR channel

The spectrum utilization is pending on several factors including waveform assumption, modulation orders and corresponding RF core requirements including emission requirements, Tx EVM and PA model.  There is trade-off between SU and RF performance (MPR, Tx EVM).

Proposal 3-8: RAN4 further evaluate spectrum utilization with trade-off between improved SU, RF performance and UE/gNB complexity with detailed parameter assumption

- RF non-linearity assumption: PA model, I/Q imbalance, PN

- TX RF core performance assumption: OBE (emission and ACLR), Tx EVM

- Waveform and modulation orders

- Spectrum sharing technologies

  - Channel raster/sync raster

Channel raster defines RF reference frequencies to identify channel position for both DL and UL (which always located in the sub-carrier which closet to channel centre with half SCS offset to right side). A global frequency raster is defined between 0-100GHz as NR-ARFCN.

And band specific band channel raster is also defined as multiple times of basic global channel raster granularity.  Considering 5G co-existence with 4G, 100kHz channel raster specified for LTE refarming bands. This brought some limitation on spectrum usage especially on less than 10MHz CHBW and enhanced channel raster feature was introduced in later stage of NR.

Considering network deployment flexibility, and spectrum usage flexibility, we didn’t see the motivation to restrict channel raster for each band. Especially, for NR, the RB allocation and SSB placement was designed based on SSB placement not related to channel raster.

Proposal 4-1: Support 5kHz channel raster in day 1 for below 3GHz

Proposal 4-2: Remove per band channel raster concept with following value per sub-frequency range basis

| Frequency range | Channel raster |
| --- | --- |
| <3GHz | 5kHz |
| 3GHz ~ 24.25kHz | 30kHz |
| 24.25GHz ~ 52GHz | 120kHz |

On sync raster, generally sync raster location was decided based on following equation with principle that 1) minimize candidate SS raster locations for initial cell search complexity; 2) allow each BW, at least one SS entries can be fully contained:

SS raster > = CHBW- SSB BW + channel raster

A global sync raster is defined for all frequencies (GSCN) which can be mapped to the frequency location if SS block.

The Sync raster was used to indicate SS block location of initial cell search.  It’s work to simplify sync raster design by consideration of following principle: 1) facilitate UE initial cell search (complexity/initial search time/power consumption 2) flexibility of SSB placement for 6GR spectrum allocation

Those sync raster design is still pending on RAN1 initial cell search design on SSB and 6GR minimum CHBW definition.

Proposal 4-3: Postpone sync raster discussion until sufficient progress reached in RAN1 on 6GR initial cell search design e.g., no early Q2’ 2026

Proposal 4-4: Further evaluate to simplify sync raster to facilitate UE initial cell search (complexity/initial search time/power consumption vs flexibility for SSB placement) with potential area

- Flexible step size per sub-frequency range/per band

- Per band group-based default SSB periodicity

- SS raster design for MRSS

  - Irregular bandwidth

In NR, under Rel-17/18 RAN4 established study item to study potential solutions on irregular channel bandwidths. The initial demand from operators were collected as show in table below.

Irregular BW from TR 38.837

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 6, 7, 11, 12 MHz |
| n12, n85 | 6, 12 MHz |
| n26 | 7 MHz |
| n28 | 13 MHz |
| n29 | 6, 11 MHz |

During the study, RAN4 identified several solutions with pros and cons summarized in the TR. Overlapping CA/Overlapping CHBW was identified as candidate solution to resolve this issue. During the study, RAN4 also identified major limitation on SSB and channel placement under irregular BW was channel raster granularity. 900kHz minimum distance between carriers is required with 100kHz channel raster and 180kHz RB size. This brings spectrum utilisation issue.  Later, to allow flexibility of channel placement, RAN4 agreed to introduce enhance channel raster to resolve this issue.

Another limitation was SSB placement for less than 10MHz, the common part over overlapped carrier is not wide enough to cover 3.6MHz SSB and 4.32MHz corset#0. Then two separate SSB per carrier with time multiplexing for 6MHz/7MHz is required which bring resource waste and scheduling complexity. To address this, 6MHz/7Mhz dedicated channel bandwidth was introduced in later stage.

For 6GR, CHBW is still the key parameters and RAN4 shall care exam channel bandwidth sets to control overall UE design/test complexity.

Proposal 5-1: RAN4 shall careful exam channel bandwidth sets to balance UE design/test complexity and flexibility to fully usage operators’ spectrum

- RAN4 requirements and system parameter design are developed based on Channel bandwidths

- Granularity of CHBW sets need to be carefully considered

Proposal 5-2: Study potential solution to support irregular spectrum with following direction:

- Overlapping CA /Overlapping CHBW from network perspective

- Channel raster/sync raster and channel allocation mapping rule design to be compatible with flexible BW

  - Device type and RF/BB capabilities

In 5G design, the initial target was for eMBB UE which brought NBC issue when new device type with less Rx antenna capability e.g. XR, Redcap and Vehicular device types and some exception was required on these device types.

RAN4 also introduce several requirements and features which have form factor dependency e.g., some PCs only target for non-handheld UE, FWA or CPE , separate requirements are specified based on UE form factor and some feature only applicable for certain device types e.g., 6RX with 4 layer and 6 layer feature. RAN4 requirements for 6Rx capabilities can be summarized in below table:

According to 6G SID, and latest RAN-P agreements some key direction can be summarized in below:

- Harmonic system design to support diverse device types in the same cell

- Define a basic set of mandatory RF/BB capability per device type (e.g.,  CHBW, Modulation order, Number of Tx/Rx , duplex mode, PC and RRM mobility), with optional capabilities on top of it

- Dynamic UE capability can be considered e.g.  operating state for foldable device, operating mode of eMBB device (power saving mode, high performance mode)

  - Cooperation between NW and UE required for UE capability update e.g. BS mandatory feature set

- RAN4 shall be deeply involved in early stage on UE device type and RF/BB capabilities

Proposal 6-1: RAN4 needs to discuss how to handle and discriminate different device types/form factors (not necessary treated as device types/capabilities) from RAN4 requirements perspective

Proposal 6-2: RAN4 shall focus on mandatory RF/BB capabilities assumption for 6GR devices

- Which can be further categorized based device type/form factor

Proposal 6-3: A basic set of mandatory RF/BB capability shall include following candidate factors with optional capabilities

- Number of Tx/Rx

- Maximum CHBW

- Modulation oder

- Duplex mode

- Power class

- RRM mobility

Proposal 6-4: At least IoT device Type and MBB device type shall be considered in RAN4

- FFS whether need to further split sub-device type under each category e.g. low-end MBB, high-end MBB

- FFS whether need to consider dedicated device type for FWA

Proposal 6-5: For handheld UE, basic UE RF/BB capabilities can be further decided by per sub-frequency rang basis

Proposal 6-7: UE capability update can be further studied e.g.  operating state for foldable device, operating mode of eMBB device (power saving mode, high performance mode)

- Cooperation between NW and UE required for UE capability update e.g., conditional BS mandatory feature set

- Conclusion

In this contribution, our preliminary view on 6G system parameter design is provided on following areas:

- Waveform, modulation

- CHBW/Numerology/FFT/Spectrum utlization

- Channel raster/Sync raster

- Irregular channel bandwidth

- Device type and RF/BB capabilities

- Overall scope

Proposal 1: RAN4 initial study on system parameters focus on following direction:

- Unify candidate numerologies (data and SSB) as sub-frequency range basis

- Decide minimum CHBW and maximum CHBW compatible with diverse device types

- Simplify sync raster and channel raster design compatible with variable CHBW and device types

- Evaluate solutions to support irregular bandwidths with minimized complexity from both gNB and UE side

- Harmonize design for diverse device types with basic RF/BB capabilities

- Waveform /Modulation order

Observation 2-1: RAN1 agreed to reuse NR waveforms and modulation orders as baseline, with necessary enhancement which pending on further discuss in RAN1.

Proposal 2-1: Postpone waveform/modulation order discussion until sufficient progress reached in RAN1 e.g., start from Q2’ 26.

- Numerology/FFT/CHBW/Spectrum utilization

Proposal 3-1: Limit to single numerology as per sub-frequency range/per band basis at least for DL/UL control channel and data channel except PRACH

Proposal 3-2: Same numerology is applied for SSB (initial cell access) and DL channel in same band

Proposal 3-3: On 5G migration spectrum/bands, harmonize the numerology between 5G and 6G by taking existing NR commercialization deployment choice into account.

Proposal 3-4: Following numerology choice is proposed on different sub-frequency ranges

| Frequency range | SCS for data/control channel except PRACH | SCS for PBCH |
| --- | --- | --- |
| Below 3GHz (FDD bands) | 15kHz | 15kHz |
| Below 3GHz (TDD bands) | 30kHz | 30kHz |
| 3GHz ~ 7.125GHz | 30kHz | 30kHz |
| 7.125GHz ~8.4GHz | 30kHz | 30kHz |
| 24.25 GHz -52GHz | 120kHz | 120kHz |

Proposal 3-5: Considering both 3MHz and 5MHz as minimum CHBW pending on operating bands

Proposal 3-6: Take 8K FFT as baseline assumption

Proposal 3-7: Specify the minimum CHBW and maximum CHBW based on numerologies

| SCS | Min CHBW | Max CHBW |
| --- | --- | --- |
| 15kHz | 5MHz 3MHz (below 1GHz bands) | 100MHz |
| 30kHz | 10MHz | 200MHz |
| 120kHz | 50MHz | 800MHz |

Proposal 3-8: RAN4 further evaluate spectrum utilization with trade-off between improved SU, RF performance and UE/gNB complexity with detailed parameter assumption

- RF non-linearity assumption: PA model, I/Q imbalance, PN

- TX RF core performance assumption: OBE (emission and ACLR), Tx EVM

- Waveform and modulation orders

- Spectrum sharing technologies

- Channel raster/sync raster

Proposal 4-1: Support 5kHz channel raster in day 1 for below 3GHz

Proposal 4-2: Remove per band channel raster concept with following value per sub-frequency range basis

| Frequency range | Channel raster |
| --- | --- |
| <3GHz | 5kHz |
| 3GHz ~ 24.25kHz | 30kHz |
| 24.25GHz ~ 52GHz | 120kHz |

Proposal 4-3: Postpone sync raster discussion until sufficient progress reached in RAN1 on 6GR initial cell search design e.g., no early Q2’ 2026

Proposal 4-4: Further evaluate to simplify sync raster to facilitate UE initial cell search (complexity/initial search time/power consumption vs flexibility for SSB placement) with potential area

- Flexible step size per sub-frequency range/per band

- Per band group-based default SSB periodicity

- SS raster design for MRSS

- Irregular bandwidth

Proposal 5-1: RAN4 shall careful exam channel bandwidth sets to balance UE design/test complexity and flexibility to fully usage operators’ spectrum

- RAN4 requirements and system parameter design are developed based on Channel bandwidths

- Granularity of CHBW sets need to be carefully considered

Proposal 5-2: Study potential solution to support irregular spectrum with following direction:

- Overlapping CA /Overlapping CHBW from network perspective

- Channel raster/sync raster and channel allocation mapping rule design to be compatible with flexible BW

- Device type and RF/BB capabilities

Proposal 6-1: RAN4 needs to discuss how to handle and discriminate different device types/form factors (not necessary treated as device types/capabilities) from RAN4 requirements perspective

Proposal 6-2: RAN4 shall focus on mandatory RF/BB capabilities assumption for 6GR devices

- Which can be further categorized based device type/form factor

Proposal 6-3: A basic set of mandatory RF/BB capability shall include following candidate factors with optional capabilities on top of that

- Number of Tx/Rx

- Maximum CHBW

- Modulation oder

- Duplex mode

- Power class

- RRM mobility

Proposal 6-4: At least IoT device Type and MBB device type shall be considered in RAN4

- FFS whether need to further split sub-device type under each category e.g. low-end MBB, high-end MBB

- FFS whether need to consider dedicated device type for FWA

Proposal 6-5: For handheld UE, basic UE RF/BB capabilities can be further decided by per sub-frequency rang basis

Proposal 6-7: UE capability update can be further studied e.g.  operating state for foldable device, operating mode of eMBB device (power saving mode, high performance mode)

- Cooperation between NW and UE required for UE capability update e.g., conditional BS mandatory feature set

Annex A RAN1#122/RAN-P#109 agreements

- Modulation orders and waveforms

RAN1#122 R1-2506550 Feature Lead summary #1 on 6GR waveform Moderator (Nokia) From Wednesday session Agreement (first agreement for 6G!!) CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. R1-2506595 Feature Lead summary #2 on 6GR waveform Moderator (Nokia) From Friday session Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc. R1-2506579 FL summary#2 for 6GR modulation, joint channel coding and modulation Moderator (Qualcomm) From Friday session Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

- Numerologies

RAN1#122 and RAN#109 agreements Agreement - Regarding the RAN1 agreement below, the following is agreed: - Keep using “around 7GHz” instead of using detailed frequency range definition in RAN1’s and RAN4’s study. Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

- Overall -Minimum CHBW/Device Type

RAN#109 - In RP-252873, proposal 1 is endorsed, proposal 3 & 4 are endorsed for RAN only (no WG discussion) Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. FFS: whether RF and BB UE BW are same or different Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type The value(s) for the identified parameters for a device type

Exemplary information on initial consideration on IoT device types capabilities