3GPP TSG-RAN WG4 Meeting #116bis	R4-2513288

# Prague, Czech Republic, October 13th – 17th, 2025

Source: 	China Telecom

Title: 	Discussion on UL CA_n5-n8 with non-concurrent n5 DL and n8 UL

Agenda Item:	5.9.3

Document for:	Discussion

Introduction

In RAN4#115 meeting, the WF [1] agreed the feasible solution for CA_n5-n8 with non-concurrent n5 DL and n8 UL.

<Way forward>: For CA_n5-n8, further consider the following solution with side conditions to support both 1UL/2DL and 2UL/2DL CA with non-concurrent n5 DL and n8 UL in Rel-19: Support case 1 and case 2 via RRC reconfiguration. - Case 1: 2UL/2DL configuration, where NW shall keep all the four carriers activated, allocate resource allocation for a dedicated user only to n5 UL, n8 UL and n8 DL (no n5 DL resource allocation to a dedicated user) and both two bands are collocated. - Case 2: 1UL/2DL configuration Companies are encouraged to study necessity of additional RRM spec or additional requirement(s) of other working groups. If there is impact on RRM or other WGs beyond the introduction of UE capability, the solution is not considered in Rel-19. Whether new capability is needed or not would be further discussed.

In RAN4#116 meeting, the detailed procedure was discussed and the WF [2] was noted. The following agreement was reached. This contribution provides our views on CA_n5-n8 related RRM impacts.

For CA_n5-n8, in RAN4#116bis, an agenda item in RRM session will be introduced to discuss and determine if there is RRM impact. If RRM impact is identified and agreed, this topic will not be further pursued. If not, the discussion will continue in main session under R19 maintenance. - When n5DL is operated, n8UL RF path needs to be switched off and the same for the other way around. - Proponent companies are encouraged to bring detailed anlysis, including the possible gain/loss. Before the conclusion on RRM impact is made, no further discussion is needed in the main session.

Discussion

From operator’s view, the bands n5 and n8 are very crucial for our network deployment especially for the scenario of deep coverage. However, the bandwidth of n5 and n8 is narrow, and there exists overlap between n5 DL and n8 UL which make the low bands more fragmented and scattered. In order to increase flexibility and spectral efficiency for the band combination CA_n5_n8, it’s very important to consider the introduction of UE capability for supporting UL CA_n5-n8 with non-concurrent n5 DL and n8 UL.

Observation 1: In order to increase flexibility and spectral efficiency for the band combination CA_n5_n8, it’s very important to consider the introduction of UE capability for supporting UL CA_n5-n8 with non-concurrent n5 DL and n8 UL.

As for whether there is RRM impact, in our views, there would be no RRM impact if UE is not scheduled to transmit on n8 UL in the slots where UE is configured to receive on n5 DL, e.g., UE is not scheduled to transmit on n8 UL in the slots where SSB and/or CSI-RS are configured on n5 DL.

Measurement and mobility requirements are typically defined based on SSB and/or CSI-RS, therefore, there is no RRM impact for measurement and mobility if UE is not scheduled to transmit on n8 UL in the slots where SSB and/or CSI-RS are configured on n5 DL.

For the detailed procedure on CA_n5-n8, as discussed in the WF [2], n5 acts as PCell for the band combination, that is, PCell is always n5, and n8 always serves as SCell. In our understanding, SCell is always activated during the procedure described in the WF [2], so there is no impact on RRM requirements for SCell activation and deactivated SCell measurement.

As discussed in the WF [2], PCell is always n5, n8 always serves as SCell, PCell and SCell is collocated, and have the same frame timing. The UL timing requirements for n5 UL and n8 UL are both based on the timing of n5 DL, there is no impact for the timing of n5 DL if UE is not scheduled to transmit on n8 UL in the slots where SSB and/or CSI-RS are configured on n5 DL, therefore, there is also no impact for UL timing requirements for n5 UL and n8 UL.

Observation 2: There is no RRM impact on measurement, mobility, SCell activation, deactivated SCell measurement, DL timing, UL timing, etc., if UE is not scheduled to transmit on n8 UL in the slots where UE is configured to receive on n5 DL.

Proposal 1: RAN4 to confirm there is no RRM impact.

Regarding capability reporting: Case 1 requires the BS to avoid allocating n5 DL resources to specific UE, meanwhile, case 2 requires the BS remove UL configuration of n8 UL to the specific UE via RRC. Although BS know the capability support when UE reporting UL CA_n5-n8, it is still beneficial to explicitly report this capability in case some UEs have other implementation for supporting UL CA_n5-n8 in future.

Proposal 2: Introduce UE capability for supporting UL CA_n5-n8 with non-concurrent n5 DL and n8 UL in RF spec.

Conclusion

In this contribution, we give our observation and proposal on left issues on RRM impact and capability reporting for CA_n5-n8 with non-concurrent n5 DL and n8 UL.

Observation 1: In order to increase flexibility and spectral efficiency for the band combination CA_n5_n8, it’s very important to consider the introduction of UE capability for supporting UL CA_n5-n8 with non-concurrent n5 DL and n8 UL.

Observation 2: There is no RRM impact on measurement, mobility, SCell activation, deactivated SCell measurement, DL timing, UL timing, etc., if UE is not scheduled to transmit on n8 UL in the slots where UE is configured to receive on n5 DL.

Proposal 1: RAN4 to confirm there is no RRM impact.

Proposal 2: Introduce UE capability for supporting UL CA_n5-n8 with non-concurrent n5 DL and n8 UL in RF spec.

- References

- R4-2507910, WF on simultaneous Rx/Tx requirements, Huawei

- R4-2511850, WF on UL CA_n5-n8, CATT, Huawei, China Telecom, China Unicom, Anterix