3GPP TSG-RAN WG4 Meeting #116bis	R4-2513294

Prague, CZ, 13th – 17th October 2025

Agenda item:	8.13

Source: 		NTT DOCOMO, INC.

Title: 	6G RAN4 Study on RAN4 operation efficiency

Document for:	Discussion

- Introduction

In RAN4#116, the plan and meeting arrangement for 6G RAN4 study were introduced through contribution R4-2511652[1]. Starting from this meeting, discussions on 6G begin within each RAN4 topic. In this contribution, we present our view on 6G in the context of RAN4 operation efficiency.

- Discussion

According to R4-2511652[1], the scope of system parameters as 6G topics was shared. We contribute the 6G concept from each RAN4 perspective.

RRM

In Release 19, the importance of improving TS 38.133[2] for better readability and maintainability was recognized, and for the initial 6G framework it is critical to address redundancy and structural inefficiencies early, as these impact both specification clarity and implementation consistency across vendors.

From an RRM perspective, TS 38.133 currently contains repeated definitions of parameters such as TPSS/SSS_sync_inter and TSSB_time_index_inter across multiple subsections like 9.3.4, 9.3.9, and 9.3.10, which increases the risk of misalignment and complicates future updates; therefore, a centralized definition approach or common reference table would be beneficial.

Similarly, tables such as 8.5.6.2-1 for FR1 and 8.5.6.2-2 for FR2 could be merged into a single unified table with FR-specific columns to simplify cross-frequency comparison and reduce document length, while tables like 9.3.10.1-1 to 9.3.10.1-4 that include identical notes could be consolidated or referenced to avoid duplication.

In addition, sections such as 8.2 on “Interruption length” create multiple tables for different conditional branches even though their structure is nearly identical, which could be addressed by introducing a conditional column or unified table format. Performance requirement tables, such as those like 10.1.2.1.1-1, are also repeated for different use cases with only requirement values changing, suggesting that a parameterized approach or annex-based structure would improve efficiency.

Furthermore, repeated references to other specifications, such as in Section 6.2.2.2 on Random Access where behaviour descriptions for pass/fail cases all conclude with “defined in clause x in TS 38.321,” could be simplified into a single generic statement.

Observation 1: There are many instances of redundancy in TS 38.133 for NR, with repeated definitions and tables appearing throughout the specification.

Proposal 1: Addressing these issues during the Release 20 study phase will allow RAN4 to define a more scalable and maintainable structure for 6G RRM specifications before formalization in Release 21.

BS Demodulation

In 5G, the BS demodulation performance requirements are defined in 3GPP specifications such as TS 38.141-1. Some of these requirements are intended to apply only to BSs that support specific features. Although these features are specified as part of a 3GPP Release, in practical BS development, they are often introduced incrementally on a feature-by-feature basis. Therefore, it is essential that the specifications provide a clear and direct mapping from each individual feature to its corresponding performance requirements.

However, the current specifications do not always explicitly define this linkage. This lack of a clear applicability rule creates ambiguity. For instance, the performance requirement for PUSCH with 0.001% BLER, specified in TS 38.141-1, clause 8.2.6, is understood to be for BSs supporting URLLC, but this applicability is not explicitly stated in the specification. This ambiguity can make it difficult for implementers to meet all applicable requirements and for testers to create comprehensive test plans. As a result, critical performance requirements may be unintentionally omitted from verification, leading to potential performance inconsistencies across the network.

For the development of 6G, we believe it is essential to create more precise and clearer specifications. To avoid the issues observed in 5G, the applicability rule should be explicitly stated. This will enhance the usability and practical value of the specifications.

Observation 2: In the 5G BS demodulation performance specifications, the applicability of certain requirements is not explicitly linked to the support of the corresponding features. This can lead to the unintentional omission of requirements and inconsistent network performance.

Proposal 2: For 6G, the specification needs to be explicitly stated so that it is always clear which requirements apply to a given BS.

- Conclusion

In this contribution, we offer our 6GR view as follows.

Observation 1: There are many instances of redundancy in TS 38.133 for NR, with repeated definitions and tables appearing throughout the specification.

Proposal 1: Addressing these issues during the Release 20 study phase will allow RAN4 to define a more scalable and maintainable structure for 6G RRM specifications before formalization in Release 21.

Observation 2: In the 5G BS demodulation performance specifications, the applicability of certain requirements is not explicitly linked to the support of the corresponding features. This can lead to the unintentional omission of requirements and inconsistent network performance.

Proposal 2: For 6G, the specification needs to be explicitly stated so that it is always clear which requirements apply to a given BS.

References

[1] 3GPP, R4-25116522, “Plan on RAN4 6G discussion and meeting arrangement in Rel-20”, RAN4 Chair (Apple).

[2] 3GPP, “NR Requirements for support of radio resource management”, v19.1.0