3GPP TSG-RAN WG4 Meeting #116-bis												        R4-2513312

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:			8.12

Source:	Huawei, HiSilicon

Title:	Consideration on testability and OTA study for 6G

Document for:	Approval

Introduction

The SID [1] for 6G was approved in RAN#108. As guided by RAN4 chair [2], this contribution discusses possible aspects of OTA test in 6G.

Discussion

The test methodology for TRP and TRS has been stable since Release 19 and new bands can be included in TRP and TRS tests. For example, 7.125-8.4 GHz and 14.8-15.35 GHz were studied in R19 SI of FS_NR_IMT_4400_7125_14800MHz and could be included. 7GHz bands would naturally fit into FR1 OTA test methods. However, more discussion is needed on test methodology and performance metrics for 14GHz frequency range as it falls in between the current FR1 and FR2 range.

Proposal 1: study and include new frequency bands such as 7.125-8.4 GHz and 14.8-15.35 GHz in TRP and TRS tests.

Apart from NTN and XR devices, wearable devices have become more and more popular, ranging from watches to braces. Additional types of wearables would surely come to 6G.

Proposal 2: develop test methodology for wearable devices

MIMO OTA has developed test methodology for dynamic channels using the current UMa and UMi channels in various segments. Under 6G deployment scenarios, a device is more likely to connect to more than one Transmission Reception Point in downlink. This necessitates MIMO OTA tests in dynamic channel under two Transmission Reception Points in order to fully assess MIMO performances under 6G.

Proposal 3: develop test methodology for MIMO OTA test under dynamic channels with connections to two Transmission Reception Points.

All communication satellites use circular polarisation while R19 test methodologies defined in 3GPP employ linear polarisation. If linear polarisation is used in test systems, devices or VSAT with circular polarisation would be unduly penalised even though those devices would have better performance in real networks. In addition, there would be no possibility to use test systems with linear polarisation to assess the performance of circular polarisation such as axial ratio. The reason for test systems to use circular polarisation is to drive devices to improve their performances in real network operation. While test systems using linear polarisation clearly fail to play such a role. If test systems cannot address these issues in 5G, clearly it should be corrected in 6G.

Proposal 4: NTN OTA tests should use circular polarisation.

AI/ML has been studied in R19. One application of AI/ML in 6G is device beamforming at high frequency range such as FR2. The associated tests could only be carried out over the air.

Proposal 5: develop OTA test methodology for AI/ML beamforming by devices.

One prominent 6G feature is sensing or ISAC (Integrated Sensing and Communication). Various sensing scenarios exist as shown below.

Tests for sensing can only be carried out over the air and existing test methodologies and test system can be modified to carry OTA tests for the following scenarios, namely TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic.

Proposal 6: develop sensing test methodology and channel models for the following scenarios, namely TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic.

If sensing channel models are beyond the scope of RAN4, a static channel can be used instead for OTA test.

More and more vehicles such as cars are getting connected to networks. Besides, sizes of anechoic chambers for OTA have been expanding. A certain portion of anechoic chambers could accommodate a vehicle. This raises the possibility of testing OTA performances of vehicles. 6G would accelerate vehicle communication further.

Proposal 7: consider developing OTA test methodology for vehicles.

Conclusion

This contribution provides the following proposals.

Proposal 1: Study and include new frequency bands such as 7.125 – 8.4 GHz and 14.8 – 15.35 GHz in TRP and TRS tests.

Proposal 2: Develop test methodology for wearable devices.

Proposal 3: Develop test methodology for MIMO OTA test under dynamic channels with connections to two Transmission Reception Points.

Proposal 4: NTN OTA tests should use circular polarisation.

Proposal 5: Develop OTA test methodology for AI/ML beamforming by devices.

Proposal 6: Develop sensing test methodology and channel models for the following scenarios, namely TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic.

Proposal 7: Consider developing OTA test methodology for vehicles.

Reference

[1] RP-251881 Study on 6G Radio

[2] R4-2511652 RAN4 6G Plan and Meeting Arrangement