3GPP TSG-RAN WG4 Meeting #116bis	R4-2513318
Prague, Czech Republic, 13 – 17 Oct 2025

Title:	RAN4 driven AI based use cases

Source:	OPPO

Agenda item:	8.9

Document for:	Approval

- Introduction

For the upcoming 6G, the SID [1] of 6GR was approved in RAN#108 meeting. The objectives related to AI air interface for 6GR are listed as follows.

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

- Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc…

Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…)

NOTE: lead WG depends on the use case.

- AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including

  - LCM procedures [RAN2, RAN1, RAN3, RAN4]

  - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1]

Note: NW for AI is assumed to be covered by new services

6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In this contribution, we share our general considerations on AI for 6G RAN4 study and present some RAN4 drive AI use cases.

- General considerations

  - Principles for selecting RAN4 AI use cases

In the 5G-Advanced phase, RAN4 has addressed requirements and testing impacts for use cases such as CSI, beam management, positioning, as well as some LCM related features, laying the groundwork for AI use case studies in RAN4. Moving into the 6G era, in addition to incorporating more inputs from RAN1 and RAN2 use cases, RAN4 could identify and study some typical use cases that exhibit distinct RAN4 characteristics.

In our view, RAN4-driven use cases should center on the direct effects of introducing AI technologies in RF, demodulation, and RRM-related aspects, while emphasizing testability considerations from the very beginning of 6G standardization. For such use cases, the guidance agreed upon during the RAN1 #122 meeting can serve as a starting point, with "testability assumptions" added from RAN4 perspective.

Proposal 1: 	For RAN4-driven use cases, reuse the guidance that agreed in RAN1 #122 meeting as starting point, and add “Assumption on testability”.

For 6GR AI use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training, finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., - no collaboration/interaction - UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact - Assumption on testability

  - Designs for holistic LCM test

Based on the description in TR38.843, following aspects are involved in LCM

- Data collection

- Model training

- Functionality/model identification

- Model delivery/transfer

- Model inference operation

- Functionality/model selection, activation, deactivation, switching, and fallback operation

- Functionality/model monitoring

- Model update

- UE capability

In 5G-A, due to time constraints and limited input from RAN4 and other working groups, RAN4 discussions only cover model inference, monitoring, activation, deactivation, switching, and fallback.

For 6G, RAN4 should take a comprehensive approach to LCM testing to fully support AI use cases throughout their lifecycle and enable native AI in 6G. More LCM features, e.g., data collection, model selection, training, update, and UE capability, need further study in RAN4 AI, including testability issues, core requirements, performance requirements. For example, RAN4 needs to define performance requirements for model selection, training, and updates. When a change in the model to be used occurs, e.g., due to selection, training, or updates by the UE or network, it is important to determine whether and how such a change is effective and testable.

If the UE identifies different deployment scenarios and conditions that require a more suitable model, it may switch between pre-prepared models, receive new model parameters from the network, or obtain an updated model through online data collection or training. In this case, the new model would differ from the model used during testing (e.g., a standardized testing model). From a RAN4 testing and performance perspective, key questions arise: Should testing focus on verifying the UE’s capability to perform model selection, training, and updates? Or is new test method needed to evaluate these LCM-related features and quickly check the new model performance? These aspects must be considered in 6G RAN4. This may also relate to post-deployment testing.

Observation 1: 	For 6G, RAN4 should take a comprehensive approach to LCM testing to fully support AI use cases throughout their lifecycle and enable native AI in 6G.

Proposal 2: 	For 6G, More LCM features, e.g., data collection, model selection, training, update, and UE capability, need further study in RAN4 AI, including testability issues, core requirements, performance requirements.

  - Specification and deployment for AI models

In 5G-A, for the CSI compression use case, the introduction of 2-sided models has led RAN4 to study standardized models in order to ensure testability. Progress has been made in R19, and work will continue in R20.

Looking ahead to 6G, further research is needed on model standardization and deployment. From our understanding, the discussion about AI models in 6G RAN4 may focus on following two aspects:

- Effective Model Design:

For different use cases, if standardized models are needed, we need to determine what kind of models will be most effective. In the R18 and R19 phases, AI use case studies in 3GPP did not pay much attention to the models themselves.

As 6G marks the start of a new generation of communication systems, it is important to carefully compare and analyze the best model design approaches for different use cases. This will help ensure that the reference models developed by RAN4 are practical and feasible for field implementation.

- Constraints on Model Deployment:

We need to define what makes a model "complex" or "simple” and understand the power consumption differences between different models, especially under various UE deployment conditions(e.g. different AI categories as described in section 2.5).

It is also important to consider how well different UEs, particularly those with varying AI categories, can support different models. This includes understanding the constraints on model deployment for different UEs, such as the type of available computing resource, the amount of computing resources, and where those resources are located.

Proposal 3: 	For 6G, further study is needed on model standardization and deployment, RAN4 needs to consider following two aspects:

      - Effective Model Design, to ensure the reference models developed by RAN4 are practical and feasible for field implementation.

      - Constraints on Model Deployment, e.g., to study and define what makes a model "complex" or "simple” and understand the power consumption differences between different models.

  - Test method for AI models

In 5G-A, we have encountered significant testability challenges, many of which remain unresolved, especially for FR2 testing. A lot of effort is spent on generating signals in a chamber environment that can serve as input for AI use case models. However, key questions still lack clear answers, e.g., how to effectively simulate beam-sweeping scenarios in a chamber with limited probes, how to model channels, and whether testing model performance under simplified CDL UMi conditions is truly meaningful. From our understanding, in current BM FR2 test discussions, the over-simplified OTA assumptions being used may offer little practical guidance for field deployment.

In 6G, RAN4 may need to specify more reference models for different use cases and potentially for different scenarios/conditions. Besides, new LCM features like model selection, model updates, online training/tuning may also kick off new needs for model tests. Furthermore, the post-deployment issue needs to be dealt with at this time, e.g., treat as parts of LCM and identify details for testing methods. So, this time if we still need to do a lot of work for OTA setup discussion and introduce complex test procedures for the model test, it will be a big challenge.

In 6G, another approach for AI tests (especially for model test) could be RAN4 may explore whether/how to decouple the signal acquisition testing(e.g., or model inputs) from model performance evaluation(e.g., for model performance) for AI use cases.

- Signal acquisition could continue to be tested via cable or OTA methods, e.g., evaluating BM Set B measurement accuracy.

- Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets), enabling a virtual testing environment (maybe somehow like a Digital Twin setup). For instance, RAN4 study and specify test dataset for models under test. TE would provide the DUT with a predefined test dataset. The DUT would then return inference results to the TE for performance evaluation. This approach would reduce the need for complex OTA tests, accelerate model validation, and bring testing closer to real-world conditions depending on the quality of the test dataset.

Observation 2:	The over-simplified OTA assumptions being used may offer little practical guidance for field deployment.

Proposal 4: 	For 6G, RAN4 could explore whether/how to decouple the signal acquisition testing (e.g., for model inputs) from model performance evaluation (e.g., for model performance) for AI use cases.

Proposal 5: 	Signal acquisition could continue to be tested via cable or OTA methods, e.g., evaluating BM Set B measurement accuracy.

Proposal 6: 	Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets), enabling a virtual testing environment.

  - AI categories in RAN4

In recent years, more and more user devices are being equipped with AI capabilities. However, this also leads to significant differences in AI performance across different devices. It is important to distinguish between devices with different levels of AI capability to define how they can best use AI, e.g., considering different computing resource, energy consumption, storage, supported models and performance requirements. This will help ensure that devices with varying AI capabilities can still effectively support the AI functions they need.

Taking available computing resources as an example, in current terminal devices, most computing resources are located outside the modem. Computing resources inside the modem, especially dedicated processors like GPUs or NPUs, are very limited. Moreover, when a UE lacks dedicated AI processors or such resources are constrained, non-AI-specific processors (CPU) may be used to run simple neural network models, like simplified MLP models. As a result, for wireless AI use cases, available computing resources may fall into: (1) AI computing resource on device, but outside modem, e.g. NPU on device for AI operation. (2) Non-AI computing resource on device, but outside modem, e.g. normal computing resource on device. (3) AI computing resource  inside modem, e.g. the NPU inside modem for AI operation. (4) Non-AI computing resource  inside modem, e,g, normal computing resource inside modem

Figure 1: Computing resources on chip

Observation 3: 	For computing resources, in current terminal devices, most computing resources are located outside the modem. Computing resources inside the modem, especially dedicated processors like GPUs or NPUs, are very limited.

Observation 4: 	For wireless AI use cases, available computing resources may fall into:

- AI computing resource on device, but outside modem

- Non-AI computing resource on device, but outside modem

- AI computing resource inside modem

- Non-AI computing resource inside modem

Besides, another example could be UE with different capabilities may support different reference models. Some UEs may be able to support relatively complex models, while others may only support simpler ones. If RAN4 defines reference models without taking into account the diverse AI capabilities of UEs, it could lead to AI features being available only to high-end devices.

From our understanding, in 6G RAN4 AI, it would be helpful to consider different factors that may affect AI features/requirements/deployments. From day1, RAN4 could try to study/identify these factors and estimate whether and how to introduce AI categories in RAN4 to serve different UE and different AI features. Following aspects could be considered first:

- Computing Resource: Different tiers could correspond to different minimum computational requirements (e.g., operations per second), or to the number of supported AI processing units.

- Energy Consumption: Different tiers could be defined based on typical power consumption levels. Some commercial devices already indicate power references for non-modem AI processing, such as typical power consumption under given conditions like sustained AI load. Additionally, the feasibility of power consumption testing could be studied from the start of 6G in RAN4 AI. Research could explore whether and how to use metrics like FLOPS/OPS to reflect AI power consumption, along with other influencing factors like different AI computing resources, their location, and deployment assumptions.

- Memory Tiers: Different tiers could be based on available memory size or the number of memory units (MUs).

- Types of Computing Resources: For example, non-AI-specific resources (like CPUs) versus AI-dedicated resources (like GPUs).

- Location of Computing Resources: Resources could be located inside or outside the modem, inside or outside the device itself.

- Supported Reference Models: This refers to the types of model architectures a UE can support, such as MLP, CNN, or Transformer-based models. A UE might support one or more of these types.

Observation 5: 	UE with different capabilities may support different reference models. If RAN4 defines reference models without taking into account the diverse AI capabilities of UEs, it could lead to AI features being available only to high-end devices.

Proposal 7: 	For 6G, RAN4 could study/identify these factors and estimate whether and how to introduce AI categories in RAN4 to serve different UE and different AI features. Following aspects could be considered first:

      - Computing Resource

      - Energy Consumption

      - Memory Tiers

      - Types of Computing Resources

      - Location of Computing Resources

      - Supported Reference Models

Proposal 8: 	The feasibility of power consumption testing could be studied from 6G day1 in RAN4 AI.

      - Whether/how to use flops/ops to reflect the AI power consumption.

      - Further study other impacts that may affect the AI power consumption (if any, e.g., due to different AI computing resource/location, different deployment assumptions).

- Use cases of AI for 6GR

In this contribution, we demonstrate some RAN4 driven AI use cases for 6G and summarized in Table 1.

Table 1: AI use case summary

| Category | AI use case |  | Definition and inputs/outputs | Loc/Training |
| --- | --- | --- | --- | --- |
| RF - AI based Non-linearity Compensation | NW side DPoD |  | - Model input: received signal after channel estimation and equalization - Model output: received signal before demodulation [or before channel decoding] | - NW-side - Offline/online training |
|  | UE side DPD |  | - Model input: transmitted signal before PA - Model output: transmitted signal before PA after DPD | - UE-side - Offline/online training |
| Demod - AI based Receiver enhancement | DMRS based channel estimation, e.g., for high accuracy CE |  | - Model input: received signal and DMRS info. - Model output: estimated channel | - NW-side - Offline training |
|  | SRS based channel estimation, e.g., for SRS insertion loss compensation |  | - Model input: received signal and SRS info. - Model output: estimated channel | - NW-side - Offline training |
|  | AI based demodulation, e.g., for high-order modulation |  | - Model input: received symbols after channel estimation and equalization, [noise power] - Model output: received signal before channel decoding, e.g., soft information (i.e., LLR) for each bit | - NW-side - Offline training |
|  | [AI based joint channel estimation + demodulation] |  | - Model input: received signal and DMRS info - Model output: received signal before channel decoding, e.g., soft information (i.e., LLR) for each bit | - NW-side - Offline training |
| RRM - AI based RRM enhancement | L1/L3 beam measurement | Time domain Intra-cell | - Input: L1/L3 measurement based on set B of beams at historic time instance(s) - Output: predicted L1/L3 measurement on set B of beams | - NW and UE side model - Offline training |
|  |  | Time & spatial domain Intra-cell | - Input: L1/L3 measurement based on set B of beams at historic time instance(s) - Output: predicted L1/L3 measurement on set A of beams. - Set B is different from Set A | - NW and UE side model - Offline training |
|  |  | Frequency & spatial domain Inter-cell | - Input: L1/L3 measurement based on set B of beams at frequency B - Output: predicted L1/L3 measurement on set A of beams at frequency A | - NW and UE side model - Offline training |
|  | L3 cell measurement | Time domain Intra-cell | - Input: L3 cell measurement at historic time instance(s) at frequency B - Output: predicted L3 cell measurement at frequency B | - NW and UE side model; - Offline training |
|  |  | Frequency domain Inter-cell | - Input: L3 cell measurement at frequency B - Output: predicted L3 cell measurement at frequency A | - NW and UE side model; - Offline training |

  - RF - AI based Non-linearity Compensation

Supporting the growing demand for high data rates with higher-order modulation requires a higher SNR, which, at a constant noise level, must be supported by increased transmission power. However, this introduces a new challenge, i.e., increasing transmission power is often limited by the nonlinear characteristics of the PA.

Currently, it is desirable for the PA to operate in its linear region, or at least for its output to meet the EVM requirements, in order to avoid degradation of data demodulation performance at the receiver due to PA nonlinearity. Additionally, non-linear compensation (NC) for the PA is another approach to address this issue, which can be implemented either at the transmitter side using Digital Pre-Distortion (DPD) or at the receiver side using Digital Post-Distortion (DPoD).

Figure 2 illustrates a method for implementing DPD at the UE side. Before the signal enters the PA, it is processed by an AI model that performs digital pre-distortion. This model can be trained either offline or online. In Appendix 5.1, some preliminary simulation results of UE-side DPD processing are provided. Under the assumed PA nonlinearity model, the AI based DPD achieves performance close to that of an ideal PA. Particularly for 256QAM, where the constellations are originally difficult to distinguish, the demodulation performance improves significantly after DPD is applied.

Figure 2: DPD at the UE side

Figure 3 illustrates a method for DPoD at the NW side. At the receiver, DPoD uses AI based post-processing to mitigate performance degradation caused by PA nonlinearity. This relaxes the EVM requirements at the transmitter, allowing the PA to operate over a wider dynamic range. Specifically, three different implementations of DPoD at the receiver could be considered:

- NW-side nonlinearity compensation (NC) via AI demodulation: This approach evaluates whether PA nonlinearity can be indirectly handled through AI-based demodulation.

- NW-side NC using a separate AI-based NC module + non-AI demodulation: An independent AI-based DPoD module is introduced before demodulation to handle PA nonlinearity. For example, the received signal can be converted to the time domain for DPoD processing and then converted back to the frequency domain. This approach may reduce model complexity and size compared to frequency-domain processing. Other implementation options can also be evaluated.

- NW-side NC using a separate AI-based NC module + AI demodulation: This method combines AI-based NC with AI demodulation to implement DPoD.

Figure 3: DPoD at the NW side

These different DPoD implementations represent distinct approaches to nonlinear compensation. Preliminary simulation results are provided in Appendix 5.1. It appears that nonlinear compensation on the network side is more challenging than on the UE side. For DPoD, introducing NW-side AI-driven NC could obtain performance gains compared to no AI-NC, especially for higher-order modulation. However, NW-side AI-NC does not achieve the same performance level as UE-side AI-NC. This may be because the signal distortion at the receiver combines the effects of channel fading, noise, PA nonlinearity, and other factors, thereby increasing the difficulty of NW-side NC.

Furthermore, the integration of channel estimation with the above schemes could also be considered. It is clear that there are numerous possible implementations for DPoD on the network side. In RAN4 studies, companies could propose solutions they consider more suitable, including DPD and DPoD, and compare different approaches based on performance gain, cost analysis and standardization impact.

Proposal 9: 	For 6G, Non-linearity Compensation (NC) could be taken into account in 6G, including:

      - Option 1: UE side DPD (Digital Pre-Distortion)

      - Option 2: NW side DPoD (Digital Post-Distortion) and Receiver

      - Note: model updates/tuning could be taken into account

Proposal 10:  	For NW side DPoD(option2), following sub cases could be evaluated in RAN4:

      - Option 2a: NW side NC by AI demod

      - Option 2b: NW side NC by a separate AI NC module +Non-AI demod

      - Option 2c: NW side NC by a separate AI NC module + AI demod

      - Option 2x: other solutions

      - Note: channel estimation may/may not be included

Based on our current understanding, both DPD and DPoD might be effective for nonlinearity compensation. In RAN4, if we intend to initiate a study on nonlinearity compensation, in addition to above candidate solutions, following issues should first be addressed:

- Alignment on simulation assumption, e.g., during/after RAN4#122bis, through offline/email discussion to agree on an aligned simulation assumption for further study and to support continued research across companies.

- Clarification of PA nonlinearity modeling, e.g., amplitude nonlinearity, phase nonlinearity, time variation, and other relevant characteristics.

- Definition of evaluation methods and metrics, e.g., determining how to quantify performance gains, e.g., gain in BLER/TPs @ a given EVM requirements

Additionally, for nonlinearity compensation, it is important to consider the ability to quickly train and update the compensation model through online training. During the 6G SI, RAN4 could include this aspect as part of its research and consider how to set up the PA nonlinear model and its time-varying characteristics.

Proposal 11: 	For Non-linearity Compensation (NC) study in RAN4,

      - Alignment on simulation assumption is needed

      - PA Non-linearity modeling should be clarified

      - Definition of evaluation method/metric is needed

  - Demod - AI based Receiver enhancement

AI based receiver enhancement could also be evaluated in RAN4. For example, employing AI for channel estimation may lead to improved performance, and in the case of SRS-based uplink channel estimation, it may also help compensate for SRS insertion loss. Furthermore, for higher-order modulation schemes, AI-based receiver design has the potential to achieve better demodulation performance gains. From our perspective, all these new cases or sub-use cases could be considered for discussion and study in RAN4. By Q1 2026, RAN4 may need to review and identify high-priority candidates for further work.

Proposal 12: 	For 6G, AI based Receiver enhancement could be taken into account in 6G, e.g., for high accuracy channel estimation, SRS insertion loss compensation, high-order modulation

  - RRM - AI based RRM enhancement

3.3.1 Use case definition and category in 6G

In 5G NR, several AI/ML based use cases have been studied e.g., CSI compression, CSI reporting, Positioning, Beam management and mobility, among which beam management and mobility are RRM related which we think will be the baseline of 6G AI/ML based RRM use cases.

For beam management, according to the WID [3] in Rel-19 NR_AIML_air, two sub-use cases for DL Tx beam prediction are included in the work item for both UE-sided and NW-sided model in which the two sub-use cases are defined as:

- Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

- Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

The inference procedure is illustrated as below (Figure 6.3.1-1 in TR 38.843):

Figure 4: An example of the inference procedure for beam management

Based on the definition, for BM-Case1, only spatial domain prediction for the same cell is included and the measurements on time domain is not reduced, i.e., the measurement results of a big set of Tx beams are predicted based on a small set of Tx beams of same cell at the same time.

For BM-Case2, Set A and Set B can be same or different (Set B is NOT a subset of Set A), or Set B is a subset of Set A. It should be noted that when Set A and Set B are the same, BM-Case2 would be a purely time domain prediction, i.e., the measurement results of a set of Tx beams are predicted by the historic measurements on the same set of Tx beams of same cell. And when Set A and Set B are different or Set B is a subset of Set A, BM-Case2 would be a combination of time domain and spatial domain prediction.

Observation 6: 	Spatial domain prediction, time domain prediction and the combination of time domain and spatial domain prediction for L1 beam measurement are studied and specified for beam management in 5G NR.

Observation 7: 	Only DL Tx beam prediction is specified for beam management in 5G NR.

For AI/ML based mobility, according to the WID [4], the following scenarios will be defined in Rel-20:

- Measurement prediction and indirect measurement event prediction including:

  - L3 Cell level prediction

    - Time domain case A and case B, and co-located frequency domain prediction for UE sided model and NW sided model

    - Spatial domain prediction for NW sided model

  - L3 Beam level prediction for NW sided model

- For indirect measurement event prediction, event A1~A6 are included.

- Direct event prediction is still open and need to be checked in RAN#112 meeting.

For each scenario of cell level and beam level prediction, the following sub-use cases are defined:

3 sub-use cases are considered for cell-level RRM measurement prediction: - RRM sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated; - RRM sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s); - RRM sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s). 3 sub-use cases are considered for beam-level RRM measurement prediction: - Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated; - Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s); - Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s).

For time domain measurement prediction, two cases are included in which case A is for 2nd study goal (improve the handover performance) and the samples with predicted results will be still actually measured, while case B is for 1st study goal (reduce measurement efforts) and the samples with predicted results will be skipped when performing measurement. The examples for case A and case B are illustrated as below (Figures in TR 38.744):

Figure 5: Example of intra-frequency temporal domain case A

Figure 6: Skipping pattern example 1 of intra-frequency temporal domain case B

Figure 7: Skipping pattern example 2 of intra-frequency temporal domain case B

For frequency domain measurement prediction, only co-located scenario is included, i.e., the cell to be measured and cell to be predicted are located in the same sector of either serving site or same neighboring site. And for the prediction method, both signal cell prediction and cluster-based prediction are considered.

For measurement event prediction, the inference procedure for direct and indirect event prediction are illustrated as below (Figures in TR 38.744):

Figure 8: Indirect measurement event prediction

Figure 9: Direct measurement event prediction

For indirect event prediction method, measurement result(s) is predicted by a RRM measurement prediction model for intra-frequency temporal domain case A, temporal domain case B, frequency domain or spatial domain at first. Afterwards, predicted and optionally actual historical measurement result(s) of the same cell(s) are used to derive whether a measurement event at one future time instance occurs, without further involvement of an AI/ML model.

For direct prediction method, measurement event is predicted directly by an AI/ML model, i.e., the output of the model is the likelihood of an event occurrence.

Observation 8: 	For measurement prediction and indirect event prediction in AI mobility, time domain and co-located frequency domain prediction for cell level measurement are included, while spatial domain prediction was not included for UE sided model in 5G NR.

Observation 9: 	L3 cell level measurement prediction for non-collocated frequency domain and L3 beam level measurement prediction for all domains are not included in 5G NR.

Based on the background above, we found that the use case definition in 5G NR is based on the measurement purpose (beam management or mobility) and the scenario category in AI mobility is based on the measurement report type (e.g., L3 cell measurement or L3 beam level measurement). And for each scenario, multiple sub-use cases are included based on the measurement type for AI model input and output, e.g., L1 beam measurement, L3 beam measurement and L3 cell measurement. We summarized the use case of AI/ML based measurement prediction and scenarios which have been studied and will be included in 5G NR as below:

Table 2: Summary of AI/ML based use case and scenarios for measurement prediction in 5G NR

| Use case (purpose) | Report type | AI Model |  | Prediction domain | Included in 5G NR |
| --- | --- | --- | --- | --- | --- |
|  |  | Input | Output |  |  |
| Beam management | L1-RSRP (beam level) | L1 beam measurement | L1 beam measurement | Time | Yes |
|  |  |  |  | Spatial |  |
|  |  |  |  | Combination of time and spatial |  |
| LTM | L1-RSRP (beam level) | - | - | - | No |
| L3 Mobility | L3-RSRP (cell level) | L1 beam measurement | L1 beam measurement | Time Frequency Spatial | Spatial domain is only supported for NW side model |
|  |  | L3 cell measurement | L3 cell measurement |  |  |
|  |  | L1 beam measurement | L3 cell measurement |  |  |
|  | L3-RSRP (beam level) | L1 beam measurement | L1 beam measurement | Time Frequency Spatial | Only supported for NW side model |
|  |  | L3 beam measurement | L3 beam measurement |  |  |
|  |  | L1 beam measurement | L3 beam measurement |  |  |

According to the summary above, we can see some problems which cause confusions, e.g.,

- There are some overlapping for AI model implementation between different use cases.

- In AI mobility, the report type, sub-use case and prediction domain are defined separately. Too many combinations will exist, but it is not explicitly clarified whether all of the combination between predicted measurement type and prediction domain are supported.

- It is not explicitly clarified whether some of the combination of different domains are supported.

- Some procedures (e.g., LTM) are missing although the similar measurement prediction approach can be applied theoretically.

For 6G discussions on AI/ML based RRM use case, we think we can reconsider the definition of use cases or scenarios by decoupling the AI/ML model implementation (input and output) and measurement report type. I.e., for use case definition, we only consider the input and predicted measurement type of AI model. Afterwards, UE can get further measurement results based on the AI output and post processing. What kinds of measurement should be reported and what is the purpose of the measurement report can follow the legacy procedure and decouple with AI model.

Observation 10:  Some issues are observed for the definition of AI/ML based RRM use cases and scenarios in 5G NR, e.g.,

      - There are some overlapping for AI prediction part between different use cases.

      - Too many combinations exist based on the report type, sub-use case and prediction domain. But it is not clear whether each combination is supported.

Proposal 13: 	Reconsider the definition of measurement prediction use cases in 6G AI/ML based RRM by decoupling the AI model implementation and report procedures, i.e., the use case is defined based on AI model input and output.

3.3.2 Use cases for measurement prediction

For a UE in CONNECTED mode, for mobility handling, it needs to perform the serving cell and neighbor cell measurement. According to the existing requirements in TS 38.133, the measurement will need measurement gap or scheduling restrictions or interruptions.

For the measurement with gap, UE is not required to transmit or receive signals during gap occasions. And for the measurement without gap, due to the time difference of cells or different subcarrier spacing or different beams, the measurement of neighbor cells may cause scheduling restrictions, i.e., UE is not expected to transmit or receive signals on SSB symbols to be measured and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. For the measurement without gap when UE report “no-gap”, according to UE capability, interruption may be needed. In summary, all the above three aspects would cause data loss and impact the system throughput.

On the other hand, for the measurement within gap, since UE can only measure one layer in a gap occasion, the measurement delay needs to be extended by the number of layers. And for the measurement without gap, due to limited UE measurement resources (e.g., searchers), the measurement delay would also be extended by the number of layers to be measured.

Observation 11: The UE measurements for mobility may need gap configuration or cause scheduling restrictions or interruptions which will impact the system throughput.

Observation 12: The UE measurements delay will to be extended due to limited measurement resources or gap sharing when multiple layers are configured.

When 6G starts, although there may be some optimization to be done for measurement, we think the baseline measurement framework can be reused and the issues mentioned above may still exist. To resolve the mentioned issues, we understand the study goals in 5G NR AI mobility (reduce the measurement efforts and improve handover performance) can be still applied in 6G AI based RRM study.

Proposal 14: 	The study goals in 5G NR including to reduce the measurement efforts and to improve handover performance can be still applied in 6G AI based RRM study.

Considering the above study goals, we think the use cases specified in 5G NR are still applicable in 6G and can be the starting point of 6G AI/ML based RRM study with consideration of new definition approach. In addition, we also noticed that there are some use cases and scenarios not included in 5G NR which makes the AI/ML utilization very limited. And we think more scenarios can be considered in 6G study.

For example, regarding L3 measurement for mobility, only cell level measurement prediction with time domain and collocated frequency domain prediction are included for UE sided model in 5G NR. Due to the limited time and RAN4 workload, L3 beam level measurement was not included in 5G NR, but considering the completion of L3 mobility and the similarity between L3 cell level and beam level in terms of AI/ML related mechanism, we think L3 beam level measurement prediction can be supported in 6G SI and the approaches of L3 cell level prediction can be extended.

Besides L3 measurement for mobility, L1 beam level measurement for mobility is also introduced in Rel-18 LTM. Referring to the requirements defined in TS 38.133 9.15.5 and 9.1.5.4, L1 beam level measurement can be used for neighbor cells which may also need measurement gaps and will participate the gap sharing with L3 measurement referring to the requirements. Reducing the measurement on L1 measurement for mobility would also be an approach to reduce the gap ratio and improve the system throughput. We think most of the approaches defined for L3 cell level and beam level measurement prediction can be reused.

Based on the previous proposal on the new approach for use case definition, we do not need to differentiate the usage and report type for each measurement prediction case, e.g., after we define the L1 beam measurement prediction, it can be applied to both beam management and L1 mobility.

Proposal 15: 	The following AI/ML based measurement prediction use cases can be included in 6G:

      - L3 cell measurement prediction

      - L3 beam measurement prediction

      - L1 beam measurement prediction

For each above use case, time domain, frequency domain, spatial domain prediction (Tx beam prediction) and the combination of different domains are possible. But we need to consider the reality of each case, e.g., regarding frequency domain prediction for beam measurement, it is not realistic to predict the measurements on different frequencies with the same Tx beam. Because the measurements on different frequencies are from different cells, even the Tx beam (i.e., SSB index) is same, the related direction can be different. And the Tx beam index which UE can receive on the two frequencies are different which may make the prediction on the same Tx beam meaningless. So the better way is to predicted the best beams on different frequencies which makes the prediction not a pure frequency domain prediction but a combination of frequency and spatial prediction. Based on such principle, we think we need to analyze the realistic prediction approach for each use case above rather than to consider all the combinations.

Proposal 16: 	For each measurement prediction use case, consider the realistic prediction approach, e.g.,

      - For L3 cell measurement, consider time and frequency domain prediction.

      - For L1/L3 beam measurement, beside the time domain prediction, consider the combination of spatial and time domain prediction, and the combination of spatial and frequency domain prediction.

For frequency domain prediction, only co-located scenario is considered in 5G NR which highly restricts the usage of frequency prediction. It is noted that for different frequencies, non-collocated scenario is also typical deployment and it deserves some studies on the possible solutions to enable frequency domain prediction. We can understand that there may be some issues and difficulties for non-collocated scenarios, e.g.,

- The channel status changes for different frequencies are independent and with less relevance. It is hard to get the correlation between non-collocated frequencies.

- Some information of neighbor cells (e.g., relative location, transmission power) is not known to UE.

- The differences between site to site may be significant which makes the generalization hard to be done.

So we think we can firstly investigate the difficulties of frequency domain prediction and potential standardized approaches to resolve identified issues. From our side, we think at least for a certain scenario e.g., the sites deployment is fixed, it is possible to find some solutions to enable the non-collocated frequency domain prediction e.g., through some assistant information. For generalization part, further study is needed and the discussion in 5G NR may be referred since it is kindly common for all the use cases.

Proposal 17: 	The frequency domain prediction for non-collocated scenario can be considered in 6G SI starting from a stationary scenario. The generalization issue needs further study referring to the discussions in 5G NR.

Among the above discussions, spatial domain prediction refers to Tx beam prediction which is included in 5G NR. Taking the following figure 10 as an example, the Tx beam prediction only focus on the Tx handling under same Rx beam (e.g., Case 1 or Case 2) which may reduce measurement efforts or network transmission overhead.

Figure 10: illustration of beam prediction

But from RAN4 perspective, for L1 and L3 measurement, Rx beam is also an important aspect since Rx beam sweeping is assumed for receiving different Tx beams which require additional measurement efforts and extend the measurement delay. So it is beneficial if some solutions can be found to predict the Rx beam or Tx-Rx beam pair (e.g., Case 3) to reduce beam sweeping and the measurement efforts on UE side. For this target, some studies can be done in 6G SI on the potential enhancements or difficulties for Rx beam or Tx-Rx beam pair prediction. For this prediction approach, it cannot be a single domain prediction and at least the combination of time domain and spatial domain prediction can be considered for intra-frequency measurement.

However, since the Rx beam sweeping is up to UE implementation and no report is needed, there are some difficulties to define the Rx beam prediction use case, e.g.,

- How to define the input and output of AI model

- How to handle the different cells with single Rx

- How to differentiate from Tx beam prediction

Since Rx beam sweeping is UE implementation, and one Rx may receive multiple Tx beams and multiple cells, the prediction results may still need to reflect on measurements on Tx beams or cells. If that is the case, it needs to be considered what is the difference from the existing Tx beam prediction.

Proposal 18: 	Some issues need to be resolved before supporting Rx beam or Rx-Tx beam pair prediction in 6G SI, e.g.,

      - The definition of AI model input and output

      - The handling of different cells for different Rx beams

      - The difference from Tx beam prediction and cross-cell prediction

With above discussions, we summarized the use cases of measurement prediction to be considered in 6G SI based on our proposals as below:

Table 3: Summary of AI/ML based use case and scenarios for measurement prediction in 6G

| Use case (measurement type) | Prediction domain | AI Model input and output | Model side and training | Note |
| --- | --- | --- | --- | --- |
| L1 beam measurement | Time domain | Input: L1 measurement based on set B of beams at historic time instance(s). Output: predicted L1 measurement on set B of beams. | NW and UE side model; Offline training | Intra-cell |
|  | Combination of time and spatial domain | Input: L1 measurement based on set B of beams at historic time instance(s). Output: predicted L1 measurement on set A of beams. Set B is different from Set A |  |  |
|  | Combination of frequency and spatial domain | Input: L1 measurement based on set B of beams at frequency B. Output: predicted L1 measurement on set A of beams at frequency A. | NW and UE side model; Offline training | Inter-cell |
| L3 beam measurement | Same as above by replacing L1 measurement with L3 beam measurement |  |  |  |
| L3 cell measurement | Time domain | Input: L3 cell measurement at historic time instance(s) at frequency B. Output: predicted L3 cell measurement at frequency B. | NW and UE side model; Offline training | Intra-cell |
|  | Frequency domain | Input: L3 cell measurement at frequency B. Output: predicted L3 cell measurement at frequency A. | Same as above | Inter-cell |

3.3.3 Other RRM procedures based on measurement prediction

As we all know, measurement is the baseline for most of RRM procedures e.g., HO/LTM, RLM/BFD/CBD, SCell related procedures, active DL/UL TCI switch, SRS antenna switch and carrier switch, etc. The measurement availability and delay may also impact the corresponding procedure delay or performance. And the measurement results report before signaling may help the NW decision on sending the corresponding command. For example, the LTM and SCell activation procedure are illustrated as below:

Figure 11: illustration of LTM procedure

Figure 12: illustration of SCell activation procedure

On top of the discussions about measurement prediction, considering the impact of measurement, if the approach can be applied to such procedures, it may reduce the procedure delay or improve the performance by early prediction and preparation. So we think this can be another study goal for AI/ML based RRM use case to improve mobility performance and the CA operation. In 5G NR, there are already some use cases to for this study goal e.g., time domain measurement prediction case A and corresponding indirect event prediction. Similar approaches can be reused in 6G and extended to SCell related enhancements in CA operation.

Proposal 19: 	RAN4 to consider new use cases by applying the measurement prediction and event prediction to existing RRM procedures, e.g.,

      - AI/ML based RLM/BFD/CBD, LTM

      - AI/ML based SCell related enhancement: SCell addition/activation/change

Above discussions are based on L1 measurement and L3 measurement respectively. However, at the meantime, we noticed that in 5G NR, there are some overlapping for L1 and L3 measurement e.g., for L1 MIMO, L1 mobility and L3 mobility. The UE measurement behavior is similar, but the measurement configurations, capabilities, mobility procedures and the requirements are defined separately. And the L1 and L3 measurement for the same cell may compete the measurement resources or gap resources which highly extend the measurement delay. To simplify the UE behavior, reduce the measurement efforts and provide flexibility of mobility management, we think a unified solution for L1 and L3 measurement can be considered.

When discussing the AI/ML based RRM use cases, the potential integration for L1 and L3 measurement should be also taken into consideration. Before the unified approach is concluded, the use cases can still be discussed, but the common design can be used as much as possible.

Proposal 20: 	In Rel-20 6G SI, the mechanism for AI/ML based L1 and L3 measurement can be designed as common as possible to accommodate the unified L1/L3 measurement framework.

- Conclusion

In this section, allow us to repeat our proposals and observations as below.

General consideration:

Proposal 1: 	For RAN4-driven use cases, reuse the guidance that agreed in RAN1 #122 meeting as starting point, and add “Assumption on testability”.

Observation 1: 	For 6G, RAN4 should take a comprehensive approach to LCM testing to fully support AI use cases throughout their lifecycle and enable native AI in 6G.

Proposal 2: 	For 6G, More LCM features, e.g., data collection, model selection, training, update, and UE capability, need further study in RAN4 AI, including testability issues, core requirements, performance requirements.

Proposal 3: 	For 6G, further study is needed on model standardization and deployment, RAN4 needs to consider following two aspects:

      - Effective Model Design, to ensure the reference models developed by RAN4 are practical and feasible for field implementation.

      - Constraints on Model Deployment, e.g., to study and define what makes a model "complex" or "simple” and understand the power consumption differences between different models.

Observation 2:	The over-simplified OTA assumptions being used may offer little practical guidance for field deployment.

Proposal 4: 	For 6G, RAN4 could explore whether/how to decouple the signal acquisition testing (e.g., for model inputs) from model performance evaluation (e.g., for model performance) for AI use cases.

Proposal 5: 	Signal acquisition could continue to be tested via cable or OTA methods, e.g., evaluating BM Set B measurement accuracy.

Proposal 6: 	Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets), enabling a virtual testing environment.

Observation 3: 	For computing resources, in current terminal devices, most computing resources are located outside the modem. Computing resources inside the modem, especially dedicated processors like GPUs or NPUs, are very limited.

Observation 4: 	For wireless AI use cases, available computing resources may fall into:

- AI computing resource on device, but outside modem

- Non-AI computing resource on device, but outside modem

- AI computing resource inside modem

- Non-AI computing resource inside modem

Observation 5: 	UE with different capabilities may support different reference models. If RAN4 defines reference models without taking into account the diverse AI capabilities of UEs, it could lead to AI features being available only to high-end devices.

Proposal 7: 	For 6G, RAN4 could study/identify these factors and estimate whether and how to introduce AI categories in RAN4 to serve different UE and different AI features. Following aspects could be considered first:

      - Computing Resource

      - Energy Consumption

      - Memory Tiers

      - Types of Computing Resources

      - Location of Computing Resources

      - Supported Reference Models

Proposal 8: 	The feasibility of power consumption testing could be studied from 6G day1 in RAN4 AI.

      - Whether/how to use flops/ops to reflect the AI power consumption.

      - Further study other impacts that may affect the AI power consumption (if any, e.g., due to different AI computing resource/location, different deployment assumptions) .

RF related use case:

Proposal 9: 	For 6G, Non-linearity Compensation (NC) could be taken into account in 6G, including:

      - Option 1: UE side DPD (Digital Pre-Distortion)

      - Option 2: NW side DPoD (Digital Post-Distortion) and Receiver

      - Note: model updates/tuning could be taken into account

Proposal 10:  	For NW side DPoD (option2), following sub cases could be evaluated in RAN4:

      - Option 2a: NW side NC by AI demod

      - Option 2b: NW side NC by a separate AI NC module +Non-AI demod

      - Option 2c: NW side NC by a separate AI NC module + AI demod

      - Option 2x: other solutions

      - Note: channel estimation may/may not be included

Proposal 11: 	For Non-linearity Compensation (NC) study in RAN4,

      - Alignment on simulation assumption is needed

      - PA Non-linearity modeling should be clarified

      - Definition of evaluation method/metric is needed

Demod related use case:

Proposal 12: 	For 6G, AI based Receiver enhancement could be taken into account in 6G, e.g., for high accuracy channel estimation, SRS insertion loss compensation, high-order modulation

RRM related use case:

Observation 6: 	Spatial domain prediction, time domain prediction and the combination of time domain and spatial domain prediction for L1 beam measurement are studied and specified for beam management in 5G NR.

Observation 7: 	Only DL Tx beam prediction is specified for beam management in 5G NR.

Observation 8: 	For measurement prediction and indirect event prediction in AI mobility, time domain and co-located frequency domain prediction for cell level measurement are included, while spatial domain prediction was not included for UE sided model in 5G NR.

Observation 9: 	L3 cell level measurement prediction for non-collocated frequency domain and L3 beam level measurement prediction for all domains are not included in 5G NR.

Observation 10:  Some issues are observed for the definition of AI/ML based RRM use cases and scenarios in 5G NR, e.g.,

      - There are some overlapping for AI prediction part between different use cases.

      - Too many combinations exist based on the report type, sub-use case and prediction domain. But it is not clear whether each combination is supported.

Proposal 13: 	Reconsider the definition of measurement prediction use cases in 6G AI/ML based RRM by decoupling the AI model implementation and report procedures, i.e., the use case is defined based on AI model input and output.

Observation 11: The UE measurements for mobility may need gap configuration or cause scheduling restrictions or interruptions which will impact the system throughput.

Observation 12: The UE measurements delay will to be extended due to limited measurement resources or gap sharing when multiple layers are configured.

Proposal 14: 	The study goals in 5G NR including to reduce the measurement efforts and to improve handover performance can be still applied in 6G AI based RRM study.

Proposal 15: 	The following AI/ML based measurement prediction use cases can be included in 6G:

      - L3 cell measurement prediction

      - L3 beam measurement prediction

      - L1 beam measurement prediction

Proposal 16: 	For each measurement prediction use case, consider the realistic prediction approach, e.g.,

      - For L3 cell measurement, consider time and frequency domain prediction.

      - For L1/L3 beam measurement, beside the time domain prediction, consider the combination of spatial and time domain prediction, and the combination of spatial and frequency domain prediction.

Proposal 17: 	The frequency domain prediction for non-collocated scenario can be considered in 6G SI starting from a stationary scenario. The generalization issue needs further study referring to the discussions in 5G NR.

Proposal 18: 	Some issues need to be resolved before supporting Rx beam or Rx-Tx beam pair prediction in 6G SI, e.g.,

      - The definition of AI model input and output

      - The handling of different cells for different Rx beams

      - The difference from Tx beam prediction and cross-cell prediction

Proposal 19: 	RAN4 to consider new use cases by applying the measurement prediction and event prediction to existing RRM procedures, e.g.,

      - AI/ML based RLM/BFD/CBD, LTM

      - AI/ML based SCell related enhancement: SCell addition/activation/change

Proposal 20: 	In Rel-20 6G SI, the mechanism for AI/ML based L1 and L3 measurement can be designed as common as possible to accommodate the unified L1/L3 measurement framework.

- Appendix

  - Preliminary Simulation results for AI based Non-linearity Compensation

Simulation assumption

| Parameters | Value |
| --- | --- |
| Carrier frequency | 3.5GHz |
| TX antenna number | 1 |
| RX antenna number | 2 |
| Waveform | CP-OFDM |
| Sub-carrier number | 64 |
| OFDM symbol number | 12 |
| Modulation | QPSK/16QAM/64QAM/256QAM |
| Channel coding | LDPC(CR=0.5) |
| Channel model | CDL-C, 100ns |
| Subcarrier spacing | 30 kHz |
| UE speed | 10m/s |
| Channel estimation | Ideal |
| PA setup (in simulation) : The nonlinearity of PA is modeled using the RAPP model [2], as shown below, where - 𝐴_𝑖𝑛 denotes the input amplitude, - 𝐴_0=1 represents the limiting output amplitude - 𝑣=1 is small signal gain - 𝑝=2 is defined as the smoothness parameter - 𝐺(⋅) signifies the output amplitude |  |

Simulation result

Preliminary simulations have been conducted to evaluate the performance of the DPD and DPoD schemes based on the assumptions provided in the previous table. The nonlinearity of the PA is modeled using the RAPP model [2], which has been widely adopted in academia. Evaluation results indicate that the PA nonlinearity introduced by the RAPP model leads to approximately 32% error vector magnitude (EVM) at the transmitter side, resulting in significant signal distortion. ​​ Accordingly​​, it is essential to incorporate a more realistic PA nonlinearity model that closely reflects practical systems in subsequent evaluations.

Figure 13 illustrates the BLER performance of the DPD scheme compared with a baseline ​​which excludes PA nonlinearity compensation​​, under different modulation orders. It can be observed that the performance degradation caused by PA nonlinearity becomes more severe as the modulation order increases. The DPD scheme is shown to effectively compensate for the impact of PA nonlinearity, maintaining performance nearly identical to that under the ideal PA assumption, even at higher modulation orders.

As a comparison, Figure 14 shows the BLER performance of the DPoD scheme. As presented in the second and third subplots of Figure 14, two variants of the DPoD scheme are evaluated: one using an AI-based nonlinear compensation (NC) module with a non-AI demodulator (referred to as DPoD-opt1), and the other employing both an AI-based NC module and an AI demodulator (referred to as DPoD-opt2). The results indicate that the introduction of DPoD yields performance gains compared to the system without AI-based NC, particularly under higher-order modulation. Furthermore, jointly optimizing both the NC module and the demodulator using AI provides more substantial gains than optimizing the NC module alone, especially at high modulation orders. This can be attributed to the fact that the receiver-side NC module alone cannot fully compensate for the distortion introduced by PA nonlinearity, while the AI demodulator is better adapted to handle the impaired signals. However, the DPoD schemes do not achieve the same performance level as DPD. A possible explanation is that the signal distortion at the receiver results from the combined effects of channel fading, noise, PA nonlinearity, and other impairments, which increases the complexity of NC at the receiver side.

In subsequent RAN4 studies, it is essential to first align simulation assumptions and evaluation metrics among companies, to validate results across different companies.

Figure 13 Evaluation results of the DPD scheme

Figure 14 Evaluation results of the DPoD scheme

The model complexity is summarized in the following table.

| Model | FLOPs | Trainable parameters |
| --- | --- | --- |
| DPD | 0.57M | 0.4K |
| DPoD(NC module） | 57.6M | 40K |
| DPoD(demod. module） | 7.0M | 4K |

- References

- RP-251881, New SID: Study on 6G Radio, RAN#108, June 2025

- Kenington, Peter B. "Methods linearize RF transmitters and power amps." Microwaves & RF 37.12 (1998): 102-116.

- RP-243244 Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-252899 Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR