3GPP TSG RAN WG4 Meeting #116-bis	R4-2513324

Prague, CZ, 13th – 17th Oct, 2025

Agenda item:			8.9

Source:	vivo

Title:	Initial discussion on 6G AI for RAN4 driven cases

Document for:	Discussion

- Introduction

The RAN WG study on 6G radio has been approved by RAN#108 as SID in [1]. The AI-relevant objective of RAN4 is copied below for reference.

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4]

Also, as indicated in [2], multiple topics are provided for initial 6G RAN4 discussion from Q4 2025 to Q1 2026. AI, which has been previously explored in AI/ML PHY and AI/ML mobility study, is one of the topics for initial 6G discussion.

In this contribution, we will analyze several potential aspects specific to RAN4 that could potentially be enhanced through AI/ML-based prediction.

- Discussion

  - AI-based Optimization for Pre-Configured Measurement Gap for Inter-frequency Measurement

    - Background

In Rel-17, RAN4 discussed and introduced the Pre-configured Measurement Gap, which enhances measurement flexibility through pre-configuration and conditional activation/deactivation mechanisms. According to TS 38.133 Section 9.1.7, Pre-MG allows the network to pre-configure gap patterns via RRC signalling and supports two activation/deactivation modes:

- UE autonomous mode: The UE adjusts the state based on events (such as BWP switching, SCell activation, measurement object changes, etc.).

- Network-controlled mode: The network controls the state via preConfGapStatus.

Compared to traditional gap configuration, Pre-MG reduces signalling overhead and introduces activation/deactivation mechanisms. However, it also has significant limitation. The activation/deactivation is driven by events that have already occurred and the UE responds to all predefined events regardless of signal stability, possibly leading to redundant gap activation in stable scenarios or untimely measurements in scenarios where signal quality changes rapidly.

Meanwhile, in the Rel-19 study item on AI/ML mobility, one of the scenarios focuses on quality prediction for inter-frequency cells, where measurements from one frequency band are used to predict the quality of another frequency band, thereby reducing or replacing the need for actual measurements on inter-frequency cells.

In the context of inter-frequency prediction, two scenarios may be considered. In one scenario, inter-frequency prediction eliminates the need for measurement entirely, allowing decisions (such as for handover) to be made directly based on the prediction results and no measurement will be performed for inter-frequency cells. In the other scenario, necessary inter-frequency measurement is still need. For example, in certain cases, even if the quality of an inter-frequency cell is predicted to be poor, the UE still needs to check the quality of the inter-frequency cell by actual measurements, which requires the use of measurement gaps.

Therefore, based on the understanding of the second scenario above, we believe that one possible enhancement direction for RAN4 is to combine AI-driven inter-frequency prediction with the activation/deactivation logic of Pre-MG.

2.1.2 Possible scenario

One possible combination is to integrate inter-frequency prediction into the Pre-MG decision as a triggering condition to the existing event-based logic. For Network-controlled mode, the AI prediction results serve as an information for network decision-making; For UE-controlled mode, AI prediction can be a basis for determining the Pre-MG state. For example, when an AI model predicts a change in inter-frequency signal quality in the near future, this prediction result can serve as a supplementary triggering condition for activating/deactivating the MG. Either the UE or the network side can determine whether and when to activate/deactivate the MG based on the prediction results.

It potentially can help to optimize the traditional event-driven Pre-MG by introducing AI prediction as a trigger condition. In scenarios with stable signals, it can deactivate unnecessary gaps in advance, extending gap-free transmission time.

Proposal 1: RAN4 to consider to integrate AI/ML-based inter-frequency prediction as a trigger condition for Pre-configured Measurement Gap (Pre-MG) activation/deactivation, to enhance measurement timeliness in dynamic scenarios and reduce unnecessary gaps in stable conditions.

  - AI-based Optimization for neighbor cell measurement initiation mechanism

2.2.1	Background

In the current NR system, s-measure serves as the neighbour cell measurement initiation threshold. It controls the start and stop of neighbour cell measurements by comparing the UE's current measurement value with a static threshold, making it a key mechanism for mobility management in the CONNECTED mode. When the serving cell RSRP measured by the UE falls below this threshold, intra-frequency, inter-frequency, or inter-RAT neighbour cell measurements are initiated.

However, the existing mechanism relies solely on the instantaneous value of the UE's current RSRP. The pre-configured s-measure threshold is difficult to accommodate diverse scenarios, including stationary/mobile UEs and cell-centre/cell-edge conditions. For example, high-mobility UEs at the cell edge experience rapid signal changes, where a fixed threshold may trigger measurements too late. Additionally, when RSRP temporarily drops below the threshold due to interference, the existing mechanism immediately initiates continuous measurements, leading to unnecessary measurement activities. Such "false triggers" caused are not effectively filtered, resulting in wasted UE power consumption.

2.2.2 Possible scenario

Based on the above analysis of the existing s-measure mechanism, we can consider leveraging AI prediction. This would implement a decision mechanism combining "static thresholds + AI prediction enhancement".

When AI predicts that the RSRP will be stable in the short term or that the UE will remain near the cell centre, even if the instantaneous RSRP drops below the threshold due to temporary interference rather than real signal degradation, continuous measurements could be avoided. The benefit of this approach is that by using AI prediction to achieve measure in advance when needed, avoid blind measurements when ineffective.

Proposal 2: RAN4 to consider to enhance s-measure mechanism by integrating AI-based prediction to enable measurement triggering and reduce unnecessary measurements.

  - AI-based Optimization for RLM Measurement Mechanism

In the current design, the Radio Link Monitoring (RLM) mechanism requires the UE to periodically measure RLM-RS (SSB or CSI-RS) to assess downlink quality, which may present certain measurement restrictions as defined in current spec:

- For SSB-based RLM, when an SSB overlaps in the time domain with CSI-RS or other SSBs under specific conditions (e.g., different SCS, UE does not support simultaneous reception with different numerologies), the UE must choose one measurement in case of conflict. This may lead to extended RLM measurement periods.

- For CSI-RS-based RLM, in FR2 frequency bands, symbol-level collisions between CSI-RS and SSBs or other CSI-RS resources can trigger similar restrictions, further increasing measurement uncertainty.

To reduce UE power consumption and alleviate the impact of measurement restrictions, one possible approach is to reduce the measurement frequency for RLM-RS. We can consider extending AI prediction technology to RLM measurement prediction. The difference between this prediction and the AI/ML for predicting RLF scenario in the Rel-19 AI/ML Mobility Study lies in the fact that RLF prediction directly or indirectly forecasts whether an RLF event will occur at a future time based on actual measurements or based on predicted and optionally actual L1-SINR results. And here, the prediction for RLM measurement is more focused on replacing actual measurements with predictions to reduce the measurement of RLM-RS, at least under stable cell quality condition.

Proposal 3: RAN4 to consider AI-based RLM measurement prediction.

  - Tx-Rx beam pair prediction Mechanism

The Rel-19 AIML BM defined two core use cases for Tx beam prediction in spatial and temporal domains: BM-Case1 and BM-Case2. In the finalized standardization in RAN4, it mainly focused solely on individual DL Tx beam prediction case. And for Tx-Rx beam pair prediction, it is reflected via UE capability that a UE can indicate to the network whether it supports the capability to provide associated Rx beam information simultaneously when acquiring predicted Tx beam information. This capability definition opens the possibility for research on the utilization of Rx beam knowledge when predicting beam pairs.

Further, for Tx-Rx beam pair prediction, we also have identified some potential benefits: Compared to individual DL Tx beam prediction, our preliminary simulation evaluation shows that Tx-Rx beam pair prediction can help improve Tx beam prediction performance. Additionally, in the current procedure, even if the network side has obtained the predicted Tx beam, without UE capability, the UE still needs to verify and determine the optimal Rx through Rx beam sweeping, which requires a relatively long time and may result in the previously predicted optimal Tx beam having already changed and become invalid by the time this process is completed.

Therefore, we propose to continue exploration based on beam pair prediction with consideration of the gains brought by improvements in Tx beam prediction performance and the utilization of Rx beam knowledge when predicting beam pairs.

Proposal 4: RAN4 to consider further exploration based on Tx-Rx beam pair prediction

  - AI-based RF transmitter/receiver

Coverage enhancement and energy efficiency are high-priority objectives for 6GR given that 6G's coverage in the 7GHz band is expected to be comparable to 5G NR's coverage in the 3.5GHz band, achievable without increasing number of sites deployed.

Increasing power level of power amplifier (PA) poses another significant challenge, often with reduced power efficiency or higher PA non-linearity, which would lead to difficulties to meet requirements on EVM, ACLR, Spectral Emission Mask (SEM), Occupied bandwidth (OBW), and In-Band Emissions (IBE). AI/ML-based techniques offer potential solutions to improve PA linearity, this is also discussed in RAN1 AI sections as part of AI use cases.

However, given AI-based PA nonlinearity improvement is highly related to RF power boosting and many RF requirements, e.g., EVM, ACLR. We think RAN4 should also start this scope discussion and identify any RF impacts.

AI-based PA nonlinearity improvement may consider different techniques, e.g, high power efficiency (HPE) waveforms, digital predistortion (DPD), and digital post-distortion (DPoD), aiming to achieve better cost-efficiency and performance trade-offs. In our preliminary simulations as shown in Table 1, we identify 1-3dB gain for different cases, e,g, AI-based DPD and DPoD shows about 3dB gain for 256 QAM for CP-OFDM.

Table 1 Performance summary of candidate schemes

| Schemes | Total Gain | Beneficial SNR range |
| --- | --- | --- |
| HPE waveform | 1.5-3 dB @QPSK/16QAM ~1.5 dB @64QAM | Low SNR medium SNR |
| DPD | ~1dB @QPSK/16QAM/64QAM ~3dB @256QAM | All SNR range |
| DPoD | ~3dB @256QAM | High SNR range |

Furthermore, if the AI-based solution is dependent on some RF requirements relaxation, e.g., UE Tx EVM relaxation (AI-based solution is considered as 6G scope), then the capability reporting might be needed as well. Overall, we think the RF requirements impact of AI-based transmitter and receiver should be studied in RAN4.

Proposal 5: RAN4 should study the UE/BS RF requirement impact for AI-based RF transmitter/receiver (e.g., PA nonlinearity improvement) to achieve better coverage.

- Summary

Proposal 1: RAN4 to consider to integrate AI/ML-based inter-frequency prediction as a trigger condition for Pre-configured Measurement Gap (Pre-MG) activation/deactivation, to enhance measurement timeliness in dynamic scenarios and reduce unnecessary gaps in stable conditions.

Proposal 2: RAN4 to consider to enhance s-measure mechanism by integrating AI-based prediction to enable measurement triggering and reduce unnecessary measurements.

Proposal 3: RAN4 to consider AI-based RLM measurement prediction.

Proposal 4: RAN4 to consider further exploration based on Tx-Rx beam pair prediction

Proposal 5: RAN4 should study the UE/BS RF requirement impact for AI-based RF transmitter/receiver (e.g., PA nonlinearity improvement) to achieve better coverage.

- Reference

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

- R4-2511652, Plan on RAN4 6G discussion and meeting arrangement in Rel-20, Apple, RAN4 115