# 3GPP TSG-RAN WG4 Meeting #116-bis                                                                    R4-2513330

Prague, CZ, October 13th – 17th, 2025

Agenda Item:	8.3

Source: 	CSCN

Title: 	Views on 6G system parameters

Document for:	Discussion

- Introduction

In RAN #108 meeting, a New SID for “Study on 6G Radio” was approved [1] aims to develop 6GR to support a broad range of use cases, and meet requirements as defined during the RAN requirements study. In RAN4#116 meeting, RAN4 6G Plan and Meeting Arrangement was approved [2], and RAN4 6G Topic Planning based on 6G SID is shown as following.

Agreement

In this contribution, some views on the topics of system parameters for 6GR are provided, especially for 6G NTN (Non-Terrestrial Network).

- Discussion

The vision for 6G is to bridge the digital divide in its entirety and realize seamless, integrated connectivity across all domains: aerial, spatial, terrestrial and maritime. In the context of 6G, NTN is not merely a significant use case, it constitutes a fundamental architectural component and is indispensable to the core technology framework.

Considerations should be given to aspects of 5GR that require refinement or simplification for 6G design. The differences between 6G and 5G considerations from the perspectives of system parameters are illustrated in the following.

  - 2.1  SCS

In 5G-NR, the SCS design is highly flexible with multiple SCS options in a band, not only different Bandwidth Parts (BWPs), but the Synchronization Signal Block (SSB) and the initial Downlink Bandwidth Part (initial DL BWP) also support different SCSs. However, in actual 5G-NR deployments, most of the aforementioned flexibilities are not realized. For example,60kHz SCS for FR1 is rarely used. Therefore,6GR can eliminate these redundant and unused flexibility to reduce unnecessary implementation and testing complexity for both gNB and UE.

From the agreements achieved in RAN1#122, we observe that for FR1 (sub 6GHz) band, support 15kHz SCS for FDD and 30kHz SCS for TDD. Meanwhile, for FR2 band (24.25GHz-52.6GHz), 120kHz SCS is supported. For 6GR, we propose that the FDD mode below 6GHz needs to support 30kHz SCS. From a technical perspective, 30kHz SCS in FDD bands demonstrates several obvious benefits over 15kHz SCS especially in frequency offset resistance and low latency performance under high speed scenarios e.g. terminal mounted over train, car, airplane and ship.

Proposal 1: For 6GR bands, consider reducing the SCS options set.

Proposal 2: For 6GR NTN bands, consider only two SCSs for each new bands.

Proposal 3: For sub-6GHz (FR1) and FR2(under 10G), 30kHz SCS should be supported for FDD mode in additional to 15kHz.

  - 2.2  Duplex Mode

In 5G-NR, both TDD and FDD modes are supported in FR1, but only TDD mode is supported in FR2-1. Half-duplex (HD-FDD) mode is beneficial for reducing device complexity and cost, and only defined for redcap UE in 5G-NR now.  In 6NR, more UE types may involve to access 6GR. Similar to TDD and FDD, HD-FDD can be configured as a fundamental capability option for UE. For example, HD-FDD is quite suitable for mobile VAST type in NTN.

Currently, many spectrum resources have already been allocated to existing TN, making spectrum resources relatively scarce, particularly in the lower bands. For 6G NTN systems, TDD may be a reasonable solution in these spectrum-constrained lower bands due to its high spectral efficiency. However, the adoption of TDD requires addressing the challenge of long Guard Periods (GP) caused by large Round-Trip Times (RTT). The existing TDD frame structure does not support such prolonged GP requirements. Thus, extension of the TDD cycle duration should be considered.

Proposal 4: In 6GR NTN, TDD mode could be adopted to improve spectrum utilization efficiency.

Proposal 5: In 6GR NTN, it is worth to study new DL-UL pattern which is suitable for NTN deployment scenario.

Proposal 6: In 6GR, HD-FDD mode should be supported for NTN device type.

  - 2.3  CHBW

In 5G-NR, maximum FFT size of 4K is assumed, which can natively support up to 50MHz/100MHz/400MHz for 15KHz/30KHz/120KHz respectively. For 6GR, it is necessary to determine the FFT size for the Orthogonal Frequency Division Multiplexing (OFDM) family of waveforms. Hardware computing resources of 6G equipment are expected to achieve a significant development compared with 5G. Assuming 6GR can support higher FFT complexity (e.g.,8K), a larger FFT size can support a wider system bandwidth (when SCS is fixed), thereby improve the peak rate and spectral efficiency of a single carrier. Hence, we have

Proposal 7: Under the premise that the maximum FFT size of 6GR is 8K at least for some special scenarios,

- For 30kHz SCS, a maximum bandwidth of 200MHz should be supported.

- For 120kHz SCS, a maximum bandwidth of 800MHz should be supported.

  - 2.4  Rx & Tx

The number of transmit and receive antennas in a terminal directly affects its performance, size, and power consumption. In 5G NR, he number of transmit antennas can be 1/2/4, while the number of receive antennas can be 1/2/4/ 8, with 8-antenna configurations typically used in Fixed Wireless Access (FWA) scenarios. Due to the predominance of Line-of-Sight (LoS) paths in satellite-terrestrial propagation channels, and considering satellite implementation complexity and power efficiency, the satellite payload employs single circular polarization antennas on UL or DL to support single-stream transmission. 5G NTN terminals for FR1 supports 2RX reception as baseline to to prevent depolarization loss and VAST for FR2 supports 1RX as baseline. Given the single-stream transmission and antenna layout constraints, it is recommended to support up to 2Tx and 2Rx.

Proposal 8: Recommend up to two receivers and two transmitters for 6GR NTN terminals.

  - 2.5  Spectrum

The currently available spectrum resources for 5G-NTN are relatively limited. On one hand, 6GR-NTN may consider reusing the existing frequency bands of 5G-NTN, on the other hand, portions of the 5G terrestrial network (TN) spectrum could also be allocated for supplementary coverage in NTN networks, though it must be ensured that NTN operations do not cause any interference to TN networks. NTN can be deployed either as an independent network or as a comprehensive coverage solution together with TN.

Proposal 9: 6GR NTN reuse 5G-NTN spectrum resources.

Proposal 10: 6GR NTN may share the spectrum resource of TN network on top of further study on TN-NTN co-existence.

  - 2. 6 Device Types

6G targets to achieve harmonized design for variety device types. There are diverse NTN device types considering different form factors and operating bands. Both handheld UE, IoT device type and VSAT can be supported for NTN operation. On VSAT devices, NR specified 6 VSAT types for supporting satellite scenario (NGSO, GSO scenario), mobility (fixed or mobile) and antenna type (electronical beam steering, mechanical beam steering). In Rel-20, VSAT with small antenna size for vehicular use case is also under discussion.

Proposal 11: 6GR NTN should support diverse NTN device types including handheld UE, IoT device and VSAT with different RF/BB capabilities.

- Conclusion

In this contribution, some views on the topics of system parameters for 6G NTN are provided.

Proposal 1: For 6GR bands, consider reducing the SCS options set.

Proposal 2: For 6GR NTN bands, consider only two SCSs for each new bands.

Proposal 3: For sub-6GHz (FR1) and FR2(under 10G), 30kHz SCS should be supported for FDD mode in additional to 15kHz.

Proposal 4: In 6GR NTN, TDD mode could be adopted to improve spectrum utilization efficiency.

Proposal 5: In 6GR NTN, it is worth to study new DL-UL pattern which is suitable for NTN deployment scenario.

Proposal 6: In 6GR, HD-FDD mode should be supported for NTN device type.

Proposal 7: Under the premise that the maximum FFT size of 6GR is 8K at least for some special scenarios,

- For 30kHz SCS, a maximum bandwidth of 200MHz should be supported.

- For 120kHz SCS, a maximum bandwidth of 800MHz should be supported.

Proposal 8: Recommend up to two receivers and two transmitters for 6GR NTN terminals.

Proposal 9: 6GR NTN reuse 5G-NTN spectrum resources.

Proposal 10: 6GR NTN may share the spectrum resource of TN network on top of further study on TN-NTN co-existence.

Proposal 11: 6GR NTN should support diverse NTN device types including handheld UE, IoT device and VSAT with different RF/BB capabilities.

Reference

- R4-2511652, “RAN4 6G Plan and Meeting Arrangement”.

- RP-251881, New SID, Study on 6G Radio, NTT DoCoMo