3GPP TSG-RAN WG4 Meeting #116bis	R4-2513334

# Prague, Czech Republic, Oct. 13-17, 2025

Source: 	KDDI　Corporation

Title: 	Discussion on spectrum sharing for 6GR

Agenda Item:	8.10

Document for:	Discussion

- 1. Introduction

In RAN#108, the Rel-20 SID was approved that aimed at enabling the 6G radio access technology for the next-generation radio access system capable of addressing the full spectrum of future use cases, operational constraints, and deployment realities [1]. And then, in RAN4#116, RAN4 6G Topic Planning based on 6G SID was also shared by RAN4 chair as follows [2]. In this contribution, we focus on 5G and 6G spectrum sharing and propose some issues.

- 2. Discussion

  - Lessons and learn from DSS between LTE and 5G NR

Rel-17 Minimum requirements for CRS-IM

In Rel-17, RAN4 standardized CRS-IM feature as “optional” and it works for mitigating NR throughput degradation in PCI Mod-3 unmatched case.

DSS interference issue between LTE and 5G NR

In Rel-17, RAN4 standardized CRS-IM feature as “optional” and it works for mitigating NR throughput degradation in PCI Mod-3 unmatched case.

With some UEs, NR throughput degradation was critical in Mod-3 unmatched case. The inability to mitigate NR throughput degradation caused by LTE CRS interference from adjacent cells is hindering the deployment of DSS infrastructure.

Observation 1: Rel-17 CRS-IM feature works for mitigating 5G NR throughput degradation in Mod-3 unmatched case.

Proposal to mandate Rel-17 CRS-IM for UE

In RAN#105, we proposed to mandate Rel-17 CRS-IM feature “crs-IM-DSS-15kHzSCS-r17” for UE and some operators supported it. However, the proposal was not agreed eventually.

Observation 2: Proposal to mandate Rel-17 CRS-IM feature for UE was not agreed in 5G NR.

  - MRSS between 5G NR and 6GR

When it comes to 5G NR, its flexibility of control channels increase compared to 4G. Actually, there are not specific fixed reference signals like LTE CRS. Therefore, regarding the study of MRSS, firstly RAN4 needs to know and understand whether there are technically any interference issues or not, based on outcomes and progress of 6GR control channels’ design in RAN1.

Proposal 1: RAN4 needs to know and understand whether there are technically any interference issues or not, based on outcomes and progress of 6GR control channels’ design in RAN1.

And then, if RAN4 identify possibilities on any interference issues based on RAN1’s outcomes and progress, RAN4 need to study candidate solutions for the issues and expect to mandate related features for 6GR UE in order to avoid any performance degradation due to interference issues between 5G NR and 6GR. This is because even if 6GR use PCI similar to LTE and 5G NR, generally speaking operators can’t ensure to construct PCI Mod-3 matched network.

Proposal 2: If RAN4 identify possibilities on any interference issues based on RAN1’s outcomes and progress, RAN4 need to study candidate solutions for the issues and expect to mandate related features for 6GR UE.

- 3. Conclusion

Observation 1: Rel-17 CRS-IM feature works for mitigating 5G NR throughput degradation in Mod-3 unmatched case.

Observation 2: Proposal to mandate Rel-17 CRS-IM feature for UE was not agreed in 5G NR.

Proposal 1: RAN4 needs to know and understand whether there are technically any interference issues or not, based on outcomes and progress of 6GR control channels’ design in RAN1.

Proposal 2: If RAN4 identify possibilities on any interference issues based on RAN1’s outcomes and progress, RAN4 need to study candidate solutions for the issues and expect to mandate related features for 6GR UE.

- 4. Reference

[1] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone, RAN#109, Sep 15-18, 2025

[2] R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 chair, RAN4#116, Aug 25–29, 2025

[3] RP-242339, Mandatory proposal for Rel-17 CRS-IM, RAN#105, KDDI, China Telecom, Orange, Sep 9-12, 2024