3GPP TSG-RAN WG4 Meeting #116bis	R4-2513344

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	6.1.2

Source:	MediaTek Inc.

Title:	Discussion on demodulation and CSI requirements for 6Rx UE

Document for:	Discussion

# Introduction

Based on the WF of demodulation and CSI requirements for 6Rx UE [1], we provide our perspectives on the unresolved issues in this contribution.

- Discussion

## 2.1 General

Issue 1-1: Test applicability rule for 6Rx tests - Proposals - Option 1: Introduce the test applicability rules by following core part agreement. - 6 MIMO layers performance requirements will be only applicable for FWA devices - 4 MIMO layers performance requirements will be applicable to both FWA and handheld UE devices. - Option 1a: Introduce UE declaration for the proper test applicability design if no explicit capability signalling finally confirmed by RAN2 - Option 2: Postpone the discussion until RAN2 concludes the UE capability signalling discussion Issue 1-3: Specification structure - Proposals - Option 1: Add a new clause for 6Rx after the current 8Rx performance requirements - Section 5.2.5 6Rx requirement - Option 2: Modify the current section numbering and put 6Rx requirements between 4Rx and 8Rx in the new Rel-19 specification - Section 5.2.3 4Rx requirements - Section 5.2.4 6Rx requirements - Section 5.2.5 8Rx requirements

With respect to UE capability, RAN#109 officially approved RAN2 CR [2][3], which introduces maxNumberMIMO-LayersPDSCH-v1900 exclusively for FWA. This capability clearly indicates the 6 MIMO layers performance is only applicable to FWA.

maxNumberMIMO-LayersPDSCH, maxNumberMIMO-LayersPDSCH-v1900 Defines the maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2 (except for FR2-NTN). If supportOf2RxXR is indicated, for single CC standalone NR, it is mandatory with capability signalling to support 2 MIMO layers in the bands specified in Table 7.3.2-2b in TS 38.101-1 [2]. If absent, the UE does not support MIMO on this carrier. The maxNumberMIMO-LayersPDSCH-v1900 is only included for FWA UE as defined in TS 38.101-1 [2]. The UE supporting maxNumberMIMO-LayersPDSCH-v1900 shall also indicate support of maxNumberMIMO-LayersPDSCH for the corresponding component carrier. When maxNumberMIMO-LayersPDSCH and maxNumberMIMO-LayersPDSCH-v1900 are reported together for a component carrier, the network which is able to decode maxNumberMIMO-LayersPDSCH-v1900 ignores maxNumberMIMO-LayersPDSCH. For the bands where pdsch-1024QAM-2MIMO-FR1-r17 is indicated, MIMO layers for 1024 QAM is the smaller value between 2 and maxNumberMIMO-LayersPDSCH.; FSPC; CY; N/A; N/A.

Proposal 1: 6 MIMO layers performance requirements are only applicable for FWA devices.

As for specification structure, modifying the current clause numbering for 4Rx and 8Rx would significantly increase the complexity of CR maintenance. Such changes could lead to unnecessary confusion and inefficiencies in managing updates and revisions. To ensure clarity and streamline the process, we propose adding new clauses specifically for 6Rx. This approach not only preserves the integrity of the existing structure but also provides a dedicated space to address the unique requirements and parameters associated with 6Rx, facilitating better organization and future scalability.

Observation 1: Modifying the current clause numbering for 4Rx and 8Rx would significantly increase the complexity of CR maintenance.

Proposal 2: Add a new clause for 6Rx after the current 8Rx performance requirements.

## 2.2 PDSCH requirements

Issue 2-2-1: Test cases and MIMO correlation for 4 MIMO layers for FWA and Handheld UE - Proposals - Option 1: One single test case that is applicable for both FWA and Handheld UE - Option 1a: α = 0 and β = 0.07179 - Option 1b: α = 0 and β = 0 - Option 2: α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively

Handheld UE and FWA devices differ significantly in terms of form factor and mobility. The form factor of FWA equipment is generally larger and intended for stationary use, providing a stable and reliable internet connection to fixed locations such as homes, offices, and buildings. These devices are often mounted on structures or installed in designated areas to ensure optimal performance. In contrast, handheld UE devices feature a compact form factor for ease of portability, allowing users to conveniently carry and operate them in various environments. Given that differences in form factor lead to variations in antenna placement, which in turn affect MIMO correlation, we propose defining two sets of requirements for 4L6R, each tailored to different MIMO correlation scenarios.

Observation 2: Handheld UE and FWA exhibit significant differences in form factor and mobility.

Proposal 3: Define two sets of requirements for 4L6R, each tailored to different MIMO correlation scenarios for FWA and Handheld UE, i.e., α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively.

Issue 2-3-1: Test applicability rule for FWA - Proposals - Option 1: FWA device can skip corresponding 4 layers test if it passes the FWA 6 layers tests - Option 2: Not introduce additional applicability rule to skip corresponding 4 layers tests if it passes the FWA 6 layers tests. Issue 2-3-2: DMRS configuration for 6 MIMO layers case - Proposals - Option 1: Rel-15 Type 1 DMRS 2+2 - DMRS ports {1000-1003, 1004, 1006} - Option 2: Rel-18 eType1 DMRS 1+1

To reduce the testing burden, we propose to introduce test applicability rules for UE skipping FWA 4 layers requirements if it can pass the FWA 6 layers requirements. As for DMRS configuration, we propose using Rel-15 2+2 to define requirements for FWA 6R6L as Rel-18 enhanced DMRS needs additional UE capabilities.

Proposal 4: Introduce test applicability rules to let UE skip FWA 4 layers requirements if it can pass the FWA 6 layers requirements.

Proposal 5: Use Rel-15 2+2 DMRS configuration to define 8T6R6L FWA requirements.

## 2.3 SDR requirements

Issue 3-1-2: SDR requirements for 1024QAM with 4 layers - Proposals - Option 1: Not Define SDR requirements for 1024QAM with 4 layers - Option 2: Consider MCS23 in Table 4 to define 6Rx SDR requirements with 1024QAM and 4 layers Issue 3-1-3: UE capability for support 1024QAM with 4 layers for 6Rx (if introduced) - Proposals - Option 1: Define UE capability pdsch-1024QAM-4MIMO-FR1-r17 for applicability rules if SDR 1024QAM 4-layer requirements for 6Rx is introduced - Option 2: Not define the UE capability pdsch-1024QAM-4MIMO-FR1-r17

Based on the predetermined static channel configuration for 4T6R, we have conducted simulations to assess the feasibility of SDR requirements for 4T6R4L 1024QAM. The results of these simulations are depicted in Figure x. The result demonstrates that the SNR required to achieve 85% throughput is below 29 dB. Given that the maximum achievable SNR for 1024 QAM is 32.04 dB (corresponding to an EVM of 2.5%), and considering the margin for RF impairments, it is appropriate to designate MCS23 as the highest achievable MCS for 1024QAM with 4 MIMO layers.

Figure 1. SDR simulation results for 4T6R4L MCS 23 (Table 4)

Observation 3: For 6Rx SDR requirements with 1024QAM and 4 layers, the SNR required to achieve 85% throughput is below 29 dB if MCS 23 in Table 4 is used.

Proposal 6: Given that the maximum achievable SNR for 1024 QAM is 32.04 dB (corresponding to an EVM of 2.5%), RAN4 can consider MCS 23 in Table 4 to define 6Rx SDR requirements with 1024QAM and 4 layers.

Currently, there are two UE capabilities related to 1024QAM defined in TS38.306, which are pdsch-1024QAM-FR1-r17 and pdsch-1024QAM-2MIMO-FR1-r17. If RAN4 agreed to define SDR 1024QAM 4-layer requirements for 6Rx, we suggest defining new UE capability pdsch-1024QAM-4MIMO-FR1-r17 for applicability rules.

| pdsch-1024QAM-2MIMO-FR1-r17 Indicates whether the UE supports 1024QAM modulation scheme for PDSCH with maximum 2 MIMO layers for FR1 as defined in TS 38.211 [6], MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [12]. UE indicating support of this feature shall also indicate support of pdsch-256QAM-FR1 and shall not indicate support of pdsch-1024QAM-FR1-r17. | Band | No | N/A | FR1 only |
| --- | --- | --- | --- | --- |
| pdsch-1024QAM-FR1-r17 Indicates whether the UE supports 1024QAM modulation scheme for PDSCH for FR1 as defined in TS 38.211 [6], MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214 [12]. UE indicating support of this feature shall also indicate support of pdsch-256QAM-FR1 and shall not indicate support of pdsch-1024QAM-2MIMO-FR1-r17. | Band | No | N/A | FR1 only |

Proposal 7: Define UE capability pdsch-1024QAM-4MIMO-FR1-r17 for applicability rules if SDR 1024QAM 4-layer requirements for 6Rx is introduced.

Issue 3-1-4: SDR requirements - Agreement - Decide the MCS values as per the agreed static channel matrix in Issue 3-1-1 and parameters in Table 5.5A-1/2/3 of TS 38.101-4 in the next meeting - QPSK/16QAM/64QAM/256QAM with 6 MIMO layers as below table Maximum number of PDSCH MIMO layers; Maximum modulation format; Scaling factor; MCS. 6; 8; 1; TBD. 6; 8; 0.8; 23. 6; 8; 0.75; 22. 6; 8; 0.4; 12. 6; 6; 1; TBD. 6; 6; 0.8; 24. 6; 6; 0.75; 23. 6; 6; 0.4; 14. 6; 4; 1; 16. 6; 4; 0.8; 16. 6; 4; 0.75; 16. 6; 4; 0.4; 11. 6; 2; 1; 9. 6; 2; 0.8; 9. 6; 2; 0.75; 9. 6; 2; 0.4; 5. Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 [12]. - 1024QAM with 2 MIMO layers as below table Supported RX antenna ports; Maximum number of PDSCH MIMO layers; Maximum modulation format; Scaling factor; MCS. 6RX; 2; 10; 1; TBD. 2; 10; 0.8; 21. 2; 10; 0.75; 19. 2; 10; 0.4; 9. Note 1: MCS Index for maximum modulation format 10 is based on MCS index Table 4 defined in clause 5.1.3.1 of TS 38.214 [12] Note 2: For the band(s) on which UE supporting “Maximum modulation format” of 10, with 2 RX and 2 MIMO layers, the MCS index is derived from the rows with “Maximum modulation format” of 8 in Table 5.5A-5.

RAN4 agreed to define SDR requirements for QPSK/16QAM/64QAM/256QAM with 6 MIMO layers and 1024QAM with 2 MIMO layers. We provide our initial evaluations in Table x, where the simulation assumptions are captured in [2].

Table x. Summary for the required SNR achieving 85% maximum throughput

| MCS | SNR(dB)@85% of maxTP |  |  |
| --- | --- | --- | --- |
|  | MCS Table 1 8T6R6L Fixed precoder with i1,1=i1,2=i2=0 | MCS Table 2 8T6R6L Fixed precoder with i1,1=i1,2=i2=0 | MCS Table 4 2T6R2L Random precoder |
| 28 | 21.2 |  |  |
| 27 | 19.2 | 26.2 |  |
| 26 | 18.1 | 25.1 |  |
| 25 | 17.1 | 24.1 | 31.6 |
| 24 | 16.1 | 23.1 | 27.1 |
| 23 | 15.0 | 22.0 | 24.8 |
| 22 |  | 20.3 |  |

Table x. EVM assumptions and corresponding SNR

| Modulation scheme for PDSCH | EVM assumptions | Corresponding SNR |
| --- | --- | --- |
| QPSK/16QAM/64QAM | 6 % | 24.44 dB |
| 256QAM | 3 % | 30.46 dB |
| 1024QAM | 2.5 % | 32.04 dB |

Based on the EVM assumptions in table x, we have the following proposal for MCS.

Proposal 8: Use the following MCS to define SDR requirements for QPSK/16QAM/64QAM/256QAM with 6 MIMO layers.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 27 |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | 28 |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Proposal 9: Use the following MCS to define SDR requirements for 1024QAM with 2 MIMO layers.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

## 2.4 CSI requirements

Issue 4-1-1: CQI test for 6Rx - Proposals - Option 1: Not define CQI test - Option 2: Only define one CQI requirements with rank 4 under static channel Issue 4-1-2: RI test for only Rank 6 - Proposals - Option 1: Not define RI tests for 6Rx - Option 2: Define RI test for 6Rx

For the CSI requirements, we believe there is minimal difference between 6Rx and 2Rx, as well as between 4Rx and 8Rx. Consequently, we suggest not defining specific CQI and RI requirements

Proposal 10: Not to define CQI requirements for 6Rx.

Proposal 11: Not to define RI requirements for 6Rx.

# Conclusion

In this contribution, we provide our views about performance requirements for 6Rx UEs. Observations and proposals are summarized:

Proposal 1: 6 MIMO layers performance requirements are only applicable for FWA devices.

Observation 1: Modifying the current clause numbering for 4Rx and 8Rx would significantly increase the complexity of CR maintenance.

Proposal 2: Add a new clause for 6Rx after the current 8Rx performance requirements.

Observation 2: Handheld UE and FWA exhibit significant differences in form factor and mobility.

Proposal 3: Define two sets of requirements for 4L6R, each tailored to different MIMO correlation scenarios for FWA and Handheld UE, i.e., α = 0 and β = 0.07179 and α = 0 and β = 0 for Handheld UE and FWA respectively.

Proposal 4: Introduce test applicability rules to let UE skip FWA 4 layers requirements if it can pass the FWA 6 layers requirements.

Proposal 5: Use Rel-15 2+2 DMRS configuration to define 8T6R6L FWA requirements.

Observation 3: For 6Rx SDR requirements with 1024QAM and 4 layers, the SNR required to achieve 85% throughput is below 29 dB if MCS 23 in Table 4 is used.

Proposal 6: Given that the maximum achievable SNR for 1024 QAM is 32.04 dB (corresponding to an EVM of 2.5%), RAN4 can consider MCS 23 in Table 4 to define 6Rx SDR requirements with 1024QAM and 4 layers.

Proposal 7: Define UE capability pdsch-1024QAM-4MIMO-FR1-r17 for applicability rules if SDR 1024QAM 4-layer requirements for 6Rx is introduced.

Proposal 8: Use the following MCS to define SDR requirements for QPSK/16QAM/64QAM/256QAM with 6 MIMO layers.

| Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- |
| 6 | 8 | 1 | 27 |
| 6 | 8 | 0.8 | 23 |
| 6 | 8 | 0.75 | 22 |
| 6 | 8 | 0.4 | 12 |
| 6 | 6 | 1 | 28 |
| 6 | 6 | 0.8 | 24 |
| 6 | 6 | 0.75 | 23 |
| 6 | 6 | 0.4 | 14 |
| 6 | 4 | 1 | 16 |
| 6 | 4 | 0.8 | 16 |
| 6 | 4 | 0.75 | 16 |
| 6 | 4 | 0.4 | 11 |
| 6 | 2 | 1 | 9 |
| 6 | 2 | 0.8 | 9 |
| 6 | 2 | 0.75 | 9 |
| 6 | 2 | 0.4 | 5 |
| Note 1: MCS Index for maximum modulation format 2,4 and 6 is based on MCS index Table 1 defined in clause 5.1.3.1 of TS 38.214 Note 2: MCS Index for maximum modulation format 8 is based on MCS index Table 2 defined in clause 5.1.3.1 of TS 38.214 |  |  |  |

Proposal 9: Use the following MCS to define SDR requirements for 1024QAM with 2 MIMO layers.

| Supported RX antenna ports | Maximum number of PDSCH MIMO layers | Maximum modulation format | Scaling factor | MCS |
| --- | --- | --- | --- | --- |
| 6RX | 2 | 10 | 1 | 24 |
|  | 2 | 10 | 0.8 | 21 |
|  | 2 | 10 | 0.75 | 19 |
|  | 2 | 10 | 0.4 | 9 |

Proposal 10: Not to define CQI requirements for 6Rx.

Proposal 11: Not to define RI requirements for 6Rx.

# Reference

- R4-2512617, Way Forward for Topic for [116][325] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei, HiSilicon

- R2-2506466, Introduction of UE capability on 6 DL MIMO layers, Huawei, HiSilicon, Samsung, MediaTek Inc., Apple

- R2-2506467, Introduction of UE capability on 6 DL MIMO layers, Huawei, HiSilicon, Samsung, MediaTek Inc., Apple

- R4-2513345, Simulation results on 6Rx UE demodulation performance requirements, MediaTek