3GPP RAN WG4 Meeting #116bis		R4-2513349

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	8.4

Source:	Apple

Title:	Initial views on 6G UE RF requirements framework

WI/SI:	FS_6G_Radio

Release:	Rel-20

Document for:	Approval

# 1	Introduction

The Rel-20 6G working group study item has been approved in June RAN #108 meeting [1]. While the RAN1 side of 6G study item objectives have been started in August working group meeting, the RAN4 part of the 6G discussions are just to be kicked off in this meeting. Following the RAN4 Chairman’s guidance on 6G topic planning based on the 6G SID as presented in August RAN4 meeting [2], the RAN4 discussions will be divided into the following eleven topics:

- System parameters

- General RF and UE RF

- BS RF

- Spectrum

- RRM

- Demod

- AI

- 5G and 6G spectrum sharing

- Sensing

- OTA and testability

- RAN4 operation efficiency

In this contribution, we focus on the UE RF aspects and share our initial views on the 6G UE RF requirements framework which are primarily based on the lessons learned from 5G.

# Discussion

Despite 5G technology is now becoming the mainstream, it still has not yet reached its full potential where part of it may only be realized by its successor also known as 6G. Therefore, 6G technology is not expected to be a whole new revolution from 5G, but more likely a 5G evolution and this perspective is also aligned with the 6G study objectives [1].

In this contribution, we discuss the UE RF aspects of the RAN4 objectives in 6G study item [1] and share our initial views on the 6G UE RF requirements framework for Tx, Rx, and CA requirements respectively which are primarily based on the lessons learned from 5G.

## 2.1	Tx requirements

UL coverage has always been the focal point of UE RF centric enhancement since Rel-15 and will continue to be the most concerned performance in driving the 6G technology. On this aspect, we propose to prioritize the MPR/A-MPR and power class enhancements among all Tx requirements as they have the most direct impact to UL coverage.

### 2.1.1	MPR/A-MPR enhancement

MPR/A-MPR requirements have been evaluated based on certain UE performance assumptions for simulation or lab measurement, such as transceiver UL image rejection ratio, counter IM3/IM5, and the standard waveform for power amplifier model calibration. Since most of the assumptions are inherited from LTE except for image rejection ratio which was improved from 25 dB to 28 dB for NR, with the continual advancement in UE RF technology for over two decades, some of the existing MPR/A-MPR requirements have been seen excessive and comfortably outperformed by practical UE implementations. However, in some cases RAN4 simulation assumptions could not reflect certain real device impairments and may lead to insufficient power back-off. In such cases, measurements are needed to assess the actual power back-off. Therefore, revisiting the MPR/A-MPR simulation assumptions to align with the UE RF technology advancement and with the assist of measurements would be a crucial step for 6G MPR/A-MPR enhancement.

Observation 1: Some of the existing MPR/A-MPR requirements have been seen excessive and comfortably outperformed by practical UE implementations. However, in some cases measurements are necessary because RAN4 simulation assumptions could not reflect certain real device impairments.

Proposal 1: Revisit MPR/A-MPR simulation assumptions, aiming to improve the accuracy of MPR/A-MPR requirements from 5G.

On the other hand, the way the current 5G MPR/A-MPR requirements are specified is rather complicated and often time-consuming to reach agreement. To improve the efficiency in defining the requirements over 5G, we propose to study on the MPR/A-MPR requirements simplification.

Observation 2: The way the current 5G MPR/A-MPR requirements are specified is rather complicated and often time-consuming to reach agreement.

Proposal 2: Study on the MPR/A-MPR requirements simplification aiming to reduce workload and improve process efficiency of requirements development over 5G.

### 2.1.2	Power class enhancement

LTE and NR FR1 UE power classes in general have been specified with 3dB power step which may potentially constrain the UE maximum output power capability under different implementations. For example, different frequency bands may be subject to different RF front-end insertion losses which would affect the maximum output power at the antenna connector. This may result in a nominal power level ranging from 22 dBm to 25 dBm under power class 3 for different bands. On the other hand, for the 3Tx feature in Rel-19 work item [3], UE may use 3 x 23dBm PA configuration where the total maximum output power would be 27.8 dBm which is not aligned with either PC2 or PC1.5 nominal maximum output power. Based on the current power class definition, UE would have to settle with PC2 at 26 dBm, meaning that its maximum output power capability is not fully utilized. Furthermore, in 5G there had been several power boosting features with 0.5dB or 1dB output power enhancement for which the UE implementations are transparent to network. If such incremental power difference can be embedded into the potential new power class definition, the true UE output power capability could be more effectively utilized.

Observation 3: LTE and NR FR1 UE power classes in general have been specified with 3dB power step which may potentially constrain the UE maximum output power capability under different implementations.

Proposal 3: Study whether there are better alternatives than the existing NR FR1 UE power class definition for which the UE’s true output power capability can be more effectively utilized for 6G.

One other power class related issue in 5G is the uplink full power transmission (ULFPTx) capability introduced in Rel-16 for UE supporting UL MIMO. To our understanding, this capability is intended for UE to indicate its PA configuration to achieve full power transmission for UL MIMO to network such that network can configure a suitable TPMI to UE for single-layer transmission. However, we have not seen the usefulness of this capability to achieve better UL coverage, especially if UE also supports Tx diversity. For example, it does not make much sense for UE to support mode 0 from coverage perspective as this would mean under 2-layer UL MIMO transmission, each Tx path can only transmit up to half of its PA output power capability. On the other hand, the mode definition for MIMO configuration with 3 or more Tx has not been very clear which may result in different interpretations. Therefore, we propose to study the necessity of ULFPTx capability in 6G.

Observation 4: 5G ULFPTx capability does not seem to be useful for achieving better UL coverage and the mode definition with 3 or more Tx has not been very clear which may result in different interpretations.

Proposal 4: Study on the necessity of ULFPTx capability in 6G.

Another power class related issue as we observed is that the current MPR requirements are different for the same power class with different number of Tx chains which had added substantial RAN4 workload and specifications complexity in 5G. As these MPR differences are mostly within one dB, it might be beneficial to have just one set of MPR requirements for the same power class with different number of Tx chains.

Observation 5: Existing MPR requirements are different for the same power class with different number of Tx chains which adds substantial RAN4 workload and specifications complexity.

Proposal 5: Study on the feasibility to have same MPR requirements for the same power class with different number of Tx chains.

Moreover, in the spirit of true UL power enhancement for high power UE (HPUE), the gain in maximum output power should be as advertised by the power class in all UL configurations which means the MPR requirements should be independent of the power class. For example, PC2 in principle should deliver 3dB more power than PC3 under the same UL configuration. However, in 5G this has not been the case for HPUE as PC2 can only deliver 3dB more power than PC3 in certain UL configurations and less than 3dB power gain in the rest of UL configurations. Due to this reason, RAN4 had to go through the process of MPR evaluation and harmonization among companies for every new power class introduced which not only adds substantial RAN4 workloads but also increases specifications complexity. To mitigate this concern, we propose to study whether it is feasible to specify MPR/A-MPR requirements which is power class independent or having one set of MPR/A-MPR requirements covering more than 3dB power range.

Observation 6: The current MPR/A-MPR requirements are per UE power class which would increase RAN4 workloads and specifications complexity when more power classes are introduced.

Proposal 6: Study on whether it is feasible to specify MPR/A-MPR requirements which is power class independent or having one set of MPR/A-MPR requirements covering more than 3dB power range.

One further power class related issue is the power class definition for inter-band UL combinations. In our view, the power class for total maximum output power for inter-band UL combinations is not so meaningful and may complicate the UL power control as well as constrain the UE per-band maximum output power capability. This latter issue has been recognized in Rel-17, and as a consequence, the higherPowerLimit-r17 capability was introduced for the mitigation. On the other hand, the coexistence between the per band power class and the UL combination power class also created lots of ambiguities on the UE UL power capability indication for which RAN4 spent near two releases of meeting cycles for the discussions and still could not reach the consensus. With that aid, we propose to study on whether it is necessary to define power class for inter-band UL CA in 6G.

Observation 7: For inter-band UL combinations, the power class for total maximum output power is not so meaningful and may complicate the UL power control as well as constrain the UE per-band maximum output power capability.

Proposal 7: Study on whether it is necessary to define power class for inter-band UL CA in 6G.

Last but not least, the existing PCMAX formula has become rather complicated and difficult to comprehend via its past evolvements where some of the terms related to certain features have been recognized either not very useful or never been deployed. Therefore, we propose to study on the possible simplification of PCMAX formula.

Observation 8: The existing PCMAX formula is rather complicated and difficult to comprehend.

Proposal 8: Study on the simplification of PCMAX formula.

## 2.2	Rx requirements

REFSENS has been the basis for most of the Rx requirements, including ACS, blocking, wide-band inter-modulation, MSD, etc. The current REFSENS and CA requirement frameworks are mostly based on what were initially developed in LTE and may need to be revisited when taking into account the UE RF technology enhancement over the past two decades as well as the new wider channel bandwidths for FDD bands and the ever-increasing new band combinations introduced in 5G where most of them may very well be reintroduced in 6G.

### 2.2.1	REFSENS

In 3GPP, the single carrier REFSENS requirements have been derived based on the following formula:

REFSENS (dBm) = -174 dBm/Hz + 10log10(BW) + NF + SNR – GDIV + IM

, where -174 dBm/Hz is the thermal noise floor, BW is the downlink carrier transmission bandwidth, NF is the UE receiver noise figure, SNR is the QPSK demodulation requirement to achieve 95% or more of the maximum throughput of the reference measurement channel, GDIV is the diversity gain, and IM is the implementation margin. To our understanding, the assumptions for single carrier REFSENS requirements such as noise figure and implementation margin have not been revamped since LTE era despite the continual UE RF technology advancement ever since. On the other hand, the ever-increasing band combinations support by UE would add substantial front-end insertion losses due to the implementation of multiplexers and cascaded switches which may not always be accounted for single carrier REFSENS requirements. Therefore, if single carrier REFSENS requirements would be revisited in 6G, both the REFSENS assumptions from technology advancement and the insertion losses from CA support need to be considered simultaneously.

Observation 9: The assumptions for single carrier REFSENS requirements such as noise figure and implementation margin have not been revamped since LTE era despite the continual UE RF technology advancement ever since.

Observation 10: The ever-increasing band combinations support by UE would add substantial front-end insertion losses due to the implementation of multiplexers and cascaded switches which may not always be accounted for single carrier REFSENS requirements.

Proposal 9: If single carrier REFSENS requirements would be revisited in 6G, both the REFSENS assumptions from technology advancement and the insertion losses from CA support need to be considered simultaneously.

### 2.2.2	REFSENS for FDD bands

There has not been clear principle in RAN4 on specifying the FDD band REFSENS with impact of Tx self-interference. On one hand UL RB number restriction was considered to reduce the Tx spectral regrowth side-lobe encroaching into DL carrier, on the other hand, the RB allocation is placed as close to DL band as possible to observe the worst-case degradation at the specified allocation number. However, the absolute worst-case degradation which normally occurs at full UL allocation has never been revealed. As there is no clear rule on choosing the restricted UL RB numbers, not only the process of reaching the agreed specifications could be rather inefficient, the physical meaning for the selected UL RB numbers could also be difficult to comprehend for specifications readers who did not physically participate in the requirement development process.

Observation 11: There has not been clear principle in RAN4 on specifying the FDD band REFSENS subject to potential impact of Tx self-interference.

Observation 12: As there is no clear rule on choosing the restricted UL RB numbers, not only the process of reaching the agreed specifications could be rather inefficient, the physical meaning for the selected UL RB numbers could also be difficult to comprehend.

Proposal 10: Study to improve the definition of FDD band REFSENS requirements subject to potential impact of Tx self-interference where the selection of UL configurations can be well comprehended.

## 2.3	CA requirements

CA was first introduced in LTE release 10. Ever since the number of band combinations has grown rapidly for every release and there are now tens of thousands band combinations in NR alone. Despite the block approval process has been introduced in RAN4 to handle enormous amount of new band combinations in every RAN4 meeting, the workloads for introducing new band combinations are still quite substantial, not only for the band combination proponents, but also for the block approval reviewing delegates. To our understanding, the most time-consuming process would be the MSD analysis and specifications development, followed by the ΔTIB/ΔRIB analysis and reviews. On the other hand, the concept of bandwidth combination set (BCS) first introduced in LTE CA and also adopted in 5G NR for CA and DC has evolved to create many unnecessary segmentations in per-band bandwidth support which would add an additional dimension of complexity in band combinations support.

### 2.3.1	MSD

The current MSD analysis has been known for consuming the most time and efforts in the new band combination introduction process. The artificially defined MSD requirements not only occupy a substantial portion of the RAN4 technical specifications (80+ pages in 38.101-1 [4] and growing) but also create enormous workloads for UE conformance tests. On the other hand, MSD requirements have been specified based on a particular worst-case carrier configuration, but not necessarily and likely not the REFSENS degradation for the configuration scheduled by the network in real field operations. As a result, there has been growing doubt on the usefulness of the existing MSD requirements.

Observation 13: The current MSD analysis has been known for consuming the most time and efforts in the new band combination introduction process.

Observation 14: The artificially defined MSD requirements not only occupy a substantial portion of the RAN4 technical specifications but also create enormous workloads for UE conformance tests.

Observation 15: MSD requirements have been specified based on a particular worst-case carrier configuration, but not necessarily and likely not the REFSENS degradation for the configuration scheduled by the network in real field operations.

Proposal 11: Study on whether there are better alternatives than existing MSD requirements.

Proposal 12: Study on whether it would be beneficial to allow network to perceive the MSD amount under real-field CA configurations.

### 2.3.2	ΔTIB/ΔRIB

ΔTIB/ΔRIB are rather tedious CA requirements which to our understanding have not been practically leveraged by most UE implementations nor precisely accounted in conformance tests. On the other hand, it is also fairly time consuming to validate the requirements during the band combination TP block approval process. Hence, we propose to study on the feasibility to either completely remove or streamline the ΔTIB/ΔRIB requirements.

Observation 16: ΔTIB/ΔRIB are rather tedious CA requirements which have not been practically leveraged by most UE implementations nor precisely accounted in conformance tests.

Observation 17: It is fairly time consuming to validate the ΔTIB/ΔRIB requirements during the band combination TP block approval process.

Proposal 13: Study on the feasibility to either completely remove or streamline the ΔTIB/ΔRIB requirements.

### 2.3.3	BCS

The concept of bandwidth combination set (BCS) was first introduced in LTE for CA which was intended to address the concern that some early implemented UE may have the constraint on supporting the maximum aggregated bandwidth from two or more component carriers. The BCS concept was also adopted by NR for both CA and DC. To our understanding, in NR the BCS has always been proposed by operators to reflect their own spectrum holdings but not UE’s capability limitation. And during the 5G time, it has evolved to create many unnecessary segmentations in per-carrier bandwidth support which would add an additional complexity in the band combination requirements. With this lesson learned, we propose RAN4 to study the necessity to introduce BCS for 6G CA and also how to address UE’s limitation on supporting the maximum aggregated bandwidth if without BCS.

Observation 18: The BCS concept has evolved to create many unnecessary segmentations in per-carrier bandwidth support which would add an additional complexity in the band combination requirements.

Proposal 14: Study the necessity to introduce BCS for 6G CA and how to address UE’s limitation on supporting the maximum aggregated bandwidth if without BCS.

# 3	Conclusion

In this contribution, we discuss the UE RF aspects of the RAN4 objectives in 6G study item and share our initial views on the 6G UE RF requirements framework for Tx, Rx, and CA requirements respectively which are primarily based on the lessons learned from 5G.

Observation 1: Some of the existing MPR/A-MPR requirements have been seen excessive and comfortably outperformed by practical UE implementations. However, in some cases measurements are necessary because RAN4 simulation assumptions could not reflect certain real device impairments.

Proposal 1: Revisit MPR/A-MPR simulation assumptions, aiming to improve the accuracy of MPR/A-MPR requirements from 5G.

Observation 2: The way the current 5G MPR/A-MPR requirements are specified is rather complicated and often time-consuming to reach agreement.

Proposal 2: Study on the MPR/A-MPR requirements simplification aiming to reduce workload and improve process efficiency of requirements development over 5G.

Observation 3: LTE and NR FR1 UE power classes in general have been specified with 3dB power step which may potentially constrain the UE maximum output power capability under different implementations.

Proposal 3: Study whether there are better alternatives than the existing NR FR1 UE power class definition for which the UE’s true output power capability can be more effectively utilized for 6G.

Observation 4: 5G ULFPTx capability does not seem to be useful for achieving better UL coverage and the mode definition with 3 or more Tx has not been very clear which may result in different interpretations.

Proposal 4: Study on the necessity of ULFPTx capability in 6G.

Observation 5: Existing MPR requirements are different for the same power class with different number of Tx chains which adds substantial RAN4 workload and specifications complexity.

Proposal 5: Study on the feasibility to have same MPR requirements for the same power class with different number of Tx chains.

Observation 6: The current MPR/A-MPR requirements are per UE power class which would increase RAN4 workloads and specifications complexity when more power classes are introduced.

Proposal 6: Study on whether it is feasible to specify MPR/A-MPR requirements which is power class independent or having one set of MPR/A-MPR requirements covering more than 3dB power range.

Observation 7: For inter-band UL combinations, the power class for total maximum output power is not so meaningful and may complicate the UL power control as well as constrain the UE per-band maximum output power capability.

Proposal 7: Study on whether it is necessary to define power class for inter-band UL CA in 6G.

Observation 8: The existing PCMAX formula is rather complicated and difficult to comprehend.

Proposal 8: Study on the simplification of PCMAX formula.

Observation 9: The assumptions for single carrier REFSENS requirements such as noise figure and implementation margin have not been revamped since LTE era despite the continual UE RF technology advancement ever since.

Observation 10: The ever-increasing band combinations support by UE would add substantial front-end insertion losses due to the implementation of multiplexers and cascaded switches which may not always be accounted for single carrier REFSENS requirements.

Proposal 9: If single carrier REFSENS requirements would be revisited in 6G, both the REFSENS assumptions from technology advancement and the insertion losses from CA support need to be considered simultaneously.

Observation 11: There has not been clear principle in RAN4 on specifying the FDD band REFSENS subject to potential impact of Tx self-interference.

Observation 12: As there is no clear rule on choosing the restricted UL RB numbers, not only the process of reaching the agreed specifications could be rather inefficient, the physical meaning for the selected UL RB numbers could also be difficult to comprehend.

Proposal 10: Study to improve the definition of FDD band REFSENS requirements subject to potential impact of Tx self-interference where the selection of UL configurations can be well comprehended.

Observation 13: The current MSD analysis has been known for consuming the most time and efforts in the new band combination introduction process.

Observation 14: The artificially defined MSD requirements not only occupy a substantial portion of the RAN4 technical specifications but also create enormous workloads for UE conformance tests.

Observation 15: MSD requirements have been specified based on a particular worst-case carrier configuration, but not necessarily and likely not the REFSENS degradation for the configuration scheduled by the network in real field operations.

Proposal 11: Study on whether there are better alternatives than existing MSD requirements.

Proposal 12: Study on whether it would be beneficial to allow network to perceive the MSD amount under real-field CA configurations.

Observation 16: ΔTIB/ΔRIB are rather tedious CA requirements which have not been practically leveraged by most UE implementations nor precisely accounted in conformance tests.

Observation 17: It is fairly time consuming to validate the ΔTIB/ΔRIB requirements during the band combination TP block approval process.

Proposal 13: Study on the feasibility to either completely remove or streamline the ΔTIB/ΔRIB requirements.

Observation 18: The BCS concept has evolved to create many unnecessary segmentations in per-carrier bandwidth support which would add an additional complexity in the band combination requirements.

Proposal 14: Study the necessity to introduce BCS for 6G CA and how to address UE’s limitation on supporting the maximum aggregated bandwidth if without BCS.

# 4	References

RP-251881 “New SID: Study on 6G Radio”, NTT DOCOMO, 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9th -13th, 2025

R4-2511652 “RAN4 6G Plan and Meeting Arrangement”, RAN4 Chair, 3GPP RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025

RP-234007 “New WID: NR MIMO Phase 5”, Samsung, 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11th -15th, 2023

3GPP TS 38.101-1 V19.2.0 (2025-06)