3GPP TSG-RAN WG4 Meeting #116bis									R4-2513359

Prague, Czech Republic, Oct. 13-17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.191 | CR |  | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on 38.191 for A-loT device RF requirement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | subclause “6.1 the general part” is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding clause 6.1 the general part. Updating sub-clause number for the remaining parts of clause 6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The general part for Tx characteristics is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- Transmitter characteristics

6.1	General

Unless otherwise stated, the transmitter characteristics are specified over the air (OTA). The power levels for all D2R signals are defined assuming a 0dBi reference antenna located at the center of the quiet zone. The minimum requirements on Effective Isotropic Radiated Power (EIRP) apply to four measurements, corresponding to incident CW signals in orthogonal polarizations and UL signals in orthogonal polarizations.

6.2	Backscatter power

6.2.1	Device backscatter power

The backscatter power is defined as mean filtered power measured over the duration of the D2R signal, excluding the power at the carrier frequency. The backscatter loss is defined as the difference between the input CW power at the device antenna in dB scale to the backscatter power at the device antenna in dB scale.

The backscatter power is measured with the test metric of EIRP as specified in clause 8.2.1. The minimum requirement on backscatter loss in Table 6.2.1.1-1 shall be met with the test parameters defined in Annex A, at the peak antenna gain direction declared by the device vendor.

Table 6.2.1.1-1: Maximum allowable backscatter loss

| Operating band | Maximum backscatter loss (dB) |  | CW power at the device antenna (dBm) |
| --- | --- | --- | --- |
| n8 | OOK | 10 | -27 |
|  | BPSK | 6 |  |
|  | OOK | 15 | -10 |
|  | BPSK | 11 |  |

6.3	Output RF spectrum emissions

6.3.1	Out of band emissions

6.3.1.1	General

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions.

6.3.1.2	Spectrum emission mask

The spectrum emission mask of the device applies to frequencies (ΔfOOB) starting from the centre of the assigned D2R channel bandwidth. For frequencies offset greater than ΔfOOB as specified in Table 6.3.1.2-1 the spurious requirements in clause 6.3.2 are applicable.

The emission within ΔfOOB of a device shall be lower than the level specified in Table 6.3.1.2-1 compared to the D2R backscatter power under same incident CW power level. The requirement is verified with the test metric of EIRP and the incident CW power level at the device set to -5 dBm. The test direction is the same as the backscatter output power as specified in clause 6.2.

Table 6.3.1.2-1 Spectrum emission mask for device

| ΔfOOB (MHz) | Spectrum emission limit (dBc) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.5*D2R CBW - FOOB | 10 | 0.5* Nominal D2R transmission Bandwidth without SFO |
| NOTE 1: FOOB is the OOB boundary frequency specified in Table 6.3.1.2-1. |  |  |

6.3.2	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 [4].

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.3.2.2-1 from the centre of the D2R channel bandwidth. The spurious emission limits in Table 6.3.2.2-2 apply for all D2R channel bandwidths. The requirement is verified with the test metric of EIRP under the incident CW power level at the device is set to -5 dBm. The test direction is the same as the transmitter output power as specified in clause 6.2.

Table 6.3.2.2-1: Boundary between out of band and spurious emission domain for device

| D2R Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| D2R CBW<1.4 MHz | max (500kHz, 10*D2R CBW) |
| D2R CBW≥1.4 MHz | 7.5 MHz |

Table 6.3.2.2-2: Requirement for spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 5 GHz | -30 dBm | 1 MHz |  |
| 5 GHz ≤ f < 12.75 GHz | -30 dBm | 1 MHz | 1 |
| NOTE 1: Applies for Band for which the upper frequency edge of the UL Band is greater than 1 GHz and less than or equal to 2.55 GHz. |  |  |  |