# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513368

# Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	6.15.2.1

Source:	China Telecom

Title:	Discussion on RRM performance requirements for On-demand SSB SCell operation

Document for:	Discussion

- Introduction

In RAN4 #116 meeting, RRM performance requirements for On-demand SSB SCell operation were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements for On-demand SSB SCell operation captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1]:

Sub-issue 1: Test scope Issue 3-1-1: Scenario - General - RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement. - No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB. - RAN4 should not define fast SCell activation test cases for OD-SSB. - RAN4 only choose one test case for case 2 to justify the effective periodicity of OD-SSB and AO-SSB. - RAN4 to further discuss the test case scope for scenario #2, scenario #2A and scenario #3B as follow. - Option 1: - In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A. - In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B. - The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell. - Option 2: RAN4 not to define test cases for scenario #3B. - Option 3: RAN4 to define the OD-SSB based test cases for - Case #1 and Scenario #2 - Case #2 and Scenario #2A - RAN4 to further discuss the test case scope for Case 2 as follow. - FFS: RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1, Alt. Time C2 and scenario 3 if RAN4 agrees to define the RRM requirements for all these scenarios.

For the general issues on scenario, in our views, it’s proposed to consider the following cases

- In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

- In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

- The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

Proposal 1: RAN4 to consider the following general cases for scenario

- In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

- In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

- The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

Status in the WF [1]:

Issue 3-1-3: Scenario – known/unknown Scell activation - Proposal 1: RAN4 to define the OD-SSB based SCell activation test case for both known SCell and unknown SCell. - Proposal 2: RAN4 to define the OD-SSB based SCell activation test case for known SCell only. - Proposal 3: RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. One in known scenario and the other in unknown scenario such as: - Case 1 in unknown scenario - Case 2 in known scenario

In the core part, requirements on OD-SSB based SCell activation are defined for both known SCell and unknown SCell, therefore, it’s proposed to define the OD-SSB based SCell activation test cases for both known SCell and unknown SCell.

Proposal 2: RAN4 to define the OD-SSB based SCell activation test cases for both known SCell and unknown SCell.

Status in the WF [1]:

Sub-issue 2: Test case list Issue 3-1-4: Test case list Agreement RAN4 to at least define the following OD-SSB test cases. - SCell activation in Case 1 - Intra-frequency measurement(deactivated SCell) in Case 1 - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B) RAN4 to further discuss whether the following OD-SSB test cases can be agreed. - SCell activation in Case 2 Alt Time C-1 - OD-SSB based multiple SCell activation - OD-SSB based direct SCell Activation at SCell addition in Case 1 - OD-SSB based direct SCell Activation at SCell addition in Case 2 - OD-SSB based PUCCH SCell Activation in Case 1 - OD-SSB based PUCCH SCell Activation in Case 2 - Fast SCell Activation with OD-SSB; - OD-SSB based SCell Activation with the L3 reporting during activation - Intra-frequency measurement(deactivated SCell) in Case 2 - Intra-frequency measurement in Case 1 - Intra-frequency measurement in Case 2 - Interruption at OD-SSB activation/deactivation - Interruptions during measurements on deactivated NR SCC

For performance requirements of NES part 1, based on the core part discussion in RAN4 previous meetings, the requirements on OD-SSB based deactivated SCell measurement with FMW, OD-SSB based SCell activation, and OD-SSB based L1 measurement are to be defined, therefore, related test cases need to be designed. Besides, two cases including Case 1 and Case 2 need to be considered for test case design.

For OD-SSB based deactivated SCell measurement with FMW, OD-SSB based NR intra-frequency measurements requirements are to be defined in the core part, Case 1 and Case 2 are also considered for OD-SSB based NR intra-frequency measurements.

For OD-SSB based SCell activation, scenarios including OD-SSB based normal SCell Activation, OD-SSB based direct SCell Activation at SCell addition, OD-SSB based PUCCH SCell Activation, and OD-SSB based SCell Activation with the L3 reporting during activation are considered in core part, therefore, it’s proposed to design test cases for these scenarios. For OD-SSB based SCell activation, OD-SSB based direct SCell Activation at SCell addition, OD-SSB based PUCCH SCell Activation, and OD-SSB based SCell Activation with the L3 reporting during activation, considering the workload, whether to design one test case or two test cases for Case 1 and Case 2 can be further discussed.

For OD-SSB based L1 measurement, it’s proposed to design test case for L1-RSRP measurements based on OD-SSB.

Proposal 3: RAN4 to design the following OD-SSB test cases

- OD-SSB based deactivated SCell intra-frequency measurement in Case 1 and Case 2

- OD-SSB based SCell Activation in Case 1 and Case 2

- OD-SSB based direct SCell Activation at SCell addition

- OD-SSB based PUCCH SCell Activation

- OD-SSB based SCell Activation with the L3 reporting during activation

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

Status in the WF [1]:

Sub-issue 3: Test configuration Issue 3-1-5: General principle RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed. - Both FR1 and FR2 - Test OD-SSB periodicities with 5ms, 20ms. 10ms - Only test non-DRX mode - The test without SSB time index detection - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting - SMTC and measCycleSCell as 40ms/160ms - Only NCD-SSB for OD-SSB test case.

For general principles, FR1 and FR2 can be considered for test case design. Besides, the principles on only testing non-DRX mode, the test without SSB time index detection, following the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting and only NCD-SSB for OD-SSB test case can be considered. For OD-SSB periodicities, 5ms, 10ms, 20ms can be considered, and SMTC and measCycleSCell can be as 40ms/160ms.

Proposal 4: RAN4 to design the OD-SSB test case with the following principles

- Both FR1 and FR2

- Test OD-SSB periodicities with 5ms, 20ms, 10ms

- Only test non-DRX mode

- The test without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

- SMTC and measCycleSCell as 40ms/160ms

- Only NCD-SSB for OD-SSB test case.

- Conclusion

The contribution presents our views on RRM performance requirements for On-demand SSB SCell operation, with the following proposals:

Proposal 1: RAN4 to consider the following general cases for scenario

- In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

- In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

- The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

Proposal 2: RAN4 to define the OD-SSB based SCell activation test cases for both known SCell and unknown SCell.

Proposal 3: RAN4 to design the following OD-SSB test cases

- OD-SSB based deactivated SCell intra-frequency measurement in Case 1 and Case 2

- OD-SSB based SCell Activation in Case 1 and Case 2

- OD-SSB based direct SCell Activation at SCell addition

- OD-SSB based PUCCH SCell Activation

- OD-SSB based SCell Activation with the L3 reporting during activation

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

Proposal 4: RAN4 to design the OD-SSB test case with the following principles

- Both FR1 and FR2

- Test OD-SSB periodicities with 5ms, 20ms, 10ms

- Only test non-DRX mode

- The test without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

- SMTC and measCycleSCell as 40ms/160ms

- Only NCD-SSB for OD-SSB test case.

- References

- R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #116, 25 – 29 August, 2025.

- R4-2509063, Topic summary for [116][220] Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #116, 25 – 29 August, 2025.