3GPP TSG-RAN WG4 Meeting #116bis	R4-2513384

Prague, Czech, 13th -17th August, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on R20 A-IoT SI coexistence scenarios and assumptions

Agenda Item:	7.11.3

Document for:	Discussion

Introduction

The new SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices was approved at the RAN#108 meeting [1]. The RAN4 relative objectives in the SID are copied as below:

Copied from RP-251884 [1]

RAN4-led

- Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios.

- Use band n8 as an example band.

NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

RAN1 had made some progress on link level simulation parameters in RAN1#122. In this paper we discuss the coexistence scenarios and parameters based on the SID and RAN1 link level simulation parameters.

Discussion

Coexistence scenarios and cases

As per SID [1], outdoor A-IoT system uses Deployment scenario 4 with topology 1 with no external carrier wave. Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum.

To reduce workload, since LTE and NR have similar coexistence performance with A-IoT, we can only study coexistence between Device 2b/Device C and NR/LTE as R19 SI.

Proposal 1: To minimize evaluation cases in RAN4, only evaluate A-IoT and NR co-existence.

Both device 2b and device C share the same coexistence cases.

It is proposed the following simulation cases are used for evaluation of coexistence:

Table 1-1: Co-existence simulation cases for device 2b/C （standalone）

| Case | Aggressor | Victim | Direction | Note |
| --- | --- | --- | --- | --- |
| e | Device | NR UL | D2R |  |
| f | NR UL | reader | D2R |  |
| c | Reader | NR DL | R2D |  |
| d | NR DL | device | R2D | A-IoT BS and NR BS belonging to different operators are not the same site. The device may be far from its own cell and closer to the cell of a different operator, resulting in a near-far effect. |

*Note: b/c/d/e refer to TR38.769

Table 1-2: Co-existence simulation cases for device 2b/C (inband)

| Case | Aggressor | Victim | Direction | Note |
| --- | --- | --- | --- | --- |
| e | Device | NR UL | D2R |  |
| f | NR UL | reader | D2R |  |
| c | Reader | NR DL | R2D | A-IoT R2D signal is orthogonal with NR DL signal，so there is no co-existence issue. No need to simulate |
| d | NR DL | device | R2D |  |

Proposal 2: Adopt the coexistence simulation cases in Table 1-1 and 1-2.

Co-existence evaluation assumptions

Deployment

Table 2. Deployment parameters for D4T1

| Parameter | D4T1 | note |
| --- | --- | --- |
| Carrier frequency | 900 MHz (Band n8) |  |
| BW for NR | 10MHz with 15KHz SCS |  |
| R2D BW | 180kHz, for 15kHz SCS | Agreement（from R1-2506567） For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings: - 180kHz as starting point - Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report Agreement（from R1-2506567） For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values: - Addition of M=1 - Addition of M=32 Removal of M=24 |
| D2R BW | 15kHz | Agreement（from R1-2506567） For Rel-20 study, D2R transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings: - 15kHz as starting point - Other values are not precluded and up to company report Companies to report which of 2SB or 1SB modulation is assumed for D2R transmission bandwidth |
| Waveform | R2D: OFDM-based OOK D2R: BPSK |  |
| A-IoT UL power control | Open loop power control | Agreement（from R1-2506567） Study necessity and feasibility of D2R Tx power control for Device 2b and for Device C, including the following aspects - Open loop and/or closed loop power control - Required measurements (e.g., signal strength) - Required information at the device - Required information at reader side Signaling overhead and power consumption |
| NR BS/A-IoT BS deployment (outdoor) | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, the minimum 2D distance between macro BS and device 2b/C as 50-500 m. | Conclusion（from R1-2506567） For Rel-20 study, it is up to RAN4 to decide the evaluation assumption on ISD for the study in outdoor scenarios. Inter-NR BS distance: 750m NR BS height: 25m |
| Frequency reuse | 1:3 | R2D is OFDM-based OOK signal. The available spectrum is divided into three parts, and these three parts are then reused by different cells in a repeating, non-adjacent pattern similar with GSM. This provides frequency isolation between adjacent cells and helps to manage intercell interference. |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IoT device under one reader at one drop Minimum distance between reader and device is 35m |  |
| NR UE dropping | For NR UE only outdoor, uniformly distributed outdoor. UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |  |
| NR BS Inter-site distance | MCL of 70 dB |  |
| Pathloss model | UMa NLOS defined in TR38.901 Penetration loss: [50%] 0dB +[50%] 20dB Shadow fading margin：For UMa NLOS: 6dB | Agreement（from R1-2506567） Shadow fading margin：For UMa NLOS: 6dB For Rel-20 study, add a new item of “Penetration margin (dB)” into link budget template and the value of X dB for evaluation purpose is from the followings: - For both R2D and D2R - X = {0, 20, 80} dB - Note: Example use cases for the values above: - 20 dB for a Device inside a car with metallized car windows - 80 dB for a Device inside a metallic shipping container - Companies can report additional example use cases Shadow fading margin：For UMa NLOS: 6dB |

A-IoT reader RF parameters

Table 3: A-IoT reader RF parameters

| A-IoT reader parameters | Values for evaluation |  |
| --- | --- | --- |
| A-IoT reader total Tx power | standalone: 43 dBm in-band：38dBm | Agreement（from R1-2506567） For Rel-20 study, BS Tx power assumption for coverage purpose is from the followings: - For standalone: 43 dBm - For in-band: - 38 or 33 dBm over R2D transmission BW - Note1: any necessary PSD limitation can be applied by RAN4 - Note2: companies should report any PSD limitation they assume in their RAN1 evaluations Send a LS to RAN4 with the above agreement, and to provide feedback to RAN1 if any. |
| A-IoT reader receiver Noise Figure (dB) | 5 |  |
| A-IoT reader antenna gain (dBi) | 17 |  |
| ACLR | [35,50] dB |  |
| ACS | [30,50] dB] |  |
| Antenna configuration | Same as legacy NR BS |  |

A-IoT device RF parameters

Device 2b generates its UL transmission internally, independent of backscatter. Considering the peak power consumption of few 100 µW and the power added efficiency (PAE) of power amplifier, the maximum transmit power is expected to be no higher than -10 dBm. We can use [-10/-20] dBm to analysis.

Device parameters are proposed as follows:

Table 4: A-IoT device RF parameters

| A-IoT device parameters | Device 2b | Device C | note |
| --- | --- | --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0dBi | 0dBi |  |
| A-IoT device output power（dBm） | -10dBm | 5dBm | From R1-2506567： Device 2b: {-10, -20} dBm Device C: {-3, 0, 5} dBm |
| A-IoT device noise figure (dB) | 15dB | 13 dB | Agreement（from R1-2506567） For Rel-20 study, Device noise figure assumption in link budget for evaluation purpose: Device 2b: 15 dB; FFS: 40dB as additional assumption for Device 2b with IF-ED Device C: 13 dB |
| ACLR | [20-30dB] | [20-30dB] |  |
| ACS | [10-30dB] | [10-30dB] | At the scenarios with 1:3 frequency reuse Pencil in a range of ACS values used for simulation |

NR BS RF parameters

NR BS RF parameters in TR38.769 can be reused.

Table 5: NR BS RF parameters

| NR BS Parameter | Values for evaluation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| BS antenna gain (dBi) and antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} BS antenna gain (dBi) (including feeder loss:15 |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |
| ACLR | Option 1: 30dBc, Option 2: 17dBc |

NR UE RF characteristics

Assumptions relevant for modelling the NR UE in TR38.769 in Table 6.6.2.3-1 and Table 6.6.2.3-2 can be reused.

Table 6: NR UE RF parameters

| NR UE Parameter | Values for evaluation |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| NR UE Antenna gain (dBi) | 0 |
| Height of UE antenna (m) | 1.5 |
| NR UE ACLR（dB） | 30 |
| NR UE Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

Proposal 3: Adopt the simulation assumptions and parameters for co-existence analysis as Table 2~ 6.

Evaluation methodology and performance metric

In R19 SI，TR38.769

Copied from TR38.769[2]

- 6.6.3.1	Coexistence evaluation methodology

The coexistence evaluation methodology can be summarized as:

1)	Aggressor and victim network are generated. NR UEs and A-IoT devices are distributed as described by parameter assumptions.

2)	UEs are associated to BS based on coupling loss, and A-IoT devices are associated to A-IoT reader or intermediate UE based on coupling.

- NOTE: For A-IoT devices are associated to A-IoT reader or intermediate UE, there are two ways applied by companies:

(a) The device access to the reader is randomly selected from candidate devices whose R2D receiving power/coupling from one reader is higher than the other. The receiving power of device meets the device receiver sensitivity.

(b) The device can access the reader with best receiving power among of 12 or 18 total readers in one building. The receiving power of device meets the device receiver sensitivity.

3)	Once association is done, round robin scheduling is used.

4) For inter-system interference (between A-IoT and NR):

-	If SINR degradation is smaller than and equal to 1 dB, it can be considered that inter-system interference is negligible.

-	Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR

Since in R20 SI, the scenario is D4T1, and there is no intermediate UE and CW, the coexistence evaluation methodology can be revised to as follows:

1)	Aggressor and victim network are generated. NR UEs and A-IoT devices are distributed as described by parameter assumptions.

2)	UEs are associated to BS based on coupling loss, and A-IoT devices are associated to A-IoT reader based on coupling.

- NOTE: For A-IoT devices are associated to A-IoT reader, there are two ways applied by companies:

(a) The device access to the reader is randomly selected from candidate devices whose R2D receiving power/coupling from one reader is higher than the other. The receiving power of device meets the device receiver sensitivity.

3)	Once association is done, round robin scheduling is used.

On performance metric, for NR system, use 5% throughput loss as performance metric as legacy.

For A-IoT system, use SINR degradation as performance metric:

For inter-system interference (between A-IoT and NR):

-	If SINR degradation is smaller than and equal to 1 dB, it can be considered that inter-system interference is negligible.

-	Note: For SINR degradation, SINR refers to the 5% and 50% CDF SINR

Proposal 4: For NR system, use 5% throughput loss as performance metric as legacy.

Proposal 5: A-IoT system, use SINR degradation as performance metric.

On SINR definition, it can be as follows:

For D4T1-C,

SINR definition for D2R:

- SINR is used as the baseline reference for co-existence evaluation for reader.

baseline SINR=received wanted signal powernoise + intra_system interference

- SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intrasysteminterference + intersysteminterference

The noise and intra-system interference are within total receiver bandwidth.

SINR definition for R2D:

- signal power of device to the noise and interference within 10MHz is baseline assumption

- Consider 180KHz noise and interference bandwidth after BB LPF as optional

Proposal 6: Adopt the following A-IoT SINR definition.

SINR definition for D2R:

- SINR is used as the baseline reference for co-existence evaluation for reader.

baseline SINR=received wanted signal powernoise + intra_system interference

- SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intrasysteminterference + intersysteminterference

The noise and intra-system interference are within total receiver bandwidth.

SINR definition for R2D:

- signal power of device to the noise and interference within 10MHz is baseline assumption

- Consider 180KHz noise and interference bandwidth after BB LPF as optional

Opinion on RAN1’s LS on BS Tx power assumption

RAN1 sent a LS on BS Tx power assumption to RAN4 and asked RAN4 to provide feedback to RAN1 if any [4].

For the Rel-20 study, RAN1 discussed BS Tx power assumption for coverage evaluation in A-IoT and made the following agreements:

Agreement For Rel-20 study, BS Tx power assumption for coverage purpose is from the followings: - For standalone: 43 dBm - For in-band: - 38 or 33 dBm over R2D transmission BW - Note1: any necessary PSD limitation can be applied by RAN4 - Note2: companies should report any PSD limitation they assume in their RAN1 evaluations Agreement For Rel-20 study, R2D transmission bandwidth assumption in link budget and LLS for evaluation purpose is from the followings: - 180kHz as starting point - Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report

From our understanding, TS36.104 specified that NB-IoT power dynamic range shall be larger than or equal to +6dB in TS36.104.There is no need to set a upper boundary for power boosting for inband.

Copied from TS36.104:

NB-IoT power dynamic range shall be larger than or equal to +6dB, except for guard band operation with E-UTRA 5 MHz channel bandwidth signal where BS manufacturer shall declare the NB-IoT dynamic range power it could support. (in this version of the specification).

The +6 dB power dynamic range is only required for one NB-IoT RB for both in-band and guard band operation modes.

Proposal 7: No need to provide feedback to RAN1 LS (R1-2506575).

Conclusion

Based on the analysis in this contribution, we have the following proposals:

Proposal 1: To minimize evaluation cases in RAN4, only evaluate A-IoT and NR co-existence.

Proposal 2: Adopt the Co-existence simulation cases in Table 1-1 and 1-2.

Proposal 3: Adopt the simulation assumptions and parameters for co-existence analysis as Table 2~ 6.

Proposal 4: For NR system, use 5% throughput loss as performance metric as legacy.

Proposal 5: A-IoT system, use SINR degradation as performance metric.

Proposal 6: Adopt the following A-IoT SINR definition.

SINR definition for D2R:

- SINR is used as the baseline reference for co-existence evaluation for reader.

baseline SINR=received wanted signal powernoise + intra_system interference

- SINR is calculated as total power ratio:

SINR=received wanted signal powernoise + intrasysteminterference + intersysteminterference

The noise and intra-system interference are within total receiver bandwidth.

SINR definition for R2D:

- signal power of device to the noise and interference within 10MHz is baseline assumption

- Consider 180KHz noise and interference bandwidth after BB LPF as optional

Proposal 7: No need to provide feedback to RAN1 LS (R1-2506575).

Reference

[1] RP-251884, New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, Moderator (RAN1 Vice-Chair), RAN#108, June,2025

[2]TR38.769, Study on solutions for ambient IoT (Internet of Things) V2.0.0

[3] R1-2506567, Session notes for 10.3 Study of Enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, RAN1#122, Aug,2025

[4] R1-2506575, LS on BS Tx power assumption for coverage evaluation in A-IoT,RAN1#122,Aug, 2025