3GPP TSG-RAN WG4 Meeting #116bis           	R4-2513398

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.15.4

Source:	Samsung

Title:	View on demodulation requirement for Ambient-IoT device 1

Document for:	Discussion

- Introduction

In the last RAN4 , the test scope and testup for  performance part of Ambient IoT (Internet of Things) in  NR was under discussion. The related agreement was captured into WF in [1] as following

- Device requirement - General - Deployment and topology - The assumed deployment and topology D1T1-B for A-IoT demodulation requirements discussion in R19 - No need to capture it in RAN4 specification - Operation mode - Standalone and in-band operation for Device 1 - Same demodulation performance requirements for standalone and in-band operation. - How to capture it in the specification can be discussed during the CR drafting if necessary - Standalone operation for BS - Demodulation requirements - Whether to define PRDCH demodulation requirements for Ambient-IoT - Option 1: Define PRDCH demodulation requirement. - Option 2: Not define PRDCH demodulation requirements for Ambient-IoT considering that it can be verified by the RF REFSENS requirement - Test methodology - 10% BLER with BLER=1- Number of responsed Message 1Number of transmitted Message 0 or BLER=1- Number of responsed Message 3Number of transmitted Message 2 - For testing - During countering the number of responded Message 1 or 3, the TE should check if the responded Message 1 or Message 3 is the correct response of Message 0 or Message 2 by checking the device ID - Further discuss the message type to be tested - Multiplexing - Single device - Antenna configuration - 1T1R - Numerology - 15kHz SCS - R2D message structure for testing - R-TAS (SIP+CAP) + PRDCH + Postamble - TBS - Option 1: 20bits - Option 2: 96 bits - Other options are not preluded - R2D coding - R2D line coding to define the corresponding PRDCH - Waveform - DFT-s-OFDM with OOK-4 - M-chips for OOK - Option 1: 6 - Option 2: 24 - Other options are not precluded - Repetition - No repetition configured - Channel bandwidth - Option 1: 2PRBs - Option 2: 3PRBs - Other options are not precluded - Channel model - Option 1: TDLA30-10 - Other options are not precluded - TE vendors are encouraged to check the feasibility to emulate TDL channel in the chamber for Device 1 testing - Specification structure (if specified) - Option 1: RAN4 should specify the Ambient IoT device demodulation requirements in “Ambient IoT device radio transmission and reception” - Option 2: The demodulation requirements can be captured in clause 10 of TS 38.191

As stated in the WID, from RAN4 perspective, RAN4 needs to specify the necessayr demodualtion requirements for device 1 and Ambient-IoT BS. In this contribution, the view on the remaining issue of test scope and test sope for demodualtion requirement for Ambient-IoT device is provided.

# 2	General Scope

In this section, the view on general scope of demodulation requirement for Ambient-IoT Device is provided.

FFS to define the PRDCH demodulation requirements

From RAN1 perspective, a new physical layer signal and channel was introduced to support Ambient IoT transmission, as PRDCH (Physical Reader to device channel (PDRCH)), which carriers the data and control information from the reader to device. It includes the new waveform, modulation, coding scheme to support A-IoT. The following is the processing procedure which includes CRC attachment, coding, OOK modulation, and waveform generation

Figure 1: PRDCH transmission structure

In general, considering it is a new physical layer design, RAN4 should define the related demodulation requirement for PRDCH transmission, considering it has the baseband impact.

In the last meeting, RAN4 has discussed whether to define PRDCH demodulation requirement. The main discussion is that whether it is necessary to define PRDCH demodulation requirement considering the PRDCH requirement can be verified by RF REFSENS requirement.

From RAN4 RF perspective, the following was agreed for RF reference sensitivity

Agreement in RAN4#115 - Using [-34dBm] as starting point to define REFSENS requirements, based on the peak gain direction of device antenna. - Consider to define orientation requirements with following options: - Option 1: min sensitivity at peak antenna gain direction and EIS spherical coverage requirements, XdBm at Xth percentage - Option 2: spatial averaging EIS - Option 3: EIS in the peak direction - Test metric: 10% miss detection rate, FFS on whether to consider false alarm - FFS for FRC, consider following as starting point SCS; 15; 15; 15; 15. PRB; 1; 2; 3; 4. TBS; 20; 20; 20; 20. CRC; 6; 6; 6; 6. M; 6; 12; 24; 24. Agreement in RAN4#116 - EIS at peak antenna gain direction is -34dBm. Additional peak EIS levels can be added in the future if needed - EIS spherical coverage requirement is defined as sensitivity over partial sphere, i.e. XdBm at Y solid angle range partial sphere . Y is suggested to be +-45 degree with respect to the bore sight direction (or UE declared direction), X is suggested to be 5.5dB worse than peak EIS - For testing - 90% success rate - Use CFRA for REFSENS test procedure - Allow set time + SFO for the device response time - Leave number of repeats to reach 90% success rate for the RAN5 to determine. - Further discuss if false alarm test can be considered in demod - FRC is listed as below (note: it does not mean UE should be tested for all specified FRC) SCS; 15; 15; 15; 15. PRB; 1; 2; 3; 4. SIP; Bit length; 8; 8; 8; 8. M_SIP; 4; 4; 4; 4. OOK. OFDM; 2; 2; 2; 2. CAP; Bit length; 4; 4; 4; 4. M; 2; 2; 2; 2. OOK. OFDM; 2; 2; 2; 2. PRDCH; TBS; Depending on the size of the MAC PDU of A-IoT paging message indicating CFA. CRC; 16; 16; 16; 16. Line encoding. OOK. M; 6; 6; 6; 6. postamble; Bit length; 4; 4; 4; 4. M; 6; 12; 2; 2. chip number except for SIP, padding; 228; 228; 228; 228. Padding; 6; 12; 2; 2.

From RF testing, the CFA based test procedure has been applied, and 90% success rate was considered as the test metric. The related FRC table has included the different configurations for SIP, CAP, PRDCH and postamble. Based on RF testing, we observed that the same test procedure and test metric was applied for both RF REFSENS testing and demodulation requirement testing.

Observation 1:  From the test procedure and test metric perspective, the same test up was applied for both RF REFSENS testing and demodulation requirement testing.

Based on FRC table, we observe the value is M selected as 6 for PRDCH. For RF REFSENS testing, it is usually defined at cell edge which means small data rate.  While for demodulation test point of view, it may target for high data rates with large M-value. Therefore, we think it is necessary to define PRDCH demodulation requirement, while we are also open to further discuss if companies think there is no need to define the related PRDCH demodulation requirement

Proposal 1:  RAN4 can consider define PRDCH demodulation requirement targeting high data rates

Test Methodology

Regarding test methodology, the device cannot access the network unless being trigged by the reader. To evaluate the receiver performance is to measure the success rate of the device that correctly responds to the paging that triggers the random-access procedure. RAN4 has discussed the test methodology in the last meeting based on random access procedure, where the BLER with 10% was considered based on response of Message 1 or response of Message 3, when message 0 or message 2 was transmitted for Ambient IoT device.

For A-IoT random access procedure, both CBRA and CFA are supported. From the PRDCH demodulation requirement perspective, we think there is no different. Therefore, to align with RF sensitivity test and simplify the test procedure, we prefer to use the message 0 to be tested.

Proposal 2: RAN4 can consider the message 0 to test A-IoT device demodulation requirement

Specification structure

According to core specification, the RF requirement is specified into the 38.191 for ambient IoT device. Sine ambient is a new feature with new physical layer design, it is straightforward to specify the corresponding Ambient IoT device demodulation requirement into 38.191. As for clause number, based on current latest specification structure, current latest specification structure, the clause 10 is voided, then the demodulation requirement can be specified into clause 10.

Proposal 3: RAN4 specifies the corresponding Ambient IoT device demodulation requirement into 38.191. And captured the demodulation requirements into the clause 10 of 38.191

# 3	Test Setup

In this section, the view on the test setup for PRDCH demodulation requirement.

M value

For R2D chip duration, the CP handling that retains sub-carrier orthogonality, the length of a R2D chip duration in one OFDM symbol is defined as

- C= 1/(SCS*M)

- Note: the total length of M OOK chips is equal to 1/SCS

Regarding the value of M, the following is agreed in the last meeting

Agreement - M = {2,6,12,24} are adopted for CAP and same M value is used for CAP and PRDCH in an R2D transmission. R2D postamble is specified with 4 ON chips corresponding to M value of the PRDCH - R2D postamble is added immediately after the PRDCH - R2D postamble has always 4 ON chips - Note: For M=24, 2 ON chips at the end of OFDM symbol for CP handling are in addition to R2D postamble, and are not part of the R2D postamble - R2D padding duration is determined after R2D postamble insertion TBS information for R2D is supported via higher layer R2D control signalling. - Send LS to RAN2 asking to include R2D TBS information (excluding CRC length) in higher layer signaling, at least for messages with variable size. Note: Exact method for determining the end of PRDCH at the device is not specified. Agreement For R2D, the minimum Btx,R2D # of PRBs associated to each agreed M value (i.e. M = 2, 6, 12 and 24) is specified as per TR38.769. Agreement For M=24 on CP handling, update the below agreement as follows Agreement For further down-selection among CP handing which retains subcarrier orthogonality, at least for PRDCH, at least Method Type 1 is supported - For supported M values <= 12 - RAN1 will not further pursue additional CP handling design - For supported M values > 12 - RAN1 will further down-select one from the followings - Option 1: Candidate 3 of M2-1-1 (as per agreements from RAN1#120) - Insert padding chips only at the end OOK chips of OFDM symbol - The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ - For the OFDM symbol contains CAP - The last 2 out of M OOK chips at the end of the OFDM symbol are always ‘ON’ - Option 3: RAN1 will not further pursue additional CP handling design

The value of M will impact on the length of R2D chip duration, also R2D postamble related operation, CP handling and the minimum Btx,R2D # of PRBs. Therefore, RAN4 should discuss the proper value of M when defining PRDCH requirement.

Observation 2: The value of M will impact on the length of R2D chip duration, R2D postamble related operation, CP handling and minimum Btx,R2D # of PRBs

According to TS 38.391, the length of the MAC PDU is expected to be larger than 20 bits. Since we are targeting higher data rate, we think M value as 12 or 24 can be considered. Since RAN1 will not further pursue additional CP handling in case the supported M values is small than 12, and specify the solution for M value is large than 12. Based on agreed M values, M as 24 can be considered as starting point when defining PRDCH requirement. And the same M value will be applied for CAP and PRDCH transmission

Proposal 4: RAN4 can consider M as 24 for specifying PRDCH requirement. And the same M value is applied for CAP and PRDCH transmission

TBS

For the R2D transmission, the maximum number of TBS is equal to the maximum message size of 1000 bits for A-IoT device.  The TBS is depending on the size of the MAC PDU of A-IoT paging message,

Pending on configurated bandwidth and M, RAN4 should further discuss the number of TBS used for defining PRDCH demodulation requirement.

According to the FRC for RF REFSENS testing, the number of TBS is configured as 96 with M=2. The value of TBS can be considered as starting point

Proposal 5: RAN4 should further discuss the number of TBS used for defining PRDCH demodulation requirement. The value of TBS as 96 can be considered as starting point

Channel Bandwidth

For the R2D transmission, the following is the minimum of PRB and associated with M value as indicated in the TR 38.769

Table 1: Mapping of minimum of PRB and associated with M value

| M | Minimum Btx,R2D # of PRBs |
| --- | --- |
| 1 | 1 |
| 2 | 1 |
| 4 | 1 |
| 6 | 1 |
| 8 | 2 |
| 12 | 2 |
| 16 | 2 |
| 24 | 3 |
| 32 | 4 |

Considering we propose M as 24, based on the mapping of minimum PRB and M value, we can consider the number of PRB as 3PRB as starting point. We are also open to further discuss whether other transmission bandwidth will be considered if other M-value will be considered.

Proposal 6: Considering the number of PRB as 3 for starting point when defining PRDCH requirement. FFS on other transmission bandwidth if other M-value will be considered

Channel Model

For A-IoT device, the main deployment should be targeting for indoor scenario with low speed, the channel varying is slow, similar as eMTC and NB-IoT devices. Based on RAN1 evaluation, AWGN and TDL-A channel were selected for feasibility evaluation. For A-IoT devices, the OTA test will be proceeded. In current NR UE demod test, only conducted test was supported in FR1, the feasibility to emulated TDL channel in the chamber of OTA testing should be discussed and confirmed from TE vendors. Therefore, we propose to use the TDL-A channel as starting point if the feasibility to emulated TDL channel in the chamber for Device 1 testing is confirmed from TE vendors

Proposal 7: RAN4 can consider the TDLA30-10 channel for defining the A-IoT device demodulation requirement if the feasibility to emulated TDL channel in the chamber for Device 1 testing is confirmed from TE vendors

# 4	Conclusion

In this contribution, the overview on performance part of A-IoT devices was provided.

FFS to define the PRDCH demodulation requirements

Observation 1:  From the test procedure and test metric perspective, the same test up was applied for both RF REFSENS testing and demodulation requirement testing.

Proposal 1:  RAN4 can consider define PRDCH demodulation requirement targeting high data rates

Test Methodology

Proposal 2: RAN4 can consider the message 0 to test A-IoT device demodulation requirement

Specification structure

Proposal 3: RAN4 specifies the corresponding Ambient IoT device demodulation requirement into 38.191. And captured the demodulation requirements into the clause 10 of 38.191

M value

Observation 2: The value of M will impact on the length of R2D chip duration, R2D postamble related operation, CP handling and minimum Btx,R2D # of PRBs

Proposal 4: RAN4 can consider M as 24 for specifying PRDCH requirement. And the same M value is applied for CAP and PRDCH transmission

TBS

Proposal 5: RAN4 should further discuss the number of TBS used for defining PRDCH demodulation requirement. The value of TBS as 96 can be considered as starting point

Channel Bandwidth

Proposal 6: Considering the number of PRB as 3 for starting point when defining PRDCH requirement. FFS on other transmission bandwidth if other M-value will be considered

Channel Model

Proposal 7: RAN4 can consider the TDLA30-10 channel for defining the A-IoT device demodulation requirement if the feasibility to emulated TDL channel in the chamber for Device 1 testing is confirmed from TE vendors

# References

RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR,

R4-2512603, WF for A-IoT demod_RAN4#116_final