3GPP TSG RAN WG4 Meeting #116-bis	R4-2513412

Prague, CZ, 13th – 17th Oct, 2025

Agenda item:			6.11.4

Source:	vivo

Title:	Discussion on RRM core requirement and testing framework for beam management

Document for:	Discussion

- Introduction

In RAN4#116 meeting, there were some further discussions on key issues including KPIs related, the update on simulation assumption of measurement error, and the potential RRM requirements for beam prediction. The latest outcome was recorded in the latest WF [1] [2].

In this contribution, we will further analyze the impacts on requirements for beam prediction and provide some simulation results on proposed KPIs/ Test Metrics with consideration of the measurement error modelling derived from the updated simulation assumption [3].

- Requirements for beam predictions

In the previous meetings, the RRM impacts on beam management was discussed. The package of requirements for beam predictions was identlified as follows:

- Prediction delay requirements

- The KPIs on prediction accuracy

- TCI state known/unknown related

- UE capability

Based on the latest progress of each bullet, we will further provide our analysis on the requirements.

  - Prediction delay requirements

Issue 2-2: Prediction report delay Agreement: - For aperiodic report, the overall prediction reporting delay at least includes measurement delay +reporting delay - The reporting delay includes the inference delay - For semi-persistent and periodic report, the overall prediction reporting delay at least includes measurement delay +inference delay+ time for the first available reporting occasion Issue 2-1: Measurement period for inference Agreement: observation period for prediction: (observation period is the amount of time during which UE samples the reference signals to make 1 prediction report) For case 1 reuse M,N,P from legacy measurement requirements For case 2, use T*M, N P (M,N,P same as legacy measurement requirement) FFS on T value, T can also be 1. T can also be based on capability This observation period is to be used for the prediction delay requirement

First, regarding the overall delay in the prediction report, the delay components for aperiodic, semi-persistent, and periodic reporting scenarios were discussed in the previous meeting.

Regarding the existing L1 RSRP measurement requirements, as shown below, a unified requirement is defined for the three reporting scenarios. For aperiodic reporting, no corresponding periodicity for reporting is configured. For instance, in formulas such as max(TReport, ceil(M*P*N)*TCSI-RS), only the latter part is taken as the delay for one measurement report, which is defined based on the maximum requirement to be met.

Table 9.5.4.2-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

As for the relevant conclusions previously reached by RAN1 regarding CSI computation time and the updates related to aperiodic reporting captured in TS38.214, it represents the minimum processing time required for the UE from receiving the PDSCH carrying the DCI with trigger signaling to the start of transmitting the PUSCH/PUCCH containing the CSI report. The actual delay reported by the UE must be larger than or equal to the number of slots corresponding to the Z3+d / Z3’+d’ values. Therefore, this delay defines the minimum requirement.

RAN1 agreement For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively Detailed values of d and d’ can be further discussed in UE feature. TS 38.214 ## 5.4 UE CSI computation time ***Unrelated part is omitted - (Z_3+d,Z_3^'+d'), with (Z_3,Z_3^') of the table 5.4-2 if reportQuantity is set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', where d and d' are according to UE reported capability as defined in [13, TS 38.306] and different values for d and d' may be reported by UE capability based on whether nroftimeinstance-r19 is configured or not within the CSI report, or

From this perspective, we believe that RAN4 should follow the legacy RAN4 definition of the maximum requirement when defining delay. On this basis, it is necessary to additionally consider the inference delay introduced by model inference. The specific value of this delay can directly refer to RAN1's definitions of the values d and d'. These specific values are still under discussion in RAN1. Specifically, for aperiodic reports, the corresponding L1 RSRP prediction delay requirement is:

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | ceil(M*P*N)*TCSI-RS+Tinference) |
| DRX cycle ≤ 320ms | ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)+ Tinference) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX+Tinference |

For semi-persistent and periodic reports, the difference from aperiodic reports lies in the network configuring a corresponding periodicity for reporting (i.e., Treport in the requirement), which corresponds to the existing requirement of calculating ‘max(TReport, ceil(M*P*N)*TCSI-RS)’. As for the inference delay, it can also refer to the definitions of the values d and d'. Accordingly, for semi-persistent and periodic reports, the corresponding L1 RSRP prediction delay requirement is:

| Configuration | TL1-RSRP_Measurement_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) +Tinference |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS) +Tinference) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX+Tinference |

Combining the requirements of the above three types of reports, the unified prediction report delay applicable to semi-persistent and periodic reports can be defined as:

- For CSI-RS based prediction , the prediction reporting delay requirements TL1-RSRP_Prediction_Period_CSI-RS for BM can be defined as

Table 1: Prediction period TL1-RSRP_Prediction_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(K*M*P*N)*TCSI-RS) +Tinference |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*K*M*P*N)*max(TDRX,TCSI-RS) +Tinference) |
| DRX cycle > 320 ms | ceil(K*M*P*N)*TDRX +Tinference |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tinference is the time required for model inference, which is defined by d/d' as specified according to clause 5.4 in TS 38.214 [26]. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

- For SSB-based prediction , the prediction reporting delay requirements TL1-RSRP_Prediction_Period_SSB for BM can be defined as

Table 2: Prediction period TL1-RSRP_Prediction_Period_SSB for FR2

| Configuration | TL1-RSRP_Prediction_Period_SSB (ms) |
| --- | --- |
| non-DRX | Max (TReport, ceil (K*M*P*N) *TSSB+Tinference) |
| DRX cycle ≤ 320 ms | Max (TReport, ceil (1.5*K*M*P*N) *max (TDRX, TSSB) +Tinference) |
| DRX cycle > 320 ms | Ceil (1.5*K*M*P*N) *TDRX+Tinference |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tinference is the time required for model inference, which is defined by d/d' as specified according to clause 5.4 in TS 38.214 [26]. |  |

On this basis, additionally, for BM-Case 2, regarding the value of T left over from the last meeting, this issue was parallelly discussed in RAN1 during the last meeting. As shown in the highlighted part of the corresponding agreement below, it was agreed that the value of K is indicated by a new UE capability. As for this capability, RAN1 is still discussing it.

Agreement: Support the following on the reporting condition of CSI reporting for inference - For BM-Case2, after the CSI report (re)configuration, serving cell activation, BWP change, or activation of SP-CSI, UE transmit a CSI report for inference only if receiving at least K latest consecutive transmission occasion for each of the RS resources in Set B no later than the corresponding CSI reference resource, where K is indicated by a new UE capability.

according to the following RAN1 conclusion, AP CSI-RS is not supported, and the corresponding requirements should exclude this scenario.

Conclusion For UE-sided model, for BM-Case 2, for inference, AP CSI-RS for Set B is not supported.

Proposal 1：For prediction reporting delay requirement for BM-Case 2, it is not applicable for aperiodic CSI-RS reporting scenario.

Proposal 2: RAN4 to define unified Prediction report delay, which applies to semi-persistent,periodic and periodic reports, and it can be defined as

- For CSI-RS based prediction , the prediction period requirements TL1-RSRP_Prediction_Period_CSI-RS for BM can be defined as

- Table 1: Prediction period TL1-RSRP_Prediction_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(K*M*P*N)*TCSI-RS+Tinference) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*K*M*P*N)*max(TDRX,TCSI-RS) +Tinference) |
| DRX cycle > 320 ms | ceil(K*M*P*N)*TDRX +Tinference |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tinference is the time required for model inference, which is defined by d/d' as specified according to clause 5.4 in TS 38.214 [26]. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

- For SSB-based prediction, the prediction period requirements TL1-RSRP_Prediction_Period_SSB for BM can be defined as

Table 2: Prediction period TL1-RSRP_Prediction_Period_SSB for FR2

| Configuration | TL1-RSRP_Prediction_Period_SSB (ms) |
| --- | --- |
| non-DRX | Max (TReport, ceil (K*M*P*N) *TSSB+Tinference) |
| DRX cycle ≤ 320 ms | Max (TReport, ceil (1.5* K*M*P*N) *max (TDRX, TSSB) +Tinference) |
| DRX cycle > 320 ms | Ceil (1.5* K*M*P*N) *TDRX+Tinference |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tinference is the time required for model inference, which is defined by d/d' as specified according to clause 5.4 in TS 38.214 [26]. |  |

Where,for BM-Case 1, K=1; for BM-Case 2, the value of K depends on the UE capability on the number of latest consecutive transmission occasions designed by RAN1.

Additionally, for scenarios supported by the core requirement, RAN4 needs further clarification. According to the latest UE feature [4] from RAN1, as highlighted, it supports the scenario of SSB predicting SSB. For a CSI-ReportConfig with reportQuantity-r19, it can be set to 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19'.

Observation 1: In RAN1 design, for a CSI-ReportConfig with reportQuantity-r19, it can be set to 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', which means it supports the scenario of predicting SSB.

However, this scenario has not been touched in RAN4 discussions. In particular, for simulation evaluation, according to the conclusion of the previous meeting [5], when defining performance metrics, only the following two scenarios are considered. Therefore, for the scenario of SSB predicting SSB, RAN4 needs to clarify whether the core part requirement (e.g., in the prediction delay requirement) considers this scenario.

Issue 2: The target scenarios to be defined Background: Scenario 1: CSI-RS predicts CSI-RS, narrow-to-narrow prediction Scenario 2: SSB predicts CSI-RS, wide-to-narrow prediction Agreement: Target both scenarios 1 and 2 for requirement definition

Proposal 3: RAN4 needs to clarify whether the requirement for the core part (e.g., in the prediction delay requirement) considers the scenario of predicting SSB.

  - TCI state known/unknown

Agreement in RAN4#116 meeting Issue 2-3: TCI State Handling Agreement: - On Detectability and SNR conditions: - The UE has sent at least 1 L1-RSRP inference report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB - Time conditions: - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam predict reporting or measurement - Agreement on QCL relationship - If the predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam, where TX beam can be both inside or outside set B, the corresponding Rx beam is known. - If the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, known TCI state conditions shall be based on UE capability.

For the known condition of the TCI state, it was basically confirmed in the last meeting. However, regarding the Time conditions, specifically the interpretation of "upon the last transmission of the RS resource for beam prediction reporting or measurement," we believe further clarification is needed. First, with regard to "the last transmission of the RS resource" mentioned here, since it refers to the period within 1280 ms before the TCI state switch command is sent, not all Tx beams in set A are actually transmitted—only those in set B are sent. Therefore, the RS here does not refer to the transmission of set A, but rather to the actual transmission of the beams within set B.

Proposal 4: For the agreed time conditions for TCI state, further clarification on ‘the last transmission of the RS resource’ is as follows:

- TCI state switch command is received within 1280 ms upon the last transmission of the RS resource, where the RS resource here refers to

- the RS resource used for the L1-RSRP measurement reporting for the target TCI state [legacy case] or

- the RS resource in resourcesForChannelMeasurement used for prediction of resourcesForChannelPrediction-r19 [The RS resource for set B measurements]

Furthermore, based on the aforementioned Agreement on QCL relationship RAN4, since the following two scenarios are distinguished, further classification within the known condition is also required.

- For the first bullet, it corresponds to:

  - Scenario 1-1: The predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam inside Set B.

  - Scenario 1-2: The predicted Tx beam in Set A is QCL Type-D to a known measured Tx beam outside Set B (corresponding to the legacy case).

- For the second bullet, it corresponds to:

  - Scenario 2: The predicted Tx beam in Set A is QCL Type-D to a Tx beam that is neither inside Set B nor a known measured Tx beam outside Set B.

For Scenario 1-1, "the last transmission of the RS resource" refers to the RS resource for Set B measurement. "The RS in the target TCI state or QCLed to the target TCI state" also refers to the RS resource for Set B measurement.

For Scenario 1-2, "the last transmission of the RS resource" refers to the RS resource for L1-RSRP measurement as specified in the existing TCI state case.

For Scenario 2, "the last transmission of the RS resource" refers to the RS resource for Set B measurement or legacy L1-RSRP measurement as described in Scenario 1-1 or Scenario 1-2. Meanwhile, "the RS in the target TCI state or QCLed to the target TCI state" corresponds to one of the reported predicted RSs (i.e., Set A), but this reported predicted RS is not included in the RS resource for Set B measurement or legacy L1-RSRP measurement. For this case, support for the UE capability defined by RAN4 is required.

Based on the above analysis, the corresponding "Known condition of TCI state" can be defined as follows(scenario 1-1 in green text, scenario 2 in purple text).

Proposal 5: RAN4 to update the known TCI state conditions as

8.10.2 Known conditions for TCI state The TCI state is known if the following conditions are met: - During the period from T0 the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement T0 - The UE has sent at least 1 L1-RSRP measurement or predicted RS quantities report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3 dB Where T0 is - the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state [legacy case] - or the last transmission of the RS resource in resourcesForChannelMeasurement used for prediction of resourcesForChannelPrediction-r19, where - the RS resource in resourcesForChannelMeasurement used for prediction of resourcesForChannelPrediction-r19 is the RS in target TCI state or QCLed to the target TCI state or - the one of RS resource in resourcesForChannelPrediction-r19 is the RS in the target TCI state or QCLed to the target TCI state but not in the RS resource in resourcesForChannelMeasurement, and UE supports [known TCI based on prediction capability], Otherwise, the TCI state is unknown.

  - KPIs related

2.3.1	RSRP prediction accuracy

2.3.1.1	Definition alignment

In the previous meeting, the following conclusions were reached regarding RSRP prediction accuracy. For the absolute/relative RSRP requirements, it was agreed that the specific RSRP reporting, including both absolute and relative, would be defined in RAN1. In RAN1's definition, only the RSRP reported for the predicted Top 1 beam corresponds to the absolute predicted RSRP. Therefore, for the absolute predicted RSRP, it can be applied to the Top-1 of predicted beams. For beams other than the Top 1 predicted beam, the differential values of the predicted RSRP relative to the Top 1 predicted beam are reported. Therefore, for beams other than the Top 1 predicted beam, the requirement to be met is the Relative RSRP accuracy, where the relative value refers to the comparison with the Top 1 predicted beam.

As for the specific definitions of Absolute RSRP accuracy and Relative RSRP accuracy, they were not reflected in the conclusions. RAN4 had previously discussed this part, and further clarification is needed based on those discussions. Absolute RSRP accuracy is defined for the Top 1 predicted beam.

Issue 2-9: Metrics/KPIs for beam ID prediction Agreement: - If RSRP prediction is reported, absolute/relative RSRP requirements applies to - reported beams where SNR side condition is met - The exact RSRP reporting including both absolute and relative will be defined in RAN1 - The related test can only apply to the top-K, where the value of K is FFS, beams. - The legacy beam management test and parameters can be taken as the reference to decide the exact value of K - The feasibility of the test will be further discussed and decided.

- Absolute RSRP accuracy

Absolute RSRP accuracy is define for the Top 1 predicted beam. It refers to the RSRP difference between the predicted value and the true value of that predicted beam.

Proposal 6: RAN4 to define absolute RSRP accuracy as one of metrics for beam management requirements/tests, the corresponding definition is:

- For BM-Case 1, the absolute RSRP accuracy = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i.

- For BM-Case 2, the absolute RSRP accuracy = predicted L1-RSRP of beam index i for time instance m – ground-truth of L1-RSRP of beam index i for time instance m

- Where, The index i corresponds to the index of Top-1 beam of predicted beams; the index m is corresponds to the index of time instance, 1<=m<=M

- Relative RSRP accuracy

Issue 2-3: Relative RSRP accuracy Agreement: - RAN4 to use the follow as baseline for further discussions, if the relative RSRP accuracy requirement is defined - Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n), [where the beam index n owns the largest reported value]. - Reported L1-RSRP can be predicted, - FFS whether reported L1-RSRP can be measured RSRP - Relative RSRP accuracy requirements apply for Case 1, and FFS on whether it can apply for Case 2

Regarding whether the Reported L1-RSRP can be predicted, based on the following RAN1 conclusion, the Reported L1-RSRP can only be predicted (assuming beam index n is also one of the predicted best Top K beams).

RAN1#116bis Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. RAN1#120 Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion.

"Regarding FFS whether the reported L1-RSRP can be measured RSRP, that is, considering beam index n is not a predicted beam but a purely measured beam, the currently agreed ground truth definition is based on the UE's reported measurement time. In this case, the reported L1-RSRP of beam index n will cancel out with the ground truth of L1-RSRP of beam index n."

Proposal 7: RAN4 to use the follow definition to define the relative RSRP accuracy requirement:

- For BM-Case 1, Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - Reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n)

- For BM-Case 2, the absolute RSRP accuracy = predicted L1-RSRP of beam index i for time instance m – ground-truth of L1-RSRP of beam index i for time instance m.

- Where, The index i corresponds to the index of Top-K (K is the number of reported predicted beam) beam of predicted beams and i >1; The beam index n owns the largest reported value; The index m is corresponds to the index of time instance, 1<=m<=M

- The reported L1-RSRP refers to predicted L1-RSRP

2.3.1.2	Simulation results with consideration of measurement error

In this section, we present simulation results based on the latest assumption [5] to evaluate the defined absolute RSRP accuracy requirements. We consider the performance impact of the following three cases on beam prediction accuracy in each KPI:

- Case 1: No error will be considered in training dataset, model input during inference and ground-truth.

- Case 3: Error will be considered in training dataset, model input during inference and ground-truth.

Here, baseband (BB) error is modeled using LLS-based measurement error modeling with an assigned SNR of -3 dB. For the RF error, we conducted simulations based on the RF model agreed in [5] as below.

- Truncated Gaussian distribution under ±4.5 dB RF error (μ=0,  σ2=4.5) is used

  - Each TX beam are Independent

  - 2Rx chains with independent RF error per chain (same distribution for each chain)

On the other hand, in addition to narrow-to-narrow spatial beam prediction (CSI-RS to CSI-RS), the previous meeting also introduced wide-to-narrow spatial beam prediction (SSB to CSI-RS) in the simulation assumptions. For this scenario, we have also provided corresponding simulation results.

Case 1: narrow-to-narrow spatial beam prediction (CSI-RS to CSI-RS)

Table 1-1 90%-tile RSRP difference between the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of the same beams (narrow-to-narrow)

| Simulation results | L1-RSRP difference (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Top K | 1 | 2 | 3 | 4 | 5 |
| Case 1 |  |  |  |  |  |
| No error | 2.76 | 2.83 | 3.08 | 3.23 | 3.65 |
| Case 3 |  |  |  |  |  |
| BB error | 5.19 | 5.21 | 5.24 | 5.50 | 5.62 |
| RF error | 4.77 | 4.72 | 4.98 | 4.91 | 5.13 |
| Both BB and RF | 4.80 | 4.84 | 5.23 | 5.16 | 5.08 |

Case 2: wide-to-narrow spatial beam prediction (SSB to CSI-RS)

Table 1-2 90%-tile RSRP difference between the predicted L1-RSRP of the Top-K (K=1,2,3,4,5) predicted beams and the ideal L1-RSRP of the same beams (wide-to-narrow)

| Simulation results | L1-RSRP difference (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Top K | 1 | 2 | 3 | 4 | 5 |
| Case 1 |  |  |  |  |  |
| No error | 1.76 | 1.70 | 1.98 | 2.31 | 2.73 |
| Case 3 |  |  |  |  |  |
| BB error | 5.17 | 5.20 | 5.87 | 5.85 | 6.22 |
| RF error | 4.34 | 4.53 | 6.00 | 6.38 | 7.15 |
| Both BB and RF | 4.48 | 4.89 | 6.91 | 6.97 | 7.81 |

Table 1-1 and Table1-2 show the simulation results of RSRP difference between the predicted L1-RSRP of the Top-K predicted beams and the ideal L1-RSRP of the same beams, respectively for narrow to narrow and wide to narrow.

For narrow to narrow, for Case 3 (Errors in Training, Inference, and Ground Truth), with only BB Error is considered, the difference is close to Case 1. For RF Error, the difference remains significant. When both BB and RF errors are considered, the performance is similar as the case that only RF error is considered.

For wide to narrow, for Case 1 (No Error), Lower RSRP differences compared to narrow-to-narrow, possibly due to wider beam coverage reducing sensitivity. However, for Case 3, unlike narrow-to-narrow, in most of cases, the performance is worse than narrow-to-narrow.

2.3.2	beam prediction accuracy

2.3.2.1	Definition alignment

Regarding the definition of beam prediction accuracy, significant progress was made in the RAN4#115, and the following conclusions were agreed upon through discussion.

Issue 2-1: Metrics/KPIs for beam ID prediction Agreement: - Metrics/KPIs for beam ID prediction, at least for the case if only beam ID is reported: The successful rate for the correct prediction, - The correct prediction is considered as maximum ground-truth RSRP among top-K predicted beams larger than or equal to the ground-truth RSRP of the strongest genie-aided beam(s) – x dB, - FFS on top-N (N</=K) predicted beams have to be considered in the success rate evaluation - FFS on N values - FFS on x values Note: if x=0 and N=1, it means Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

Regarding this conclusion, there are still several points to be discussed. We will analyze these points one by one.

Issue 1: Regarding Top-N

The currently approved KPI definition compares the predicted largest beam's ideal RSRP with the actual largest beam's ideal RSRP. Based on our understanding of Top-N here (the conclusion above does not clearly elaborate on Top-N; this understanding is derived from the papers submitted by companies and the discussions, in addition to this comparison, it is also necessary to compare the predicted i-th largest beam's ideal RSRP (i = all predicted beams) with the actual i-th largest beam's ideal RSRP (i = all predicted beams). This method is intended to verify whether the difference between the reported beam's actual RSRP and the corresponding ranked beam's RSRP falls within a certain range, ensuring to some extent that all beams are usable.

However, we believe the necessity of verifying this KPI is not very strong, mainly for the following reasons:

This definition is very similar to what we previously discussed in the RSRP difference KPI: comparing the predicted RSRP of the Top-K beams with the actual RSRP of the corresponding ranked beams. The only difference here is that the comparison is between actual RSRP values. According to the conclusions of previous meetings, the RSRP difference has already been agreed upon as a KPI, and its definition is quite similar to the two definitions mentioned above.

To some extent, this involves redundant comparisons. For example, the second-largest predicted RSRP beam not only needs to be compared with the second-largest actual RSRP beam but also, based on the approved part of the definition, if the second-largest predicted RSRP beam happens to be the largest actual RSRP beam, it must also be compared with the largest actual RSRP beam. This leads to too many comparison combinations to consider.

Proposal 8: For Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, RAN4 not to further consider the verification of top-N (N</=K) predicted beams in the success rate evaluation

Issue 2: Whether Beam ID Prediction Is Only Applied When Only Beam ID Is Reported

For cases where both RSRP and beam ID are reported, we prefer that beam ID accuracy should also be checked. The main reasons are:

- RSRP prediction (same ID) only ensures that the difference between the predicted and actual values falls within a certain range but does not guarantee that the Tx beam is the best. Legacy methods only consider RSRP accuracy because all values are measured. If the reported beam only satisfies RSRP accuracy (e.g., Beam 1 meets the requirement), Beam 1 may not necessarily be the strongest beam but could merely be among the actual Top-K beams. This prediction result does not reflect whether the model can correctly predict the best beam.

- If only RSRP is considered when both beam ID and RSRP are reported, and only the predicted Top-1 RSRP is checked, a possible scenario is: the predicted Top-1 is Beam 1, and its RSRP meets the accuracy requirement. However, Beam 1 might not even be among the reported Top-K beams (extreme case), making this beam practically unusable.

- From a monitoring perspective, the existing RAN1 metric for monitoring requires that at least one of the actual top-N beams (N=1 or 2) is within the predicted Top-K beams. It is unlikely that RSRP-related KPIs will be introduced. If the prediction metric is defined solely based on RSRP, a situation may arise where RSRP meets the accuracy requirement, but monitoring still fails because the two KPIs are evaluating different aspects.

Proposal 9: For Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, it will apply to the case only beam ID is reported and the case both beam ID and L1-RSRP are reported.

Issue 3: FFS on X Values

Regarding the value of X, in the simulation assumptions [2], we included three values for X: 0, 1, 2, and 3. We believe this value can be determined based on the simulation results and analysis submitted by each company.

KPI 1: Beam prediction accuracy - The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 0, 1, 2, 3dB

Proposal 10: For the X value of Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, it can be determined based on the simulation results

Specifically, according to the results discussed in RAN1#120 meeting, the step size of RSRP is 1dB. Therefore, the possible values of X can only be positive integers, which are relatively straightforward to determine.

Agreement For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size. Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

2.3.2.2	Simulation result with consideration of measurement error

Here, we also conducted simulation verification on the successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 0, 1, 2, 3dB. The simulation results are shown below.

Case 1: narrow-to-narrow spatial beam prediction (CSI-RS to CSI-RS)

Table 2-1 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction (narrow-to-narrow)

| Beam prediction accuracy (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Top-1 | Top-2 | Top-3 | Top-4 | Top-5 |
| Case 1 |  |  |  |  |  |  |
| No error | X=0 | 79.07 | 91.25 | 94.67 | 96.42 | 97.12 |
|  | X=1 | 88.93 | 94.58 | 96.77 | 97.77 | 98.22 |
|  | X=2 | 92.98 | 96.50 | 97.85 | 98.47 | 98.73 |
|  | X=3 | 95.40 | 97.65 | 98.58 | 98.97 | 99.13 |
| Case 3 |  |  |  |  |  |  |
| BB error | X=0 | 60.67 | 83.27 | 89.80 | 93.82 | 95.70 |
|  | X=1 | 72.47 | 89.10 | 93.28 | 95.92 | 97.13 |
|  | X=2 | 81.40 | 93.03 | 95.60 | 97.27 | 98.00 |
|  | X=3 | 87.78 | 95.17 | 97.17 | 98.20 | 98.63 |
| RF error | X=0 | 57.93 | 81.05 | 88.63 | 92.77 | 94.90 |
|  | X=1 | 69.48 | 87.27 | 92.88 | 95.17 | 96.58 |
|  | X=2 | 78.63 | 91.70 | 95.18 | 96.70 | 97.67 |
|  | X=3 | 85.22 | 94.15 | 96.67 | 97.77 | 98.38 |
| Both BB and RF | X=0 | 60.68 | 81.70 | 88.57 | 92.50 | 94.68 |
|  | X=1 | 71.80 | 87.38 | 92.32 | 94.83 | 96.25 |
|  | X=2 | 80.47 | 91.68 | 95.05 | 96.53 | 97.45 |
|  | X=3 | 86.73 | 94.22 | 96.78 | 97.72 | 98.17 |

Case 2: wide-to-narrow spatial beam prediction (SSB to CSI-RS)

Table 2-2 Evaluation results of prediction accuracy for BM-Case1 DL Tx beam prediction (wide-to-narrow)

| Beam prediction accuracy (%) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Top-1 | Top-2 | Top-3 | Top-4 | Top-5 |
| Case 1 |  |  |  |  |  |  |
| No error | X=0 | 85.92 | 95.98 | 98.50 | 99.27 | 99.55 |
|  | X=1 | 94.85 | 98.52 | 99.43 | 99.68 | 99.78 |
|  | X=2 | 97.22 | 99.48 | 99.78 | 99.90 | 99.92 |
|  | X=3 | 98.50 | 99.75 | 99.90 | 99.98 | 99.98 |
| Case 3 |  |  |  |  |  |  |
| BB error | X=0 | 60.28 | 83.50 | 91.87 | 95.70 | 97.45 |
|  | X=1 | 71.45 | 90.07 | 95.60 | 97.82 | 98.78 |
|  | X=2 | 81.02 | 94.50 | 97.95 | 99.05 | 99.53 |
|  | X=3 | 87.68 | 96.72 | 98.87 | 99.53 | 99.78 |
| RF error | X=0 | 57.77 | 81.97 | 90.47 | 94.67 | 97.00 |
|  | X=1 | 68.57 | 88.50 | 94.58 | 97.15 | 98.48 |
|  | X=2 | 77.85 | 93.35 | 97.38 | 98.72 | 99.45 |
|  | X=3 | 84.68 | 95.82 | 98.58 | 99.35 | 99.77 |
| Both BB and RF | X=0 | 57.32 | 80.48 | 89.43 | 94.05 | 96.42 |
|  | X=1 | 68.05 | 87.57 | 93.72 | 96.67 | 98.12 |
|  | X=2 | 78.00 | 92.67 | 96.67 | 98.23 | 99.10 |
|  | X=3 | 84.63 | 95.47 | 98.05 | 99.00 | 99.55 |

For Narrow-to-Narrow Spatial Beam Prediction, in the no error case (Case 1), accuracy improves significantly with a higher Top-K. Relaxing the X threshold (X=3) boosts Top-1 accuracy to 95.4% when K=1. For the combined errors considered in Case 3, it shows that when X=0, K must be larger than 4, , the accuracy can exceed 90%.

For Wide-to-Narrow Spatial Beam Prediction, in the no error case (Case 1), the baseline accuracy is significantly higher than that of narrow-beam. For the combined errors considered in Case 3, it also shows that when X=0, K must be larger than 4, or when K=2 and X=1, the accuracy can exceed 90%.

  - UE capability

For scenarios where the predicted Tx beam in Set A is not QCL Type-D to a known Tx beam, RAN4 has agreed to introduce the relevant UE capability. We propose the specific description of this UE capability as follows:

NR_AIML_air

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_AIML_air |  | Support known Rx beam with Tx beam prediction | Indicates whether UE can know corresponding Rx beam for the predicted Tx beam | At least one among RAN1 feature 58-1-2, 58-1-3, 58-1-4, 58-1-5 | Yes | No | Rx beam sweeping is needed, e.g., during TCI state switching procedure | Per UE | No | FR2 only | N/A |  | Optional with capability signalling |

- Summary

Proposal 1：For prediction reporting delay requirement for BM-Case 2, it is not applicable for aperiodic CSI-RS reporting scenario.

Proposal 2: RAN4 to define unified Prediction report delay, which applies to semi-persistent,periodic and periodic reports, and it can be defined as

- For CSI-RS based prediction , the prediction period requirements TL1-RSRP_Prediction_Period_CSI-RS for BM can be defined as

- Table 1: Prediction period TL1-RSRP_Prediction_Period_CSI-RS for FR2

| Configuration | TL1-RSRP_Prediction_Period_CSI-RS (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(K*M*P*N)*TCSI-RS+Tinference) |
| DRX cycle ≤ 320 ms | max(TReport, ceil(1.5*K*M*P*N)*max(TDRX,TCSI-RS) +Tinference) |
| DRX cycle > 320 ms | ceil(K*M*P*N)*TDRX +Tinference |
| NOTE 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tinference is the time required for model inference, which is defined by d/d' as specified according to clause 5.4 in TS 38.214 [26]. NOTE 2: the requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3. |  |

- For SSB-based prediction, the prediction period requirements TL1-RSRP_Prediction_Period_SSB for BM can be defined as

Table 2: Prediction period TL1-RSRP_Prediction_Period_SSB for FR2

| Configuration | TL1-RSRP_Prediction_Period_SSB (ms) |
| --- | --- |
| non-DRX | Max (TReport, ceil (K*M*P*N) *TSSB+Tinference) |
| DRX cycle ≤ 320 ms | Max (TReport, ceil (1.5* K*M*P*N) *max (TDRX, TSSB) +Tinference) |
| DRX cycle > 320 ms | Ceil (1.5* K*M*P*N) *TDRX+Tinference |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Tinference is the time required for model inference, which is defined by d/d' as specified according to clause 5.4 in TS 38.214 [26]. |  |

Where,for BM-Case 1, K=1; for BM-Case 2, the value of K depends on the UE capability on the number of latest consecutive transmission occasions designed by RAN1.

Observation 1: In RAN1 design, for a CSI-ReportConfig with reportQuantity-r19, it can be set to 'p-ssb-index-r19' or 'p-ssb-index-RSRP-r19', which means it supports the scenario of predicting SSB.

Proposal 3: RAN4 needs to clarify whether the requirement for the core part (e.g., in the prediction delay requirement) considers the scenario of predicting SSB.

Proposal 4: For the agreed time conditions for TCI state, further clarification on ‘the last transmission of the RS resource’ is as follows:

- TCI state switch command is received within 1280 ms upon the last transmission of the RS resource, where the RS resource here refers to

- the RS resource used for the L1-RSRP measurement reporting for the target TCI state [legacy case] or

- the RS resource in resourcesForChannelMeasurement used for prediction of resourcesForChannelPrediction-r19 [The RS resource for set B measurements]

Proposal 5: RAN4 to update the known TCI state conditions as

The TCI state is known if the following conditions are met:

-	During the period from T0 the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state

-	TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement T0

-	The UE has sent at least 1 L1-RSRP measurement or predicted RS quantities report for the target TCI state before the TCI state switch command

-	The TCI state remains detectable during the TCI state switching period

-	The SSB associated with the TCI state remain detectable during the TCI switching period

-	SNR of the TCI state ≥ -3 dB

Where T0 is

-	the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state [legacy case]

-	or the last transmission of the RS resource in resourcesForChannelMeasurement used for prediction of resourcesForChannelPrediction-r19, where

-	the RS resource in resourcesForChannelMeasurement used for prediction of resourcesForChannelPrediction-r19 is the RS in target TCI state or QCLed to the target TCI state or

-	the one of RS resource in resourcesForChannelPrediction-r19 is the RS in the target TCI state or QCLed to the target TCI state but not in the RS resource in resourcesForChannelMeasurement, and UE supports [known TCI based on prediction capability],

Otherwise, the TCI state is unknown.

Proposal 6: RAN4 to define absolute RSRP accuracy as one of metrics for beam management requirements/tests, the corresponding definition is:

- For BM-Case 1, the absolute RSRP accuracy = predicted L1-RSRP of beam index i – ground-truth of L1-RSRP of beam index i.

- For BM-Case 2, the absolute RSRP accuracy = predicted L1-RSRP of beam index i for time instance m – ground-truth of L1-RSRP of beam index i for time instance m

- Where, The index i corresponds to the index of Top-1 beam of predicted beams; the index m is corresponds to the index of time instance, 1<=m<=M

Proposal 7: RAN4 to use the follow definition to define the relative RSRP accuracy requirement:

- For BM-Case 1, Relative RSRP accuracy for reported beams during inference reporting = (predicted L1-RSRP of beam index i - Reported L1-RSRP of beam index n) - (ground truth of L1-RSRP of beam index i - ground truth of L1-RSRP of beam index n)

- For BM-Case 2, the absolute RSRP accuracy = predicted L1-RSRP of beam index i for time instance m – ground-truth of L1-RSRP of beam index i for time instance m.

- Where, The index i corresponds to the index of Top-K (K is the number of reported predicted beam) beam of predicted beams and i >1; The beam index n owns the largest reported value; The index m is corresponds to the index of time instance, 1<=m<=M

- The reported L1-RSRP refers to predicted L1-RSRP

Proposal 8: For Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, RAN4 not to further consider the verification of top-N (N</=K) predicted beams in the success rate evaluation

Proposal 9: For Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, it will apply to the case only beam ID is reported and the case both beam ID and L1-RSRP are reported.

Proposal 10: For the X value of Metrics/KPIs for beam ID prediction agreed in RAN4#114bis, it can be determined based on the simulation results

- References

- R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated

- R4-2511849, WF on [116][132] NR_AIML_air_part2, Ericsson

- R4-2511797, WF on updated simulation assumption for AI based beam management, vivo, NTU, Nokia, Ericsson, Qualcomm, Xiaomi, Huawei, Hisilicon, Mediatek, OPPO, APPLE, Rohde & Schwarz, CATT, Samsung, Intel, ZTE Corporation, Sanechips, CAICT

- R1-2506627, Updated RAN1 UE features list for Rel-19 NR after RAN1 #122, AT&T, NTT DOCOMO, INC.

- R4-2511892, AH minutes for BM simulation assumption in AI/ML for air interface [116][131], vivo, Qualcomm Incorporated