3GPP TSG-RAN WG4 Meeting #116bis	R4-2513423

Prague, Czech Republic, October 13 – 17, 2025

Agenda Item:	6.11.6

Source:	Huawei, HiSilicon

Title:	Discussion on RRM performance requirements for beam management and positioning accuracy

Document for:	Discussion

- Introduction

According to WF in RAN4 #115 [1], we discuss the remaining issues related to the Discussion on RRM performance requirements for beam management.

- Measurement error impact on prediction accuracy in AI-BM

2.1 Measurement error modelling in AI-BM

In the following, we provide the relationship between SNR and measurement error under Gaussian distribution assumption.

Figure 1. Relationship between SNR and baseband measurement error

Observation 1: With Gaussian distribution assumption of baseband measurement error, under different SNRs, the mean values are all close to 0, while the variance increases as the SNR drops.

Based on the generated error distribution, the error is sampled and then added for each individual Tx-Rx beam pair according to the SNR. For RF error, we employ the assumption described in [2], i.e., truncated Gaussian distribution under ±4dB (μ=0,  σ=2), regardless of the SNR.

2.2 Simulation results

In [2], two categories of KPI are considered. The first one is beam prediction accuracy, while the second on is absolute RSRP accuracy.

Updated simulation assumptions for beam prediction [2] At least the following performance characteristics are to be provided KPI 1: Beam prediction accuracy - The successful rate for the correct prediction which is considered as maximum ground truth RSRP among top-K predicted beams is larger than the ground truth RSRP of the strongest genie-aided beam – x dB, where K=1,2,3,4,5, X= 1, 2, 3dB KPI 2: Absolute RSRP accuracy - 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1-RSRP of the same beams Note 1: the ‘ground truth’ underlined refers to: - Ideal value without consideration of measurement error (for Case 1 and Case 2a defined in clause 6) - Measurement result with consideration of measurement error (for Case 2b and Case 3 defined in clause 6) Note 2: 90%-tile L1-RSRP difference = max (abs (95%-tile L1-RSRP), abs (5%-tile L1-RSRP))

In the following, we provide the simulation results regarding to KPI 1, i.e., beam prediction accuracy with X=1, 2, 3dB margin.

Proposal 1: The simulation results for different prediction accuracy regarding to KPI 1 is shown in Table 1.

Table 1. Measurement error impact on prediction accuracy regarding to KPI 1

| KPI-1 |  | Model input w/o measurement error | Model input w measurement error |
| --- | --- | --- | --- |
| maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB | x = 1dB | 83.02% | 60.75% |
|  | x = 2dB | 92.49% | 72.87% |
|  | x = 3dB | 96.29% | 81.84% |

From Table 1, we observe that measurement error impacts prediction accuracy, and the more stringent the KPIs are, the greater the impact.

Observation 2: The more stringent the KPIs are, the greater the impact of the measurement error on the prediction accuracy will be.

In the following, we provide the simulation results regarding to KPI 2, i.e., 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1-RSRP of the same beams.

Proposal 2: The simulation results for different prediction accuracy regarding to KPI 2 is shown in Table 2.

Table 2. Measurement error impact on prediction accuracy regarding to KPI 2

| KPI-2 |  |  | Model input w/o measurement error (case 1) | Model input w measurement error (case 2b) |
| --- | --- | --- | --- | --- |
| 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1- RSRP of the genie aided strongest beam | Avg. | CDF_strong_1 | 2.547dB | 5.871dB |
|  |  | CDF_strong_3 | 2.345dB | 5.392dB |
|  |  | CDF_strong_5 | 2.209dB | 4.893dB |
|  | w/o avg. | CDF_strong_Top1 | 2.547dB | 5.871dB |
|  |  | CDF_strong_Top2 | 2.380dB | 5.492dB |
|  |  | CDF_strong_Top3 | 2.139dB | 4.781dB |
|  |  | CDF_strong_Top4 | 2.049dB | 4.445dB |
|  |  | CDF_strong_Top5 | 1.967dB | 4.044dB |
| 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1-RSRP of the same beams | Avg. | CDF_same_Top1 | 2.151dB | 7.250dB |
|  |  | CDF_same_Top1&2&3 | 2.236dB | 7.406dB |
|  |  | CDF_same_Top1&2&3&4&5 | 2.419dB | 7.444dB |
|  | w/o avg. | CDF_same_Top1 | 2.151dB | 7.250dB |
|  |  | CDF_same_Top2 | 2.200dB | 7.314dB |
|  |  | CDF_same_Top3 | 2.435dB | 7.702dB |
|  |  | CDF_same_Top4 | 2.674dB | 7.730dB |
|  |  | CDF_same_Top5 | 2.689dB | 7.287dB |

Besides reaching the same observations as under KPI 1, we note that regarding RSRP prediction accuracy, measurement errors have a larger impact on the Top 1 predicted beam than on the Top 5th predicted beam.

Observation 3: Regarding RSRP prediction accuracy, measurement errors have a larger impact on the Top 1 predicted beam than on the Top 5th predicted beam.

Proposal 3: RAN4 define requirements with measurement error as model input.

Proposal 4: RAN4 will not select KPI under which acceptable performance cannot be achieved, including

- the KPI under which the prediction accuracy is less than 80%, e.g, Top K/1 with K<4 without RSRP margin.

- the KPI under which the 90%-tile L1-RSRP difference is larger than 5dB, e.g, maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB with x<3dB.

- Considerations on requirement definition under LLS

We understand that in RAN4 legacy, only the link-level channel model is used for defining test cases and requirements. Since there may exist a gap between the impact derived from system-level simulations and the impact derived from link-level simulations, how to use the evaluation results with system-level simulation assumptions for requirement definition study is still an open issue.

- Conclusions

According to the discussion, following proposals and observations are provided:

Observation 1: With Gaussian distribution assumption of baseband measurement error, under different SNRs, the mean values are all close to 0, while the variance increases as the SNR drops.

Proposal 1: The simulation results for different prediction accuracy regarding to KPI 1 is shown in Table 1.

Table 1. Measurement error impact on prediction accuracy regarding to KPI 1

| KPI-1 |  | Model input w/o measurement error | Model input w measurement error |
| --- | --- | --- | --- |
| maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB | x = 1dB | 83.02% | 60.75% |
|  | x = 2dB | 92.49% | 72.87% |
|  | x = 3dB | 96.29% | 81.84% |

Observation 2: The more stringent the KPIs are, the greater the impact of the measurement error on the prediction accuracy will be.

Proposal 2: The simulation results for different prediction accuracy regarding to KPI 2 is shown in Table 2.

Table 2. Measurement error impact on prediction accuracy regarding to KPI 2

| KPI-2 |  |  | Model input w/o measurement error (case 1) | Model input w measurement error (case 2b) |
| --- | --- | --- | --- | --- |
| 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1- RSRP of the genie aided strongest beam | Avg. | CDF_strong_1 | 2.547dB | 5.871dB |
|  |  | CDF_strong_3 | 2.345dB | 5.392dB |
|  |  | CDF_strong_5 | 2.209dB | 4.893dB |
|  | w/o avg. | CDF_strong_Top1 | 2.547dB | 5.871dB |
|  |  | CDF_strong_Top2 | 2.380dB | 5.492dB |
|  |  | CDF_strong_Top3 | 2.139dB | 4.781dB |
|  |  | CDF_strong_Top4 | 2.049dB | 4.445dB |
|  |  | CDF_strong_Top5 | 1.967dB | 4.044dB |
| 90%-tile L1-RSRP difference between the predicted L1-RSRP of the Top-1/ Top-3/ Top-5 predicted beam(s) and the ground truth L1-RSRP of the same beams | Avg. | CDF_same_Top1 | 2.151dB | 7.250dB |
|  |  | CDF_same_Top1&2&3 | 2.236dB | 7.406dB |
|  |  | CDF_same_Top1&2&3&4&5 | 2.419dB | 7.444dB |
|  | w/o avg. | CDF_same_Top1 | 2.151dB | 7.250dB |
|  |  | CDF_same_Top2 | 2.200dB | 7.314dB |
|  |  | CDF_same_Top3 | 2.435dB | 7.702dB |
|  |  | CDF_same_Top4 | 2.674dB | 7.730dB |
|  |  | CDF_same_Top5 | 2.689dB | 7.287dB |

Observation 3: Regarding RSRP prediction accuracy, measurement errors have a larger impact on the Top 1 predicted beam than on the Top 5th predicted beam.

Proposal 3: RAN4 define requirements with measurement error as model input.

Proposal 4: RAN4 will not select KPI under which acceptable performance cannot be achieved, including

- the KPI under which the prediction accuracy is less than 80%, e.g, Top K/1 with K<4 without RSRP margin.

- the KPI under which the 90%-tile L1-RSRP difference is larger than 5dB, e.g, maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB with x<3dB.

References

- R4-2508080, “WF on AI ML for air interface study”, 3GPP RAN4 #115, 2025.

- R4-2508081, “Updated simulation assumptions for AI/ML BM”, 3GPP RAN4 #115, 2025.