# 3GPP TSG-RAN WG4 Meeting #116bis	  R4-2513430

Prague, Czech Republic, October 13 – 17, 2025

Title: 	Discussions on demodulation and CSI requirements for SCM

Source: 	Huawei, HiSilicon

Agenda item:	7.5.3

Document for:	Discussion

Background

A new WID[1] was approved, where one objective is to specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1

- Specify optional performance requirements under a single spatial channel model for SU-MIMO in FR1 - Using rCDL-C1 channel model detailed in TR 38.753 as starting point - Specify PDSCH demodulation performance requirements for the 8T8R 2 codeword use case and the 4T4R case. - Study, and if/when performance alignment on PMI reporting is achieved, specify requirements for PMI performance with 4T4R and 8T4R eTypeII precoding. - Note 1: The rCDL-C1 based spatial channel model detailed in TR 38.753 needs to be updated to address issues identified in the SI, e.g., frequency generalization and PMI bias - Note 2: The outcome from RAN1 SI (FS_NR_7_24GHz_Chmod) to update the antenna model and scaling of angles in TR 38.901 needs to be considered.

In this contribution we provide our related views.

- Discussions

Channel model

- rCDL-C1 channel model was agreed for requirements definition, but there are so many left issues to be addressed. Before discussing the detailed test parameters, it’s necessary to consolidate the rCDL-C1 study and address all issues identified in study item. Below text list all identified issues:

          - Study if rCDL-C1 can be applicable for all frequency range in FR1

          - Introduce time varying spatial property for rCDL-C1 channel to enable valuable PMI test

          - The generation of rCDL-C1 channel is not aligned with the that defined in latest version of 39.901

The issues will be discussed one by one in below text.

Frequency generation

FR1 spans a large frequency range from 700MHz to 3.5GHz, which means the angular spread is quite different for FR1 bands. Lower frequency resulting in large angular spread and 8 layers may perform worse compared to high frequency band,  We propose RAN4 to evaluate the 8Rx and 4Rx performance with CDL channel based on some typical 8Rx and 4Rx bands  respectively. If all typical bands exhibit similar performance, RAN4 can define single CDL channel for all 8Rx and 4Rx requirements.

Proposal 1: RAN4 to evaluate the 8Rx and 4Rx performance with CDL channel based on some typical 8Rx and 4Rx bands  respectively. If all typical bands exhibit similar performance, RAN4 can define single CDL channel for all 8Rx and 4Rx requirements.

PMI bias

Beam steering defined in current spec is applied to legacy PMI test, where the intention is to make the cluster of TDL channel vary to verify UE’s capability to perform PMI selection correctly under channel cluster with different direction. Reusing beam steering approach for CDL channel is technically feasible, but the issue is that it artificially changes the inherent spatial properties of CDL channel matched with real deployment of CDL channel, so that the practicality will be destroyed.

Observation 2: Reusing beam steering approach for CDL channel is technically feasible, but the issue is that it artificially changes the inherent spatial properties of CDL channel matched with real deployment of CDL channel, so that the practicality will be destroyed.

We propose RAN4 to exploit a new approach which remains the inherent spatial properties of CDL channel.

Proposal 2: To address the PMI bias issue, RAN4 shall exploit a new approach which remains the inherent spatial properties of CDL channel.

UE antenna modelling

Two UE antenna models are defined in latest version of 38.901: Handheld UE and FWA.

Follow text summarizes the antenna models for handheld UE:

- Antenna placement candidate locations and some typical cases for orientation are specified

- Polarized antenna modelling is specified

- Radiation power pattern of a single antenna element is specified.

For antenna models for FWA, antenna placement candidate locations are specified. Figure 2-2 illustrates the antenna placement candidate locations for handheld UE and FWA:

Figure 2-2: Antenna placement candidate locations for handheld UE and FWA

Below text specifies some typical cases for orientation for handheld UE.

For calibration with spatial non-stationarity modelling: - For one-hand blockage, ΩUT,α = 0 – 360 degrees, ΩUT,β = 45 degrees, ΩUT,γ = 0 degrees, - For dual-hand blockage, ΩUT,α = 0 – 360 degrees, ΩUT,β = 0 degrees, ΩUT,γ = 45 degrees, - For hand and head blockage, ΩUT,α = 0 – 360 degrees, ΩUT,β = 90 degrees, ΩUT,γ = 0 degrees, - For all other cases: - ΩUT,α = 0 – 360 degrees, ΩUT,β = 45 degrees, ΩUT,γ = 0 degrees,

rCDL-C1 is created based on XPL antenna model with omni radiation pattern, which should be updated according to the above content. For cases with 4Rx, handheld UE modelling should be used, for cases with 8Rx, FWA modelling should be used.

Proposal 3: RAN4 to update the antenna modelling of rCDL-C1 to be aligned with the modelling defined in v19.0.0 of 38.901.

- For cases with 4Rx, handheld UE modelling should be used

- For cases with 8Rx, FWA modelling should be used.

Angular scaling

RAN1 also updated the angular scaling procedure, it’s proposed to update the rCDL-C1 based on the consideration of new angular scaling procedure.

Proposal 4: RAN4 to update the rCDL-C1 based on the new angular scaling procedure.

Test cases

PDSCH requirements

Considering the new channel model has no impact on baseband processing and the requirements are optional, it’s proposed to define limited number of test cases. I.e. One case for 4T4R case and one case for 8T8R case. It’s proposed to reuse the test setup of Rel-19 SI for R20 PDSCH requirements definition.

Proposal 5: RAN4 shall reuse the test setup of Rel-19 SI for R20 PDSCH requirements definition based on the updated channel model.

PMI requirements

Companies’ results for PMI test cases in Rel-19 are not aligned. Even RAN4 has never aligned the target SNR for PMI test, but RAN4 always collected the target SNR for PMI test and the SNR was always well aligned. Aligning SNR help align the channel implementation and verify the correction of PMI selection algorithm. Our proposal is that RAN4 continue to do the PMI simulations and strive to align the simulation results based on the updated channel model and spatially varying approach and make the decision according to the situation.

Proposal 6: RAN4 continue to do the PMI simulations in Rel-20 and strive to align the simulation results based on the updated channel model and spatially varying approach and make the decision according to the situation.

For PMI requirements, it’s proposed to reuse the test setup of Rel-19 for evaluation and requirements definition (If agreed)

Proposal 7: For PMI requirements, it’s proposed to reuse the test setup of Rel-19 for evaluation and requirements definition (If agreed)

- Conclusion

In this contribution we provide our views on demodulation and CSI requirements for SCM, the proposals and observations are:

Proposal 1: RAN4 to evaluate the 8Rx and 4Rx performance with CDL channel based on some typical 8Rx and 4Rx bands  respectively. If all typical bands exhibit similar performance, RAN4 can define single CDL channel for all 8Rx and 4Rx requirements.

Observation 2: Reusing beam steering approach for CDL channel is technically feasible, but the issue is that it artificially changes the inherent spatial properties of CDL channel matched with real deployment of CDL channel, so that the practicality will be destroyed.

Proposal 2: To address the PMI bias issue, RAN4 shall exploit a new approach which remains the inherent spatial properties of CDL channel.

Proposal 3: RAN4 to update the antenna modelling of rCDL-C1 to be aligned with the modelling defined in v19.0.0 of 38.901.

- For cases with 4Rx, handheld UE modelling should be used

- For cases with 8Rx, FWA modelling should be used.

Proposal 4: RAN4 to update the rCDL-C1 based on the new angular scaling procedure.

Proposal 5: RAN4 shall reuse the test setup of Rel-19 SI for R20 PDSCH requirements definition based on the updated channel model.

Proposal 6: RAN4 continue to do the PMI simulations in Rel-20 and strive to align the simulation results based on the updated channel model and spatially varying approach and make the decision according to the situation.

Proposal 7: For PMI requirements, it’s proposed to reuse the test setup of Rel-19 for evaluation and requirements definition (If agreed)

- Reference

[1]   RP-252957	New WID: NR demodulation performance: Phase 6