3GPP TSG-RAN WG4 Meeting #116bis	R4-2513458

# Prague, CZ, 13-17 Oct, 2025

Source: 	Xiaomi

Title: 	(NR_LPWUS-Core) Discussion on remaining issues on RRM core requirements for LP-WUS/WUR

Agenda Item:	4.22.3

Document for:	Discussion

# 1Introduction

In RAN4#116 meeting, the RRM impacts on low-power wake-up signal and receiver for NR (LP-WUS/WUR) was discussed and a WF was approved [1]. In this contribution, we provide our further analysis on RRM impacts of LP-WUS/WUR features.

- Discussion

Issue 1-2-12: UE behaviro when both Rel-16 relaxation and Rel-19 LP-WUR offloading/relaxation are satisfied; or when both Rel-17 relaxation for Redcap and Rel-19 LP-WUR offloading/relaxation are satisfied Agreement: UE is allowed to follow the most relaxation requirements when only Rel-16 not at cell edge is configured, and Rel-16 not at cell edge and Rel-19 LP-WUR offloading/relaxation are both satisfied and Rel-16 EMR is not configured or Rel-16 EMR is configured and T331 is not running.

In last meeting, RAN4 discussed the combination scenario when UE is configured and satisfied with different kind of relaxation cases. Similar discussion happened in Rel-17 when both Rel-16 and Rel-17 criteria are configured and satisfied. And the conclusion is that UE is allowed to meet the requirements that are the most relaxed out of Rel-16 and Rel-17 requirements. We understand that some companies have concern that, the principle of low mobility criterion is not the same as not at cell edge. However, RAN2 has reached conclusion that RAN2 assumes UE low mobility criterion is not included for Rel-19 LP-WUS RRM relaxation/offloading mode, which means UE would not evaluate UE mobility state for entering/leaving Rel-19 relaxation mode. We prefer RAN4 to take the previous conclusion as baseline, i.e., UE is allowed to follow the most relaxation requirements when multiple relaxation criteria are configured and satisfied.

Proposal 1: UE is allowed to follow the most relaxation requirements when multiple relaxation criteria are configured and satisfied.

Issue 1-5-3: MR wake up delay after exiting case 1 due to offloading conditions cannot be met Agreement: When exiting case 1 due to exiting conditions of offloading is met, MR reuse the existing wake up delay. Discuss on the measurement periodicity after wake up. Discuss on whether the wake up delay apply in the scenario when exiting from case 3.

Similar discussion happened for the case when existing from case 3. The main difference from case 1 and case 3 is on the MR status. MR is off for case 1, while is in relaxation for case 3. The wake up delay defined in RAN1 comprise of the time for MR ramping up and for MR synchronization, which from our understanding is not needed for MR in case3.

Proposal 2: The wake up delay does not apply in the scenario when exiting from case 3.

The next issue is on the measurement periodicity after wake up. Generally, we think this issue can be further considered as what is the UE behavior after existing case1/case3. There are three measurement states in Rel-19, with different relaxation level. In our understanding, UE behavior after existing relaxation cases can be up to UE implementation. From requirements perspective, it is reasonable to define the less stringent requirement.

Proposal 3: UE shall meet the less stringent requirement after existing case 1/case 3 due to not fulfilling the entry condition, e.g.

- After existing Case 1, UE shall meet the requirements for Case 3 (if the related relaxation criteria are configured);

- After existing Case 3, UE shall meet the legacy requirements.

- Conclusion

Proposal 1: UE is allowed to follow the most relaxation requirements when multiple relaxation criteria are configured and satisfied.

Proposal 2: The wake up delay does not apply in the scenario when exiting from case 3.

Proposal 3: UE shall meet the less stringent requirement after existing case 1/case 3 due to not fulfilling the entry condition, e.g.

- After existing Case 1, UE shall meet the requirements for Case 3 (if the related relaxation criteria are configured);

- After existing Case 3, UE shall meet the legacy requirements.

- Reference

- R4-2512139, WF on RRM requirements for NR_LPWUS, Vivo