3GPP TSG-RAN WG4 Meeting #116bis                   	R4-2513463

Agenda item:			7.6.3

Source:	Xiaomi

Title:	Discussion on R20 HPUE NTN TX requirement

Document for:	Discussion

Background

In t he RAN#109 meeting, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2 by the meeting aims to complete missing requirements for NR-NTN (Non-Terrestrial Networks) and IoT-NTN that were not able to be concluded in Rel-19.

HPUE for NTN Core Part

- Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types

  - Example bands are Band 256 and 255

  - Necessary Tx requirements, at least including

    - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

  - Necessary Rx requirements, at least including

    - Specify the RSD requirements on the example bands, if necessary

  - Specify the necessary signaling to support the above objectives

- The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

In this document, we would like to discuss the Tx and Rx requriements for HPUE for IoT-NTN.

Disucussion

To specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands, n256 and n255 are specified as the example bands, PC1.5 with 1Tx for handheld and non-handheld device types are the two types should be discussed.

Tx requirements

In the release-19, the coexistence study did not cover the case of PC1.5 NB-IoT for HPUE NTN. As the important value to determine the determine MPR and AMPR, it is necessary to discuss how to detetrmine this value for PC1.5. One way is to rerun the simulation and analyse the simulation result again. However, the simulation assumptions should be discussed again in order to do this work. Another way is the pick a middle value from Release-19 study between PC1 and PC2. But this way should based on the fully discussion that the simulation assumptions of PC2 and PC1 can cover the situation of PC1.5.

RAN4 should consider the ALCR of PC1.5 NB-IoT based IoT-NTN in FR1-NTN bands

- In Release-19, the SEM agreement for PC2/1 reapplies the SEM requirements originally defined for PC3 to NB-IoT-based IoT-NTN. Based on previous RAN4 discussions, the SEM represents a major bottleneck for higher-power systems. Consequently, relaxing SEM requirements for high-power UEs is not feasible and has not been evaluated. Therefore, following the Release-19 agreement to reuse PC3's SEM requirements for NB-IoT-based IoT-NTN remains the best approach.

Reusing SEM requirements defined for PC3 for NB-Iot based Iot-NTN

- In release-19 the MPR and AMPR calibration assumption for PC2 and PC1 shown as below:

- Agreement: - [3] dB post-PA insertion losses for calibration purpose - For PC2, PC3 NR PA can also be considered for reference architecture - Calibration points for PC2 (PC3 NR PA) (further update based on offline common view): - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage: 28 dBc - IQ image: 28 dBc - CIM3: 60 dBc - NR ACLR: 30dBc, other number is not precluded - Calibration based on NB-IOT signal is not precluded. - For PC1, consider adding [-34] dBc IQ-image and LO-leakage and 60 dBc CIM3 as another option for evaluation - Calibration points for PC1 (based on NR PC2 PA) (further update based on offline common view): - DFT-s-OFDM QPSK 20 MHz - 100RB0 - 4dB post PA loss - 1dB MPR - Carrier leakage: 28 dBc - IQ image: 28 / [-34]dBc - CIM3: 60 dBc - NR ACLR: 31dBc, other number is not precluded - Calibration based on NB-IOT signal is not precluded.

- The major different is the ACLR values, but only 1dB different. Therefore, it is good to use the rel-19 calibration assumptions as the baseline to trigger the discussion of MPR and AMPR simulation.

Using Rel-19 assumptions as the baseline of PC1.5 MPR and AMPR simulation.

Conclusion

- RAN4 should consider the ALCR of PC1.5 NB-IoT based IoT-NTN in FR1-NTN bands

- Reusing SEM requirements defined for PC3 for NB-Iot based Iot-NTN

- Using Rel-19 assumptions as the baseline of PC1.5 MPR and AMPR simulation.