3GPP TSG-RAN WG4 Meeting #116bis	R4-2513474

Prague, Czech Republic, 13-17 October 2025

Source: 	Huawei, HiSilicon

Title:  	Discussions on RRM performance requirements for Rel-19 ATG

Agenda Item:	6.7.3

Document for:	Discussion

Introduction

In the last meeting, we discussed the Rel-19 ATG CA enhancement. The TU schedule of #114bis shows that the performance part for Rel-19 ATG is starting. This paper presents our observations and views on RRM impacts of performance parts on ATG CA.

RRM performance requirement analysis

In this part, we provide our analysis of the performance requirements to support CA in case-by-case manner in the following part.

Test scope

In the last meeting, the companies provided their views on which test cases the Rel-19 ATG is needed for. The following is the summary captured from WF [1].

Issue 2-1-3: Test scope Agreement: - Introduced test cases: - Interruptions during measurements on deactivated NR SCC in FR1 - test together with Event triggered reporting on SCC with deactivated SCell test under non-DRX, with measurement cycle of no smaller than 640ms - If it is feasible to test both neighbour cell measurement skipping and not skipping in this test case, - not introduce the test case for “Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle”. - If not feasible, - introduce one more test case for “Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle”

During the last meeting, we discussed the test case design of SCell activation. The key issue is whether or not to define separate test cases for “neighbour cell measurement skipping” and “neighbour cell measurement non-skipping”. In our view, it is better to use separate test cases to test whether or not UE support the Rel-19 measurement enhancement that neighbour cell measurement skipping. And using different cycles, 160ms and 640ms, in the test design. For instance,

      - Event-triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle with the SCell measurement with neighbour cell measurement skipping.

      - Event-triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle

Therefore, for the test case of Interruptions during measurements on deactivated NR SCC in FR1, Rel-19 ATG defines the above test cases.

Proposal 1: for the test case of Interruptions during measurements on deactivated NR SCC in FR1, Rel-19 ATG defines the following test cases:

1. Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle with the SCell measurement with neighbour cell measurement skipping.

2. Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle

Conclusion

Proposal 1: for the test case of Interruptions during measurements on deactivated NR SCC in FR1, Rel-19 ATG defines the following test cases:

1. Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle with the SCell measurement with neighbour cell measurement skipping.

2. Event triggered reporting on SCC with deactivated SCell test under non-DRX for 640 ms SCell measurement cycle

References

[1] R4-2512122 WF on RRM requirements for NR_ATG_enh