3GPP TSG-RAN WG4 Meeting #116bis	R4-2513475

# Prague, Czech Republic, 13-17 October 2025

Agenda item:	7.4.3.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on (e)RedCap UE enhancement for Rel-20

Document for:	Discussion

- Introduction

The enhancement of the (e)RedCap UE in Release 20 is intended to achieve lower cost, reduced complexity and a smaller battery size for RedCap devices. Building on the objectives outlined in WID [1], the new RRM requirement supersedes the previous RRM specifications of RLM and BFD and supports channel bandwidths of less than 5 MHz. In this contribution, we provide our views on the enhancement of (e)RedCap.

- Discussion

Relaxed RLM/BFD RRM requirements

Criteria for relaxation requirements

In The WID [1] introduced that the RLM/BFD relaxation measurement defined in Rel-17 will apply to the 1Rx (e)RedCap and 2Rx (e)RedCap. Because the requirements for 1Rx and 2Rx are different, the RLM/BFD relaxation requirement discussion for the (e)RedCap of 1Rx must be treated separately from that of 2Rx.

Proposal 1: RAN4 discussing the RLM/BFD relaxation requirement for the (e)RedCap of 1Rx must be treated separately from 2Rx.

When we look at the requirement for the 2Rx (e)RedCap, the specification is applied without any relaxation, and the level of relaxation is considered reasonable. In our view, the RLM/BFD relaxation requirement for the 2Rx (e)RedCap is able to proceed with no conditions. For the 1Rx (e)RedCap, however, a modest relaxation is already built into the definition; the additional relaxation that will be introduced in Rel-20 would have to be discussed in the context of the applicable conditions.

Observation 1: The discussion on the RLM/BFD relaxation for Rel-20 (e)RedCap, with the condition for 1Rx (e)RedCap.

To extend the Rel-17 RLM/BFD relaxation measurement to (e)RedCap in Rel-20, the criteria should consider the following aspect for 1Rx (e)RedCap and 2Rx (e)RedCap:

- The length of DRX

- serving cell quality criteria of RLM/BFD relaxation

- mobility criterion

Proposal 2: RAN4 consider the criteria of RLM/BFD relaxation for 1Rx (e)RedCap and 2Rx (e)RedCap in the following aspects:

1.	The length of DRX

2.	Serving cell quality criteria

3.	Mobility criterion

Relaxation factors

In release 17, RAN4 discussed the relaxation factor for RLM/BFD defined the K value in 2 and 4 for FR1 and 2 for FR2 SSB. This can be the starting point of the Rel-20 (e)RedCap measurement relaxation. From our perspective, the 2Rx (e)RedCap shares the same requirement as the NR UE, but the 1RX (e)RedCap might be relaxed by double with certain conditions. Therefore, the starting point of the relaxation factor for different Rx RedCap devices can be:

  - Relaxation factor 2 and 4 for 2Rx (e)RedCap.

  - Relaxation factor 2 for 1Rx (e)RedCap.

  - The applicable conditions can be further discussed.

Proposal 3: The starting point of the relaxation factor for different Rx RedCap devices can be:

•	Relaxation factor 2 and 4 for 2Rx (e)RedCap.

•	Relaxation factor 2 for 1Rx (e)RedCap.

•	The applicable conditions can be further discussed.

Applicability rules

A key issue for this feature is how to enable and disable the relaxation of the requirement. To our understanding, the Rel-17 rule for the feature applicability rule that the network to enable and disable this feature is workable for the (e)RadCap to apply this feature well.

Proposal 4: Network to enable and disable the RLM/BFD relaxation for (e)RedCap.

Support of less than 5 MHz channel bandwidth

The less than 5MHz and feature apply for the 1Rx (e)RedCap are the key points to figure out the RRM impact. In the discussion of the less than 5 MHz is also need to separate the 1Rx (e)RedCap and 2Rx (e)RedCap. It is possible to reuse the Rel-18 less than 5 MHz requirement for 2Rx (e)RedCap, and apply for the 1Rx (e)RedCap with some limitations.

Observation 2: It is possible to reuse the Rel-18 less than 5 MHz requirement for 2Rx (e)RedCap, and apply it for the 1Rx (e)RedCap with some limitations.

SSB based RLM/BFD

Given that the bandwidth for the 1Rx (e)RedCap is longer than the 2Rx (e)RedCap. The current PDCCH BW for 1Rx (e)RedCap is 48 PWB, which is wider than the 2Rx (e)RedCap. This design is to ensure the coverage of 1Rx (e)RedCap and 2Rx (e)RedCap is the same. To apply the less than 5MHz feature, the bandwidth of the PDCCH will be 15PRBs and 12 PRBs for both 1Rx (e)RedCap and 2Rx (e)RedCap. This will cause the coverage of 1Rx (e)RedCap and 2Rx (e)RedCap to be different.

Observation 3: Applying the less than 5MHz SSB based RLM and BFD will cause the coverage of 1Rx (e)RedCap and 2Rx (e)RedCap to be different.

The TEvaluate_out_SSB,RedCap is defined in the requirement of SSB based RLM with different values for 1Rx (e)RedCap and 2Rx (e)RedCap. The details are captured from Chapter 8.1B of 38.133 V19.1.0[2]:

Table 8.1B.2.2-1: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 2Rx RedCap UE Configuration; TEvaluate_out_SSB,RedCap (ms); TEvaluate_in_SSB,RedCap (ms). no DRX; Max(200, Ceil(10 P) TSSB); Max(100, Ceil(5 P) TSSB). DRX cycle≤320 ms; Max(200, Ceil(15 P) Max(TDRX,TSSB)); Max(100, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle>320 ms; Ceil(10 P) TDRX; Ceil(5 P) TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length.. Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE Configuration; TEvaluate_out_SSB,RedCap (ms); TEvaluate_in_SSB,RedCap (ms). no DRX; Max(400, Ceil(20 P) TSSB); Max(100, Ceil(5 P) TSSB). DRX cycle≤320 ms; Max(400, Ceil(30 P) Max(TDRX,TSSB)); Max(100, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle>320 ms; Ceil(20 P) TDRX; Ceil(5 P) TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length..

For the 1Rx (e)RedCap device, the detection SNR is lower than the 2Rx (e)RedCap. The reason why the TEvaluate_out_SSB,RedCap for 1Rx RedCap is 20 is that working in a low SNR environment needs more samples to maintain the performance. In our view, if RAN4 introduces the less than 5 MHz feature to Rel-20 (e)RedCap while retaining the same PDCCH bandwidth, 12 PRBs and 15 PRBs, the TEvaluate_out_SSB,RedCap can be set to the same value for both 1Rx (e)RedCap and 2Rx (e)RedCap.

Proposal 5: If RAN4 introduces the less than 5 MHz feature to Rel 20 (e)RedCap while retaining the same PDCCH bandwidth, 12 PRBs and 15 PRBs, the TEvaluate_out_SSB,RedCap can be set to the same value for both 1Rx (e)RedCap and 2Rx (e)RedCap.

On the other hand, for the SSB bases RLM and BFD requirement, the Aggregation level in different bandwidths with different values, e.i, 4 and 8. For 2Rx (e)RedCap, this feature can be reused, and RAN4 is necessary to discuss the applicability of this to 1Rx (e)RedCap. Because the Aggregation level is related to coverage of RedCap, and it is unclear whether or not the 12 PRBs case with the aggregation level equal to 4 is enough for 1Rx (e)RedCap to maintain the coverage. Therefore, RAN4 needs to discuss and evaluate the case where the PDCCH bandwidth is 12 PRBs, with the aggregation level of 4, for a 1‑Rx (e)RedCap.

Proposal 6: It is proposed to discuss the feasibility of extending the number of control OFDM symbols from 2 to 3 for 1‑Rx (e)RedCap the BFD parameter with PDCCH bandwidth of 12 PRBs.

- Conclusions

Proposal 1: RAN4 discussing the RLM/BFD relaxation requirement for the (e)RedCap of 1Rx must be treated separately from 2Rx.

Observation 1: The discussion on the RLM/BFD relaxation for Rel-20 (e)RedCap, with the condition for 1Rx (e)RedCap.

Proposal 2: RAN4 consider the criteria of RLM/BFD relaxation for 1Rx (e)RedCap and 2Rx (e)RedCap in the following aspects:

1.	The length of DRX

2.	Serving cell quality criteria

3.	Mobility criterion

Proposal 3: The starting point of the relaxation factor for different Rx RedCap devices can be:

•	Relaxation factor 2 and 4 for 2Rx (e)RedCap.

•	Relaxation factor 2 for 1Rx (e)RedCap.

•	The applicable conditions can be further discussed.

Proposal 4: Network to enable and disable the RLM/BFD relaxation for (e)RedCap.

Observation 2: It is possible to reuse the Rel-18 less than 5 MHz requirement for 2Rx (e)RedCap, and apply it for the 1Rx (e)RedCap with some limitations.

Observation 3: Applying the less than 5MHz SSB based RLM and BFD will cause the coverage of 1Rx (e)RedCap and 2Rx (e)RedCap to be different.

Proposal 5: If RAN4 introduces the less than 5 MHz feature to Rel 20 (e)RedCap while retaining the same PDCCH bandwidth, 12 PRBs and 15 PRBs, the TEvaluate_out_SSB,RedCap can be set to the same value for both 1Rx (e)RedCap and 2Rx (e)RedCap.

Proposal 6: It is proposed to discuss the feasibility of extending the number of control OFDM symbols from 2 to 3 for 1‑Rx (e)RedCap the BFD parameter with PDCCH bandwidth of 12 PRBs.

4. References

[1] RP-252955 WID: NR Radio Resource Management (RRM) Phase 6

[2] 3GPP TS 38.133 V19.1.0