3GPP TSG-RAN WG4 Meeting #116bis	R4-2513496

Prague, Czech Republic, October 13 – 17, 2025

Agenda Item:	7.12.2

Source:	Huawei, HiSilicon

Title:	Discussion on general aspects and workplan for AI mobility

Document for:	Discussion and Decision

- Introduction

According to new WI on AI/ML for mobility in NR [1], the core part work of interoperability and RRM requirement in RAN4 includes:

For both RRM measurement prediction and measurement event prediction [RAN4]: - Specify RRM performance requirements and test cases for the corresponding use cases - Specify necessary performance requirements and test cases for LCM procedures, if any

In this contribution, we discuss general aspects and workplan for AI mobility.

- Discussion on work plan

- For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

In R19, RAN4 studied interoperability, testability, and RRM requirements for AI-enabled RRM measurement prediction and measurement event prediction in RAN4.

- RRM measurement prediction: The impact on L3 cell-level RSRP prediction (for scenarios/sub-use cases defined in relevant sections) has been studied. According to the new WID, L3 beam-level prediction and spatial domain prediction for UE-sided model are not supported. Absolute and relative accuracy of predicted L3-RSRP are potential test metrics, with definitions provided for intra-frequency and inter-frequency scenarios. Ground truth definitions for L3-RSRP differ between FR1 and FR2, and factors like UE speed, measurement errors (modeled via link-level simulation and truncated Gaussian distribution for BB/RF errors), and inter-frequency conditions (e.g., EPRE difference, cluster approaches) are considered for prediction accuracy.

- For measurement event prediction: Impacts of both direct and indirect event prediction have been studied; for indirect prediction, requirements like delay, accuracy of predicted event-triggered reports (and associated RSRP accuracy if applicable) are considered. According to the new WID, there is a check point in RAN#112 to decide whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions. For direct event prediction, study feasibility of core requirement specification in Q2 2026.

Considering that the model output of RRM measurement prediction and indirect measurement event prediction could be same, the core requirements can be reused as much as possible.

Proposal 1: Reuse core requirement of RRM measurement prediction for indirect measurement event prediction, unless specific issues are identified for indirect measurement event prediction.

- Discussion on general aspects

In RAN2, the AI model is trained by system level channel model and verified in terms of the accuracy of measurement prediction under the same channel model. However, in RAN4 legacy, only link level channel model is considered for defining test cases. There are two potential testing options for RAN4 in AI RRM measurement prediction.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

Proposal 2: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy.  Pros and Cons need to be analyzed.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

There are two options for defining the testing and training dataset. One is using the channel model and the sampling rules. Another is that the dataset is specified or partially specified in a sample-by-sample manner.

Proposal 3: RAN4 will study the following two potential options for defining the testing and training dataset.

  - Option 1: Use the channel model and the sampling rules.

  - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner.

- Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: Reuse core requirement of RRM measurement prediction for indirect measurement event prediction, unless specific issues are identified for indirect measurement event prediction.

Proposal 2: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy.  Pros and Cons need to be analyzed.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

Proposal 3: RAN4 will study the following two potential options for defining the testing and training dataset.

  - Option 1: Use the channel model and the sampling rules.

  - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner.

References

- RP-252899, “Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”, 3GPP RAN #109, 2025.