3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513498

Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Discussion on impacts for measurement event prediction

Source: 	Huawei, HiSilicon

Agenda item:	7.12.4

Document for:	Discussion

Introduction

The revised WID on AI/ML for mobility in NR agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI The objectives of this work item focused on AI/ML-assisted mobility in RRC_CONNECTED mode, wherein RRM measurement prediction and measurement event prediction could be applied. Only functionality-based LCM procedures are supported. In general, the RRM measurement configuration and reporting frameworks in RRC layer are taken as baseline. The objectives are as the following: - Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible. - For both RRM measurement prediction and measurement event prediction for UE sided model [RAN4]: - Specify core requirements for the use cases and scenarios supported by the previous objectives for RRM measurement prediction and indirect measurement event prediction - Note 6: RAN4 is expected to investigate whether the core requirement for RRM measurement prediction can be reused for indirect measurement event prediction - For direct event prediction, study feasibility of core requirement in Q2 2026. - Specify LCM-related core requirements to support the LCM functionality management , if any Note 7: Can check in RAN#112 whether direct event prediction can be supported depending on RAN2 and RAN4 conclusions

This contribution focuses on measurement event prediction for AI/ML mobility.

Discussion

In RANP#109, it is agreed RAN4 specify core requirements for the use cases and scenarios for indirect measurement event prediction. The direct event prediction will be discussed in Q2 2026. In this meeting we focus the discussion on indirect event prediction.

- On direct event prediction: - RAN2 to discuss KPI for performance monitoring in Q1 2026 - RAN4 to discuss requirements in Q2 2026 - Check point in RAN#112 for final decision on whether direct event prediction will be supported depending on RAN2 and RAN4 conclusions.

In indirect event prediction, measurement prediction AI/ML model is used to predict L3 RSRP, and afterwards measurement event is derived by some post processing.

Figure 4.3-1: Indirect measurement event prediction [TR38.744]

According to RAN2 discussion in R19 SI phase, the handover procedure based on event prediction for temporal domain prediction is optimized with three options. In TR38.744 [4], option1, 2 and option 3 are with regard to how to decide on the time point to transmit handover command. Option1/2/3 can reduce handover failure number.

Cited from TR38.744 Figure 5.5.1-1: Handover model option 1 Option 1 is illustrated in Figure 5.5.1-1. At current time i.e. t0 measurement event e.g. A3 event is predicted at some point of time in future. Network will not transmit handover command until a real measurement event is reported for the same neighbouring cell. In this way, the main benefit of this option is to save handover preparation time. Figure 5.5.1-2: Handover model option 2 Option 2 is illustrated in Figure 5.5.1-2. At current time i.e. t0 measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entry condition of the predicted measurement event is met based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier. If measurement event is predicted based on intra-frequency temporal domain case B, there is option 3 w.r.t. how to decide on the time point to transmit handover command: Figure 5.5.1-3: Handover model option 3 Option 3 is illustrated in Figure 5.5.1-3. Once a predicted measurement event e.g. A3 event is received network can transmit handover command immediately after handover preparation is completed. UE will report predicted measurement event at the time instance it is to be triggered.

From handover delay requirements perspective, in legacy the starting point of handover delay is from the instance UE received the handover command. In AI event prediction model, although the time point of HO preparation and the time instance of handover command indication are different from legacy, the UE subsequent behaviour (e.g. AGC, fine time tracking, RACH) after receiving HO command still follows legacy way in option 1, 2 or 3.

Proposal 1: If the starting point of legacy handover delay is reused (i.e, when UE receives HO CMD), the AI event prediction has no impact on the requirements of handover delay.

In RAN4#116 meeting, the indirect event prediction was discussed [2].

Issue 3-1: Performance metric for indirect measurement event prediction Agreement: - On top of the agreement made in RAN4 #115, RAN4 to discuss whether/how to define additional performance metrics for indirect measurement event prediction in WI phase. - Capture the candidate proposals in the WF for information purpose.

In RAN2 evaluation, performance metric F1 score is introduced [TR38.744]:

F1 score = 2*Precision*Recall/(Precision + Recall) Where: Precision = n3/(n1+n3) Recall =n3/(n2+n3)

For indirect event prediction, the counter n1, n2 and n3 in the formula are defined as following [TR38.744]:

- Counter n3(true event prediction): it increases by 1 when a ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa

- Counter n1(false event detection): it increases by 1 when no ground-truth event occurs around a predicted event with ETD, whose range is [0, maximum ETD]

- Counter n2(missed event detection): it increases by 1 when no event is predicted around a ground-truth event with ETD, whose range is [0, maximum ETD]

where ETD is the timing difference between the instance ground-truth event occurs and the instance predicted event occurs.

Figure 5.3.1-1: illustration of ETD [TR38.744]

From defining requirements perspective, there are some options.

Option 1: Define requirements for indirect event prediction

Option 1-1: define ETD of event prediction as a requirement, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD compared with ground truth event occurrence instance, under the condition that precision and recall or F1 score consistently is above a value.

Option 1-2: define precision and recall or F1 score of event prediction as a requirement, under the condition of certain fixed ETD. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently above a value, under the condition that measurement event is predicted within certain ETD compared with ground truth event.

Option 2: Not define specific requirement for indirect event prediction

In indirect event prediction, measurement prediction model is used to predict L3 RSRP, and afterwards measurement event is derived by some post processing without further involvement of an AI/ML model. Companies may have an impression that no indirect event prediction requirements are needed, as the measurement prediction accuracy is able to be satisfied. However in our understanding, although if the measurement prediction accuracy is satisfied during PW (prediction window) for measurement prediction, it can not represent the UE capability of event prediction, such as ETD and F1 score.

Moreover if the MR includes the intermediate predicted RSRP corresponding to the predicted event occurrence time, the intermediate RSRP shall satisfy the absolute and relative measurement prediction accuracy. This intermediate predicted RSRP is what we are discussing in measurement prediction objective.

Proposal 2: RAN4 to discuss the following options for indirect event prediction

- Option 1: Define requirements for indirect event prediction

- Option 1-1: define ETD of event prediction as a requirement, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD compared with ground truth event occurrence instance, under the condition that precision and recall or F1 score consistently is above a value.

- Option 1-2: define precision and recall or F1 score of event prediction as a requirement, under the condition of certain fixed ETD. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently above a value, under the condition that measurement event is predicted within certain ETD compared with ground truth event.

- Option 2: Not define specific requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied (not preferred)

Predicted event triggered reporting delay

In legacy, the measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface, which is 2 x TTIDCCH. The legacy measurement reporting delay is defined at the measurement reference point where the actual channel meets event.

In legacy, the event triggered measurement reporting delay is defined as cell identification duration (T identify), where UE really realizes that the channel changes and the event is fulfilled.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than T identify_intra_with_index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

In R19 indirect event predication, at current time (i.e. t0) measurement event is predicted at some point of time in future (i.e. t1), and UE will report the event prediction @t0 to network. The legacy measurement reporting delay can be applied for the predicated event reporting, however it is meaningless as the requirement is not able to represent UE event prediction capability/performance. In addition, the legacy event triggered measurement reporting is not applicable for event predication delay anymore, since the event prediction delay has nothing with Tidentify_intra.

Figure 1. Event prediction delay

Regarding indirect event prediction, we suggest RAN4 to discuss whether “event prediction delay” needs to be specified. The event prediction delay is the time is the time between the instance (at t0) where measurement event is predicted and reported, and the instance of predicted measurement event (at t1). The event prediction delay can represent the UE capability of how long UE can accurately predict event. It shall be noted that event prediction delay is different as ETD as shown in Figure 1. We are also thinking whether PW (prediction window) for measurement prediction can characterize the same concept of event prediction delay. RAN4 can carry out further discussion.

Proposal 3: RAN4 to study the requirement of event prediction delay which is the time between the current time (at t0) where event is predicted, and the future time (at t1) of the occurrence of predicted event.

Conclusion

The contribution provides analysis on RRM measurement event prediction in AI/ML mobility. The following proposals are proposed:

Proposal 1: If the starting point of legacy handover delay is reused (i.e, when UE receives HO CMD), the AI event prediction has no impact on the requirements of handover delay.

Proposal 2: RAN4 to discuss the following options for indirect event prediction

- Option 1: Define requirements for indirect event prediction

- Option 1-1: define ETD of event prediction as a requirement, under the conditions of certain fixed precision and recall or F1 score. More specifically, the measurement event shall be predicted within certain ETD compared with ground truth event occurrence instance, under the condition that precision and recall or F1 score consistently is above a value.

- Option 1-2: define precision and recall or F1 score of event prediction as a requirement, under the condition of certain fixed ETD. More specifically, the qualified UE shall satisfy precision and recall or F1 score consistently above a value, under the condition that measurement event is predicted within certain ETD compared with ground truth event.

- Option 2: Not define specific requirement for indirect event prediction, as long as the measurement prediction accuracy is satisfied (not preferred)

Proposal 3: RAN4 to study the requirement of event prediction delay which is the time between the instance (at t0) where measurement event is predicted and reported, and the instance of predicted measurement event (at t1).

Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO

- R4-2512127, WF on AI/ML Mobility Part 1, Nokia