3GPP TSG-RAN WG4 Meeting #116bis	R4-2513501

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	On maintenance issues for RRM enh5

Agenda Item:	4.15.1

Document for:	Discussion

Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

The RRM core requirement part has officially closed at RAN4 116 meeting. In this contribution, we provided our views on remaining issues at maintenance phase.

Discussion

In this contribution, the following open issues are discussed based on WF at RAN4 116 meeting.

- Issue 1-1-4: Requirements when UE reports different BSF values for different bands

Way forward:

Further discuss this issue in the maintenance stage:

  - Option 1:

    - It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured.

For simplicity, the solution in option 1 can be used.

Proposal 1: When multiple bands are involved and UE reports different BSF values for different bands, the requirement can be based on the largest BSF value reported over all the bands where an MO is configured.

- Issue 1-1-5: Revision of the agreement on applicability requirement

Agreement in Tuesday ad-hoc:

- The FBS enhancement applies for the case when the target and serving carrier frequency are same.

- FFS in maintenance which CC to apply the FBS when the target and serving carrier frequency are on different.

Agreement in Thursday online:

When the target and serving carrier frequency are different,

- FBS requirement does not apply to inter-frequency measurement without measurement gap.

- FFS inter-frequency measurement with per-FR measurement gap.

- Note: inter-RAT is not in the scope.

Proposal 2: For which CC to apply the FBS when the target and serving carrier frequency are on different, introducing UE capability which indicate UE can support FBS on both carrier frequency could be considered.

- Issue 1-1-2: Re-evaluation mechanism

Agreement:

FFS following options in the maintenance stage:

- Option 1: Introduce a predefined time duration T2, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. Value of T2 can be:

    - Alt 1: Use the values for T1 as a starting point.

    - Alt 2: 5 s, which is predefined in RAN4 spec.

- Option 2: It is up to UE implementation when to re-enter the FBS after quitting the last FBS.

- Option 3: It is up to UE implementation when to re-evaluation of the activation condition for FBS after quitting the last FBS.

Agreement in Friday online:

- No consensus to introduce a predefined time duration T2 in the WI core part in RAN4.

Since there is no consensus to have any predefined value for T2, it is suggested no need further discussion on T2 in the maintenance phase.

Proposal 3: No need further discussion on T2 at the maintenance phase.

Conclusion

In this contribution, we provide our considerations on the remaining issues for RRM enh5 and have the following observations and proposals.

Proposal 1: When multiple bands are involved and UE reports different BSF values for different bands, the requirement can be based on the largest BSF value reported over all the bands where an MO is configured.

Proposal 2: For which CC to apply the FBS when the target and serving carrier frequency are on different, introducing UE capability which indicate UE can support FBS on both carrier frequency could be considered.

Proposal 3: No need further discussion on T2 at the maintenance phase.

- References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, RANP 105

- R4-2512124, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN4 116, Apple

- R4-2512125, WF on RRM requirements for NR_RRM_Ph5_Part2, RAN4 116, CATT