# 3GPP TSG-RAN WG4 Meeting #116bis														R4-2513562

# Prague, Czech Republic, Oct. 13-17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.191 | CR | - | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on requirement applicability for bandwidth of AIoT device |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025.09.30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The requirement applicablity for bandwidth is not clearly stated in the spec, e.g., backscatter loss, reference sensitivity |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Align the terminology of R2D channel bandwdith - Clarify the backscatter loss is applied to all devcie D2R channel bandwidth - Clarify the Reference sensitivity and EIS partial sphere coverage are applied to all R2D channel bandwdith. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement applicability is ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.2, 5.3.1.3, 5.3.1.4, 5.3.1.5, 6.2.1,7.2.2, 7.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

### 5.3.1	R2D Channel bandwidth

#### 5.3.1.1	General

The R2D channel bandwidth supports a single reader RF carrier in R2D link at the reader.

The relationship between the R2D channel bandwidth, the guardband and the transmission bandwidth is shown in figure 5.3.1.1-1.

Figure 5.3.1.1-1: Definition of channel bandwidth and transmission bandwidth configuration for one reader channel

#### 5.3.1.2	R2D Transmission bandwidth

The transmission bandwidth NRB for each R2D channel bandwidth and subcarrier spacing is specified in table 5.3.1.2.-1.

Table 5.3.1.2-1: R2D Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 200 kHz | 400 kHz | 600 kHz | 800 kHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 15 | 1 | 2 | 3 | 4 |

NOTE:	All BS Tx and device Rx requirements are defined based on transmission bandwidth configuration specified in table 5.3.1.2-1.

#### 5.3.1.3	Minimum guardband and R2D transmission bandwidth configuration

The minimum guardband for each R2D channel bandwidth and SCS is specified in table 5.3.1.3-1.

Table 5.3.1.3-1: Minimum guardband (kHz) (FR1)

| R2D CBW | 200kHz | 400kHz | 600kHz | 800kHz |
| --- | --- | --- | --- | --- |
| Minimum guardband(kHz) | 2.5 | 12.5 | 22.5 | 32.5 |

The number of RBs configured in any R2D channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.1.3-1: reader PRB utilization

#### 5.3.1.4	RB alignment

For each R2D channel bandwidth, BS transmission bandwidth configuration must fulfil the minimum guardband requirement specified in clause 5.3.1.3.

#### 5.3.1.5	R2D channel bandwidth per operating band

The requirements in this specification apply to the combination of R2D channel bandwidths, SCS and operating bands shown in table 5.3.1.5-1 for FR1. The transmission bandwidth configuration in table 5.3.1.2-1 shall be supported for each of the R2D channel bandwidths within the BS capability. The R2D channel bandwidths are specified for the Tx path.

Table 5.3.1.5-1: R2D channel bandwidths and SCS per operating band

| NR Band | SCS (kHz) | Reader channel bandwidth (kHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 200 | 400 | 600 | 800 |
| n8 | 15 | 200 | 400 | 600 | 800 |

<< Next change >>

## 6.2	Backscatter power

### 6.2.1	Device backscatter power

The backscatter power is defined as mean filtered power measured over the duration of the D2R signal, excluding the power at the carrier frequency. The backscatter loss is defined as the difference between the input CW power at the device antenna in dB scale to the backscatter power at the device antenna in dB scale.

The backscatter power is measured with the test metric of EIRP as specified in clause 8.2.1. The minimum requirement on backscatter loss in Table 6.2.1.1-1 shall apply to all device D2R channel bandwidth defined in Table 5.3.2.3-1 and shall be met with the test parameters defined in Annex A, at the peak antenna gain direction declared by the device vendor.

Table 6.2.1.1-1: Maximum allowable backscatter loss

| Operating band | Maximum backscatter loss (dB) |  | CW power at the device antenna (dBm) |
| --- | --- | --- | --- |
| n8 | OOK | 10 | -27 |
|  | BPSK | 6 |  |
|  | OOK | 15 | -10 |
|  | BPSK | 11 |  |

<< Next change >>

## 7.2	Reference sensitivity power level

### 7.2.1	General

The reference sensitivity power level REFSENS is defined as the EIS level at the centre of the quiet zone in the RX gain peak direction, at which the successful detection rate shall meet or exceed the requirements for the specified reference measurement channel.

### 7.2.2	Reference sensitivity power level

The successful detection rate shall be ≥ 90% of the reference measurement channels as specified in Annexes B.1, B.2 and B.3 with peak reference sensitivity specified in Table 7.2.2-1. The requirement is applied to all R2D channel bandwidth defined in Table 5.3.1.5-1 and verified with the test metric of EIS at the peak antenna gain direction as specified in clause 8.2.1.

Table 7.2.2-1: Reference sensitivity

| Operating band | Sensitivity Level | REFSENS (dBm) |
| --- | --- | --- |
| According to subclause 5.2 | L1 | -34 |

NOTE:	The peak reference sensitivity is measured at the low, middle and high frequency of the supported

band(s), and the average value is verified against the requirement.

### 7.2.3	EIS partial sphere coverage

The reference measurement channels and detection criterion shall be as specified in clause 7.2.2

The maximum EIS measured over the partial sphere around the device is defined as the partial sphere coverage requirement and is found in Table 7.2.3-1 below. The requirement is applied to all R2D channel bandwidth defined in Table 5.3.1.5-1 and verified with the test metric of EIS as specified in clause 8.2.1.

Table 7.2.3-1: EIS partial sphere coverage

| Operating band | EIS (dBm) | Angular width |
| --- | --- | --- |
| According to subclause 5.2 | -28.5 | ±45 degrees |

NOTE:	The EIS partial sphere coverage requirement is verified at the middle frequency of the supported

band(s).

<< End of change >>