3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513563

Prague, CZ, October 13th – 17th, 2025

Source: 	vivo

Title: 	Discussion on UE RF requirements for Rel-20 LBCA

Agenda Item:	7.8.3

Document for:	Approval

Introduction

In RAN#109, a Rel-20 WI [1] for LB CA in RAN4 package has been approved. This is enhancement based on Rel-19 WI NR_LBCA_Sw.

In this discussion paper, a summary of Rel-19 work was done, and some possible RF impact analysis for Rel-20, and the difference between Rel-19 WI were made.

Background

The current WI is based on Rel-19 WI [2].

Rel-19:

In Rel-19, a low band CA via switching WI [2] is done. A TR [3] was drafted for that WI, including all the studies and analysis, and the feature CR was agreed in RAN4 in [4].

In that Rel-19 WI, the switching is between the following two cases:

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

In another word, there is no simultaneous DL transmission in the FDD carrier and SDL carrier.

During the study, one of the main discussions point in the WI is the switching period location and relationship with the switching gap which is defined in RAN1. The results were captured in the TR report and also copied below for reference:

Based on these analysis, an time mask for switching was defined and captured into 36.101 as following:

In this requirement, only Tx part is considered which is aligned with legacy time mask requirements.

The switching gap is configured by RRC in terms of number of symbols blanked by the network, while the actual location of the switch is specified by time masks similarly to UL Tx switching.

For the switching period, a set of numbers {35us, 70us, 140us} were agreed in RAN4, and were defined as UE capability in RAN2.

Rel-20:

In Rel-20, a new switching case is introduced. As in [1], requirements would be studied for switching between {case 1, case 3} which is as following:

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2.

  - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1.

It can be seen that the case 1 here is the same to the case 1 in Rel-19, and Case 3 is a new case that there would be DL carrier aggregation (simultaneous DL reception in FDD carrier 1 and SDL carrier 2).

In addition, there is the following restrictions in the Rel-20 WID [1]:

-	RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary

-	Specify the switching delay and time mask for carrier switching [RAN4]

This means that RAN1 would use basically similar way as Rel-19.

In this paper, the RF requirements for Rel-20 cases were studied based on Rel-19 requirements.

Discussion

Tx Part

From Tx point of view, the switching between case1 and case 2, is actually quite similar to case 1 and case 3. The only Tx RF chain belongs to the FDD part, and SDL is DL only. The “tx switching” is actually can not happen, and only ON and OFF for FDD carrier can be defined and specified.

Observation 1: The switching of case 1 <-> case 2 and case 1 <-> case 3 is quite similar for the Tx part.

The switching delay and time mask are surely need to be defined as in the WID objective.

For the switching delay, it seems that it is highly likely that those numbers {35us, 70us, 140us} can be reused, since similar implementation is expected. The DL CA newly introduced is not likely have significant impact on those requirements.

Proposal 1: Rel-19 switching delay requirements {35us, 70us, 140us} can be used as a starting point for Rel-20.

In addition, there is one guideline in the WID that:

Note 2: Strive to minimize the RAN1 impact

If certain RAN1 specification can be reused, e.g. that switching gap located in the switching from carrier, then the ON/OFF mask requirements would be pretty much the same as Rel-19. It is highly likely.

Proposal 2: Reusing Rel-19 ON/OFF mask for case 1 <-> case 2 as starting point for Rel-20 case 1 <-> case 3, and confirmation may need RAN1 involvement.

Rx Part

Rx part is the main difference between Rel-20 and Rel-19, in that Rel-19 do not support DL CA while Rel-20 do in case 3.

This is actually quite similar to what Rel-16 Tx switching case, in that DL interruption need to be considered. However, this is in RRM scope and would not be discussed in detail here.

Observation 2: DL interruption need to be considered for case 3, which belongs to RRM scope.

Proposal 3: Study interruption requirements in RRM scope.

Others

Furthermore, currently many CA only requirements, such as insertion loss has been clarified that not applicable to Rel-19 LB CA via switching combination, since it is not really abide by CA architecture. Now with the introduction of case 3 in Rel-20, those clarifications need to be re-checked.

Proposal 4: Re-check the requirements related to CA applicability for LB CA.

Conclusion

In this paper, an initial study was conducted for the RF requirements impact for Rel-20 enhancement of low NR band carrier aggregation via switching, the following observations and proposals are provided:

Observation 1: The switching of case 1 <-> case 2 and case 1 <-> case 3 is quite similar for the Tx part.

Proposal 1: Rel-19 switching delay requirements {35us, 70us, 140us} can be used as a starting point for Rel-20.

Proposal 2: Reusing Rel-19 ON/OFF mask for case 1 <-> case 2 as starting point for Rel-20 case 1 <-> case 3, and confirmation may need RAN1 involvement.

Observation 2: DL interruption need to be considered for case 3, which belongs to RRM scope.

Proposal 3: Study interruption requirements in RRM scope.

Proposal 4: Re-check the requirements related to CA applicability for LB CA.

- References

[1] RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, RAN#109

[2] RP-243317, New WID on low band carrier aggregation via switching, RAN#106

[3] TR 38.768, NR; Low NR Band Carrier Aggregation via Switching

[4] R4-2511894, (NR_LBCA_Sw-Core) Introducing low NR band aggregation via switching to TS38.101-1, Apple, RAN4#116