3GPP TSG-RAN WG4 Meeting # 116bis	R4-2513564

Prague, CZ, October 13th – 17th, 2025

Source: 	vivo, MediaTek

Title: 	Discussion on PC1.5 HPUE Requirements for Rel-20 NTN

Agenda Item:	7.6.3

Document for:	Approval

Introduction

In RAN#109, a Rel-20 WI [1] for NTN in RAN4 package has been approved. In this WI, the first part is the HPUE which is as following:

HPUE for NTN Core Part

- Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types

  - Example bands are Band 256 and 255

  - Necessary Tx requirements, at least including

    - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

  - Necessary Rx requirements, at least including

    - Specify the RSD requirements on the example bands, if necessary

  - Specify the necessary signaling to support the above objectives

- The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

In this discussion paper, an overall discussion was provided on the RF impact on these objectives. Unless other wise stated, the analysis in this paper is focused on NB-Iot based Iot-NTN, since the NR NTN would be discussed in other WI in TN.

Since there are actually a lot can be re-used from Rel-19 work, including reference architectures, evaluation results, and even requirements structures, a preliminary draft CR is also provided in this meeting [2].

Background

Iot-NTN

In Rel-19, PC2 and PC1 HPUE were introduced for NB-Iot based Iot-NTN. For eMTC-based Iot-NTN, only PC2 were introduced in the final stage since lack of contribution and industry interest for eMTC based. The example bands are 255 and 256. Final Big CR was agreed in [3].

The co-existence study has been done, and results show no problem under current ACLR requirements.

For NB-Iot based Iot-NTN, an extensive feasibility study have been done to verify the feasibility of whether both handheld and non-handheld UE type can be applied Power Class 1. The conclusion was made in the WF [4] and [5] on the applicability of handheld and non-handheld for PC1 and also no striction on resource allocation.

For NB-Iot based PC2, a power boosting scheme was also defined. Originally this was targeted to reflect the possible insertion loss improvement of half-duplex NB-Iot in a full duplex UE, but gradually it has become an optional feature with capability that mainly reflect the power boosting potential of using NR PC3 PA implementation which results were presented in [6].

In Rel-19, no PC1.5 was considered when the WI is setting up, since this has no precedence in NB-Iot and eMTC, and also because PC1.5 in NR has an assumption of 2Tx only which is also not applicable to NB-Iot and eMTC.

Still, there was some support for PC1.5, since the gap between PC2 and PC1 is fairly large, i.e. 5dB, and there could be some room in the market and also implementation for an additional power between them. This turns out to be the main scope in Rel-20 for NTN HPUE.

For eMTC based, no more power class was proposed in Rel-20 since not too much industry interest.

NR-NTN

For NR-NTN, PC2 and PC1 requirements were defined for NR in Rel-19. Although there was some discussion in Rel-19 for PC1.5 and also some agreements, such as 2Tx MPR should reuse TN requirements, PC1.5 was not completed and removed from Rel-19 as agreed in [7].

However, in Rel-20, the PC1.5 NR NTN was not explicitly listed in the scope in NTN WI [1]. Instead, there was a clarification as following:

“The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.”

In the meantime, there is another WI [8] for UE RF enhancements for NR FR1 was agreed in Rel-20, in which PC1.5 with 2Tx for FDD and PC1.5 with 1Tx for TDD would be specified.

The understanding is that there is no need to do specification work dedicated for NR NTN before the Rel-20 TN work progress stable enough, and duplicate work in TN and NTN should be avoided. The detailed scope and time plan should be clarified.

Architecture & MPR requirements (NB-Iot based)

General

In Rel-19, it was generally agreed that NB-Iot based Iot-NTN would use some existing RF chains and components as baseline. In the WF [9], the following reference components were agreed for PC1.

  - For NB-Iot based Iot-NTN, both GSM PA and NR 1Tx PC2 PA are taken into consideration in developing unified for one power class;

Throughout the measurement evaluations, such as in [10][11][12] and [13][14], GSM PA and NR PC2 PA are the baselines for evaluation for power class 1, and the final requirements have considered both implementations.

Observation 1: GSM PA and NR PC2 PA are the baselines implementation for PC1 NB-Iot based Iot-NTN.

However, one issue was raised on the PC2 baseline reference architecture. In [13] there was a proposal:

“PC2 NB-IoT-NTN MPR is based on a PC3 NR PA”

Although this has never become an official agreement, this has eased the difficulty of using too “powerful” baseline for PC2 resulting to possibly too stringent requirements, and has become the de facto reference component in later MPR requirements discussion. The results presented in [6] which is based on this reference also played an important role in the confirmation of MPR and power boosting for PC2.

Observation 2: An NR PC3 PA can be regarded as a baseline implementation for PC2 NB-Iot based Iot-NTN although no previous agreement.

Admittedly, these reference components are not explicitly specified and can only be seen in the evaluation process. However, these reference baseline implementations, PC3/2 NR PA and GSM PA, still the baseline implementations for PC1.5. It is very natural to try to use the same references. Thus the evaluation previous done can still be reused.

Observation 3: The baseline reference components evaluation results in Rel-19 for PC1/2, can still be applied in Rel-20 for PC1.5 since these references still hold.

To be specifically, the measurements and simulation previous done are still there, we only apply the different reference components to a different power class (PC1.5) which is between PC1 and PC2. In another word, there will be no extra information if we duplicate the measurements in Rel-19, since the potential of current reference components have been explored thoroughly.

Proposal 1: Reuse the study for the reference components in Rel-19 for PC1/2, and no new simulation and/or measurements needed.

Based on this spirit, some analyses were provided for two different options in the following sections to obtain a set of requirements for PC1.5 in Rel-20:

- Option 1: Rel-19 PC1 requirements as baseline. (i.e. NR PC2 PA or GSM PA as reference components)

- Option 2: Rel-19 PC2 requirements as baseline. (i.e. Basically NR PC3 PA as reference components)

The following proposal is provided:

Proposal 2: Using either PC1 or PC2 requirements as baseline to derive PC1.5 MPR requirements.

Option 1: PC1 baseline

Using PC1 requirements as baseline for PC1.5, as discussed early, basically means NR PC2 PA or GSM PA as reference components. Those evaluations and simulations already reflected into the requirements and we can directly using requirements as baseline.

It should be noted that compared to the case that declared to PC1, the implementation difficulty for the single tone case were generally relaxed a bit, leaving more margin for UE implementation. This may be one of the key target of introducing PC1.5 with PC1 already specified.

The following PC1 MPR requirements have been defined in Rel-19:

Table 6.2B.2-4: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ 2 dB | 0 dB | ≤ 2 dB | ≤ 3.5 dB | 0 dB | ≤ 3.5 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table 6.2B.2-5: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 2 dB | ≤ 1 dB |  | ≤ 2 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 2.5dB |  | ≤ 2.5dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 4 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Based on those requirements, the following tentative MPR requirements for PC1.5 are proposed. The requirements are based on the principle of 2dB less compared to PC1 which is the power difference between PC1 and PC1.5 (29dBm):

Table X1: [Tentative] Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ [0] dB | 0 dB | ≤ [0] dB | ≤ [1.5] dB | 0 dB | ≤ [1.5] dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table X2: [Tentative] Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [0] dB | ≤ [0] dB |  | ≤ [0] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [0.5]dB |  | ≤ [0.5] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [2] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Proposal 3: An MPR = maximum {0, PC1 MPR - 2dB} can be considered for PC1.5 as starting point if using PC1 requirements and reference components as baseline.

In addition, although it seems there would be some margin for boosting in single tone case, these boosting would make this new PC1.5 implementation no difference with PC1, and can not achieve the effect of some relaxation, so this boosting seems not needed.

Proposal 4: No power boosting need to be defined for PC1.5, even if PC1 requirements is used as baseline .

Option 2: PC2 baseline

If using PC2 requirements as baseline for PC1.5, this basically means using NR PC3 PA as a baseline. Similar to previous case, the PC2 requirements are listed as following:

Table 6.2B.2-2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0 | 1-10 | 11 | 0-3 | 4-43 | 44-47 |
| MPR | ≤ 0.5 dB | 0 dBNote 2 | ≤ 0.5 dB | ≤ 0.5 dB | 0 dBNote 2 | ≤ 0.5 dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. Note 2: For tone positions 3 to 8 for 15kHz SCS and 8 to 39 for 3.75 kHz, 1dB MPR applies when UE indicates support for UE capability [powerBoosting-NBIotNTN-r19] and IE [powerBoostNBIotNTN-r19] is set to 1. |  |  |  |  |  |  |

Table 6.2B.2-3: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 2, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Similarly to PC1 case, the MPR for PC1.5 can be obtained by adding the PC2 MPR by 3dB for PC1.5 to account for a  similar implementation. It should be noted that the power boosting may be considered for single tone case, since this should actually be a rather typical case for NR PC3 PA. We are assuming this boosting capability should be there, or this UE is too far away from PC1.5 and should declare as a PC2 UE instead.

Table X3: [Tentative] Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | [0-2] | [3-8] | [9-11] | [0-7] | [8-39] | [40-47] |
| MPR | ≤ [3.5] dB | [1~3] dB | ≤ [3.5] dB | ≤ [3.5] dB | [1~3] dB | ≤ [3.5] dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table X4: [Tentative] Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [3.5] dB | [3] dB |  | ≤ [3.5] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [4] dB |  | ≤ [4] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [5] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Proposal 5: An MPR = maximum {PC2 MPR + 3dB} can be considered for PC1.5 as starting point if using PC2 requirements and reference components as baseline and the PC2 power boosting may be considered.

Between those two options, currently we have a preference on the option 1 which using PC1 as baseline, since the PC2 baseline requirements seems a bit too far away from PC1.5 and even center single tone case can not achieve the nominal power class of 29dBm. However, we still would like to discuss this issue.

Proposal 6: Prefer to use PC1 requirements (option 1) as starting point for PC1.5.

Other RF Requirements (NB-Iot based)

General

In this WI, there is no feasibility related objective, and the requirements developed for PC1.5 NB-Iot based Iot-NTN would be both for handheld and non-handheld. The is a natural extension from Rel-19 study that the requirements for both PC1 would be applied to both handheld and non-handheld and references is in the background part. PC1.5 is somehow downgrade from PC1 and should be easier to implement. In all, with PC1 and PC2 requirements can be applied both handheld and non-handheld, there is no meaning for PC1.5 do not follow this.

Observation 4: For PC1 and PC2, single set of requirements was applied to both handheld and non-handheld UE.

The following table can be used as a reference for information.

Table: Requirements’ consideration and feasibility for NB-Iot based Iot-NTN (Information)

|  | Power Class | Antenna Numbers | Handheld and/or non-handheld | Defined Release |
| --- | --- | --- | --- | --- |
| NB-Iot based | PC1 | 1Tx | Both | Rel-19 |
|  | PC1.5 | 1Tx | Both | Rel-20 |
|  | PC2 | 1Tx | Both | Rel-19 |

Based on the analysis that similar reference components were used, it is also suggested to use single set of requirements for both handheld and non-handheld for PC1.5.

Proposal 7: Single set of PC1.5 requirements would be applied for both handheld and non-handheld UE.

The detailed requirements were discussed below in this clause.

Tx Requirements

MOP & Tolerance

Although the nominal MOP for NB-Iot based Iot-NTN for PC1.5 has never been officially confirmed, the already used value of 29dBm defined in NR TDD is the only and natural reference. Theoretically, different nominal maximum output power can be considered, since it was defined in spec 36.102, which is different from NR-NTN and NR TN. However, misalignment of nominal MOP of same power class between different RAT may create a lot of confusion and there is no clear benefit for this.

Proposal 8: Define the nominal MOP of PC1.5 for NB-Iot based Iot-NTN as 29dBm which is the same to NR TN case.

For the tolerance, it is suggested to use +2/-3 which is aligned with PC1.

Proposal 9: For MOP tolerance, +2/-3dB is proposed for PC1.5.

ACLR

In Rel-19 NTN HPUE, the NB-IoT based IoT-NTN PC2 and PC1 ACLR reused from NB-IoT based IoT-NTN PC3 UTRA_ACLR. The GSM_ACLR has been deleted since this was not the most stringent case. The co-existence study in [15] has include those study and final requirements. The ACLR needed for co-existence is considerably more relaxed compared the PC3 ACLR, thus the original ACLR was kept.

For PC1.5, the co-existence study is not needed, since PC1 and PC2 had already been evaluated. The ACLR can be the same to PC1/2 as a natural extension, and this is also aligned with the possible reference implementations, and also been sufficiently evaluated.

Proposal 10: For ACLR, the current UTRA_ACLR used for PC1/2/3 is reused for PC1.5.

SEM

The spectrum emission mask for NB-Iot based Iot-NTN is basically re-used from NB-Iot, and has been defined which is irrespective of the power classes. For power class 3/2/1, the same SEM was used.

It seems that there is no reason why SEM should be changed based on this condition, and it is proposed to reuse the requirements for PC1.5.

Proposal 11: For SEM, the current SEM used for PC1/2/3 is used for PC1.5.

A-MPR

The A-MPR for band 255 and 256 has been studied in Rel-19 for power class 2 and power class 1. The derivation is based on analysis and the baseline PC3 requirements.

Table 6.2B.3-2: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1 and PC2

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |  |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |  |
| NS_24 | 6.5B.4.4.3 | 256 | PC1 | PC2 |
|  |  |  | ≤ 10 | ≤ 5.5 |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |  |

Table 6.5B.3.3.1-1: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± 0.09–0.28 | 6 for PC1 1 for PC2 -2 for PC3 -5 for PC5 | 4 kHz |
| ± 0.28–0.85 | -4 for PC1 -9 for PC2 -12 for PC3 -15 for PC5 | 4 kHz |
| ± >0.85 | -13 for PC1, PC2, PC3 and PC5 | 4 kHz |

For band 255, NS_02N, A-MPR is not needed, since the FCC SEM is relative to in-band power, and requirements are scaled according to the power class. It has been analysed in [16] that the additional SEM for NS_02N is always more relaxed compared to baseline SEM as the following figure.

So for NS_02N, the A-MPR is still proposed to be not needed.

Proposal 12: For band 255, A-MPR is not needed for PC1.5, since it was not needed for PC1.

For band 256, the NS_24 requirements for PC1 and PC2 baseline is scaled from lower power class. The additional requirement for NS_24 and PC3/5 A-MPR are as following:

As pointed out in [16], although there was earlier proposals to also consider MPR, but the final A-MPR simply use the offset between the power classes as baseline to omit the different modulation and resource allocation to better accommodate the current single-value A-MPR for NB-Iot. In this spirit, the PC1 A-MPR is increased by 4.5dB which is almost the power offset between PC1 and PC2.

For PC1.5, it is proposed to use similar method and consider an A-MPR value between 5.5dB (PC2) and 10dB (PC1), and an A-MPR value of [8 ~ 8.5]dB is proposed. A brief list table of the NS_24 A-MPR is as following.

| Power Class | MPR range (dB) | NS_24 A-MPR (dB) | MPR+A-MPR |
| --- | --- | --- | --- |
| PC3 | 0 ~ 2 | 3.5 | 3.5 ~ 5.5 |
| PC5 | 0 ~ 2 | 0.5 | 0.5 ~ 2.5 |
| PC2 | 0 ~ 2 | 5.5 | 5.5 ~ 7.5 |
| PC1 | 0 ~ 4 | 10 | 10 ~ 14 |
| PC1.5 | Option 1: [0 ~ 2] Option 2: [1 ~ 5] | [8 ~ 8.5] | Option 1: [8 ~ 10.5] Option 2: [9 ~ 13.5] |

Proposal 13: For band 256, the A-MPR for NS_24 is proposed to be [8 ~ 8.5] dB for PC1.5.

Pcmax

The Pcmax part were re-write in Rel-19. Those equations and supporting part are generally agonistic to different power classes. The power boosting is only for PC2 and not for other power classes. There was also proposals that no other power classes can apply power boosting.

For other parts, after brief check, currently it seems that no revision is needed for Pcmax sections.

Proposal 14: No need to revise the Pcmax clause with the introduction of PC1.5.

SAR solution

In release 19, P-MPR has been introduced as a SAR solution for NB-Iot based Iot-NTN. A lot of other schemes were also discussed, but no conclusion can be made. Some duty cycle related discussion for NR-NTN would be continue in TN as a maintenance.

In the current study, The SAR solution is not explicitly listed in the scope. In addition, it is not promising for other schemes to converge, and the schedule is tight that the CA NTN is still waiting the complete of HPUE part, it is proposed not to discuss other SAR solutions.

Proposal 15: No need to discuss further SAR solution apart from existing P-MPR as used in PC1/2.

Rx Requirements

For NB-Iot based Iot-NTN, only half-duplex is possible and no reference sensitivity is needed.

In this sense, no work is forseen for Rx requirements for PC1.5.

Proposal 16: No Rx requirements impact for PC1.5.

NR-NTN work plan

In the WID objective, there is following objective:

- The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

As described in the background part, the actual work is now in the TN part and no discussion in this agenda in current stage.

One agreement in R19 WF in [7] is：

Agreement:

Remove PC1.5 from the WI. The WID can be updated in the coming plenary.

The conclusions on PC1.5 which have been agreed under this WI can be directly used in the future release, e.g. R20, if the related work is approved.

An interesting part is that there is some previous PC1.5 agreement in Rel-19 NTN HPUE, such as re-using PC1.5 2Tx TDD MPR. So theoretically there might be some potential misalignments in future TN discussions with legacy requirements.

Observation 5: The NR NTN is expected to re-use NR TN requirements for PC1.5 as in the WID, but further check of the requirements applicability may be needed.

Anyway, we think that the following way is possible:

Phase 1: Suspend dedicated NR NTN PC1.5 discussion, and concentrated on TN PC1.5 discussion.

Phase 2: When the NR NTN PC1.5 is almost ready, some check is done for the applicability to NR NTN, and fit the requirements in NR-NTN spec.

Proposal 17: According to WID objective, the following brief work plan is proposed for NR NTN PC1.5:

- Phase 1: Suspend dedicated NR NTN PC1.5 discussion, and concentrated on TN PC1.5 discussion.

- Phase 2: When the NR NTN PC1.5 is almost ready, some check is done for the applicability to NR NTN, and fit the requirements in NR-NTN spec.

Conclusion

Architecture & MPR requirements (NB-Iot based)

General:

Observation 1: GSM PA and NR PC2 PA are the baselines implementation for PC1 NB-Iot based Iot-NTN.

Observation 2: An NR PC3 PA can be regarded as a baseline implementation for PC2 NB-Iot based Iot-NTN although no previous agreement.

Observation 3: The baseline reference components evaluation results in Rel-19 for PC1/2, can still be applied in Rel-20 for PC1.5 since these references still hold.

Proposal 1: Reuse the study for the reference components in Rel-19 for PC1/2, and no new simulation and/or measurements needed.

Proposal 2: Using either PC1 or PC2 requirements as baseline to derive PC1.5 MPR requirements.

Option 1: PC1 baseline

Proposal 3: An MPR = maximum {0, PC1 MPR - 2dB} can be considered for PC1.5 as starting point if using PC1 requirements and reference components as baseline.

Table X1: [Tentative] Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ [0] dB | 0 dB | ≤ [0] dB | ≤ [1.5] dB | 0 dB | ≤ [1.5] dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table X2: [Tentative] Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [0] dB | ≤ [0] dB |  | ≤ [0] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [0.5]dB |  | ≤ [0.5] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [2] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Proposal 4: No power boosting need to be defined for PC1.5, even if PC1 requirements is used as baseline.

Option 2: PC2 baseline:

Proposal 5: An MPR = maximum {PC2 MPR + 3dB} can be considered for PC1.5 as starting point if using PC2 requirements and reference components as baseline and the PC2 power boosting may be considered.

Table X3: [Tentative] Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | [0-2] | [3-8] | [9-11] | [0-7] | [8-39] | [40-47] |
| MPR | ≤ [3.5] dB | [1~3] dB | ≤ [3.5] dB | ≤ [3.5] dB | [1~3] dB | ≤ [3.5] dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table X4: [Tentative] Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [3.5] dB | [3] dB |  | ≤ [3.5] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [4] dB |  | ≤ [4] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [5] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

Proposal 6: Prefer to use PC1 requirements (option 1) as starting point for PC1.5.

Other RF Requirements (NB-Iot based)

General:

Observation 4: PC1.5 requirements would be applied for both handheld and non-handheld UE.

Proposal 7: Single set of PC1.5 requirements would be applied for both handheld and non-handheld UE.

MOP & Tolerance

Proposal 8: Define the nominal MOP of PC1.5 for NB-Iot based Iot-NTN as 29dBm which is the same to NR TN case.

Proposal 9: For MOP tolerance, +2/-3dB is proposed for PC1.5.

ACLR

Proposal 10: For ACLR, the current UTRA_ACLR used for PC1/2/3 is reused for PC1.5.

SEM

Proposal 11: For SEM, the current SEM used for PC1/2/3 is used for PC1.5.

A-MPR

Proposal 12: For band 255, A-MPR is not needed for PC1.5, since it was not needed for PC1.

Proposal 13: For band 256, the A-MPR for NS_24 is proposed to be [8 ~ 8.5] dB for PC1.5.

Pcmax

Proposal 14: No need to revise the Pcmax clause with the introduction of PC1.5.

SAR solution

Proposal 15: No need to discuss further SAR solution apart from existing P-MPR as used in PC1/2.

Proposal 16: No Rx requirements impact for PC1.5.

NR-NTN work plan

Observation 5: The NR NTN is expected to re-use NR TN requirements for PC1.5 as in the WID, but further check of the requirements applicability may be needed.

Proposal 17: According to WID objective, the following brief work plan is proposed for NR NTN PC1.5:

- Phase 1: Suspend dedicated NR NTN PC1.5 discussion, and concentrated on TN PC1.5 discussion.

- Phase 2: When the NR NTN PC1.5 is almost ready, some check is done for the applicability to NR NTN, and fit the requirements in NR-NTN spec.

In addition, a draft CR for 36.102 is presented in [2] to provide a clear view how to introduce PC1.5 requirements for NB-Iot based Iot-NTN.

- References

[1] RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, RAN#109

[2] R4-2513565, draft CR for Introduction of PC1.5 for NB-Iot based Iot-NTN, vivo, RAN4#116bis

[3] R4-2510268, Big CR for 36.102 for Iot-NTN HPUE, vivo, RAN4#116bis

[4] R4-2505114, WF on HPUE RF requirements for IoT-NTN, vivo, RAN4#114bis

[5] R4-2508122, WF on IoT-NTN HPUE, vivo, RAN4#115

[6] R4-2509230, MPR input for PC2 NB-IoT-NTN, Skyworks, RAN4#116

[7] R4-2511793, WF on NR_IoT_NTN_HPUE part 1, Moderator (Samsung), RAN4#116

[8] RP-252952, UE RF enhancements for NR FR1, Phase 5, Moderator (vivo), RAN4#116

[9] R4-2503006, WF on HPUE UE requirement for IoT-NTN, vivo, RAN4#114

[10] R4-2500764, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, SmarterMicro, RAN4#114

[11] R4-2503842, Iot NTN HPUE Tx Requirements and feasibility analysis, vivo, SmarterMicro, RAN4#114bis

[12] R4-2506858, Iot NTN HPUE Tx Requirements and further evaluation, vivo, SmarterMicro, RAN4#115

[13] R4-2505834, NR and GSM PAs measurements for NB-IoT-NTN HPUE, Skyworks, RAN4#115

[14] R4-2509243, MPR input for PC1 NB-IoT-NTN, Skyworks, RAN4#116

[15] TR 38.863, Non-terrestrial networks (NTN) related RF and co-existence aspects

[16] R4-2510265, Iot NTN HPUE Tx Requirements, vivo, RAN4#116