# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513567

# Prague, CZ, 13th – 17th Octobor, 2025

Source: 	vivo

Title: 	Initial views on HPUE for NR single carrier operation

Agenda Item:	7.1.4

Document for:	Approval

Introduction

In RAN#109, a new WID for UE RF enhancements for NR FR1 for Phase 5 was approved [1]. One objective is to introduce HPUE for single carrier operation, i.e. single carrier TDD to support 4Tx for CPE/FWA UE, PC1.5 with 2Tx for FDD bands for handheld and FWA UE. In this contribution, we provide our initial views on these two HPUE topics.

Discussion

Single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations

The objective for single carrier TDD to support 4Tx for CPE/FWA is captured as follows:

- Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class

The 3GPP standardization effort is considering defining a maximum transmitter output power level of 32 dBm. Two primary technical approaches have been identified: one way is to reuse PC1 and another is to define a new power class.

From our perspective, we prefer to reuse PC1 instead of introducing a new power class in Rel-20. Usually, PC1 is often associated with a nominal max of 31 dBm with +2/-3 dB tolerance. Actually, it is 33 dBm with +2dB tolerance, which can support up to 32 dBm maximum output power. However, strictly speaking, power tolerance cannot always guarantee the maximum output power 32dBm, e.g. when the tolerance is -3dB. It is OK for us just reusing current PC1 definition.

If we define the maximum output power up to 32dB without considering tolerance, the general power booting scheme with 1dB can be considered with PC1 definition. Power boosting scheme was widely specified in current TN and NTN specifications. It allows UE to transmit at a slightly higher output power than its baseline power class (e.g., +1dB above PC1’s nominal maximum) under specific conditions.

From our perspective, reusing PC1 has the minimal standard impact compared to the way of introducing new power class. Changes would focus primarily on specifying the boost conditions and tolerances.

Proposal 1: It is suggested to reuse PC1 with or without general power boosting scheme to achieve up to 32 dBm in Rel-20.

PC1 was introduced for band n14 for public safety service in Rel-17 [2]. It is generally acknowledged that PC1 UE for band n14 is not targeted for smartphone form factor. Then other bands were added for the support of PC1 in Rel-18 for FWA UE and vehicle mounted UE. In current specification, MPR requirements were defined for band n14 and other bands.

The MPR requirements for PC1 in band n14 was captured:

Table 6.2.2-5 Maximum power reduction (MPR) for power class 1 for Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | ≤ 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

The MPR requirements for PC1 in bands other than n14 was captured:

Table 6.2.2-4b: Maximum power reduction (MPR) for power class 1 for bands other than Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK w/ Rel-15 DMRS | (NOTE 1) | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | (NOTE 1) | 0 | 0 |
|  | QPSK | (NOTE 1) | ≤ 1 | 0 |
|  | 16 QAM | (NOTE 1) | ≤ 2 | ≤ 1 |
|  | 64 QAM | (NOTE 1) | ≤ 2.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 4.5 |  |
| CP-OFDM | QPSK | (NOTE 1) | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | (NOTE 1) | ≤ 3 | ≤ 2 |
|  | 64 QAM | (NOTE 1) | ≤ 3.5 |  |
|  | 256 QAM | (NOTE 1) | ≤ 6.5 |  |
| NOTE 1: MPR for all modulations for Edge RB allocation is defined as following for two distinguished channel bandwidths groups as: Within the <50MHz channel bandwidth group: MPR= CEIL(7.2 dB- 6 dB∙ CBW100 MHz, 0.5 dB) Within the ≥50MHz channel bandwidth group: MPR= CEIL5.35 dB+3.15 dB∙ CBW100 MHz, 0.5 dB where CEIL(x,0.5 dB) means rounding x upwards to the closest multiple of 0.5 dB. |  |  |  |  |

However, in NR, above MPR requirements were derived from one PC1 PA linearity, which are not appropriate for 4Tx 32dBm case. It needs new MPR evaluation for 4Tx 32dBm case.

Evaluating MPR for a 4Tx system in 3GPP standards is a multi-faceted process. The core principle is that MPR allows a UE to reduce its maximum transmit power under specific conditions to maintain signal quality and comply with regulatory limits. For 4Tx, the assessment must account for the signal waveform (e.g., higher PAR with CP-OFDM versus DFT-s-OFDM), the modulation order (e.g., higher MPR for 256QAM than QPSK), the number and location of allocated Resource Blocks (RBs), and the critical impact of multiple PAs transmitting simultaneously. The aggregate signal from these parallel transmissions can lead to a higher PAPR, often necessitating a greater MPR than in single-PA scenarios.

Observation 1: Currently, in TS 38.101-1, the MPR requirements are based on one PC1 PA linearity, which are not suitable for 4Tx 32dBm case.

Proposal 2: It is suggested to re-evaluate MPR for PC1 with 4Tx for FWA UE.

For 4Tx UL MIMO &TxD, RAN4 had extensive discussion on PC1.5 with 4*23 dBm in Rel-18. It would be a lot easier for specifying requirements for PC1 with 4*26dBm with the old experience in Rel-18. The core RF requirements and methodologies established for PC1 with 4*23dBm provide a validated and adaptable foundation for the more challenging PC1 for FWA UE. The total workload for this part would not be heavy. For maximum output power for UL MIMO & TxD, it is defined as sum of each Tx, which depends on the discussion of reusing the PC1 definition. For MPR evaluations, it needs further evaluation based on 4*26 PA configurations for FWA UE. For other requirements or descriptions, we think minimal change is needed.

Observation 2. Leveraging the substantial experience gained from specifying 4Tx MIMO&TxD for PC1.5 in Rel-18, the standardization effort required to define 4Tx MIMO&TxD for PC1 in Rel-20 is expected to be limited.

PC1.5 with 2Tx for FDD bands for handheld and FWA UE

Another aspect of HPUE for NR single carrier operation is to specify UE RF requirements for PC1.5 with 2Tx for FDD bands for handheld and FWA UE. The related objectives were captured as follows:

- PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25

RAN4 already specified PC2 for FDD band in Rel-18. Besides adding higher power class for FDD band, other RF requirements can refer to TDD HPUE requirements. For above PC1.5 with 2Tx for FDD bands for handheld and FWA UE, we didn’t expect many technical issues for supporting this HPUE for FDD bands. Requirements for PC1.5 with 2Tx for TDD bands can be used as a starting point for PC1.5 with 2Tx for FDD bands for handheld and FWA UE.

Proposal 3: Requirements for PC1.5 with 2Tx for TDD bands can be used as a starting point for PC1.5 with 2Tx for FDD bands for handheld and FWA UE.

Conclusion

This contribution provides our initial views on HPUE requirements for NR single carrier operation. The following observations and proposals are made:

Proposal 1: It is suggested to reuse PC1 with or without general power boosting scheme to achieve up to 32 dBm in Rel-20.

Observation 1: Currently, in TS 38.101-1, the MPR requirements are based on one PC1 PA linearity, which are not suitable for 4Tx 32dBm case.

Proposal 2: It is suggested to re-evaluate MPR for PC1 with 4Tx for FWA UE.

Observation 2. Leveraging the substantial experience gained from specifying 4Tx MIMO&TxD for PC1.5 in Rel-18, the standardization effort required to define 4Tx MIMO&TxD for PC1 in Rel-20 is expected to be limited.

Proposal 3: Requirements for PC1.5 with 2Tx for TDD bands can be used as a starting point for PC1.5 with 2Tx for FDD bands for handheld and FWA UE.

- References

[1] RP-252952, New WID: UE RF enhancements for NR FR1, Phase 5, RAN#109.

[2] RP-210190, PC1 and PC3 Updates for Band n14, AT&T, RAN#91.