# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513568

# Prague, CZ, 13th – 17th Octobor, 2025

Source: 	vivo

Title: 	Initial views on enhanced requirements for NR NTN and IoT NTN Phase 2

Agenda Item:	7.6.4.1

Document for:	Approval

Introduction

In RAN#109, a new WID was approved on enhanced requirements for NR NTN and IoT NTN phase 2 [1]. One objective is to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies. Phase 1 work is Ku band with FR1 numerology and Phase 2 work is Ku band and Ka band with FR2 numerology. Phase 1 will start in this RAN4 meeting and Phase 2 will start upon completion of Phase 1 and RAN1/2 availability. In this contribution, we provide our initial views on Ku band with FR1 numerology.

Discussion

The objective for HD-FDD for NTN core part is captured as follows:

HD-FDD for NTN Core Part - Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies - Phase 1: Ku band with FR1 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 2: Ku band and Ka band with FR2 numerology - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type - Phase 1 work will commence immediately. Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability - Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

The market shows big interest in HD-FDD for NTN. A key advantage of HD-FDD (Half-Duplex Frequency Division Duplex) is its significant potential for cost reduction. Crucially, this technology completely avoids the need for the complex and expensive duplexers and high-performance RF filters required in traditional FDD systems. This design simplification substantially lowers the Bill of Materials (BOM) cost, design complexity, and manufacturing challenges associated with the terminal equipment. This cost advantage directly enhances the terminal's economic competitiveness, driving its market expansion and penetration potential.

In the objective, there is a note saying ‘Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.’ In Rel-19 NTN Phase 3, two major aspects for HD-FDD were maximum output power and REFSENS requirements.

For maximum output power, there was power boosting scheme supported. Originally, power boosting was proposed by company due to the insertion loss reduction without using duplexer or filer for HD-FDD. Later, this power boosting was developed as a general scheme for improving the power as long as UE has this capability. For Rx requirements, HD-FDD REFSENS was defined.

However, for Ku band with FR1 numerology, radiated RF requirements were introduced. All the solution for RedCap NTN HD-FDD solution are conducted, which can not be leveraged directly.

Proposal 1: Discuss the minimum peak EIRP improvement for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna considering:

- The insertion loss reduction due to avoidance of duplexer

For UE Rx RF requirements for (e)RedCap NTN in Half-Duplex mode, the only focus is on REFSENS requirements. There are two considerations for HD-FDD REFSENS:

- Compared to FD-FDD, RF front end may adopt filters with switch for HD-FDD, instead of using duplexer; the insertion loss for HD-FDD may be smaller than FD-FDD due to the difference in RFFE.

- For HD-FDD, receiving DL will not have the interference from UL transmission;

Based on these two considerations, the REFSENS may need tightening for HD-FDD, compared with FD-FDD. However, the tightening values need to be studies case by case.

Proposal 2: Discuss EIS tightening for HD-FDD compared to FD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna considering:

- Compared to FD-FDD, RF front end may adopt filters with switch for HD-FDD, instead of using duplexer; the insertion loss for HD-FDD may be smaller than FD-FDD due to the difference in RFFE.

- For HD-FDD, receiving DL will not have the interference from UL transmission;

The automotive antenna market, specifically applications like Mobile VSAT terminals utilizing small antennas, presents a compelling use case for HD-FDD. These vehicular satellite communication systems demand small form factors, low cost, and minimal power consumption. HD-FDD's elimination of bulky, costly duplexing components makes it highly attractive. It enables the development of compact, more affordable solutions tailored to meet the core requirements of this demanding segment.

The reduction to a 20×20 cm aperture imposes severe RF performance trade-offs:

Firstly, large gain reduction for this small size VSAT type is expected at Ku-band compared to traditional VSAT terminals. Normally, VSAT UE antenna is equipped with 60 cm equivalent aperture diameter. Due to the gain is proportional to A/λ² (A is equivalent aperture area), the gain reduction by 8.5 dB is estimated compared to the normal VSAT UE. Beam width will expand 2.8 times, degrading spatial discrimination and increasing vulnerability to adjacent-satellite interference.

Secondly, Receiver Sensitivity Degradation is expected from dual effects: gain loss in reception and elevated system noise temperature due to wider beam capturing more ground/space noise and thermal coupling of LNAs to radiators.

Thirdly, EIRP Collapse & Transmitter Challenges are expected. To match traditional terminals’ EIRP, Tx power must surge 7 times within extreme volumetric and thermal constraints. ACLR/SEM demands GaN PAs with aggressive DPD, yet power backoff for linearity sacrifices efficiency.

In a summary, achieving viable 20×20 cm VSATs demands is quite challenging: HD-FDD operation, monolithic phased arrays, and intelligent RF compensation become foundational to overcome inherent 8.5 dB performance deficits.

Observation：Due to antenna size reduced to 20*20 cm, the expected performance deficit is expected for antenna gain, which will have impact on EIRP and EIS requirements for Mobile VSAT type with 20*20 cm.

Currently Mobile VSAT 5 with electronic steering antenna was defined. Compared to this new mobile VSAT type with 20*20 cm, the difference is the antenna size. We can take the requirements for VSAT type 5 as a starting point for this newly introduced mobile VSAT and further discuss the requirements impact due to the reduced antenna size and HD-FDD mode.

Proposal 3. It is suggested to take the radiated RF requirements for VSAT type 5 as a starting point for this newly introduced mobile VSAT and further discuss the requirements impact due to the reduced antenna size and HD-FDD operation mode.

Conclusion

This contribution provides our initial views on requirements for HD-FDD for Ku band with FR1 numerology. The following observations and proposals are made:

Proposal 1: Discuss the minimum peak EIRP improvement for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna considering:

- The insertion loss reduction due to avoidance of duplexer

Proposal 2: Discuss EIS tightening for HD-FDD compared to FD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna considering:

- Compared to FD-FDD, RF front end may adopt filters with switch for HD-FDD, instead of using duplexer; the insertion loss for HD-FDD may be smaller than FD-FDD due to the difference in RFFE.

- For HD-FDD, receiving DL will not have the interference from UL transmission;

Observation：Due to antenna size reduced to 20*20 cm, the expected performance deficit is expected for antenna gain, which will have impact on EIRP and EIS requirements for Mobile VSAT type with 20*20 cm.

Proposal 3. It is suggested to take the radiated RF requirements for VSAT type 5 as a starting point for this newly introduced mobile VSAT and further discuss the requirements impact due to the reduced antenna size and HD-FDD operation mode.

- References

[1] RP-252962, New WID: Enhanced requirements for NR NTN and IoT NTN Phase 2, moderator (Qualcomm).