3GPP TSG-RAN WG4 Meeting#116bis 	R4-2513573

Prague, Czech, Oct. 13-17, 2025

Source: 	vivo

Title: 	Discussions on details of XR metric

Agenda Item:	4.13.1.1

Document for:	Approval

- Introduction

In RAN4#116 meeting, it was agreed that NHPRP/NHPRS can be added into TR [1]. In this contribution, we provide our views on the details of NHPRP/NHPRS.

- Discussion

  - NHPRP/NHPRS for XR

Last RAN4 meeting, RAN4 agreed NHPRP and NHPRS as optional metric for XR. However, the details angle and equations for these two metrics are not fully concluded. There are two options for the Near-horizon region discussed last meeting:

- elevation range ±30°

- elevation range ±45°

Issue 1-2-2: If TRP/TRS is kept as baseline, whether a new additional XR performance metric can be considered (as optional)?

Agreements:

  - For XR, keep current TRP/TRS metric as baseline and add NHPRP/NHPRS into TR. RAN4 is not intended to preclude NHPRP/NHPRS as long as it is technically justified.

  - FFS future requirements, single metric will be used for future XR OTA requirements.

In CCSA and CTIA, both NHPRP±30 and NHPRP±45 is captured in the test spec, which is informative performance metric for consideration. For XR performance in 3GPP, as we already agreed in RAN4 that TRP/TRS is the baseline metric and this near-horizon partial sphere metric is optional, so it would be good to harmonize with CCSA and CTIA and capture both two elevation angles into TR.

Another reason is that, due to limited number of commercially available XR devices in the market, it is still questionable which elevation angle is the proper range to quantify XR near-horizontal performance and well presenting the end-users’ experience.

Overall, we suggest capturing both NHPRP±30 and NHPRP±45 into TR as informative metric for XR. For future work of XR requirements, down-selection would be required if near-horizontal metric is finally selected as the single metric.

Proposal 1: Capture both ±30° and ±45° for NHPRP/NHPRS into TR as informative metric for XR. The applicability should also be clearly stated.

- Conclusion

We have the following proposal for XR OTA:

Proposal 1: Capture both ±30° and ±45° for NHPRP/NHPRS into TR as informative metric for XR. The applicability should also be clearly stated.

A CR will be prepared accordingly based on agreements.

- References

- R4-2512693 Way Forward for [116][337] TRP_TRS_MIMO_OTA_Ph3, vivo, RAN4#116.